3GPP TSG-RAN Meeting #131-bis	R2-2506931

# Prague, Czech, 13th  – 17th October,  2025

Agenda item:	8.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues for RLC

Document for:	Discussion and Decision

- Introduction

The following agreements related to timely RLC retransmissions were reached in RAN2#131 [1] meeting as given below, we will discuss the remaining issues in RLC specification in this document.

- [RLC-10] There is no need to introduce limitations or associations for the configuration of UL stopReTxObsoleteSDU and DL t-RxDiscard. This can be left to network implementation. - [RLC-11] When all RLC SDUs with SNs up to POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter may stop and reset the running t-PollRetransmit. Capture this as a note. - [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change). - (RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility. It can be left to UE implementation whether to re-send the gap report after HO. - (RLC-14) The RLC entity does not discard the RLC SDU or RLC SDU segment as in legacy, although the RLC SDU or RLC SDU segment indicated by the discard indication from upper layers has been submitted to the lower layers, i.e., no RLC specification change required. - (PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement. - (UE capability-04) UE supporting txRLC-StopReTxDiscardedSDU-r19 need not indicate support for rxRLC-Discard-r19. - Try to clarify in RLC specs that “RLC SDU segment(s) for transmission or retransmission” covers RLC SDU segment(s) which have been already considered or pending for transmission/retransmission. - Can try to clarify together with the previous agreement. - It is up to UE implementation to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value. No spec change.

- Discussion

2.1 stop unnecessary retransmission

As agreed that the Tx RLC entity can stop unnecessary transmission for the outdated RLC SDU or RLC SDU segment(s), the RLC specification has been modified as below [2]:

When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall: - send an indication to the upper layers of successful delivery of the RLC SDU; - set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet. If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. NOTE Y: The corresponding RLC SDU or RLC SDU segment(s) above includes RLC SDU or RLC SDU segment(s) which have been already considered or pending for transmission or retransmission.

The newly added paragraph describes the behaviours of the Tx RLC entity when stopReTxDiscardedSDU is configured, e.g. shall not consider the corresponding RLC SDU or RLC SDU segement(s) for transmission or retransmission.

However, it should be noted that according to Caluse 5.4 of TS 38.322 [3], the Tx RLC entity shall discard the RLC SDU according to the assocaited discard indication if the RLC SDU has not been intially transmitted. If the RLC SDU already been discarded, we don’t need to worry about its transmission or retansmission.

5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Observation 1: As per legacy specification, the untransmuted RLC SDU will be discarded when its associated discard indication is received, regardless of whether stopReTxDiscardedSDU is configured.

Therefore, we believe that there is no need to explicitly mention that the Tx RLC entity shall stop retransmitting the outdated untransmitted RLC SDU when stopReTxDiscardedSDU is configured.

Proposal 1: In the RLC running CR, the description on stopping retransmitting the untransmuted RLC SDU when stopReTxDiscardedSDU is configured should be removed from the main text and Note.

Finnaly, a modified TP for running RLC CR is provided as given below:

If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission. orThe transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for retransmission if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. NOTE Y: The corresponding RLC SDU or RLC SDU segment(s) above includes RLC SDU or RLC SDU segment(s) which have been already considered or pending for transmission or retransmission.

Proposal 2: RAN2 to adopt the proposed TP.

# 3. Conclusion

This contribution makes the following proposals:

Observation 1: As per legacy specification, the untransmuted RLC SDU will be discarded when its associated discard indication is received, regardless of whether stopReTxDiscardedSDU is configured.

Proposal 1: In the RLC running CR, the description on stopping retransmitting the untransmuted RLC SDU when stopReTxDiscardedSDU is configured should be removed from the main text and Note.

Proposal 2: RAN2 to adopt the proposed TP.

# 4. Reference

[1] Chair note of RAN2#131,

https://www.3gpp.org/ftp/Meetings_3GPP_SYNC/RAN2/Inbox/Chair_Notes/R2_131%20R19%20XR%20and%20LTE%20Broadcast%20notes%20(Dawid)_EOM.docx

[2] R2-2506338

[3] TS 38.322