3GPP TSG-RAN WG2 Meeting #131bis                                                              R2-2506943

Prague, Czech Republic Oct 13th – 17th, 2025

Source:	CATT

Title:	Discussion on Topology-2 for Ambient IoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

This contribution gives an initial discussion on the D2T2-B for device 1, with the objective of the WID [1].

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

Section 2 gives our detailed analysis, including

- Inventory

- Command after inventory

- Handover/RLF of UE reader

Section 3 summarizes the contribution of this paper.

2. Discussion

## 2.1 Inventory

As mentioned in the WID, only RRC-based solution for D2T2-B is considered in R20. A candidate inventory procedure for RRC-based architecture in D2T2-B was captured in TR 38.769 [2] based on the SI discussion. In this chapter, we analyse the inventory procedure according to the candidate procedure captured in TR 38.769.

Figure 1: Message flow for A-IoT inventory in Topology 2 RRC-based solution

### 2.1.1 Pre-condition on UE reader

Before performing the inventory procedure, the UE reader should register to the CN so that the CN or gNB can perform the reader selection for a certain task. The CN cannot predict when the A-IoT service will come and thus the gNB may release these UE readers to RRC_IDLE/RRC_INACTIVE. However, based on the WID, we only focus on the RRC_CONNECTED UE. Note that this can rely on NW implementation to not release the UE readers after registering to the CN. So RAN2 can assume the RRC_CONNECTED UE readers are always available when the CN is going to perform the A-IoT service.

Proposal 1: RAN2 starts our work based on the assumption that the RRC_CONNECTED UE readers are always available when the CN is going to perform the A-IoT service.

### 2.1.2 A-IoT radio resource allocation

Dedicated A-IoT radio resource

Upon receiving the inventory request from the CN (Step 1a), the gNB allocates/coordinates the A-IoT resources (Step 1b) for the UE readers to perform the transaction on the A-IoT radio interface. During the SI discussion, the radio resource allocation was captured in TR 38.769:

6.3.6 Topology 2 aspects on the interface between UE reader and RAN The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported.

The intention of the above description is the gNB should allocate the isolated resources for different UE readers to avoid the potential interference due to overlapped coverage by the UE readers. So we prefer to keep the SI conclusion as a start point for further discussion.

Proposal 2a: RAN2 confirms the shared resource pool amongst UE readers is not supported in R20.

How to trigger resource allocation

A subsequent issue is how to trigger the gNB to allocate A-IoT radio resources. In TR 38.769, there are two alternatives for A-IoT radio resources allocation:

6.3.6 Topology 2 aspects on the interface between UE reader and RAN For the radio resources allocation request, following alternatives are studied: - Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

For Alt 1, actually it is introduced during the SI discussion to support the UP-based architecture for topology-2 (captured in SA2 TR 23.700).

Figure 2: Protocol stack for the user-plane architecture for Topology 2

According to the above protocol stack, the gNB is not aware of the A-IoT service. So it’s up to UE reader to request the A-IoT radio resource by Alt 1. However, in the RRC-based solution, the gNB can be aware of the A-IoT service by the inventory request of step 1a in Figure 1, so it’s straightforward to go with Alt 2 for the RRC-based solution.

Proposal 2b: RAN2 agrees to adopt Alternative 2 for triggering the gNB to allocate A-IoT resources to UE readers, i.e., BS configures/allocates the A-IoT radio resource to the UE reader, based on the service request from CN.

Validity of the A-IoT radio resource

For the validity of the A-IoT radio resource, it was discussed during the SI phase:

- Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

Since the A-IoT radio resource is allocated to UE reader by dedicated signalling, it can be up to gNB implementation to release the corresponding resource. So for the case without consideration of UE reader cell switch (i.e., handover, RRC re-establishement), the radio resources remain valid until the NW releases them explicitly. For the timer-based solution, it’s hard to determine a suitable timer value. Therefore, we suggest Option 1.

Proposal 2c: For the case without consideration of UE reader cell switch, the A-IoT radio resource remains valid until the gNB releases them explicitly.

### 2.1.3 RRC inventory request/response interaction with UE reader

In the Step 1a, the gNB send the RRC inventory request message to trigger the inventory procedure at the A-IoT radio interface. We further discuss the potential content of the RRC inventory request message.

Recalling R19, the NGAP INVENTORY REQUEST message towards the gNB includes the following information:

INVENTORY REQUEST IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. Inventory Request Transfer; M; OCTET STRING; Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1; YES; reject. 9.3.x.1 Inventory Request Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. A-IoT Device Identification Requested; M; 9.3.3.xx5; YES; reject. Requested Service Area Information; M; 9.3.3.xx3; YES; reject. Inventory Assistance Information; M; 9.3.3.xx4; YES; reject. Follow on Command Indication; O; ENUMERATED (true, ...); This IE indicates that there is command(s) to be transmitted.; YES; reject.

AIOTF Identifier/A-IoT Correlation Identifier

Firstly, UE reader needs to generate the transaction ID based on the A-IoT Correlation Identifier. Secondly, the A-IoT Correlation Identifier jointly with the AIOTF Identifier uniquely identifies an A-IoT service when there are parallel services towards a UE reader (Note that this is not parallel service over the A-IoT radio interface, because it’s up to reader implementation to execute the service one by one). So when performing inventory reporting to the gNB, UE reader needs to use this AIOTF Identifier/A-IoT Correlation Identifier to identify to which service the report corresponds.

A-IoT Device Identification Requested

It is straightforward to be aware by the UE reader, because the UE reader needs to page the targeted devices.

Requested Service Area Information

In R19, the requested service area information is used by the gNB to select the readers co-located within the gNB, because one gNB maybe have multiple readers. When considering R20, this information is useless to UE reader, because whether to perform the UE reader functionality is under the Gnb/CN control.

Inventory assistance information

In R19, the CN may provide the assistance information to the reader, for example, the approximate number of target A-IoT devices that may be involved in the inventory procedure for the efficient RACH resource allocation by the reader, or the expected D2R message size to support not segmented D2R transmission of the device ID. Such assistance information should also be applied for UE reader.

Follow-on command indication

For the command after inventory, in R19, the NGAP inventory request message further indicates to the reader that there is a follow-on command. Based on this indication, in topology-1, the reader will allocate the NGAP ID when reporting each device ID to the CN and map the NGAP ID with the AS ID. Upon receiving the follow-on command for that device, the reader can know using which AS ID to send this command to the specific device. The similar functionality should also be applied to UE reader in topology-2.

Content related to A-IoT radio resource

Besides above information, on the provision of the A-IoT radio resource configuration to UE reader, there are two candidate options via the dedicated signalling.

Option 1: Using the legacy RRC message, e.g., RRCReconfiguration before the start of the A-IoT service

In this option, upon UE reader finishes the register to the CN, CN may indicate to the gNB to allocate the A-IoT radio resource for the UE reader. By this way, the gNB needs to reserve these resources even if there is no A-IoT service for a long time. So this is not resource-efficient.

Option 2: Introducing a new RRC message, e.g., RRC inventory request message

This alternative involves including A-IoT radio resource configuration in the RRC Inventory Request message upon triggering the UE reader to perform the inventory procedure. Option 2 resolves the resource inefficiency issue present in Option 1.

Based on the above analysis, we prefer Option 2, i.e., A-IoT radio resource configuration is included in the RRC inventory request message.

In summary, we propose that,

Proposal 3: Introduce a new RRC message (i.e., RRC inventory request message) to trigger the UE reader to perform the inventory procedure at the A-IoT radio interface. The RRC inventory request message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- A-IoT Device Identification Requested information;

- Inventory assistance information;

- Follow-on command indication;

- A-IoT radio resource configuration.

Regarding Step 2c, the intention is to follow the R19 design in topology-1 that the gNB may reject the A-IoT service request from CN due to the potential gNB payload problem. However, for topology-2, it’s up to gNB whether to provide the A-IoT radio resources to the UE reader based on the gNB payload. If it is provided, the UE reader should follow the gNB control to perform the A-IoT service, i.e., there is no need to introduce the inventory response mechanism for the UE reader to reject the A-IoT service.

Proposal 4: The UE reader should perform the A-IoT service under the gNB control, if the gNB provides the A-IoT radio resource to the UE reader. No need to introduce the inventory response mechanism from UE reader side.

### 2.1.4 Inventory procedure at A-IoT radio interface

Since the topology should be transparent to the A-IoT device, the inventory procedure at the A-IoT radio interface in the Step 3 should follow the same one as R19.

Proposal 5: For inventory procedure at A-IoT radio interface for topology-2, the same procedure of D1T1-B should be applied.

### 2.1.5 RRC inventory report

Recalling the NGAP INVENTORY REPORT message in R19,

9.2.x.4 INVENTORY REPORT IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. Inventory Report Transfer; M; OCTET STRING; Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4; YES; reject. 9.3.x.4 Inventory Report Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; 9.3.3.xx1. Global gNB ID; M; 9.3.1.6. Reader Report List; 1. >Reader Report Item; 1..< maxnoofReaders>. >>Reader Index; M; 9.3.1.aa1. >>Device Report List; 1. >>>Device Report Item; 1..<maxnoofDevices>. >>>>A-IoT NAS PDU; M; OCTET STRING. >>>>RAN A-IoT Device NGAP ID; O; 9.3.3.yy1. Inventory Complete Indication; O; ENUMERATED (true, ...).

AIOTF Identifier/A-IoT Correlation Identifier

The AIOTF Identifier and A-IoT Correlation Identifier need to be included in the RRC inventory report to identify to which service the report corresponds, due to the parallel services executed by the reader (Note that this is not parallel service over the A-IoT radio interface, because it’s up to reader implementation to execute the service one by one).

Global gNB ID/Reader index

The purpose of introducing Global gNB ID/Reader index is to enable the device positioning at the granularity of reader level. This is meaningless for UE reader case, because the UE reader may move anywhere and thus cause the device to be not covered.

A-IoT NAS PDU (Device ID info)

The A-IoT NAS PDU needs to be included in the RRC inventory report, which is the response to the inventory service.

RAN A-IoT Device NGAP-like ID

As analysed in the section 2.1.3, if the follow-on command is included in the RRC inventory request message, similar to topology-1, the UE reader should allocate a UE A-IoT Device Uu ID when reporting each device ID to the gNB. The UE A-IoT Device Uu ID is used to map the AS ID so that upon arrival of the follow-on command for that device, the UE reader can know using which AS ID to address the corresponding device.

Inventory Complete Indication

For topology-2, similar functionality should be supported, i.e., the inventory complete indication needs to be included in the RRC inventory report so that the gNB can forward this information to the A-IoTF. Then the A-IoTF can send the report content to the AF.

Based on above summary, it is proposed that,

Proposal 6a: Introduce a new RRC message (i.e., RRC inventory report message) to report the inventory response from the UE reader at the A-IoT radio interface. The RRC inventory report message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- Per device report item, with each item including,

- A-IoT NAS PDU (Device ID info);

- UE A-IoT Device Uu ID;

- Inventory Complete Indication.

Proposal 6b: If UE reader receives the follow-on command indication in the RRC inventory request message, it allocates the UE A-IoT Device Uu ID associated with the device and maintains the device context at UE side by mapping the UE A-IoT Device Uu ID with the AS ID.

In addition, in R19, the NGAP INVENTORY REPORT message only contains the report content for one A-IoT service. If the gNB has multiple reports associated with different services, it’s up to gNB to trigger multiple NGAP INVENTORY REPORT procedures. For UE reader, it seems we can rely on multi-entry in one RRC inventory report message with each entry including the information suggested by our Proposal 6.

Proposal 7: Support the RRC inventory report message including the response for multiple A-IoT services.

## 2.2 Command after Inventory

### 2.2.1 A-IoT radio resource allocation

A candidate command procedure for RRC-based architecture in D2T2-B was captured in TR 38.769 [2] based on the SI discussion,

Figure 3: Message flow for A-IoT command in Topology 2 RRC-based solution

Upon receiving the command request towards the gNB from CN, the current procedure triggers the gNB to allocate the A-IoT radio resources for the command service. It is still open during SI discussion whether this step is needed,

1. The Inventory procedures are performed in step 1, which are the same as in Figure 6.5.3.1.1-1 in step 1-4. NOTE 1: Step 1 is performed for the "A-IoT Inventory and Command" case, Whether and under which conditions step 1 may be skipped in case of "Command-only" and which consequences this would have for the overall message flow depends on discussions led by SA2, SA3 and RAN2. NOTE 2: Whether and which way steps 2a, 2b and 2c may be part of step 1 is dependent on discussions led by SA2, SA3 and RAN2. 2a. The A-IoT CN sends a Command Request message to the A-IoT enabled gNB. 2b. The A-IoT enabled gNB coordinates A-IoT radio resources and allocates A-IoT radio resources to the A-IoT enabled UE accordingly. 2c. The A-IoT enabled gNB sends a RRC Command Request message to the A-IoT enabled UE.

From our view, the command will trigger the inventory procedure first (command-only is not supported in R19), thus there is no need to trigger the gNB to perform the resource allocation because this had been done during the inventory procedure, i.e., once UE reader is allocated with A-IoT radio resource, it will use this resource to finish the whole A-IoT related service over the A-IoT radio interface until the resource is released by the gNB.

Proposal 8: The command request from CN does not trigger the gNB to perform the A-IoT radio resource allocation. Once the A-IoT radio resource is assigned to UE reader by the inventory procedure, it will be used to finish the whole A-IoT related service over the A-IoT radio interface until the resource is released by the gNB.

### 2.2.2 RRC command request

Regarding what information that needs to be forwarded to UE reader by the RRC command request message, we can also take R19 as baseline,

9.2.x.5 COMMAND REQUEST IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. Command Request Transfer; M; OCTET STRING; Containing the Command Request Transfer IE specified in subclause 9.3.x.5.; YES; reject. 9.3.x.5 Command Request Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. A-IoT NAS PDU; M; OCTET STRING; YES; reject. Command Assistance Information; O; 9.3.3.yy3; YES; reject.

AIOTF Identifier/A-IoT Correlation Identifier

The AIOTF Identifier and A-IoT Correlation Identifier need to be aware by UE reader for which service the follow-on command is, due to the potential parallel services towards the UE reader.

RAN A-IoT Device NGAP-like ID, i.e., UE A-IoT Device Uu ID

As described in section 2.1, UE A-IoT Device Uu ID needs to be aware by UE reader for which device the command is applied.

A-IoT NAS PDU

This is the upper layer command (i.e., read, write or disable) for UE reader to be sent to the corresponding device addressed by the associated AS ID.

Command Assistance Information

This command assistance information includes the expected D2R message size to assistance the efficient scheduling by the reader. So it’s better to be aware by the UE reader.

In summary, we propose that,

Proposal 9: Introduce a new RRC message (i.e., RRC command request message) to trigger the UE reader to perform the command procedure at the A-IoT radio interface. The RRC command request message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer command);

- Command assistance information.

### 2.2.3 Command procedure at A-IoT radio interface

Similar as our Proposal 5, the command procedure at the A-IoT radio interface in the Step 3 should follow the same one as R19, due to the transparent topology to the device.

Proposal 10: For command procedure at A-IoT radio interface for topology-2, the same procedure of D1T1-B should be applied.

### 2.2.4 RRC command response

Regarding the RRC command response, according to the NGAP COMMAND RESPONSE message,

9.2.x.6 COMMAND RESPONSE IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; 9.3.3.xx2; YES; reject. A-IoT Correlation Identifier; M; 9.3.3.xx1; YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. Command Response Transfer; M; OCTET STRING; Containing the Command Response Transfer IE specified in subclause 9.3.x.6.; YES; reject. Criticality Diagnostics; O; 9.3.1.3; YES; ignore. 9.3.x.6 Command Response Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; 9.3.3.xx1. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1. A-IoT NAS PDU; M; OCTET STRING. Criticality Diagnostics; O; 9.3.1.3.

AIOTF Identifier/A-IoT Correlation Identifier

Similar to the RRC inventory report message, this information is also needed to be included in the RRC command response to identify to which service the response corresponds, due to the parallel services executed by the reader.

RAN A-IoT Device NGAP-like ID, i.e., UE A-IoT Device Uu ID

UE A-IoT Device Uu ID needs to be included in the RRC command response to identify which device is responding the command.

A-IoT NAS PDU (upper layer response message)

This is the upper layer response to be included in the RRC command response message.

In summary, we propose that,

Proposal 11: Introduce a new RRC message (i.e., RRC command response message) to report the command response from the UE reader at the A-IoT radio interface. The RRC command response message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer response message);

## 2.3 Handover/RLF of UE reader

### 2.3.1 UE reader handover

For the RRC_CONNECTED UE reader, it may be handovered to a new cell due to mobility. The WID had been updated to consider the HO scenario. From our view, there are following basical issues to solve.

Whether the UE continues serving as reader in the target gNB (cell)

To my understanding, this is under the target gNB (cell) control. At least there is an extreme case that the target gNB may be a non A-IoT functionality supported node. For other potential cases, it may depend on the criterion of reader selection, which is under RAN3/SA2 scope.

Observation 1: Whether to continue serving as UE reader in the target cell depends on the criterion of reader selection, which is under RAN3/SA2 scope.

However, from the UE reader perspective, it can follow the NW indication. According to the WID, the A-IoT radio resource is only valid in the current serving cell. That means if the target cell wants to continue this UE as a reader, it should provide the new A-IoT radio resource in the target cell within the HO command. So this A-IoT radio resource configuration can be implicitly used to indicate whether UE to continue the reader functionality in the target cell.

Proposal 12a: The A-IoT radio resource configuration should be able to be provided in the HO command.

Proposal 12b: Upon receiving the HO command, the UE continues to serve as a reader in the target cell if the HO command provides the A-IoT radio resource configuration.

Whether to continue reporting the content from the previous serving cell

Upon receiving the HO command, the UE reader may be performing the inventory or command procedure over the A-IoT radio interface. That is to say, there may be some pending inventory report or command response to be sent to the gNB side.

Generally, there are three candidate ways to handle these pending data (i.e., RRC inventory report or RRC command response), if the UE is still allowed to serve as reader in the target cell upon receiving the HO command:

Option 1: Cancel the data collected in the source cell, release all the device-specific context and wait for further inventory or comand request from the target gNB.

Option 2: Continue reporting the data collected in the source cell. After that, release all the device-specific context and wait for further inventory or comand request from the target gNB.

Option 3: Based on NW configuration to go with Option 1 or Option 2.

Option 1 is the simplest way. For Option 2, the target gNB needs to be aware of the device specific context, so that it can establish the RAN A-IoT Device NGAP ID for the device in the target gNB and map it to the corresponding UE A-IoT Device Uu ID for the pending data reception. Note that this involves RAN3 scope on the potential Xn interaction during HO.

Proposal 13: RAN2 discusses how to handle the pending data collected in the source cell (i.e., RRC inventory report or RRC command response) at the UE reader side, if the UE is allowed to continue serving as reader in the target cell after HO procedure.

### 2.3.2 Cell switch due to HO failure or RLF

The cell change can also occur during HO failure (for example, T304 expires) or RLF, which leads UE reader to initiate RRC re-establishment procedure to a new cell. So the similar issues as UE reader handover scenario are also found.

Upon finishing RRC re-establishment, the NW will send the first RRC Reconfiguration to provide the configuration for the re-established cell. So similar with HO case, whether the UE continues serving as a reader in the re-established cell can be implicitly indicated by the provision of A-IoT radio resource configuration in the first RRC Reconfuguration followed by the RRC re-establishment procedure.

Proposal 14: Upon finishing RRC re-establishment, the UE continues serving as a reader in the re-established cell if the first RRC Reconfuguration followed by the RRC re-establishment procedure provides the A-IoT radio resource configuration.

Then upon the initiation of RRC re-establishment, it needs to further discuss how to handle the A-IoT radio resource in the source cell. Whether to perform the UE reader at the re-established cell is controlled by the target gNB (cell) by provision of the A-IoT radio resource configuration in the first RRC Reconfuguration followed by the RRC re-establishment procedure. So UE reader should release the A-IoT radio resource in the source cell upon initiation of RRC re-establishment.

Proposal 15: UE reader releases the A-IoT radio resource in the source cell upon initiation of RRC re-establishment.

Regarding the pending data, upon initiation of RRC re-establishment, UE cannot predict whether the re-established cell will allow it to continue serving as a reader, so we suggest UE reader to cancel the pending data collected in the source cell and release the device specific context at the UE side, i.e., AD ID and UE A-IoT Device Uu ID.

Proposal 16: Upon initiation of RRC re-establishment, the UE reader cancels the pending data collected in the source cell and releases the device specific context at the UE side.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Observation 1: Whether to continue serving as UE reader in the target cell depends on the criterion of reader selection, which is under RAN3/SA2 scope.

Inventory

Proposal 1: RAN2 starts our work based on the assumption that the RRC_CONNECTED UE readers are always available when the CN is going to perform the A-IoT service.

Proposal 2a: RAN2 confirms the shared resource pool amongst UE readers is not supported in R20.

Proposal 2b: RAN2 agrees to adopt Alternative 2 for triggering the gNB to allocate A-IoT resources to UE readers, i.e., BS configures/allocates the A-IoT radio resource to the UE reader, based on the service request from CN.

Proposal 2c: For the case without consideration of UE reader cell switch, the A-IoT radio resource remains valid until the gNB releases them explicitly.

Proposal 3: Introduce a new RRC message (i.e., RRC inventory request message) to trigger the UE reader to perform the inventory procedure at the A-IoT radio interface. The RRC inventory request message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- A-IoT Device Identification Requested information;

- Inventory assistance information;

- Follow-on command indication;

- A-IoT radio resource configuration.

Proposal 4: The UE reader should perform the A-IoT service under the gNB control, if the gNB provides the A-IoT radio resource to the UE reader. No need to introduce the inventory response mechanism from UE reader side.

Proposal 5: For inventory procedure at A-IoT radio interface for topology-2, the same procedure of D1T1-B should be applied.

Proposal 6a: Introduce a new RRC message (i.e., RRC inventory report message) to report the inventory response from the UE reader at the A-IoT radio interface. The RRC inventory report message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- Per device report item, with each item including,

- A-IoT NAS PDU (Device ID info);

- UE A-IoT Device Uu ID;

- Inventory Complete Indication.

Proposal 6b: If UE reader receives the follow-on command indication in the RRC inventory request message, it allocates the UE A-IoT Device Uu ID associated with the device and maintains the device context at UE side by mapping the UE A-IoT Device Uu ID with the AS ID.

Proposal 7: Support the RRC inventory report message including the response for multiple A-IoT services.

Command after inventory

Proposal 8: The command request from CN does not trigger the gNB to perform the A-IoT radio resource allocation. Once the A-IoT radio resource is assigned to UE reader by the inventory procedure, it will be used to finish the whole A-IoT related service over the A-IoT radio interface until the resource is released by the gNB.

Proposal 9: Introduce a new RRC message (i.e., RRC command request message) to trigger the UE reader to perform the command procedure at the A-IoT radio interface. The RRC command request message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer command);

- Command assistance information.

Proposal 10: For command procedure at A-IoT radio interface for topology-2, the same procedure of D1T1-B should be applied.

Proposal 11: Introduce a new RRC message (i.e., RRC command response message) to report the command response from the UE reader at the A-IoT radio interface. The RRC command response message at least can include the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer response message);

Handover/RLF of UE reader

Proposal 12a: The A-IoT radio resource configuration should be able to be provided in the HO command.

Proposal 12b: Upon receiving the HO command, the UE continues to serve as a reader in the target cell if the HO command provides the A-IoT radio resource configuration.

Proposal 13: RAN2 discusses how to handle the pending data collected in the source cell (i.e., RRC inventory report or RRC command response) at the UE reader side, if the UE is allowed to continue serving as reader in the target cell after HO procedure.

Proposal 14: Upon finishing RRC re-establishment, the UE continues serving as a reader in the re-established cell if the first RRC Reconfuguration followed by the RRC re-establishment procedure provides the A-IoT radio resource configuration.

Proposal 15: UE reader releases the A-IoT radio resource in the source cell upon initiation of RRC re-establishment.

Proposal 16: Upon initiation of RRC re-establishment, the UE reader cancels the pending data collected in the source cell and releases the device specific context at the UE side.

4. References

- RP-252894 Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2

- 3GPP TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19)”, V19.0.0 (2024-12)

- RP-252894  Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2