3GPP TSG-RAN WG2 Meeting #131bis	R2-2506953

Prague, Czech Republic, Oct. 13th - 17th

Agenda Item:	8.4.2

Source: 	NEC

Title:  	Discussion on LP-WUS RRC remaining issue

Document for:	Discussion and Decision

- 1 Introduction

From RAN2 point of view, the Rel-19 work item NR_LPWUS-Core is completed. In this contribution we further discuss RRC remaining issue related to different type of LR device.

- 2 Discussion

- 2.1 Remaining issue related to different type of LR device

During past meeting discussion, the agreements related to entry/exit condition of LP-WUS monitoring considering different type of LR, i.e., OOK-based and OFDM-based are made as following:

RAN2#127

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

RAN2#130

It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types.

RAN2#131

RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

In running CR TS 38.331, for RRC_IDLE/INACTVE LP-WUS, the lowPowerConfig-r19 is provided in DownlinkConfigCommonSIB, and entry/exit condition of LP-WUS monitoring is provided within lowPowerConfig-r19 to determine whether the UE supporting LP-WUS can apply LP-WUS feature or not during RRC_IDLE/INACTIVE. Note that RAN2 has agreed that “The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration”.

For the entry condition of LP-WUS monitoring, we have different type of threshold for evaluation considering different device (i.e., MR or LR) and different RS (i.e., LP-SS or SSB).

entryEvaluationOnMR-ForLR-OnLPSS:

Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine entry condition based on MR

entryEvaluationOnMR-ForLR-OnSSB:

Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine entry condition based on MR

entryEvaluationOnLR-ForLR-OnLPSS:

LR evaluation is optional. Indicates the threshold(s) for a UE supporting OOK based LP-WUR or OFDM based LP-WUR measuring on LP-SS to determine entry condition based on LR
entryEvaluationOnLR-ForLR-OnSSB:

LR evaluation is optional. Indicates the threshold(s) for a UE supporting OFDM based LP-WUR measuring on SSB to determine entry condition LR.

We had agreement that “It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types.” However we also need to consider what if the UE only supports OOK LP-SS based measurement?

According to the current spec, it is up to NW implementation to determine which threshold will be provided considering the cell supporting situation (Cond SupportLR-OnLPSS / SupportLR-OnSSB). If NW only provide SSB-based threshold (e.g., entryEvaluationOnMR-ForLR-OnSSB and entryEvaluationOnLR-ForLR-OnSSB), the spec lacks of specifying the UE behaviour when the UE only supports LP-SS based measurement. Therefore, the spec should clarify that if the NW only provide SSB-based threshold for LP-WUS monitoring, the UEs only supporting OOK LP-SS measurement is not allowed to use LP-WUS.

Proposal-1: RAN2 should clarify in the spec that, if NW only provide SSB based threshold for LP-WUS monitoring (i.e., entryEvaluationOnMR-ForLR-OnSSB / entryEvaluationOnLR-ForLR-OnSSB), the UEs only supporting OOK LP-SS measurement is not allowed to use LP-WUS.

Furthermore, we believe the UE either supporting OFDM-based LR or OOK-based LR can at least measure on LP-SS, therefore, there is no issue for NW to only provide LP-SS based threshold of LP-WUS monitoring. No UE behaviour should be clarified for this case.

- 3 Conclusions

Proposal-1: RAN2 should clarify in the spec that, if NW only provide SSB based threshold for LP-WUS monitoring (i.e., entryEvaluationOnMR-ForLR-OnSSB / entryEvaluationOnLR-ForLR-OnSSB), the UEs only supporting OOK LP-SS measurement is not allowed to use LP-WUS.

- 4 References