3GPP TSG-RAN WG2#131-bis	R2-2506964

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	8.7.3

Title:	On the definition of non-delay-reporting PDCP SDU

Document for:	Discussion and decision

Introduction

During the post RAN2#131 meeting email discussion on list of open issues for Rel-19 XR PDCP CR, an issue was raised by Huawei in H001 that it is good to align with the definition with the procedure text for non-delay-reporting PDCP SDU [1]. The proposal by the PDCP CR rapporteur is as follows:

Proposal 2 (H001). Discuss whether to change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive.

In this contribution, we would like to provide our view on the definition of non-delay-reporting PDCP SDU and provide our TP to TS38.323 [2].

Discussion

The issue on definition of non-delay-reporting PDCP SDU has been brought up several times in previous email discussions.

The issue with the current definition is that the non-delay-reporting PDCP SDUs associated with different reporting thresholds are not exclusive. For example, a non-delay-reporting PDCP SDU associated with i-th reporting threshold is also a non-delay-reporting PDCP SDU associated with (i+x)-th reporting threshold. It relies on the procedural texts to remove the overlapping parts, which is not a good way for definition. Thus, the definition and the procedural texts for data volume calulation for DSR are not aligned.

Observation 1: In current PDCP CR, the non-delay-reporting PDCP SDUs associated with different reporting thresholds are not exclusive. A non-delay-reporting PDCP SDU associated with i-th reporting threshold is also a non-delay-reporting PDCP SDU associated with (i+x)-th reporting threshold.

Observation 2: The definition of non-delay-reporting PDCP SDU is not aligned with the procedural texts for data volume calculation for DSR.

Another issue with non-exclusive definition of non-delay-reporting PDCP SDU is on the DSR data indication to lower layers for the i:th DSR-ReportingThreshold. Currently the notification is for each reporting threshold (see below box). If current definition is used, it will cause confusion on which threshold a non-delay-reporting PDCP SDU is indicated since a non-delay-reporting PDCP SDU may be associated with multiple reporting thresholds, as mentioned above.

The transmitting PDCP entity provides a DSR data indication associated with the i:th DSR-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU (if dsr-ReportNonDelayCriticalData is configured) associated with the i:th DSR-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU (if dsr-ReportNonDelayCriticalData is configured) associated with the i:th DSR-ReportingThreshold.

Observation 3: The non-exclusive definition of non-delay-reporting PDCP SDU leads to ambiguity on the DSR data indication associated with the i:th DSR-ReportingThreshold to lower layers.

We agree with the intention of H001. However, we proposed the following change on top of proponent’s TP in the email discussion to avoid self-reference in their proposed TP and also make the non-delay-reporting PDCP SDUs associated with different thresholds exclusive.

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting or non-delay reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i.

Proposal 1: Change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive.

The procedural texts can be simplified by the change of definition of the non-delay-reporting PDCP SDU.

The final TPs on the definition of non-delay-reporting PDCP SDU and the procedure of data volume calculation for delay status reporting are proposed in the Annex.

Proposal 2: Adopt the TPs in Annex on the definition of non-reporting PDCP SDU and the procedure of data volume calculation for delay status reporting.

Conclusion

In this paper we discuss the issues on the definition of non-delay-reporting PDCP SDU and have come up with the following observations and proposals:

Observation 1: In current PDCP CR, the non-delay-reporting PDCP SDUs associated with different reporting thresholds are not exclusive. A non-delay-reporting PDCP SDU associated with i-th reporting threshold is also a non-delay-reporting PDCP SDU associated with (i+x)-th reporting threshold.

Observation 2: The definition of non-delay-reporting PDCP SDU is not aligned with the procedural texts for data volume calculation for DSR.

Observation 3: The non-exclusive definition of non-delay-reporting PDCP SDU leads to ambiguity on the DSR data indication associated with the i:th DSR-ReportingThreshold to lower layers.

Proposal 1: Change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive.

Proposal 2: Adopt the TPs in Annex on the definition of non-reporting PDCP SDU and the procedure of data volume calculation for delay status reporting.

References

- R2-2507130, List of open issues for Rel-19 XR PDCP CR, 3GPP RAN2#131bis

- R2-2506337, Introduction of R19 XR enhancements for PDCP spec, 3GPP RAN2#131

Annex (TP to TS38.323)

Start of the Change

- 3.1	Definitions

>>> unchanged part skipped>>>

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i.

Next Change

- 5.15	Data volume calculation for delay status reporting

>>> unchanged part skipped>>>

For the purpose of MAC multiple entry delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of DSR-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th DSR-ReportingThreshold:

-	the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th DSR-ReportingThreshold where k < i;

-	the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th DSR-ReportingThreshold where k < i;

-	if dsr-ReportNonDelayCriticalData is configured:

-	the non-delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and have not been submitted to lower layers;

-	if i = 1, the PDCP Control PDUs;

-	if i = 1, for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	if i = 1, for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

End of Change