3GPP TSG RAN WG2 Meeting #131-bis	R2-2506974

Prague, Czech Republic 13-17 Oct 2025

Agenda Item:	10.3.3	 Common User plane and Control plane

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Support for Location Dependent Data Collection

Document for:	Discussion

# Introduction

Crowdsourcing information from multiple UEs enables location-dependent network monitoring and training of AI/ML models with strong generalization capabilities. This requires consistently labeled, high-quality, and well-structured data to ensure usability for improving service quality and network performance.

In Release 19, 3GPP introduced AI/ML-based UE location determination but relied on legacy methods for ground-truth labeling, which exhibit degraded accuracy in non-line-of-sight (NLOS) and multipath environments. Similarly, NTN access depends on GNSS-derived UE location - a vulnerability to local jamming or spoofing that crowdsourced UE data could detect via cross-UE consistency analysis.

A standardized data collection mechanism would enhance cross-vendor interoperability and model generalization, thereby directly improving service quality and user experience.

This contribution demonstrates how expanding standardized data collection for location dependent data collection could benefit the 6G systems

# Use of independent (non-legacy) methods for ground truth labels for location services AI/ML use case

For AI/ML methods, the inference result depends on the quality of training data. For positioning and sensing applications, part of the training label may consist of location, movement and orientation information.  For the AI/ML based positioning introduced in Rel. 19, the training data is based on:

UEs configured as PRUs – which are generally deployed stationary. UEs whose location can be calculated with certain accuracy.

Use of PRU type UEs limit the data collected to static UEs and spatially limited to where the PRUs are provisioned. In addition, this is not scalable to build a large dataset.

- Use of PRU type UEs limit the data collected to static UEs and spatially limited to where the PRUs are provisioned.

For the non-PRU type UEs, the labels are based on legacy positioning methods, the methods that were known to have accuracy and reliability issues in scenarios dominated with non-line of sight and multipath conditions.

As a result, labels with good quality are collected at locations where legacy methods already perform well. For the challenging conditions, it becomes increasingly difficult to obtain labels. For example, GNSS is not available indoors. Indoor positioning (e.g. TDOA, Multi-RTT, UL-TDOA) do not perform well in NLOS and multipath conditions.

If we note that the only labels the network has available are the ones that the NW computes using legacy approaches (e.g. UL-TDOA, multi-RTT) or from UE-reported values, network does not have good quality labels to train the models.

- Good quality labels with current approach are collected at the location where the legacy methods already perform well, and fail to give good labels under challenging conditions.

On the other hand, camera-based methods have good ability to self-localise themselves, and provide good quality labels. They can establish their range to an object whose location and characteristics are known, i.e. a landmark. It can also be observed that the state-of-the-art in location services utilises information from outside sources, such as maps of buildings. Now, if a UE is given assistance information to search for certain information in the vicinity, and localise itself with this information, this approach could be a good way forward in collecting ground truth labels.

Likewise, information collected at application layer, that are location dependent could also serve as ground truth labels (e.g. RFIDs, beacons etc).

It is clear that image processing for self-localisation is computationally expensive procedure and may be limiting for UE to perform such procedures for localisation. However, for data collection in challenging areas for crowdsourcing training data, so that the models are trained with good quality labels, this would be an enabler.

For 6G location services (positioning, sensing etc), it would be meaningful to associate the ground truth information available from application layer (e.g. with camera) with the radio measurements from lower layers, for the purpose of training, validation and monitoring of AI/ML models.

Proposal 1:   Location services based on AI/ML shall utilise association of radio signal measurement with ground truth obtained from alternative sources (like UE apps), such as camera-based self-localisation, utilising assistance from the network about the features in image to search and localize.

# UE-aided detection of threat to GNSS systems and assistance data signaling

NTN operation relies on GNSS-generated UE location to determine satellite location, as well as to determine parameters for uplink transmission, such as timing advance. It is well-known that the GNSS signals can be easily jammed or spoofed.

The interference may be inter-system interference due to co-existing system operating on the same radio frequency or it may be a result of a malicious activity (such as moving jammer). A common example is the one where a vehicle driver installs a portable jammer to prevent the system from tracking his location. An example of inter-system interference is the interference caused to GNSS devices on E5/E6 bands from DME or TACAN systems.

Spoofing misleads a GNSS receiver to a wrong position or time. Generally, spoofing attacks are targeted at a few receivers at most. However, the spoofed signal in air could affect other receivers in the vicinity of the spoofed receiver.

Ideally, both the RAN-nodes and the UE nodes could monitor the impairing effects of  jamming, spoofing and interference. If only the RAN-nodes were to monitor the jamming and spoofing events, then this could give rise to the well-known hidden node problem, where the effects on the UE are local and cannot be observed by the NG-RAN node. This situation arises particularly for large cell-sizes, where the UE and the NG-RAN node may be experiencing different local environments scenario. Alternatively, the NG-RAN node could detect integrity issues in its vicinity but this may not be relevant for the area where the UE is currently located. This leads to an exposed node problem.

Clearly, interference/jamming and spoofing effects relate to the environment the UE finds itself in. Therefore, it is not possible to process and interpolate observations from a network of reference stations (like in GPS to determine correction parameters) alone in order to assess such errors. Hence, there is a need for crowd sourced UE observations of the measurement errors. If the UEs in a region can be selected and configured to provide observations relevant for the region, then this information can be provided to the network, which can provide crowd-sourced information to other UEs when entering the region.

- The network can crowd-source information about regional errors such as interference, jamming and spoofing using configured measurements and observations from capable UEs.

- Based on crowd-sourced data, the network can provide regionalized integrity information to UEs entering a specific region to inform about local GNSS impairments.

Furthermore, it is a well-known observation [1] [2], that the current GNSS receivers are impacted mainly by jammers or spoofers carried on-board on vehicles in the highway. A possible scenario is shown in Figure 1, where a moving jammer or a spoofer is impacting the signal reception by two UEs within a certain area.

Figure 1: Depiction of a moving jammer or a spoofer along a highway together with victim UEs.

This means the affected area is dynamically changing, and the situation needs to be monitored by more than one entity in the network. The entities may be RAN-nodes or UEs having the corresponding capability.

- Moving jammers and spoofers cause the affected area to change dynamically.

The network may provide information to the UE if it is known that certain area is suffering from impairments, so that the UE does not attempt to initiate connection using spoofed GNSS measurements for NTN access.

Proposal 2: Crowdsourced data collection of GNSS impairment in an area shall be enabled to assist other UEs attempting to connect to the NTN network using GNSS as location source for determining NTN parameters.

# Conclusions

In this contribution, we propose the following potential enhancement to the data collection framework for 6G systems

Proposal 1:   Location services based on AI/ML shall utilise association of radio signal measurement with ground truth obtained from alternative sources (like UE apps), such as camera-based self-localisation, utilising assistance from the network about the features in image to search and localize.

Proposal 2: Crowdsourced data collection of GNSS impairment in an area shall be enabled to assist other UEs attempting to connect to the NTN network using GNSS as location source for determining NTN parameters.

# References

| [1] | A. Hashemi, S. Thombre, N. Giorgia Ferrara, M. Zahidul, H. Bhuiyan and M. Pattinson, "STRIKE3-Case Study for Standardized Testing of Timing-Grade GNSS Receivers Against Real-World Interference Threats.," in International Conference on Localization and GNSS (ICL-GNSS), Nuernberg,Germany, 2019. |
| --- | --- |
| [2] | R. Mitch, R. Dougherty, M. Psiaki, S. Powell, B. O’Hanlon, J. Bhatti and T. Humphreys, "Signal characteristics of civil GPS jammers," in Proceedings of the 24th International Technical Meeting of the Satellite Division of The Institute of Navigation, Portland, OR, 2011. |
| [3] | TS 22.870, "Study on 6G Use Cases and Service Requirements," 3GPP, 2025-06. |