3GPP TSG-RAN WG2 Meeting #131bis		R2-2507017

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	List of RLC open issues for R19 XR

Agenda Item:	8.7.1

Document for:	Discussion and Decision

- Introduction

Per instruction from the chairlady, this document is to collect RLC open issues identified for Rel-19 XR. More specifically, please provide here open issues that were identified but not addressed during the running CR review phase or we expect to come from other WGs.

Please provide your input no later than Friday September 19 18:00 UTC. After the deadline, I will provide a draft CR for companies to review before submission. In addition, I will also identify issues that are difficult to resolve and should be discussed by contributions, if any, as suggested by the chairlady.

- Contact information

Please provide your contact information in the table below.

| Company | Name | Email |
| --- | --- | --- |
| Vivo | Chenli | Chenli5g@vivo.com |
| Xiaomi | Yanhua Li | Liyanhua1@xiaomi.com |
| Ericsson | Nithin Srinivasan | nithin.srinivasan@ericsson.com |
| Huawei, HiSilicon | Yinghao Guo | yinghaoguo@huawei.com |
| Nokia | Nishant Raina | Nishant.raina@nokia.com |
| Samsung | Vinay Shrivastava | shrivastava@samsung.com |

- Open issues

  - DSR enhancements

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Xiaomi | A open issue we put on the post email discussion: In last meeting, we have agreed: - [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC If all the RLC SDUs that RLC needs to calculated are indicated from PDCP, then RLC can indeed calculate the data based on PDCP’s indication. Then RLC calculation of the none delay reporting data can be removed. However, is that possible that RLC put some packets which are not know in PDCP before the data of one portion or is there possible that before DSR is triggered, there has already some data before one portion? Then in this way, since PDCP is not aware of data in RLC’s buffer, then I guess the part that RLC calculation of the none delay reporting data still can not be removed. And Rapp’s response is: “ My understanding is all SDU in RLC comes from PDCP, so PDCP is aware of all packets status (remaining time). RLC just needs to determine which one/segments should be calculated in the DSR, e.g. based on whether delivery to lower layer/retx. It seems no problem for this. We could think more if any issue, and further discuss it during maintenance phase. ” In last F2F meeting offline in India, after checking with LG, Sharp, we are not sure on this. We would like to hear more opinions from other companies. [Rapp] The issue is related to the implementation on RLC handling, e.g. whether PDCP and RLC have the same packet placement order, and whether PDCP and RLC share single IP packet copy in physical memory but maintaining respective protocol headers at UE side. The reasonable implementation way should be: SDU handling in RLC layer follows the same order as in PDCP layer. But companies are welcome to provide views on this issue. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-X01 |

  - Avoid unnecessary RLC retransmissions

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-V01 |
| Ericsson | The current implementation for the t-RxDiscard configured and not running is if RX_Next_Highest > RX_Next + 1 or if RX_Next_Highest = RX_Next + 1. However, the second condition might be too aggressive and could result in a situation where the RLF detection procedure is affected. For example, if the initial state is SN = 0 with segmentation and the RLC Rx entity receives byte segments out of order. Then, RX_Next = 0, RX_Next_Highest = 1, RX_Next_Discard_Trigger = 1 If the last segment of SN = 0 is received but others are not by the time t-RxDiscard expires, then the RLC Rx window advances with Rx_Next = 1 and sends an ACK for SN = 1. This behaviour could keep happening with the Tx entity assuming things are working well but the Rx entity seeing virtually no data. [Rapp]: If t-RxDiscard expires and the SDU with SN = 0 in your case has not been fully received by the receiver, it is considered outdated and no longer needs to be transmitted. This is exactly how the “avoid unnecessary retransmission” mechanism is intended to work, so we do not see any issue here. But the proponent is welcome to rais proposal on this. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Proposal from Proponent is welcome. Issue Number: RLC-E01 |
| Huawei, HiSilicon | The following paragraph has been added for avoiding unecessary retransmissions. If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. NOTE Y: The corresponding RLC SDU or RLC SDU segment(s) above includes RLC SDU or RLC SDU segment(s) which have been already considered or pending for transmission or retransmission. However, according to Caluse 5.4 of TS 38.322 [3], the Tx RLC entity shall discard the RLC SDU according to the assocaited discard indication if the RLC SDU has not been intially transmitted. If the RLC SDU already been discarded, we don’t need to worry about its transmission or retansmission. 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.. Also, we think that The behaviour of stopping retransmitting the RLC SDU or RLC SDU segment(s) that is pending for retransmission should be added in the main text. If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission. orThe transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for retransmission, or shall cancel the pending status if the corresponding RLC SDU or RLC segment(s) is pending for retransmission, if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. NOTE Y: The corresponding RLC SDU or RLC SDU segment(s) above includes RLC SDU or RLC SDU segment(s) which have been already considered or pending for transmission or retransmission.. [Rapp]: It is true there is no case on “not consider the corresponding RLC SDU for transmission”. The legacy wording, “not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission”, was primarily for simplicity in the specification. Regarding cancelling pending retransmissions, I assume there is no different to capture it in the note or normative text. My initial thinking is to keep the current wording, as there is issue, and it was reviewed by companies several times. But I am open to hear other companies’ view. Let’s discuss it during the CR phase. | Issue Type: Not essential not important How to address it: Discuss the possible wording update during the CR phase. Companies are welcome to provide views by then. Issue Number: RLC-H01 |
| Huawei, HiSilicon | In section 5.3.4, the content when STATUS PDU is triggered by other triggering conditions are reused. However, when t-RxDiscard expires, RX_Next is updated to RX_Next_Discard_Trigger and after the update, RX_Next might be larger than RX_Highest_Status. Hence, the following condition does not hold anymore. - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: Hence, when the status report is triggered by t-RxDiscard, a straightforward solution would be just to set the ACK_SN to the updated RX_Next. Then, the SDUs before RX_Next are indicated as ACK The TP can be follows: When constructing a STATUS PDU not triggered by the expiry of t-RxDiscard, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: =====omitted=== When constructing a STATUS PDU triggered by the expity of t-RxDiscard, the AM RLC entity shall: - set the ACK_SN as the updated RX_Next [Rapp]: As you mentioned, when RX_Next is larger than RX_Highest_Status, the condition of “RX_Next <= SN < RX_Highest_Status” is not satisfied. Therefore, the current specification means only ACK_SN will be included in the STATUS PDU, i.e. the current specification works well and we think no change is needed. | Issue Type: Not essential not important How to address it: No issue from Rapporteur point of view. Please let me know if any spec update is needed during the CR phase. Issue Number: RLC-H02 |
| Nokia | [N001] RX_Next in clause 7.1 The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x. Our suggestion is: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires. [Rapp]: It is true, and we need to update it. Your poropsal is one way, which may miss some information for legacy case. Another way is to capture like: if t-RxDiscard is configured, xxx. Otherwise, xxxx. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-N01 |
| Nokia | [N002] Below NOTE in clause 5.3.3.3 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU. Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard. Our suggestion is: NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. [Rapp]: This issue was discussed in RAN2#130 based on Apple’s proposal. The agreement includes the ACK/NACK/Discard cases. I understand the motivation in Apple’s contribution is that in the case where a NACK has just been received, there is no need for polling since the status is already known, and the retransmission can be performed directly. While the current Note in the specification also states “may stop and reset,” so I think this should not be a major issue. But considering it has not been extensively discussed during RAN2#130, let’s discuss it. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-N02 |

  - Ensure timely RLC retransmissions

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | In RAN2#131 meeting, it was agreed to clarify in the RLC spec that “the pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU”. However, this has not been captured due to less consensus. We could continue to discuss how to capture it in the RLC spec to ensure the packets which have already been positively acked will not be retransmitted. | Issue Type: Not essential but important How to address it: Offline discussion during meeting. Issue Number: RLC-V02 |

  - Others, please specify

Companies are invited to describe any other identified open issues not currently included within this document.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Samsung | Impact of Rel-19 RLC AM enhancements on PDCP SN gap report triggering: [PDCP-2] Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers. The above issue was discussed in RAN2#131, however, discussion was not conclusive. While almost all companies acknowledged the issue arising with the Rel-19 RLC AM enhancements, one company remarked that the issue would also be relevant for the scenario of multiple RLC entities configured with the PDCP. Particularly this should be applied “also for completely submitted SDUs” and condition for “successful delivery of discarded PDCP SDU(s) have not been confirmed” be checked. However, we think this is not appropriate as the submitted SDUs could be subject to on-going HARQ transmissions and no clear status about the successful delivery can be judged at the time of Discard SN gap report triggering. Considering to keep the solution simple and less impact to specification, we think the new condition can specifically restricted and be addressed as below: If stopReTxDiscardedSDU is configured in all RLC entities: - at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. It is worthwhile to carefully consider the issue further as there is no clear conclusion so far, and not pursuing it also does not really resolve the problem. [Rapp]: My understanding is this issue was discussed in RAN2#130 meeting, and the following agreement was achieved: - (PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement. But the proponent is welcome to rais proposal on this. | Issue Type: Not essential not important How to address it: This issue was discussed and concluded during RAN2#130. Proposal from Proponent is welcome. Issue Number: RLC-S01 |

- Conclusion

In this contribution, we collect the open issues for XR enhancements in RLC as below:

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02.

Proposal 2: Proponent companies are invited to provide proposals on the following issues, which Rapporteur understands there is no problem: RLC-E01, RLC-H02, RLC-S01.

Proposal 3: Following open issues are suggested to be discussed during the CR phase: RLC-H01, RLC-V02.

- References

- R2-2506338, Introduction of R19 XR enhancements for RLC spec., vivo.