3GPP TSG-RAN WG2 Meeting #131-bis                                                                     R2-2507034

Prague, Czech Republic, Oct. 13th – 17th

Agenda Item:	10.3.1

Source:	HONOR

Title:	Discussion on User plane for 6GR

Document for:	Discussion and Decision

- Introduction

6GR study item has been approved in the RAN plenary #108 meeting and revised in RAN plenary #109 meeting [1], from which, the objective of radio interface protocol architecture and procedures for 6GR are,

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

and the objective of initial access for 6GR is,

(2) Physical Layer structure for 6GR, …… h) Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

In this contribution, we will systematically review the functionalities of each layer in user plane in 5G NR, and analyze their necessity and inheritability for various potential 6G scenarios. In the second part, we will discuss the RA procedure with the consideration of 6G scenarios, UE types/features and energy saving.

- Discussion

  - Essential user plane functions

In the 5G NR, the several UE types (e.g. Redcap/eRedcap) and different services (EMBB/URLLC/MMTC/XR/IAB/sidelink) are supported. However, the UE types and some services are considered in the later release. In the 6G Day 1, the diverse UE types and use cases will be supported, which have different bandwidth or different delay requirements. For example, the device types contain one or multiple eMBB (or 6G IoT) device types and other device types, e.g. XR/immersive experiences, FWA, vehicle UEs (VUEs), wearables/RedCap, sensing (as standalone or complementary), NTN (standalone or complementary), AI agents, collaborative robots, etc. Therefore, different protocol layers with different functions should be designed to make their configuration more flexible and adaptable during deployment. This session gives an overview functionality of each protocol layer in the current 5G system and analyses motivation whether they are needed or not for the 6GR.

- SDAP

The main services and functions of SDAP include:

-	Mapping between a QoS flow and a data radio bearer

On the core network side, data packets are classified into different QoS flows based on their service requirements, e.g. latency, reliability, and priority. On the radio access network side, data is ultimately transmitted over the air interface using Data Radio Bearers.

For downlink/uplink data, the SDAP layer is responsible for mapping the QFI of data packets to an appropriate DRB.

6G systems will support more services with different QoS requirements, such as AI agents and XR. Therefore, the SDAP functionality needs to be retained. As new service types and core network requirements are considered, the SDAP layer may need to be enhanced, but its QoS flow mapping can remain the same in 6G systems.

-	Marking QoS flow ID (QFI) in both DL and UL packets

For downlink/uplink data, the SDAP layer in the gNB/UE adds an SDAP header to the packet header, allowing the UE/gNB to identify the QoS flow the packet belongs to and forward it correctly to the appropriate upper-layer processing entity or core network.

This is a crucial operation for the receiver to identify the QoS flow, and therefore can be inherited in 6GR.

Observation 1: The mapping between QoS flow and DRB of SDAP layer can be inherited in 6GR.

- PDCP

The main services and functions of PDCP include:

-	Transfer of data (user plane or control plane);

The PDCP layer provides its services to the RRC or SDAP layers, and performs the following operations on the data:

-	Maintenance of PDCP SNs;

-	Duplicate discarding.

In the PDCP layer, each SDU associates with a Sequence Number (SN). This serves as the foundation for reordering, duplicate discarding, ciphering, and integrity protection. Duplicate reception may occur due to handover, duplication or HARQ. Identifying and discarding duplicate SDUs based on the PDCP SN is an important function to ensure data accuracy and prevent resource waste. Therefore, the maintenance of PDCP SNs and duplicate discarding are fundamental features that support SDU transmission and should still be supported in 6GR.

-	Header compression and decompression using the ROHC protocol;

-	Header compression and decompression using EHC protocol;

-	Compression and decompression of uplink PDCP SDUs: DEFLATE based UDC only;

The 5G compression algorithms (e.g., ROHC, EHC and UDC) serve as enhanced features for specific scenarios or services. For example, the ROHC is crucial for small-packet traffic (such as VoIP) or bandwidth-constrained environments. And UDC is an enhanced feature that compresses application-layer data payload, improving energy efficiency and spectral efficiency.

Considering that 6G will still support services such as small-packet traffic and vertical industries, etc., compression and decompression should also be supported in 6G and deployed as needed.

-	Ciphering and deciphering; Integrity protection and integrity verification;

Ciphering and integrity protection serve for security. In 5G NR, both are mandatory for control plane data, but optional for user plane data. Some services such as app traffic (e.g., eMBMS) may choose to operate without ciphering and integrity protection. Similarly, in 6G, there will still be traffics that do not require these security features. Therefore, ciphering and integrity protection should continue to be provided as optional functions.

-	Timer based SDU discard;

In order to clear the buffer in time and meet the overall transmission delay, the PDCP support the timer based SDU discard. If the SDU discarded in the PDCP layer, the PDCP will indicate discard indication to the RLC layer. In the later release, it is introduced PDU set in the XR. And the PDCP support PDU-set-based discard and PSI-based discard. If 6G Day 1 support the PDU set transmission, the corresponding enhancements can be considered.  Meanwhile, in the AI data transmission, the discard timer may be not necessary, the 6G can consider whether the timer based SDU discard is a mandatory function for the PDCP.

-	Duplication;

To improve reliability, the PDCP can also duplicate SDUs and transmit the copies over two different logical channels. It is designed as enhancements for 5G URLLC services. Similarly, for 6GR, this function can be configured for specialized services with high reliability requirements.

-	For split bearers and DAPS bearer, routing;

For bearers in Dual Connectivity (DC) and DAPS, the UE is allowed to simultaneously establish connections and transmit data with two different cells. If UE need to simultaneously transmit data to non-single cell in 6G, then these features can be considered.

-	Reordering and in-order delivery; Out-of-order delivery;

Which delivery is required depends on the type of service. Such as TCP-based traffic, in-order delivery is essential. The PDCP receiver uses sequence numbers (SNs) to reorder the SDUs and deliver them in the correct sequence to the upper layers. However, for services that can tolerate out-of-order delivery (e.g., UDP-based streaming applications), in-order processing is not strictly necessary.

The 6G should support various services, such as services that require high reliability but are not sensitive to latency (e.g., file downloads, emails), or services that are extremely sensitive to latency but can tolerate a small amount of packet loss (e.g., voice and video). Therefore, the in-order delivery (with reordering) and out-of-order delivery shall be supported for 6GR.

Observation 2: In 6GR, the maintenance of SNs and duplicate discarding are fundamental functions, and other functions of PDCP layer can be deployed as needed.

- RLC

The RLC sublayer supports three transmission modes:

-	Transparent Mode (TM);

-	Unacknowledged Mode (UM);

-	Acknowledged Mode (AM).

For SRB0, paging and broadcast system information, TM mode is used. For other SRBs AM mode used. For DRBs, either UM or AM mode are used.

The main services and functions of the RLC sublayer depend on the transmission mode and include:

-	Transfer of upper layer PDUs;

The most fundamental function of all RLC modes is to transmit upper layer PDUs.

A TM RLC entity can be configured to submit/receive RLC PDUs through the common channels, which carry fixed-size signaling intended for periodic broadcasting or repeated transmission to multiple UEs (e.g., SIBs and paging messages).

As a transparent mechanism, it operates data transmission without RLC headers, segmentation or retransmission, and meets the requirements of public signaling transmission. As a fundamental signaling transmission mode, the data processing method used in this TM mode should also be supported in 6G Day 1.

-	Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs;

-	Reassembly of SDU (AM and UM);

In addition to public signaling, RLC should also support the transmission of service data and dedicated signaling. These types of services generate variable data sizes, so the UM/AM RLC must have segmentation/reassembly capabilities to accommodate the variable transport block sizes from the lower layer. Therefore, to achieve the adaptation of 6G data to transmission resources, both RLC UM and RLC AM shall support segmentation/reassembly of SDU.

In AM RLC, an RLC SDU segment needs to be re-segmented, when a portion of the segment need to retransmit or the segment does not match the transport block size. Therefore, if 6GR supports retransmission, a re-segmentation function is essential.

-	Sequence numbering independent of the one in PDCP (UM and AM)

To enable the ARQ and the RX to correctly reassemble data packets, the RLC TX must associate sequence numbers with the RLC SDUs. Therefore, the sequence number is the fundamental feature that supports reassembly and retransmission.

-	RLC SDU discard (AM and UM);

To support the Timer based SDU discard at the PDCP layer, both UM and AM RLC are capable of RLC SDU discard. In R19, the "RLC SDU discard" in RLC AM mode was enhanced to prevent retransmission of outdated data packets. Previous analysis showed that for some services (such as AI data), the discarding is not necessary. Therefore, in 6G, a similar RLC discard function shall be optionally enabled based on the PDCP discard function, and further consideration can be given to merging and migrating these two functions.

-	Error Correction through ARQ (AM only);

The ARQ within the RLC sublayer has the following characteristics:

-	ARQ retransmits RLC SDUs or RLC SDU segments based on RLC status reports;

-	Polling for RLC status report is used when needed by RLC;

-	RLC receiver can also trigger RLC status report after detecting a missing RLC SDU or RLC SDU segment.

The ARQ mechanism enables the TX retransmit the RLC SDUs or RLC SDU segments when receiving a NACK in status report. Status reports can be triggered either by the TX using polling, or by the RX detecting SN gap.

In R19, the retransmission and polling based on remaining time were introduced for XR. However, the remaining time of a packet is calculated by the PDCP layer's discard timer, so these enhanced mechanisms introduce more inter-layer interactions, which increase the complexity of UE implementation.

Therefore, if the first version of 6G supports short-latency services similar to XR, and incorporates these enhanced features, then functional migration (related to the remaining time of an SDU) should be considered for the initial release.

-	Duplicate Detection (AM only);

Due to HARQ or handover, the RLC RX may receive multiple PDUs with the same sequence number. This feature detects and discards such duplicate PDUs, ensuring that the upper layer receives only one copy of the PDU. Since the PDCP layer supports duplicate discarding, the duplicate detection in RLC is not strictly necessary.

Relying on MAC HARQ retransmission is insufficient to meet the packet loss rate requirement of 10⁻⁵ for high-reliability services. Therefore, the AM mode is a necessary mechanism to ensure service reliability.

Observation 3: In summary, the services provided by RLC TM/UM/AM are fundamental services, forming the basis of 6GR data transmission.

- MAC

The main services and functions of the MAC sublayer include:

-	Mapping between logical channels and transport channels; Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

The above is a basic transmission architecture inherited from LTE to NR, so 6GR can directly inherit this function.

-	Scheduling information reporting;

Scheduling information report includes SR/BSR and PHR. The DSR procedure is introduced for XR service. In 6G system, latency-sensitive services may still exist. Dual reporting of data volume and latency information ensures unified consideration of latency-sensitive services, addressing the limitation of reporting only data volume. Therefore, the 6G Day 1 can consider reporting data volume, latency, and other related information.

-	Error correction through HARQ (one HARQ entity per cell in case of CA);

This is the basic function of the data transmission which shall be supported in 6GR.

-	Priority handling between UEs by means of dynamic scheduling; Priority handling between logical channels of one UE by means of logical channel prioritisation; Priority handling between overlapping resources of one UE;

The above priority handlings are effective resource utilization and allocation strategy, which can transmit data according to the overall network load and service priority. The LCP is enhanced for XR to support delay-critical data, however this enhancement is not critical. Therefore, the basic priority handling scheme can be reused in 6GR.

-	Padding.

This is the basic function of the data transmission which shall be supported in 6GR.

Observation 4: The basic data transmission functions of the MAC layer should be retained in 6GR.

As analysis in the above, the user plane functions can serve as baseline for the 6GR. However, 6G Day 1 may consider new services, such as AI agents and XR, and some features will require enhancements and considerations of whether to apply them to all services.

Proposal 1: The functions of user plane supported by 5G NR can be as baseline for 6GR protocol functions.

Proposal 2: The following functions of user plane can be inherited to 6GR:

- QoS flow mapping;

- Maintenance of SNs;

- Duplicate discarding/detecting;

- Compression and decompression;

- Ciphering and deciphering;

- Integrity protection and integrity verification;

- Timer-based SDU discard;

- Re-ordering, in-ordering and out-of-ordering delivery;

- Segmentation, re-segmentation and reassembly of SDU;

- ARQ;

- Mapping between logical channels and transport channels;

- Multiplexing/demultiplexing of MAC SDUs;

- Scheduling information reporting, e.g. SR/BSR/DSR/PHR;

- HARQ handling;

- Priority handling between UEs/logical channels/overlapping resources.

Proposal 3: 6GR study the possible protocol stack and mandatory/optional function set for each layer.

In the above analysis, some functions are enhanced considering PDU set transmission. In the current specification, the definition of PDU set is as following:

PDU Set: one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s) or video slice(s) etc. for XR services), as defined in TS 23.501. A PDU in the PDU Set corresponds to a PDCP SDU.

This definition can be extended in 6GR for future services, e.g. AI agent, such as one or more PDUs with associated relationship can be classified in a PDU set. Application can generate such PDU set by its requirements, and the AS layer can provide transmission strategies for such PDU set.

Proposal 4: For 6G Day 1, study how to support PDU set transmission.

  - RA procedure

The NR system supports 4-step RA procedure and 2-setp RA procedure. 2-step RA procedure is introduced in Rel-16, which is used to reduce the delay for RA when the UE is under good coverage. The MsgA contains preamble and PUSCH payload, it requires higher link quality and has restriction for cell edge users to use. Meanwhile, 2-step RA has a higher probability of collision and may fall back to 4-step RA due to PUSCH payload transmission failure. Therefore, 4-step RA cannot completely be replaced by 2-step RA, which should be a supplementary technology. Compared with 5G systems, 6G systems may provide richer experiences, such as extreme low-latency requirements and massive communications. Therefore, the access delay and small signaling overhead of 2-step RA may be more suitable for 6G systems. Considering some defects of 2-step RA, the 2-step RA of 6G system may not be a simple 2-step RA translation of 5G, but an enhancement and evolution to provide fast access.

4-setp RA procedure can support a larger coverage area, has a relatively mature conflict handling mechanism, a relatively simple process, and high reliability. But it may cause large signalling overhead and delay in the dense link scenarios.

Therefore, both the 4-step RA procedure and 2-step RA procedure can be as a baseline in the 6G Day 1 and can be studied for 6G scenarios.

Proposal 5: The 4-step/2-step RA procedure of 5G NR can be as baseline for 6GR, and study possible enhancements for 6G scenarios.

In 5G NR, the 4-step RA procedure was introduced in Release 15, followed by the 2-step RA in Release 16. From Release 17 onward, various enhancements to the RA procedure have been introduced, including support for RedCap, network slicing, MSG3 repetition, and MSG1 repetition.

The 6G systems will support an extensive range of device types, from low-power (e.g., LPWA and IoT) devices to high-performance terminals (e.g., CPE and premium smartphones). These devices exhibit significant disparities in capabilities, power constraints, and QoS requirements, which motivates the classification of devices into distinct types. This direction has been explicitly recognized in 6G Study Items.

In this context, it is essential that specific device capabilities and features can be efficiently identified and differentiated during the RA procedure. This will enable the network to dynamically assign appropriate radio resources and access parameters, thereby improving resource utilization, increasing access success rate, and enhancing the overall service experience. Therefore, the design of the RA procedure should incorporate flexible support for diverse device types and features to meet the complex and varied access requirements of 6G networks.

Proposal 6: For 6G Day 1, study the RA procedure to support diverse UE types and features.

In the RAN1#122 meeting, the following agreement was raised：

Agreement • Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s).

It is important to consider energy saving in Day 1 6G. The network monitors PRACH at each random access occasion and transmits the SSB/SI in a period, which cause a lot of energy consumption for the network. In the 5G NR, PRACH adaptation, on-demand SSB and on-demand SIB1/OSI were introduced to save network energy. For the PRACH adaptation, the network can configure legacy PRACH configuration with a long period and activate additional PRACH resource when RA conflict occurs. For the on-demand SSB, it is applied in CA scenario when the UE is in RRC connected state, and the UE does not need to request SSB. For the on-demand SIB1, it requires a cell A to provide the SIB1 configuration. Due to the limitations of 5G signal design and the existence of legacy UEs, 5G's network energy saving technology still has significant room for improvement.

However, regardless of which method for network energy saving, the energy consumption of UE will increase. For the 6G Day 1, the joint energy saving of UE and NW will be considered. For the PRACH adaptation, on-demand SSB and on-demand SIB1/OSI, each mechanism may be needing the RA procedure to support. Therefore, in the 6G Day 1, the RA procedure may support energy saving both for network and UE.

Proposal 7: For 6G Day 1, study the RA procedure to support joint energy saving for UE and network, e.g. RACH adaptation, on-demand SSB/SI, etc.

- Conclusions

In this contribution, we discussed user plane and made the following observations and proposals:

Observation 1: The mapping between QoS flow and DRB of SDAP layer can be inherited in 6GR.

Observation 2: In 6GR, the maintenance of SNs and duplicate discarding are fundamental functions, and other functions of PDCP layer can be deployed as needed.

Observation 3: In summary, the services provided by RLC TM/UM/AM are fundamental services, forming the basis of 6GR data transmission.

Observation 4: The basic data transmission functions of the MAC layer should be retained in 6GR.

Proposal 1: The functions of user plane supported by 5G NR can be as baseline for 6GR protocol functions.

Proposal 2: The following functions of user plane can be inherited to 6GR:

- QoS flow mapping;

- Maintenance of SNs;

- Duplicate discarding/detecting;

- Compression and decompression;

- Ciphering and deciphering;

- Integrity protection and integrity verification;

- Timer-based SDU discard;

- Re-ordering, in-ordering and out-of-ordering delivery;

- Segmentation, re-segmentation and reassembly of SDU;

- ARQ;

- Mapping between logical channels and transport channels;

- Multiplexing/demultiplexing of MAC SDUs;

- Scheduling information reporting, e.g. SR/BSR/DSR/PHR;

- HARQ handling;

- Priority handling between UEs/logical channels/overlapping resources.

Proposal 3: 6GR study the possible protocol stack and mandatory/optional function set for each layer.

Proposal 4: For 6G Day 1, study how to support PDU set transmission.

Proposal 5: The 4-step/2-step RA procedure of 5G NR can be as baseline for 6GR, and study possible enhancements for 6G scenarios.

Proposal 6: For 6G Day 1, study the RA procedure to support diverse UE types and features.

Proposal 7: For 6G Day 1, study the RA procedure to support joint energy saving for UE and network, e.g. RACH adaptation, on-demand SSB/SI, etc.

- References

- RP-252912