3GPP TSG-RAN WG2 Meeting #131bis	R2-2507047

Prague, Czech Republic, 13th - 17th October, 2025

Agenda Item:	8.9.4

Source: 	OPPO

Title:  	Discussion on other open issues for Rel-19 IoT-NTN

Document for:	Discussion and Decision

- Introduction

Most of remaining open issues in 36304 are from Store & Forward satellite operation. In last RAN2 meeting, the following agreements for Store & Forward satellite operation were made.

Agreements:

1.	The UE forwards the S&F mode indication (e.g. sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

2.	The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

3.	A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review)

4.	We clarify the stage-2 description in 36.300 to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are set consistently, i.e. with the same value, when they refer to the same satellite (FFS on the actual wording)

5.	We don’t increase the size of satellite ID and or define any new satellite ID as part of this Rel-19 WI

This contribution aims to provide our considerations on the remaining open issues in 36304 and 36306, mainly covering Store & Forward satellite operation.

- Discussion

  - NAS configured S&F monitoring list

In RAN2#130 meeting, the following agreement was made. It is still FFS on the usage of NAS configured S&F monitoring list in AS layer, e.g., for discontinuous coverage procedure.

Agreements:

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

Note that in TS 23.401, based on NOTE 5 and 7 in clause 4.13.9 as shown below, it is fully up to UE implementation to use the S&F monitoring list in NAS layer, even though in this case there is increased probability that it will not be able to complete the NAS procedure.

NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

Considering that even in NAS layer the S&F monitoring list is informative, i.e., the NAS behavior related to S&F monitoring list is up to UE implementation, there is also no motivation for AS layer to specify the related AS operation. We understand that it is proper to also leave to UE implementation to use the NAS configured S&F monitoring list in AS layer for AS operations, e.g., discontinuous coverage procedure, or whether to prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the NAS configured S&F monitoring list during cell reselection procedure. No any AS impact is needed.

- Up to UE implementation on the usage of the NAS configured S&F monitoring list in AS layer, i.e., no any AS impact needed.

  - Paging enhancement for S&F Mode operation

Based on the outcome of the post-meeting email discussion [Post131][308], paging enhancement for S&F Mode operation is still an open issue, which is expected to provide company contribution for discussion. The main intention for this enhancement is to relax paging monitoring for power saving in case that cell operating in S&F mode doesn’t have any MT data buffered for UE.

We understand although the power saving gain for this enhancement is clearer, when we look into the detailed solution, additional discussion is needed, e.g., for a cell operating in S&F mode has MT data buffered for UE, if the stored MT data has already been transmitted, whether the indication needs to be changed or not, thus whether SI modification notification is needed or not. Considering Rel-19 feature frozen in RAN2, it is suggested not to consider further enhancement to avoid additional debate of detailed solution.

- RAN2 does not consider paging monitoring relaxation for cell operating in S&F mode without any stored MT data in Rel-19.

  - AS Capability for support of S&F

Based on the outcome of the post-meeting email discussion [Post131][309], one open issue for capability is whether to introduce a UE AS capability with signalling for support of S&F mode or not, given S&F mode capability is already exchanged between UE and MME in NAS layer. The decision on this issue depends on whether eNB need to know this information, e.g., to decide whether to keep the UEs or release the UEs when feeder link connection is lost or will be lost soon.

For UE in RRC_CONNECTED mode, we understand to rely on NW implementation for UE handling if eNB switches between S&F operation mode and normal mode, and the different ways of NW handling in AS layer includes: 1) release UE to RRC_IDLE mode, 2) trigger handover to switch the UE to a correct cell, or 3) update the radio bearer configuration to adapt the operation mode of the current cell, for Rel-19 UEs supporting S&F operation. Therefore, to assist eNB to make the proper implementation, eNB also needs to know the information of whether the UE supports S&F operation or not.

Considering that NAS layer capability for UE supporting S&F operation has already been introduced, we understand eNB could know this information of UE based on the network-side interaction between eNB and MME by network implementation. For example, when MME detaches the UE by MME-initiated Detach Procedure in NAS layer, at the same time MME also sends S1 Release Command message to eNB to release the S1-MME signalling connection for the UE. Therefore, eNB can simply follow the indication of MME to perform AS handling accordingly. It is redundant for UE to indicate the capability in both AS layer and NAS layer. Considering that the issue is more about network side, it is also suggested to send LS to RAN3 to check and confirm if any issue/impact on RAN3.

- RAN2 assumes no need to introduce AS capability signalling for UE supporting of S&F mode for now, given the introduced NSA capability for this. Send LS to RAN3 for further check/confirmation.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

NAS configured S&F monitoring list

- Up to UE implementation on the usage of the NAS configured S&F monitoring list in AS layer, i.e., no any AS impact needed.

Paging enhancement for S&F Mode operation

- RAN2 does not consider paging monitoring relaxation for cell operating in S&F mode without any stored MT data in Rel-19.

AS Capability for support of S&F

- RAN2 assumes no need to introduce AS capability signalling for UE supporting of S&F mode for now, given the introduced NSA capability for this. Send LS to RAN3 for further check/confirmation.