3GPP TSG-RAN2#131bis	Tdoc R2-2507073

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	10.3.2 – Control plane

Source:	Ericsson

Title:	RRC, ASN.1 and other signalling for 6G aspects

Document for:	Discussion, Decision

# 1	Introduction

In this document we present our view on the RRC signalling of the 6G radio interface. We analyse problems observed in previous generations and suggest high level principles to avoid those in 6GR.

# 2	Discussion

## 2.1	Simpler, leaner, unambiguous and extensible

LTE’s and NR’s RRC ASN.1 messages use a tree-like data structure for configuring protocols, channels, and signals. Due to the different architecture options (EN-DC, NR-SA, NR-DC, etc.), the support for dual connectivity, carrier aggregation and bandwidth parts (BWP) as well as due to the large number of configuration parameters for the physical layer, NR’s RRC ASN.1 messages became large and deeply nested (many levels of hierarchy for CellGroups, ServingCells and BWPs). The division into common and dedicated parameters as well as the separation of uplink and downlink fields created additional hierarchy levels. While this seemed convenient at a first glance, it makes the signalling unnecessary difficult to extend, i.e., to introduce new functionality which does not fit into the previously established structure. This happened already in the later phase of Rel-15 when the introduction of BWPs did not fit well with the original separation into common and dedicated parameters nor to the structure of the Serving Cells. Ultimately, this led e.g. to the problem that BWPs could not be used to switch a RedCap UE between several narrow channels within a wider carrier. The reason was that the “channel BW” configuration landed outside the BWP configuration and inside the ServingCellConfigCommon branch that could not be modified by dedicated signalling. Similarly, the introduction of various multi-TRP schemes required configuring additional PDCCHs and/or PDSCHs which were in the same frequency band but (sometimes) associated with a different cell (i.e., PCI).

Hierarchically structured configuration messages hinder the introduction of new functionality and thereby drive complexity and development costs.

Another weakness of LTE’s and NR’s configuration signalling is the almost exclusive use of non-critical extensions, i.e., the addition of parameters to the end of a previously introduced information element. To facilitate that, RAN2 defines most fields in an IE as OPTIONAL which allows omitting the field when it becomes obsolete for the functionality of a later release. After a couple of releases many of the information elements (PDSCH-Config, PDCCH-Config, PUSCH-Config, …) comprise of many optional fields which suggests a great configuration flexibility. However, it is often ambiguous which combination of values is legitimate. RAN2 tries to capture constraints in field descriptions, condition tables or procedural text. But those are typically not complete and not easy to test and validate.

Continued non-critical extensions of large IEs tend to make it ambiguous which combination of parameters and values is valid

The use of a critical extension could be a viable alternative in many such cases. The new version of an IE type (e.g. PDSCH-Config-r22) would only contain the fields required to configure this new PDSCH version.

However, critical extensions had far-reaching consequences in NR’s ASN.1 structure. One reason is that we typically configure the original IE type in AddMod-lists:

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet OPTIONAL,

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId OPTIONAL,

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace OPTIONAL,

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId OPTIONAL,

Therein, each element was associated with an ID that was used within the original IE type:

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

Had we defined a critical extension of the IE (e.g. SearchSpace-r21) one would have had to define at least a new AddMod list for that new type which bears the risk of race conditions e.g. if both lists contain an element with the same ID.

Furthermore, RAN2 tends to restrict the size of the IDs to the bare minimum required for the original version of the IE type. Consequently, also a new ID type had to be introduced in some cases (e.g. ControlResourceSetId-r16 replacing the original ControlResourceSetId). This can have far reaching impact on other parts of the configuration structure if other IEs (e.g. SearchSpace) used the original ID to refer to the original IE type. In such cases, also those other IEs would need to use (also) the new ID type to refer to instances of the new IE type. This would cause additional complexity and ambiguity in those other IE types.

We suspect that these were the primary reasons why RAN2 used critical extensions of IEs only rarely despite the potential benefits outlined above.

For 6G RAN2 should develop a configuration messages structure that is easy to use and extend. For example, RAN2 could prepare the later use of critical extensions by defining “super-types” already in the first version of the specification and to use those in the AddMod/Release-lists. If sufficiently large IDs refer to those super-type elements, critical extensions (new variants of the super-types) can be introduced later without affecting other parts of the specification.

Furthermore, we observe that hierarchical nesting levels should be minimized. This makes it easy from a signalling point of view to introduce new features which require configuring different types or cardinalities or different IE types.

6G’s configuration message structure should be convenient to use in products and easy to extend in later releases.

## 2.2	The delta

To reduce the downlink signalling overhead, LTE and NR supported delta signalling. Traditionally, RAN2 assumes that the use of delta signalling is the default for reconfigurations within a gNB but also for mobility across gNBs. RAN2 specifies delta signalling by capturing what a UE shall do if ASN.1 fields were previously configured but are absent in a subsequent RRC reconfiguration message. Those rules are captured in need codes (comments in the definitions in the ASN.1 syntax), field descriptions, condition tables and procedural text. While this seems simple at a first glance, it turns out to be ambiguous and impossible to automate and hence results in errors in implementations and testing.

LTE’s and NR’s ASN.1 delta signalling cannot be automated since rules are captured in need codes, field descriptions, condition tables and procedural text.

Those challenges led to the unfortunate situation that delta signalling is hardly used in products – especially not for inter-node mobility where small configuration messages would have been particularly beneficial. The reason is that inter-node support would require that a target gNB parses an incoming UE configuration produced by the source node and determines unambiguously which of the child branches/fields therein it needs to release or modify. While this would be possible for fields corresponding to functionality supported by this target gNB, it isn’t feasible for all other fields that this gNB does not support but that a source node might have used.

NR’s delta signalling isn’t feasible for inter-node reconfigurations since the target node would need to support releasing features which it does not support itself.

It should be noted that a UE does not face the same problem since it may assume that a gNB configures only those fields that the UE supports according to its UE capabilities. Nevertheless, implementing NR’s or LTE’s delta signalling is extremely challenging also for UEs. And in cases where we try to use delta signalling for (intra-gNB) reconfigurations we have noticed that there is often some UEs/chipsets that fail to process those messages correctly.

NR’s way of supporting delta signalling by means of need-codes is error-prone and results in error cases in live networks despite extensive IODT.

6G should therefore use a simple yet efficient in-line definition of delta signalling. It is vital that the delta signalling rules (which fields to release, which fields to keep) are embedded in the ASN.1 itself so that they survive the ASN.1 compilation, i.e., that they remain visible in the resulting source code. This would allow UE- and network implementations to automate the handling of delta signalling and thereby remove today’s ambiguity. Special attention should also be given to forward compatibility, i.e., to the case where the source gNB configured fields for a feature which the target node does not implement. Preferably, delta signalling should be specified such that the target gNB can still generate a delta configuration that is syntactically correct and by which the UE releases those features.

6G’s delta signalling should be specified in a machine-readable manner to avoid ambiguity and implementation errors. 6G’s delta signalling should support the inter-node reconfigurations with reasonable implementation complexity.

## 2.3	A holistic view on control plane

In 3GPP RAN2 the term control plane is typically associated with the RRC(Re)Configuration procedures and messages. But there are of course many more messages and procedures by which the RAN controls the UE’s connection to the gNB. Control messages are defined also in the MAC specification (MAC CEs for UL reports and DL commands) and in the L1 specifications (DCI, UCI).

The Uu interface’s control plane spans way beyond “RRC”. Control messages are also specified in MAC CEs and in L1’s Downlink- and Uplink Control Information (DCI/UCI).

3GPP intentionally defines functionality in protocol layers but many control procedures span anyway across several layers and depend on functionality described by multiple working groups in various specifications. In many cases this leads to a suboptimal and unnecessary complex design and in hard-to-read specifications. For example, NR’s connection setup procedure begins with a set of rules defined in 38.331 followed by the transmission of a RA preamble defined in a RAN1 specification according to a procedure captured in the MAC specification. MAC defines the format of the Random Access Response message while the meaning of its “UL Grant” field is defined in L1 specs. The payload of Msg3 (sent on PUSCH) is either defined in the RRC specification (in case of CCCH) or in the MAC spec or in combinations of both. Even within one specification related information is often spread into different sections which, besides being hard to consume, causes both duplication and lack of information.

In LTE and NR many procedures span across many protocol layers and involve messages described in various specifications. This makes specifications difficult to maintain and use.

We propose to aim for a more holistic view when analysing shortcomings of previous generations and when developing concepts for 6G. Working groups should develop control plane procedures and messages jointly to avoid duplicate and competing solutions.

Maintain a holistic view of the control plane messages when analysing shortcomings of previous generations and when developing concepts for 6G. Don’t think in “RAN2-UP”, “RAN2-CP”, “RAN1 MIMO”, … boxes.

## 2.4	Ownership

RAN2 claims ownership of the RRC specifications and the ASN.1 message structures therein. And with no doubt it would be important that a group of experts develops and maintains a message structure and generic functionality for the Uu configuration- and capability signalling.

However, according to the current work-split between RAN1, RAN4 and RAN2 it is rather so that RAN1 defines configuration parameters for new physical layer features in lengthy but incomplete and poorly documented Excel tables. RAN2 tries to translate them into an ASN.1 syntax, which is challenging given the lack of information and the (surprising) unwillingness of many companies to analyse and improve the structure from a signalling perspective. RAN2 attempts to describe the meaning of the configuration parameters in field descriptions – sometimes assisted by clarifications from RAN1. However, in many cases RAN1 amends, overrides or restricts in their own specifications what the description in the RRC specification seems to say. This work-split is certainly cumbersome for RAN2, but the real problem is that it makes some of RAN1 specifications extremely hard to read and more ambiguous with every subsequent addition.

The same problem arises for UE capabilities which RAN1 defines and for which RAN2 captures the ASN.1 signalling and the descriptions in two different specifications.

The current work-split between RAN1, RAN2 and RAN4 and the way how control messages are specified increases the workload and results in ambiguous specifications with inefficient signalling structures.

For 6G RAN2 should discuss whether there are better ways to capture configuration- and capability signalling with less time-consuming and error-prone inter-(working-)group communication. For example, we could envision that RAN2’s control plane experts develop and maintain the overall signalling structure including top-level messages and tools such as delta signalling, full configuration, critical- vs. non-critical extensions. Feature experts working with e.g. L2’s user plane, L1’s PDSCH or CSI-RS should more actively be involved in creating and maintaining the low-level IEs (e.g. RLC-Config, PDSCH-Config, …) that are used to configure “their” functionality, i.e. value ranges, default values, configuration restrictions. For example, already today we are capturing in the BSR-section the relevant fields for the BSR-procedure, however this is not the case for all specifications and for all functionality, but if we would consistently describe applicable fields, their relation, etc., in the section for the relevant functionality, we would likely get a specification which is easier to read and understand since the sections would be more self-contained.

This would hopefully leave more time for RAN2 to maintain the overall structure and avoid that the specification becomes convoluted and unusable after a few releases. Of course, RAN2 should also review the signalling contributed by other groups and request necessary alignment to established signalling principles.

Improve the work split between the RAN2 control plane experts and other (sub-)working groups with respect to creation and maintenance of configuration- and capability messages.

# 3	Conclusion

In this paper we have reached these observations:

Observation 1	Hierarchically structured configuration messages hinder the introduction of new functionality and thereby drive complexity and development costs.

Observation 2	Continued non-critical extensions of large IEs tend to make it ambiguous which combination of parameters and values is valid

Observation 3	LTE’s and NR’s ASN.1 delta signalling cannot be automated since rules are captured in need codes, field descriptions, condition tables and procedural text.

Observation 4	NR’s delta signalling isn’t feasible for inter-node reconfigurations since the target node would need to support releasing features which it does not support itself.

Observation 5	NR’s way of supporting delta signalling by means of need-codes is error-prone and results in error cases in live networks despite extensive IODT.

Observation 6	The Uu interface’s control plane spans way beyond “RRC”. Control messages are also specified in MAC CEs and in L1’s Downlink- and Uplink Control Information (DCI/UCI).

Observation 7	In LTE and NR many procedures span across many protocol layers and involve messages described in various specifications. This makes specifications difficult to maintain and use.

Observation 8	The current work-split between RAN1, RAN2 and RAN4 and the way how control messages are specified increases the workload and results in ambiguous specifications with inefficient signalling structures.

We propose:

Proposal 1	6G’s configuration message structure should be convenient to use in products and easy to extend in later releases.

Proposal 2	6G’s delta signalling should be specified in a machine-readable manner to avoid ambiguity and implementation errors. 6G’s delta signalling should support the inter-node reconfigurations with reasonable implementation complexity.

Proposal 3	Maintain a holistic view of the control plane messages when analysing shortcomings of previous generations and when developing concepts for 6G. Don’t think in “RAN2-UP”, “RAN2-CP”, “RAN1 MIMO”, … boxes.

Proposal 4	Improve the work split between the RAN2 control plane experts and other (sub-)working groups with respect to creation and maintenance of configuration- and capability messages.