3GPP TSG-RAN2#131bis	Tdoc R2-2507075

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	10.4 Mobility

Source:	Ericsson

Title:	6G Mobility

Document for:	Discussion, Decision

# 1	Introduction

In this document we present our view on mobility aspects of the 6G radio interface. We analyse problems observed in previous generations and suggest high level principles for 6G to avoid the same problems again.

# 2	Discussion

## 2.1	High level principles

In NR, the network is in control of mobility procedures in connected mode, and the UE is in control of the mobility procedures in idle and inactive mode. From a network perspective, UEs in connected mode contribute to the load in the network and it is important to the network to be able to steer UEs to certain frequencies to achieve the best possible load distribution. UEs have no knowledge of the load in the network and cannot take mobility decisions on their own as it may cause overload of certain frequencies leading to performance degradation. In addition, there are other network conditions that the UEs are not aware of like network energy saving measures where certain cells/network entities may be switched off or operator policies where some resources are reserved for prioritized users.

UEs have no knowledge of the network conditions and cannot take mobility decisions on their own in connected mode without the risk of network performance degradation.

It is the network operators’ decision how to use the network resources and how to steer the traffic.

UEs in idle and inactive mode perform mobility based on cell reselection. We have not observed any major issues with this approach and see hence no reason to change this. For 6G the same principle can be adopted.

The network is in control of connected mode mobility and idle/inactive mobility is based on (cell) reselection.

## 2.2	Measurements framework

Below is a summary of the various connected mode measurements in NR. The summary shows the large variety of measurement and reporting functions that exist in NR and that large variety causes complexity and larger implementation costs.

- Rel-15: Introduced Layer 3 (L3) measurement-based handover (HO), beam selection and beam switch based on L1-RSRP.

- Rel-16: Added support for L1-SINR measurements.

- Rel-17: Introduced inter-cell beam management, allowing different Transmission Reception Points (TRPs) to have different cell IDs and transmit different RS and the possibility of reporting L1-RSRP measurements associated to such TRPs.

- Rel-18: Added measurements to support lower layer triggered mobility (LTM).

- Rel-19: Extended LTM to inter-CU and added support for event triggered measurement reporting for L1-RSRP and UE initiated/event-triggered beam management under MIMO.

- Rel-20: Aims to extend the LTM CSI measurements for SCells.

Looking at the evolution of the measurement framework in 5G, it is possible to notice that there has been a fragmentation on how measurements are configured and reported, give also whether such measurements were measured and evaluated at the L1 or L3 layer.

#### 2.2.1.1	Configuring same reference signals on different layers

According to the current L1 and L3 measurement framework, it is evident that current signalling allows network to configure same type of reference signals on different configuration level of the RRC structure hierarchy. Considering that the biggest different between L1 and L3 measurements is basically the L3 filter the UE applies to the sample which has measured, in 6G it would be good to make things simple and build a measurement configuration framework where basically reference signals to be measured by the UE are indicated by the network in a unique configuration.

Such a unified measurement configuration framework would remove redundant signalling and eliminate the current layering complexity, where basically the same components of the measurement framework are defined separately at different levels. Instead, in 6G reference signals should be part of a single configuration that encapsulates the necessary parameters, while the network specifies whether the reporting should be based on raw (L1-like) or filtered (L3-like) samples. This approach would simplify both standardization and implementation, reducing ambiguities in interpretation across vendors.

Current 5G NR RRC signalling allows network to configure same type of reference signals on different configuration level of the RRC structure hierarchy.

#### 2.2.1.2	Report same measurement quantity on different layers

NR defines means for sending the same/similar type of measurement reports on different uplink channels. For instance, L1-RSRP reports are used in legacy CSI measurements for RRM, in MIMO, and also in LTM. Nevertheless, 5G UEs need to send this measurement quantity as L1 UCI on PUCCH for LTM and MIMO, and in MAC CEs (L2) for LTM. Furthermore, beam level measurements can also be reported via L3 on the RRC layer.

This unnecessary signalling complexity occurred partially because different features were standardized in different releases of NR. However, also the fact that in 5G NR the L1 measurements framework and L3 measurements framework have distinct measurement configurations and reporting configurations contributed to the diverse reporting schemes. Considering the fact that the samples which the UE measures are actually the same for both L1 and L3 measurements, we believe that in 6G we can do better and thus try to unify they way how UE report measurements to the network.

NR defines means for sending the same/similar type of measurement reports on different uplink channels causing unnecessary signalling complexity.

Given this, it is of upmost importance to learn from the mistake made in 5G and make sure that the measurement framework in 6G is lean and less complex. This will avoid dealing with redundancy, increased signaling complexity, and vendor implementation overhead (which is what has happened in 5G). For these reasons, we propose:

The 6G measurement framework defines a unified measurement configuration and reporting framework, where reference signals to be measured and measurement quantities to be reported are defined within one configuration entity of the RRC structure, with characteristic related to current L1 and L3 measurements (e.g., L3 filter) defined as attributes of the same configuration.

## 2.3	Connected mode mobility

In NR, several different mobility procedures have been defined. In Rel-15, the layer 3 mobility handover was standardized as part of the first release of NR. In Rel-16, conditional handover was standardized with the purpose of improving the handover robustness and DAPS with the purpose of minimizing the interruption time. Later, conditional PSCell change was specified and various variations such as subsequent CPAC and CHO with candidate SCG. In Rel-18 lower layer triggered mobility (LTM) with lower latency was standardized, with advantages compared to DAPS, and in Rel-19 conditional LTM improved the robustness even further. In several cases, different procedures address very similar or even identical purposes (e.g., CHO and C-LTM). Multiple procedures with similar benefits increase the risk of fragmented implementation, where different vendors support different procedures.

All above-mentioned schemes as well as the beam-management solutions defined by RAN1 address the same primary purpose: The UE is supposed to follow another synchronization reference signal and to use it for transmitting and receiving other physical channels and signals. In 6G it is hence desirable to cover all mobility options in a harmonized solution.

The mobility procedures in NR serve different purposes, but in the end multiple options have been specified addressing similar requirements.

In general, mobility can be used for connection continuation when the UE moves, for load balancing, for QoS maintenance and for avoiding and minimizing the service interruption time in case of failures, e.g. hardware failures etc. Mobility in 6G needs to support multiple use cases with different requirements. Both short interruption time and high robustness are important aspects that need to be taken into account already in the first release of in 6G. There is no reason why both aspects should not be prioritized when there already is a framework to build upon and experiences from earlier generations are available. Also, the mobility procedures should support the avoidance of RLF in general. RLF is a procedure that takes a long time and using a mobility procedure instead of triggering RLF improves the end user experience.

The mobility framework in 6G supports both short interruption time and high robustness in the first release.

One consequence of different mobility procedures being specified in different releases is that they were specified one by one without any possibility to consider the overall impacts on the specifications. In 6G, the framework should already from the start consider the different aspects of mobility so that the specification impacts are more unified in the sense that a field with a specific purpose, e.g. a condition or a target candidate configuration, is only defined once.

In 6G, both conditional and immediate mobility need to be supported as conditional mobility provides higher robustness, but there will also always be cases where target candidates have not been prepared and unconditional mobility needs to be executed. A drawback with conditional mobility is that network resources need to be kept reserved, and possibly for a long time. Also, configuration updates increase processing in the network and signalling to the UE. In 6G, it should be studied whether these drawbacks could be resolved or reduced. The advantage of using conditional configurations for resilience purposes should also be taken into account.

UL and DL pre-synchronization done as part of LTM decrease the interruption time as the handover execution procedure is faster. Hence, the pre-synchronization is beneficial and should be implemented for all type of handover procedures in the first release of 6G.

A unified handover execution procedure is a reason for selecting RRC for triggering the handover execution. An RRC message is flexible and can be used in all cases, regardless of the type of the triggered handover. RRC also has the benefit of providing security. A drawback with RRC could be the longer execution time. However, the difference in execution between a MAC CE and a short RRC message may not be very large if the message content is similar. A cell switch command can be a very short message. Also, in many cases, the configuration changes between the source configuration and the target candidate configuration are limited. In NR, there is no large benefit between sending a short RRC message and a long RRC message as the UE performance requirements from an RRC processing point of view are the same. If shorter RRC message(s) with limited content and tight performance requirements would be defined and used in the cases where only a cell switch command is sent or when most of the source configuration is kept, it would provide not only a fast handover execution, but also flexibility and a unified mobility framework.

The requirements on the 6G mobility scheme can be summarised as in the following proposal:

6G mobility has a single framework that supports: 
- both conditional and immediate execution
- both UL and DL pre-synchronization
- operation with and without candidate pre-configurations
- short and flexible execution command

## 2.4	Idle/Inactive mode mobility

When a UE enters an energy saving state (RRC_IDLE or RRC_INACTIVE) in NR, it finds a suitable cell to camp on through the cell selection procedure. The frequency selected at the cell selection procedure is up to UE implementation. This is then followed by a cell reselection procedure where the UE updates the cell, frequency and/or RAT to camp on. The cell reselection procedure is controlled by the network through cell reselection priorities, which influence which frequency and/or RAT that the UE shall prioritize to camp on. The cell reselection priorities should enable the network to steer where UEs camp and then perform access based on e.g. load situations in the network and expected service requirements for the UE. This possibility for the network to control where UEs camp in the network should be supported also in 6G.

For the inter-frequency reselection procedure, the UE is required to perform inter-frequency measurements. However, the time requirements for inter-frequency measurements are scaled by the number of frequencies configured as part of SIB4. This in turn leads to a situation where the UE often transitions to connected state without completing these inter-frequency measurements and the corresponding reselection procedure to a prioritized frequency.

For example, in NR with a configuration of 3 carriers on FR1 (as in Figure 1) a UE shall perform inter-frequency reselection to a previously undetected cell on a higher priority frequency within an allowed maximum time of ~120 seconds (when a suitable cell exists on the higher priority frequency), according to TS 38.133. Even for the case of a known cell, the time it takes a UE to complete cell reselection measurements (according to NR requirements) is significantly longer than the time that a UE typically spends in an energy saving state (RRC_IDLE or RRC_INACTIVE) before transitioning to connected state.

UE time spent in energy saving state is often short compared to the requirements for inter-frequency measurements.

Figure 1: An example with cell reselection priorities configuration with 3 frequencies.

A consequence of time requirements that scale with the number of frequencies is that the provided cell reselection priorities do often not impact where the UEs actually camp and perform their access at the end, even if there are suitable cells for the UE on the higher priority frequencies. As explained above, this can be due to that the UE returns to connected state before completing the cell reselection procedure.

Instead of relying on inter-frequency cell reselection priorities in RRC_IDLE/RRC_INACTIVE mode the network could use inter-frequency measurements and trigger inter-frequency handovers for the UE when it is in connected mode. However, such measurements and handovers are likely to impact the performance of the ongoing data transfer and create additional control signalling.

Figure 2 Field data with measurement results collected from multiple UEs in a network. The two graphs show the number of received measurement results for different RSRP values, the lower graph for a cell (SCell) on a higher priority frequency and the upper graph for a cell (PCell) on a lower priority frequency.

Observations from the field show that UEs often perform access to a cell on a lower priority frequency even though they are in coverage of a cell on a higher priority frequency. An example can be seen in Figure 2  with measurement results from UEs directly after entering RRC_CONNECTED. The measurement results are for the PCell, which is on a frequency that had a lower cell reselection priority, and for a cell on a frequency that was configured with a higher cell reselection priority (here setup as an SCell). The results indicate that even though UEs are in coverage, with higher RSRP values than the configured threshold, of the cell on the higher priority frequency, they do not camp and perform their access on that intended frequency.

UEs often do not reselect to higher priority frequencies even in the cases where coverage is present, as observed by the connected mode measurements.

6G should aim to find solutions for Idle/Inactive mobility that benefits both the UE and network in terms of performance and energy efficiency. The goal is that the UE camps on a frequency intended by the network so that the network can best serve the UE while avoiding unnecessary handovers in the connected state.

The prioritization of frequency/RAT to camp on while in idle/inactive should be under network control. RAN2 should, together with RAN4, study how to ensure that UEs camp on a frequency where the network wants to serve the UE in connected mode.

## 2.5	Inter-RAT

According to the 6G study WID [RP-252912], migration from 5G to 6G will be based on 5G/6G MRSS and inter-RAT handover. MRSS will ensure smooth integration from 5G to 6G where deployed, but MRSS may not be deployed on all carriers and thus to complement MRSS there is still a need for efficient 5G/6G inter-RAT handover procedure to ensure service continuity.

For RRC CONNECTED UEs, an efficient 5G/6G inter-RAT handover procedure is required to ensure service continuity when moving between 5G and 6G coverage

For UEs in inactive mode, the network keeps a UE context, including security context and established bearers to support a fast resume procedure for transitioning to connected mode when there is data to transmit. Maintaining this UE context also for the case of inter-RAT mobility would allow UEs to freely roam between 5G and 6G coverage while in inactive mode but would create a dependency between 5G and 6G specifications and severely limit the possibilities for 6G. Another issue is reachability, i.e. paging is performed in the UE serving system where the UE is registered. Assuming UE triggers registration when moving to another RAT, a new UE context is created in the new RAT and the UE context in the old RAT would be removed. This was discussed also during early 5G standardization but was dismissed for these reasons. We think the same principle should apply also for 6G, i.e. mobility between 5G and 6G in inactive mode should go via idle.

For UEs in inactive mode, keeping the inactive context when moving between 5G and 6G will introduce dependency between 5G and 6G specs, and additional complexity, and should be avoided.

Study support of connected mode Inter-RAT handover between 5G and 6G. Mobility between 5G and 6G in inactive mode goes via idle.

# 3	Conclusion

In this paper we have reached these observations:

Observation 1	UEs have no knowledge of the network conditions and cannot take mobility decisions on their own in connected mode without the risk of network performance degradation.

Observation 2	It is the network operators’ decision how to use the network resources and how to steer the traffic.

Observation 3	Current 5G NR RRC signalling allows network to configure same type of reference signals on different configuration level of the RRC structure hierarchy.

Observation 4	NR defines means for sending the same/similar type of measurement reports on different uplink channels causing unnecessary signalling complexity.

Observation 5	The mobility procedures in NR serve different purposes, but in the end multiple options have been specified addressing similar requirements.

Observation 6	UE time spent in energy saving state is often short compared to the requirements for inter-frequency measurements.

Observation 7	UEs often do not reselect to higher priority frequencies even in the cases where coverage is present, as observed by the connected mode measurements.

Observation 8	For RRC CONNECTED UEs, an efficient 5G/6G inter-RAT handover procedure is required to ensure service continuity when moving between 5G and 6G coverage

Observation 9	For UEs in inactive mode, keeping the inactive context when moving between 5G and 6G will introduce dependency between 5G and 6G specs, and additional complexity, and should be avoided.

We propose:

Proposal 1	The network is in control of connected mode mobility and idle/inactive mobility is based on (cell) reselection.

Proposal 2	The 6G measurement framework defines a unified measurement configuration and reporting framework, where reference signals to be measured and measurement quantities to be reported are defined within one configuration entity of the RRC structure, with characteristic related to current L1 and L3 measurements (e.g., L3 filter) defined as attributes of the same configuration.

Proposal 3	The mobility framework in 6G supports both short interruption time and high robustness in the first release.

Proposal 4	6G mobility has a single framework that supports:  - both conditional and immediate execution - both UL and DL pre-synchronization - operation with and without candidate pre-configurations - short and flexible execution command

Proposal 5	The prioritization of frequency/RAT to camp on while in idle/inactive should be under network control. RAN2 should, together with RAN4, study how to ensure that UEs camp on a frequency where the network wants to serve the UE in connected mode.

Proposal 6	Study support of connected mode Inter-RAT handover between 5G and 6G. Mobility between 5G and 6G in inactive mode goes via idle.