3GPP TSG-RAN WG2 Meeting #131bis	R2-2507081

Prague, Czech Republic, Oct. 13th-17th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on AI/ML framework and data transfer

Agenda Item:	10.3.3

Document for:	Discussion and Decisions

- Introduction

During the RAN#108 meeting, the new SID on “Study on 6G Radio” was approved, and it was further revised in RAN #109 meeting[1]. The new SID describes the objective of the work on AI/ML for 6GR and RAN as below, in which the study on AI/ML framework needs to be led by RAN2.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Besides, the SID also put the data transfer as one of the objectives in the 6GR interface protocol architecture and procedure design as below,

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In parallel, the RAN level study on the requirements of new and existing services was conducted in RAN #109 meeting, and following services are approved to be captured in TR38.914 section 5.4.

- Mobile broadband

- Immersive Communication

- Massive Communication (IoT)

- Sensing

- AI

- Voice

- Regulatory services

- Mobile broadband

- Immersive Communication

- Massive Communication (IoT)

- Sensing

- AI

- Voice

- Regulatory services

This paper discusses the AI/ML framework for 6GR and the data transfer for existing and new services.

- AI/ML framework

  - Lesson from 5G(A)

The AI/ML framework defined in 5G(A) was developed in a use-case-driven manner, where each use case (e.g., AI/ML for beam management, CSI compression, or L3 mobility optimization) was studied and specified individually. As a result, the framework was retrofitted into the existing 5G architecture rather than being designed as a unified and scalable system from the beginning. This approach led to fragmentation and duplication in the specification, since many AI/ML-related procedures—such as data collection, model transfer, and performance monitoring—were defined separately for each use case, often with different assumptions or signaling mechanisms. Consequently, interoperability and extensibility across use cases remain limited, and the reuse of common AI/ML enablers across different RAN functions is inefficient.

Observation 1: The AI/ML framework in 5G(A) was per use cases and retrofitted, which makes the related description in specification fragmented and duplicated.

In 5G(A), the data collection options for UE-side model training were analyzed to understand potential specification impact as well as the different levels of MNO controllability and visibility over the collected data. However, it was concluded that except for the case of standardized data content, the data content visibility and any level of controllability could be achieved via SLA which is out of RAN2 scope. In practice, this means that MNOs cannot effectively monitor the completeness, freshness, or integrity of the data used for AI/ML model training, nor can they flexibly manage when and how the data is collected or uploaded. Such a lack of standardized control mechanisms introduces operational uncertainty, complicates model lifecycle management, and makes it difficult to guarantee data quality for AI/ML model training in live networks.

Observation 2: The UE data collection in 5G(A) was not fully specified, and MNO cannot have full visibility and controllability.

The need for UE consent in NW data collection, particularly when data is used for NW-side model training, has not yet been standardized. While the importance of user consent is widely recognized, RAN2 cannot determine this issue independently, as it involves broader privacy and security considerations under SA3’s responsibility. The clarification of consent handling will be essential for 6G to ensure transparency, user trust, and regulatory compliance.

Observation 3: The UE consent for NW data collection in 5G(A) was not thoroughly discussed, and there was not clear conclusion on whether user consent is needed and how it can be achieved (as defined for MDT or new solution).

  - Proposals for 6G

For 6G, there is motivation to design a common AI/ML framework that can flexibly accommodate various use cases across RAN1, RAN2, and RAN3. However, achieving such unification raises multiple design challenges, such as defining common interfaces, reusable signaling structures, and cross-layer coordination mechanisms. The balance between generality (to avoid redundancy) and specificity (to ensure efficiency) must be carefully studied. Therefore, the 6G study should investigate the unified framework that can support a set of verified and representative AI/ML use cases, rather than attempting to cover all potential use cases in a generic way. In particular, model-based LCM use cases involving two-sided models should be carefully evaluated to verify their actual necessity and practical benefits before inclusion.

Proposal 1: RAN2 should study the unified framework for life cycle management for AI/ML operation.

- The LCM framework specified in 5G(A) Rel-19 is used as the baseline/staring point.

- The use cases and necessity of model-based LCM should be verified before studying the feasibility.

The requirements for UE data collection differ between Mobile Network Operators (MNOs) and UE vendors: From the MNO perspective, the key requirements include visibility, configurability, and controllability. From the UE vendor perspective, the priorities lie in protecting user privacy, minimizing power and storage consumption. The coexistence of these requirements implies the need for a standardized yet flexible control mechanism, allowing operators to manage UE data collection without compromising UE privacy and power/storage efficiency.

In 6G, there will be more requirements for NW AI/ML operation, such as data collection for model training, dataset and model (parameter) sharing for inter-vendor collaboration for two-sided model. It is essential to define the role of UE consent when data collected from UE is used for NW AI/ML operations, to ensure transparency and compliance with privacy regulations.

In addition, the NW initiated data transfer for AI/ML-based services will also much increase in 6G. A key question is whether a unified solution or common transport framework should be designed to handle both directions consistently. Despite their functional differences, both share similar requirements on security, latency, reliability, and traffic differentiation.  A unified design could improve efficiency by reusing the same data plane mechanisms and reducing signaling overhead.

Proposal 2: RAN2 study on AI/ML for 6GR should consider following key principles:

- MNO should have the visibility of the collected data.

- Efficient AI/ML data transfer mechanism should be supported.

- Unified data collection framework for both UE side model and network side model should be supported.

- AI/ML Performance monitoring should be supported, and fallback to legacy NW operation mechanism should be ensured.

- Customer Privacy (e.g. user consent) should be considered for AI/ML data collection.

- Data transfer

  - Prospects of new services

Future 6G services such as AI/ML- and sensing-based applications will impose diverse and stringent data transmission demands beyond the legacy solutions. These services require flexible QoS, ultra-high throughput, and ultra-low latency for both user/data and control/signaling exchanges among the functions. 6G may also need to support data transfer across multiple endpoints, e.g. UE–RAN, UE–CN, UE-UE, and RAN–RAN, to enable diverse use cases like AI/ML model updates or real-time sensing data fusion. Meeting these needs calls for a rethinking of 5G(A) protocol and procedure design, ensuring dynamic adaptability to different AI/ML and sensing requirements while maintaining reliability and spectrum efficiency.

Observation 4: Flexible QoS, high throughput, low latency and transmission between a broader set of endpoints such as UE to RAN, UE to CN, UE to UE, and RAN to RAN are proposed for the new services such as AI/ML and Sensing.

The control and user plane mechanisms in 5G(A), designed for legacy services, may not meet the massive data and model exchange demands of 6G new services. Current SRBs restrict RRC message size, while DRBs remain bound to UPF-based PDU sessions, restricting direct UE–gNB data transfer.

Observation 5: Existing control plane and user plane solutions in 5G(A) may be inadequate for transferring the large volumes of data and models that will be required in 6G.

  - Proposals for 6G

The traffic pattern and characteristics of new 6G services should be carefully studied before defining interface protocols and procedures. The study may start from XR (immersive communication) as the baseline, given its strong linkage to AI/ML and sensing and the availability of prior analysis on traffic behavior. Based on XR findings, it can be extended to AI/ML, sensing, and other new services following RAN plenary decisions and/or by coordinating with SA1 to better understand the data transfer requirements, ensuring protocol and procedure design reflects real traffic features.

Proposal 3: The traffic pattern and/or characteristics of new identified services should be studied thoroughly before defining the necessary interface protocol and procedure

- The new services may start from XR as the baseline/starting point, and include AI/ML, Sensing, and other new services according to RANP finding.

- RAN2 should initiate a study and/or coordinate with SA1 to better understand the data transfer requirements of emerging use cases.

A unified data transfer framework for difference services offers clear benefits in simplifying architecture and improving efficiency across diverse 6G services. However, the study should also take into account the existing legacy services, which already rely on MDT- and positioning-based mechanisms for data reporting and management. These legacy solutions are well established and widely deployed, providing a proven foundation in terms of triggering logic, measurement control, and data handling. Therefore, MDT and positioning can serve as the practical baseline for new services like AI/ML and Sensing, while the evolution toward a unified framework could be further investigated by considering the traffic pattern and/or characteristics of new identified services, such as a new data plane which enables new service data to be transmitted through dedicated bearers, minimizing interference with legacy traffic and improving efficiency.

Proposal 4: RAN2 should study the unified data transfer framework for both the legacy services and new identified services.

- The MDT-based and/or positioning-based solution should be assumed as the baseline/starting point.

- If data transfer requirements allow, to support a new data-plane for data transfer for new services (e.g. AI/ML, sensing, etc) to make data transfer more efficiently.

As data transfer becomes central to AI/ML and sensing services, 6G should ensure operator-controlled data management with user consent and visibility. Operators must be able to initiate, monitor, and terminate data transfers while guaranteeing user awareness and control over how data is collected and used. To achieve this, mechanisms such as mutual authentication, data encryption, and policy-based access control should be studied to protect data integrity and confidentiality. This approach will enable secure and transparent AI/ML operations, ensure regulatory compliance, and strengthen user trust in AI/ML- and sensing-based 6G services.

Proposal 5: RAN2 should study the operator-controlled data management (i.e. initiate, monitor, and termination) with UE consent and visibility.

- Inter-WG issues

In 5G(A), RAN3 and SA working groups were involved in discussions on data visibility, controllability, and security. However, the resulting solutions were compromised and fragmented, such as the MAC CE-based NCC delivery for inter-CU LTM and UE data collection for AI/ML.

In 6G, these challenges will persist and become even more critical. The volume, diversity, and sensitivity of exchanged data will continue to grow, not only for new services but also for broader network optimization, automation, such as further mobility enhancement. This evolution requires standardized, end-to-end (E2E) mechanisms to guarantee visibility, controllability, security, and efficiency of data transmission. Achieving this will demand tight coordination among RAN and SA working groups to ensure architectural consistency and security assurance.

Proposal 6: RAN2 shall coordinate with RAN3 and SA WGs on following issues:

- SA1: The data transfer requirements of emerging use cases like sensing.

- RAN3, SA2, SA5: The QoS requirements for new services and E2E solution for data transfer and/or data collection, new plane and/or RAN-CN interface.

- RAN3, SA3, SA5: The security and UE consent of model and data transfer and/or data collection solutions, e.g. MAC CE encryption and integrity protection.

- Conclusion

In this contribution, we have the following observations and proposals,

AI/ML framework

Observation 1: Fragmentation of AI/ML framework in 5G(A).

Observation 2: Lack of comprehensive MNO visibility and controllability of UE data collection.

Observation 3: Unresolved issues of user consent for network-driven data collection.

Proposal 1: RAN2 should study the unified framework for life cycle management for AI/ML operation, and the LCM framework specified in 5G(A) Rel-19 is used as the baseline/staring point; the use cases and necessity of model-based LCM should be verified before studying the feasibility.

Proposal 2: RAN2 study on AI/ML for 6GR should consider following key principles:

- MNO should have the visibility of the collected data.

- Efficient AI/ML data transfer mechanism should be supported.

- Unified data collection framework for both UE side model and network side model should be supported.

- AI/ML Performance monitoring should be supported, and fallback to legacy NW operation mechanism should be ensured.

- Customer Privacy (e.g. user consent) should be considered for AI/ML data collection.

Data transfer

Observation 4: Potential requirements for new services such as AI and Sensing.

Observation 5: Inadequacy of existing control and user plane solutions for large-volume AI/ML data and model transfer.

Proposal 3: RAN2 should coordinate with SA1 to clarify their data transfer requirements and study the traffic patterns and characteristics of new services (starting from XR and extending to AI/ML, sensing, etc.) before defining related interface protocols and procedures.

Proposal 4: RAN2 should study a unified data transfer framework covering both legacy and new services, using MDT/positioning-based mechanisms as the baseline/starting point, and consider introducing a new data plane (if data transfer requirements allow) to efficiently support emerging services such as AI/ML and sensing.

Proposal 5: RAN2 should study the operator-controlled data management (i.e. initiate, monitor, and termination) with UE consent and visibility.

Inter-WG issues

Proposal 6: RAN2 shall coordinate with RAN3 and SA WGs on following issues:

- SA1: The data transfer requirements of emerging use cases like sensing.

- RAN3, SA2, SA5: The QoS requirements for new services and E2E solution for data transfer and/or data collection, new plane and/or RAN-CN interface.

- RAN3, SA3, SA5: The security and UE consent of model and data transfer and/or data collection solutions, e.g. MAC CE encryption and integrity protection.

References

- NTT DOCOMO, CMCC, AT&T, Vodafone, RP-252912, “Revised SID: Study on 6G Radio,” Sep. 2025.

- CMCC (moderator), RP-252960, “pCR for 38.914 on Requirements of new and existing services for 6G” Sep.2025.