3GPP TSG-RAN2 Meeting #131bis	 R2-2507084

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.7.3

Source: 	Xiaomi Communications

Title:  	[RLC-X01] Discussing on issues on DSR and proposed TP

Document for:	Discussion and Decision

Introduction

In this contribution, we provide some views on an open issue of RLC-X01 on Rapporteur’s List of RLC open issues for R19 XR [1].

Discussion

In last meeting, we have discussed the issue on whether to remove the non-delay reporting RLC SDU from RLC specification. And proponents think the that if dsr-ReportNonDelayCriticalData is configured, PDCP indicates non-delay reporting PDCP PDUs as well as the delay-reporting RLC SDUs to RLC. Hence, all the description related to non-delay reporting RLC SDU from RLC specification can be removed as shown below [2].

[RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

And this has been captured in the draft TS 38.323:

The transmitting PDCP entity provides a DSR data indication associated with the i:th DSR-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU (if dsr-ReportNonDelayCriticalData is configured) associated with the i:th DSR-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU (if dsr-ReportNonDelayCriticalData is configured) associated with the i:th DSR-ReportingThreshold.

For the data pending for initial transmission in RLC, since PDCP will indicate for a PDCP Data PDU submitted to RLC whether the corresponding PDCP SDU becomes a delay-reporting or non-delay-reporting of i:th DSR-ReportingThreshold, RLC is going to calculate delay reporting data volume of i:th DSR-ReportingThreshold based on PDCP’s indication. An example is shown below:

For i=1 Portion:

- Data volume calculated in PDCP:

SDU4 and SDU7 (delay reporting),

SDU5 and SDU6 (none delay reporting buffered in front of SDU7);

- Data volume calculated in RLC:

SDU2 (delay reporting as indicated by PDCP),

SDU1(none delay reporting buffered in front of SDU1 as indicated by PDCP),

SDU3(none delay reporting buffered in front of SDU4 as indicated by PDCP);

Please note that PDCP decides the non-delay-reporting PDCP SDU of the i:th DSR-ReportingThreshold based on its own buffer status as shown above.

For RLC, it will get the indication of delay-reporting PDCP SDU of the i:th DSR-ReportingThreshold from PDCP (SDU2, SDU4, SDU7) for the portion of i:th DSR-ReportingThreshold as well as for the non-delay-reporting PDCP SDU (SDU1,SDU3,SDU5,SDU6) which reflects the PDCP buffer. However, we are not sure in RLC whether only the packets that has been indicated by PDCP are put in front of the delay-reporting ones. Is that possible that RLC put some other packets which are not known in PDCP before the data of the portion of i:th DSR-ReportingThreshold?

Currently, we are not sure that SDU handling in RLC layer follows the same buffer status as in PDCP layer. If not, then the calculation of non-delay reporting data should follow RLC’s own buffer status while the delay reporting data can follow PDCP’s indication as its calculation is not related to the buffer status.

Hence, we suggest RAN2 to confirm whether the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status or just follow PDCP’s indication. If the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status, then PDCP only needs to indicate the delay reporting data to RLC. Thus, RLC will calculate the non-delay reporting RLC SDU based on its own buffer status.

Hence, we propose:

Proposal 1.  	RAN2 to confirm whether the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status or just follow PDCP’s indication.

Proposal 2.  If the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status, then PDCP only needs to indicate the delay reporting data to RLC. Thus, RLC will calculate the non-delay reporting RLC SDU based on its own buffer status.

For PDCP, a TP is attached as shown below. And for RLC, we need to roll back to the previous version.

Conclusions

Based on the discussion, our proposals are provided as follows:

Proposal 1.  	RAN2 to confirm whether the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status or just follow PDCP’s indication.

Proposal 2.  If the calculation of non-delay reporting data in RLC should follow RLC’s own buffer status, then PDCP only needs to indicate the delay reporting data to RLC. Thus, RLC will calculate the non-delay reporting RLC SDU based on its own buffer status.

References

[POST131][507][XR] Final 38.322 CR (vivo) / RLC open issue for XR

RAN2#131 meeting minutes

Annex for TP for DSR on TS 38.323:

Start of changes

## 5.15	Data volume calculation for delay status reporting

The transmitting PDCP entity provides a DSR data indication associated with the i:th DSR-ReportingThreshold for the PDCP Data PDU to lower layers when:

-	the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold; or

-	the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold.

End of changes