3GPP TSG-RAN WG2 Meeting #131bis	R2-2507095

Prague, Czech, October 13th – 17th, 2025

Agenda Item:	10.4

Source: 	OPPO

Title:                     Initial considerations on 6G Mobility

Document for:	Discussion and Decision

1	Introduction

In RAN#108 meeting, 6G SID [1] has been approved and the mobility related objectives are listed as following:

The detailed objectives of the study are: (4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] (7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. c)Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

In this contribution, we would like to present our initial considerations on 6G mobility enhancement.

2	Discussion

2.1	Intra-RAT: Scenarios

Mobility Management is one of the core functions of mobile communication system, aiming to ensure that user equipment (UE) can maintain seamless and continuous communication services when moving within the network coverage area.

In 5G, the support of NTN was introduced in Rel-17, and further enhanced in later releases. In 5G NTN, three types of cell coverage are supported, those are 1) Earth-fixed, 2) Quasi-earth-fixed and 3) Earth-moving. gNB operating with MEO or LEO satellite can provide either Quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while gNB operating with GEO satellite can provide Earth-fixed cell coverage.

For intra-RAT mobility, both NTN-NTN mobility and NTN-TN mobility can be supported, with some NTN specific characteristic considered, such as the high altitude of satellite, the high motion of satellite, and much larger coverage for NTN compared with TN. The NTN specific feature for intra-RAT mobility is summarized in the following table.

Table 1 NTN-Specific part for Mobility Procedure

| 5G Status | NTN specific part for RRC_IDLE mobility | NTN specific part for RRC_CONNECTED mobility |
| --- | --- | --- |
| Rel-17 | For (Quasi-)Earth-fixed cell, support time based and location-based measurement initiation | For (Quasi-)Earth-fixed cell, support location-based measurement event, and time based and location-based CHO triggering event |
| Rel-18 | For Earth-moving cell, support location-based measurement initiation NR NTN->NR TN mobility: TN coverage can be provided by NW to assist UE skipping TN measurements. NR TN-> NR NTN mobility: support a TN cell to provide satellite assistance information for NTN neighbour cells | For Earth-moving cell, support location-based measurement event and introduce location-based CHO triggering event For Quasi-Earth fixed cell, support satellite switch with-resync For (Quasi-)Earth-fixed cell and Earth- moving cell, support RACH-less handover |

In 6G NTN, all the scenarios including Earth-fixed cell, Quasi-Earth-fixed cell and Earth-moving cell should be supported, and TN/NTN interworking needs to be considered for ubiquitous coverage and service continuity.

According to 6G SID, 6G aims at a harmonized design for TN and NTN. So, it is recommended that intra-RAT mobility in 6G could be applicable to both TN and NTN. For NTN-NTN mobility and NTN-TN mobility, 5G features e.g., time based and location-based mobility, can be taken as baseline, and further enhancements can be studied.

- In 5G intra-RAT mobility, both NTN-NTN mobility and NTN-TN mobility are supported, with NTN specific characteristic being introduced, such as the high altitude of satellite, the high motion of satellite, and larger coverage for NTN compared with TN.

- For 6G Intra-RAT mobility, RAN2 study starts from TN/NTN-common issues and takes into account of the NTN-specific issues.

2.2	Intra-RAT: Procedures (RRC_CONNECTED)

In 5G, the mobility procedure is categorized into two types of mobility: cell level mobility and beam level mobility.

- Cell Level Mobility: which is mainly L2/L3 oriented, i.e., cell switch which triggered by RRC signaling, MAC CE or executed by UE autonomously (relying on pre-configuration from network).

- Beam Level Mobility: which does not require explicit RRC signaling to be triggered, but rely on L1 signaling instead. Beam level mobility can be within a cell, or between cells, the latter is referred to as inter-cell beam management (ICBM).

Figure 1 Mobility procedure

- 5G mobility in RRC_CONNECTED state is categorized into two types of mobility: L2/L3 oriented via cell level mobility and L1-oriented via beam level mobility.

In the evolution of 5G communication system, cell level mobility mechanisms have been continuously improved, with various innovative mechanisms introduced from Rel-15 to Rel-19. These include traditional Handover (HO), Dual Active Protocol Stack (DAPS), Conditional Handover (CHO), L1/L2 Triggered Mobility (LTM), and Conditional Triggered LTM (CLTM), etc. And a serious of PSCell change/addition procedures have been also investigated for multi-connectivity scenario, e.g., PSCell Change, Conditional PSCell Change (CPC), Conditional PSCell addition (CPA), Subsequent Conditional PSCell Addition or Change (subsequent CPAC) and SCG LTM, etc.

The design and implementation of these handover mechanisms across different protocol versions have led to a certain degree of repetition and redundancy in configurations and processes, while increasing the complexity of system implementation.

Table 2 Mobility Procedure Design in 5G

| 5G Status | Feature | Details |
| --- | --- | --- |
| Rel-15 | L3 Handover | Handover procedure triggered by RRC signaling. |
|  | Network triggered PSCell Change | PSCell change procedure triggered by RRC signaling. |
| Rel-16 | DAPS | A handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. |
|  | CHO | A handover that is executed by the UE when one or more handover execution conditions are met. |
|  | CPC | A PSCell change procedure that is executed only when PSCell change execution condition is met. |
| Rel-17 | CPAC | PSCell addition or change procedure that is executed only when PSCell addition or change execution condition is met. |
|  | CHO with SN | for configuration and execution of CHO with SN which includes the cases where the SN is kept, changed or added. |
| Rel-18 | Intra-gNB LTM | gNB receives L1 or L3 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signaled via a MAC CE. |
|  | SCPAC | A conditional PSCell addition or change procedure that is executed based on pre-configured subsequent CPAC configuration which is not implicitly released by the UE upon (conditional or non-conditional) PSCell addition, (conditional or non-conditional) PSCell change, (conditional or non-conditional) PCell change or SCG release. |
|  | CHO with candidate SCG | A PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. |
|  | RACH-less HO | Handover procedure allowing UE to transmit first UL message using configured or dynamic signaled UL grant with available timing advance instead of triggering RACH procedure. |
| Rel-19 | Inter-gNB LTM | Support inter-gNB mobility by using LTM mechanism. |
|  | CLTM | LTM cell switch which executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met. |
| Rel-20 | LTM with SCell | Support NW triggered LTM SCell activation with LTM cell switch. |

- In each release of NR, new mobility features were introduced to improve mobility performance and extend the applicable scenarios.

Since the features are introduced in difference releases of NR:

- Separate IEs are used for configuring the candidate/target cells. For example, the conditionalReconfiguration is used for providing the candidate configuration as well as execution conditions for CHO/CPAC/SCPAC, while ltm-Config is used for LTM candidate configuration, among others.

- Separate procedure texts are specified for each handover procedures, including L3 HO, CHO and LTM.

For each row in the table, the yellow-highlighted parts are defined for similar purposes or share significant commonalities according to 5G specifications.

Table 3 Redundancy / Duplication between 5G Mobility Schemes

|  | L3 HO DAPS NW-triggered PSCell addition or change | CHO CHO with SN CHO with candidate SCG CPA, CPC | SCPAC | (C-)LTM |
| --- | --- | --- | --- | --- |
| Target/Candidate Cell Configuration | RRCReconfigurationWith Sync | ConditionalReconfiguration -Candidate cell ID -Candidate cell configuration | ConditionalReconfiguration -Candidate cell ID -Candidate cell configuration -Reference configuration -Complete config indication -sk counter configuration for inter-SN SCPAC -Indication for inter/intra-SN determination for SCPAC | LTM-Config/LTM-ConfigNRDC -Candidate cell ID -Candidate cell configuration -Reference configuration -Complete config indication -sk counter configuration for inter-CU SCG LTM -Indication for inter/intra-CU/DU determination. |
| L3 Meas/Report Configuration | L3 Meas/Report | L3 Meas/Report | L3 Meas/Report | L3 Meas/Report |
| L1 Meas/Report Configuration | N.A. | N.A. | N.A. | L1 Meas/Report |
| Execution condition configuration | N.A. | -Execution conditions | -Execution conditions -Execution conditions for subsequent CPAC | -Execution conditions for CLTM -Execution conditions for subsequent CLTM |
| Other Configuration | N.A. | N.A. | N.A. | -TCI state info -Early UL sync config -Indication for UE-based TA measurement -CSI report config for early CSI acquisition |
| Evaluation | N.A. | Conditional reconfiguration evaluation based on L3 measurement | Conditional reconfiguration evaluation based on L3 measurement | LTM cell switch evaluation based on L1/L3 measurement for CLTM |
| Execution | >Apply RRCReconfiguration | > Apply RRCReconfiguration | SCPAC Execution: > Inter/intra-SN determination > L2 reset handling > Complete config generation > Apply RRCReconfiguration | LTM/CLTM Execution: > Inter/intra-CU/DU determination > L2 reset handling > Complete config generation > Apply RRCReconfiguration |

- Independent procedures are specified for each mobility feature, which bring redundant signalling and procedure text overheads.

To address the above issue, 6G may consider designing a unified mobility management mechanism that integrates the current handover procedures, e.g., (C)LTM and L3 (C)HO, thereby forming a unified mobility management framework for RRC_CONNECTED. This unified mobility management mechanism aims to simplify handover processes, reduce signaling overhead, and further improve handover efficiency through universal configurations and operational steps.

With various mobility features, it is important to identify the most appropriate one for each corresponding scenario. For instance,

- LTM procedure enables the UE to perform an early uplink/downlink synchronization procedure with candidate cell(s), thereby reducing cell switch interruptions. The TCI (Transmission Configuration Indicator) states of the LTM candidate cells can be pre-activated before any of them becomes the serving cell, which facilitates a faster handover to one of these cells when the cell switch is triggered. This makes LTM more suitable for handling typical beam-based coverage scenarios.

- However, compared to traditional L3 handover, the LTM procedure requires the network to prepare more candidate cells in order to facilitate early candidate configuration and subsequent cell switch execution, which results in higher resource reservation. Therefore, L3 HO can effectively mitigate this consumption, particularly in low-frequency and low-mobility scenarios.

I.e., it seems both LTM-based and L3-based HO has its advantage and thus applicable scenario.

- LTM procedure is more proper to handle typically beam-based-coverage scenario at higher frequency scenario, while L3 procedure has the advantage of lower resource reservation and more appropriate for lower frequency scenario.

In RRC_CONNECTED, both layer 1 (L1) and layer 3 (L3) based measurements are supported, L1 measurement reporting is evaluated based on beam quality and L3 measurement reporting is evaluated based on cell quality.

For L3 based measurement, the UE measures multiple beams (at least one) of a cell and the measurements results (power values) are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB.

For L1 based measurement, the UE measures the beams configured by NW. Filtering takes place at the physical layer to derive the beam quality. The measurement reports contain the measurement results of the X (at least one) beams and reporting is with MAC CE or UCI.

The detailed info of L1 and L3 measurement are summarized as below:

Table 4 5G L1&L3 measurement

|  | L3 meas for Mob | L1 meas for LTM | L1 meas for MIMO |
| --- | --- | --- | --- |
| Measurement config | measConfig MeasObjectNR reportConfig | LTM-CSI-ResourceConfig LTM-CSI-ReportConfig | CSI-ResourceConfig CSI-ReportConfig |
| Measurement objective | Serving and neighbor cell | Serving and candidate cell | Serving cell and inter-cell TRPs |
| Report content | Cell quality and L3 beam quality | L1 Beam quality | L1 Beam quality |
| Report format | RRC signaling | MAC CE and UCI | UCI |

- Both L1 and L3 measurement can be used for the purpose of mobility management while separate schemes are specified in 5G, including configuration signalling as well as reporting formats.

To address such redundancy, 6G measurement can study a unified L1 and L3 measurement from signalling and reporting overhead reduction perspective. And as indicated in the SID [1], the RRM aspects are mainly led by RAN4 and RAN1, thus RAN2 can wait RAN4/RAN1 progress before starting the corresponding work.

d)	RRM aspects for 6GR [RAN4, RAN1, RAN2]

	RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

	Study how to improve 6G requirement specification, including structure and drafting principles

- For 6G Intra-RAT Mobility of RRC_CONNECTED, RAN2 study to 1) support unified mobility procedure by harmonizing the signalling and configuration, taking 5G mobility features, e.g., L3 (C)HO and (C)LTM, as starting point; 2) support unified L1/L3 measurement framework, and RAN2 waits RAN4/RAN1 progress first.

Energy efficiency and energy saving is one of the key performances as determined by the RAN requirements in [2]. Supporting mobility management while taking into account the reduction of power and resource consumption is also necessary. Let’s first re-screen the legacy 5G state transition as well as mobility support in each RRC state to understand the pros and cons.

Table 5 5G RRC State Handling Schemes

| Index | When there is no data ongoing | Upon data arrival | Consumption when no data ongoing | Latency upon data arrival | Mobility support when there is data on-going | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5G RRC_IDLE | 5G RRC_ CONNECTED | Low | Long | Y | Typical way |
| 2 | 5G RRC_ INACTIVE | 5G RRC_ CONNECTED | Low | Medium (due to UE context retrieve) | Y | Limited Deployment |
| 3 | 5G RRC_ CONNECTED | 5G RRC_ CONNECTED | Medium if tuning DRX properly (due to measurement and signaling for mobility) | Low | Y |  |
| 4 | 5G RRC_ INACTIVE | 5G RRC_ INACTIVE | Low | Medium (due to UE context retrieve) | N | Limited to small data packet |

Firstly, for Approach-3, the drawback is the power / resource consumption level when there is no data.

As indicated in In 5G, the long DRX cycle and RRM relaxation can help to reduce the UE power / resource consumption to a minimum level if there is no mobility. But when there is mobility, the measurement, report and the associated signaling is unavoidable, for 5G RRC_CONNECTED state.

I.e., If there is no mobility, the long DRX cycle and RRM relaxation can help to reduce the UE power / resource consumption to a minimum level. But when there is mobility, the measurement, report and the associated signaling is unavoidable, for 5G RRC_CONNECTED state.

Take LTM procedure as an example,

- Step-1/2/3/4b/5/6/7 includes signaling exchange as consumer for resource, and

- Measurement for step-1/5, sync for step-1/4a, are all consumer for UE power,

- Measurement for step-4b/7 is consumer for network power

- When there is no data is on-going, there could be a concern on keeping the UE in 5G RRC_CONNECTED state, considering the power/resource consumption due to mobility in 5G RRC_CONNECTED state.

Figure 2 5G LTM Procedure

On the contrary, approach-1/2 solve the concern above, but end up with medium/high level of latency upon data arrival.

Specifically, in approach-1, the steps from 1 to 8a are all necessary, covering jobs for NAS to prepare UE AS context, NAS and AS security activation, and SRB/DRB configuration, which is quite long latency.

While for approach-2, although the NAS/AS UE context preparation steps can be saved, but when the serving RAN node is not the anchor RAN node, the reactive UE context retrieve procedure introduce additional latency, which happens between step-1 and step-4.

- There could be a concern to release the UE to 5G RRC_IDLE or 5G RRC_INACTIVE when there is no data activity, considering the latency to resume UP activity when there is DL/UL data arrival.

Figure 3 5G RRC Setup Procedure

Figure 4 5G RRC Resume Procedure

Finally, for approach-4, performance-wise it is quite similar to approach-2, in the sense that UE context retrieval is still needed, although RRCResume message is saved. While the key problem here is the incapability to support mobility during data traffic period.

- 5G small-data-transmission (i.e., SDT) cannot be used as a main approach to handle eMBB traffic, due to incapability to support mobility during data traffic period.

Figure 5 5G SDT Procedure

Given the analysis above, the 6G mobility should aim at improvement on top of the 5G design, where approach-2 and approach-3 seem to be a good starting point.

Table 6 Comparison between Approach-2/3

| Index | When there is no data ongoing | Upon data arrival | Consumption when no data ongoing | Latency upon data arrival | Potential Improvement |
| --- | --- | --- | --- | --- | --- |
| 2 | 5G RRC_ INACTIVE | 5G RRC_ CONNECTED | Low | Medium (due to UE context retrieve) | Change reactive UE context retrieve to proactive UE context retrieve |
| 3 | 5G RRC_ CONNECTED | 5G RRC_ CONNECTED | Medium if tuning DRX properly (due to measurement and signaling for mobility) | Low | Change instant mobility control to conditional mobility control to save measurement |

Comparing the two-starting points:

If one starts from approach-2, to optimize the latency upon data arrival, the key should be to solve the latency due to RACH procedure and the re-active UE context retrieval. Which can be in various forms, which can include:

- Enhancement to reduce the RACH latency, e.g., to extend the applicability of RACH-less functionality;

- Enable proactive context relocation, as we did in RRC_CONNECTED. But in order to keep the advantage of low consumption when no data on-going, this proactive context relocation has to be pre-configured / conditionally-configured.

Where component-1 has less impact to the whole framework, while the latter has large change but bring large benefit (with the cost of more additional resource reservation).

Or if one starts from approach-3, to optimize the consumption when there is no data, the key should be to relax/remove the measurement and to simplify/remove the signaling for measurement report and mobility command. But then to implement that, both components above can be employed as well.

Then the two seem end up with a same result, essentially. Nevertheless, R2 should start the study on RRC modeling by defining the requirement, which outlines the key directions for solution(s) identification.

- For 6G mobility of RRC_CONNECTED, RAN2 to study to further reduce power/resource consumption to optimize when there is no data traffic on-going.

5G failure detection and recovery mechanism is implemented with two levels:

1）Beam failure detection/recovery (BFD/BFR): For beam failure detection, the gNB configures the UE with beam failure detection reference signals and the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expires. And the BFD/BFR is perform with the granularity of TRP. After beam failure is detected on an SCell or on a TRP, the UE triggers beam failure recovery by initiating a transmission of a BFR MAC CE for this SCell/TRP. Once BFR is triggered on both TRPs associated with SpCell, UE triggers RACH procedure for recovery.

2）Radio link monitoring (RLM): In RRC_CONNECTED, the UE performs radio link monitoring in the active BWP based on reference signals and signal quality thresholds configured by the network. The RLM is only performed in SpCell. Upon radio link failure (RLF) declaration, RRC connection re-establishment procedure is executed, where the UE firstly performs cell reselection and then initiates RRC re-establishment to the selected cell.

- 5G failure detection and recovery mechanism is implemented with two levels: beam level and cell level.

If carrier aggregation is configured,

- When PCell encounters RLF, UE will initiate RRC re-establishment procedure regardless of the radio condition of SCell, which may cause additional service interruption.

- When SpCell encounters beam failure, RACH procedure will be initiated even if SCell is alive, which also causes unnecessary RACH attempt.

- 5G failure detection and recovery mechanism mainly consider the radio condition of primary cell/carrier.

In NR Rel-19, RAN2 studied and evaluated potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, which mainly focus on AI/ML based RRM measurement and event prediction:

- With measurement predication, the measurement overhead in time, frequency and spatial domain can be reduced significantly;

- With event predication, the mobility performance KPIs can be improved, including Ping-pong handover, handover failure/radio link failure, time of stay and handover interruption, etc.

- AI/ML brings new opportunities for mobility/measurement performance improvement in 5G.

For 6G failure detection, RAN2 study to enhance the failure detection/recovery mechanism to improve robustness.

- For 6G mobility of RRC_CONNECTED, RAN2 study to enhance the failure detection/recovery mechanism to improve robustness.

2.3	Intra-RAT: Procedures (RRC_IDLE, RRC_INACTIVE)

Common signals can be categorized into two groups:

- Dynamic signals, which are influenced by UE density, including Paging and RACH

- Static signals, which are independent of UE density, including SSB and SIBs

The load associated with dynamic signals can vary significantly depending on factors such as UE activity and time of day.

In 5G, to manage dynamic load changes, networks typically deploy additional carriers as different cell(s), each accompanied by cell-specific overhead (e.g., SSB, SIB). However, this load distribution is performed statically, relying on fixed/static cell (re)selection parameters.

- From the network perspective, this requires deploying new cells;

- From the UE perspective, it involves executing cell reselection procedures based on updated parameters.

- This approach is resource-consuming and poorly suited to dynamic load scenarios.

- In 5G RRC_IDLE/RRC_INACTIVE, cell selection/reselection is used for UE to switch between frequencies for cell camping.

Besides, based on the requirement from 6G SID, the 6G design needs to target diverse device types with diverse bandwidth capability and take into account diverse spectrum allocations:

2. Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] 9. Target scalable and forward compatible design for diverse device types. 10. Improved spectrum utilization and operations taking into account diverse spectrum allocations.

To meet these demands, a unified yet scalable initial access framework is essential, which ensures compatibility across diverse UE types having different bandwidth capabilities. Additionally, to support the anticipated increase in device volume, enhanced initial access capacity is needed. It requires flexible load distribution across multiple frequency layers (e.g., coverage and capacity layers), which expands available resources beyond a single frequency layer and enables more efficient handling of dynamic traffic loads.

- In 6G, the legacy spectrum aggregation scheme can be further enhanced to allow more flexible (than cell (re)selection) load distribution.

- For 6G mobility of RRC_IDLE, RAN2 study mechanism for UE to flexibly switching between frequencies, in order for dynamic load (e.g., RACH and Paging) re-distribution. FFS for RRC_INACTIVE.

2.4	Inter-RAT: Scenarios

According to 6G SID [1], one of the objectives is to study the migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR.

(1) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: a) 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] b) Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. c) Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Correspondingly, RAN2 should focus on the study of inter-RAT mobility between 6G and 5G, while deferring the discussion on inter-RAT mobility between 6G and 4G to a later phase.

- According to 6G SID, mobility between 5G and 6G should be studied and inclusion of 4G/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary.

In 5G, the following scenarios are supported for inter-RAT mobility between 5G and 4G as specified in TS38.300 and TS 36.300.

- Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE.

- Cell reselection from NR RRC_INACTIVE to E-UTRA RRC_IDLE and from E-UTRA RRC_INACTIVE to NR RRC_IDLE.

- Handover between NR RRC_CONNECTED and E-UTRA RRC_CONNECTED.

- In 5G, the following scenarios are supported for inter-RAT mobility between NR and E-UTRA: 1) Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE, 2) Cell reselection from NR RRC_INACTIVE/E-UTRA RRC_INACTIVE to E-UTRA RRC_IDLE/NR RRC_IDLE, and 3) Handover between NR and E-UTRA.

The RRC_INACTIVE state was introduced in 5G primarily to reduce control plane latency; however, its commercial deployment has been limited. As outlined in our contribution [3], it is proposed that RAN2 should consider whether the RRC_INACTIVE state is retained in 6G. Consequently, inter-RAT mobility in RRC_CONNECTED state and RRC_IDLE state should be supported in 6G, while RAN2 further study whether to support inter-RAT mobility in the RRC_INACTIVE state. For 5G inter-RAT mobility, mobility between 4G TN and 5G NTN in RRC_IDLE are supported, with some NTN specific characteristic considered as in the table below. Inter-RAT mobility between 4G TN and 5G NTN in RRC_CONNECTED will be studied in R20 NTN WID.

Table 7 NTN Specific Enh for Inter-4/5G TN/NTN Mobility

| Scenario | NTN specific enhancement |
| --- | --- |
| 4G TN => 5G NTN (RRC_IDLE) | Support an LTE TN cell to provide satellite assistance information for NR NTN neighbor cells |
| 5G NTN => 4G TN (RRC_IDLE) | Support a 5G NTN cell to provide TN coverage for 4G TN cells to assist UE skipping TN measurements. |

- Inter-RAT mobility between 4G TN and 5G NTN in RRC_IDLE are supported in 5G, with NTN specific enhancement being introduced, such as the high altitude of satellite and larger coverage for NTN compared with TN.

According to the SID, 6G aims to achieve a harmonized radio design for TN and NTN. For inter-RAT mobility, it is recommended to begin with the study of TN-related scenarios while taking into account its applicability to NTN-related scenarios.

- For 6G-related inter-RAT mobility, RAN2 study to support inter-RAT mobility between 6G and 5G for RRC_CONNECTED and for RRC_IDLE, starting from TN/NTN-common issues, and taking into account of NTN-specific issue(s). FFS on 6G to 4G mobility. FFS for RRC_INACTIVE.

2.5	Inter-RAT: Procedures

Procedure-wise, inter-RAT mobility in 5G is performed as follows:

For UEs in 5G RRC_CONNECTED, inter-RAT mobility is performed through L3-based handover. The source RAT configures Inter-RAT measurement and reporting for the target RAT, based on which it determines whether to initiate a handover to the target RAT. The UE then performs the handover according to the RRC Reconfiguration message sent by the source RAT, which is prepared by the target RAT. There are two approaches for control plane and user plane handling during this procedure:

Table 8 Inter-RAT Mobility in 5G

|  | Intra-system inter-RAT mobility (i.e., intra-5GC) | Inter-system inter-RAT mobility (i.e., between 5GC and EPC) NOTE: if N26 is unavailable, re-direction is needed |
| --- | --- | --- |
| Control Plane handling | Delta configuration for radio bearer configuration is used to keep SDAP and PDCP configuration | Full configuration is required to reset the radio bearers. |
| User plane handling | In-sequence and lossless handover is supported. | Seamless session continuity (e.g., for voice services) can be supported. |

For UEs in 5G RRC_IDLE or 5G RRC_INACTIVE, inter-RAT mobility relies on the cell reselection procedure. The UE evaluates neighboring cells based on criteria such as signal strength and system information, and reselects to a target RAT without NW intervention.

- In 5G, inter-RAT mobility in RRC_CONNECTED relies on L3 handover procedure, inter-RAT mobility in RRC_IDLE relies on cell reselection procedure.

- For intra-system inter-RAT mobility in RRC_CONNECTED, in-sequence and lossless handover is supported by keeping the SDAP and PDCP configurations. For inter-system inter-RAT mobility, handover or redirection is performed based on radio criteria and availability of the N26 interface.

LTM is introduced in 5G to further reduce mobility latency. In this procedure, the UE can be configured with a set of candidate cell configurations for cell switch triggered by L1/L2 signaling or upon fulfilling the evaluation conditions. In LTM, L1 measurement is used for faster cell switch reaction and methods such as early UL/DL sync to the candidate cell(s) are supported for latency reduction. Compared to L3 handover, tighter interworking between the source and target cells is required to achieve the benefits of LTM as detailed in the table below, which may put a higher requirement for inter-RAT scenario.

Table 9 Comparison between Intra-RAT L3 HO and LTM in 5G

| Phases | LTM (e.g., inter-gNB LTM) | L3 handover |
| --- | --- | --- |
| Preparation | S-gNB to C-gNB(s): Request for handover preparation | S-gNB to C-gNB(s): Request for handover preparation |
|  | C-gNB(s) to S-gNB: Candidate cell configuration, CSI resource configuration and early sync information. S-gNB to C-gNB(s): Common CSI resource configuration and LTM configuration ID mapping list. C-gNB(s) to S-gNB: Updated candidate cell configuration. | C-gNB to S-gNB: Target cell configuration |
| Early UL sync | C-gNB to S-gNB: TA value, CFRA resource information, candidate cell ID | N.A. |
| Execution | S-gNB to T-gNB: Target cell ID, TCI state ID. | N.A. |
| Completion | T-gNB to S-gNB: Handover success message. S-gNB to T-gNB: SN status transfer. T-gNB to S-gNB: UE context release. | T-gNB to S-gNB: Handover success message. S-gNB to T-gNB: SN status transfer. T-gNB to S-gNB: UE context release. |
|  | T-gNB to C-gNB(s): LTM configuration update message, e.g., including new security key(s) to be used with the UE. |  |

- Compared to L3 handover, LTM-like procedure requires tight interworking between source cell and target cell which may put a higher requirement for inter-RAT scenario.

The 5G inter-RAT mobility procedure can serve as a baseline for 6G. Further discussion is needed on whether to extend the benefits of LTM to inter-RAT scenarios.

- For 6G-related inter-RAT mobility, RAN2 study the 1) For inter-RAT mobility from/to 6G RRC_CONNECTED, support L3-based procedure as baseline. 2) For inter-RAT mobility from/to 6G RRC_IDLE, 5G cell (re)selection procedure is supported as starting point. FFS on RRC_INACTIVE.

3	Conclusion

Based on the discussion in section2, we have following proposals:

Proposal 1	For 6G Intra-RAT mobility, RAN2 study starts from TN/NTN-common issues and takes into account of the NTN-specific issues.

Proposal 2	For 6G Intra-RAT Mobility of RRC_CONNECTED, RAN2 study to 1) support unified mobility procedure by harmonizing the signalling and configuration, taking 5G mobility features, e.g., L3 (C)HO and (C)LTM, as starting point; 2) support unified L1/L3 measurement framework, and RAN2 waits RAN4/RAN1 progress first.

Proposal 3	For 6G mobility of RRC_CONNECTED, RAN2 to study to further reduce power/resource consumption to optimize when there is no data traffic on-going.

Proposal 4	For 6G mobility of RRC_CONNECTED, RAN2 study to enhance the failure detection/recovery mechanism to improve robustness.

Proposal 5	For 6G mobility of RRC_IDLE, RAN2 study mechanism for UE to flexibly switching between frequencies, in order for dynamic load (e.g., RACH and Paging) re-distribution. FFS for RRC_INACTIVE.

Proposal 6	For 6G-related inter-RAT mobility, RAN2 study to support inter-RAT mobility between 6G and 5G for RRC_CONNECTED and for RRC_IDLE, starting from TN/NTN-common issues, and taking into account of NTN-specific issue(s). FFS on 6G to 4G mobility. FFS for RRC_INACTIVE.

Proposal 7	For 6G-related inter-RAT mobility, RAN2 study the 1) For inter-RAT mobility from/to 6G RRC_CONNECTED, support L3-based procedure as baseline. 2) For inter-RAT mobility from/to 6G RRC_IDLE, 5G cell (re)selection procedure is supported as starting point. FFS on RRC_INACTIVE.

4	Reference

[1] RP-251881   New SID: Study on 6G Radio

[2] RP-250810   RAN requirements for 6G

[3] R2-2506846  Discussion on 6GR control plane