3GPP TSG-RAN WG2 Meeting #131bis	R2-2507103

# Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.13.2

Source:	Apple

Title:	ASN.1 issues for SI/Paging forwarding (A500/O505/X501/K002/H451)

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In the last RAN2 meeting (RAN2#131 [1]), the forwarding of SI & Paging has been discussed and there are following conclusions:

- Management of paging monitoring information at the relay UEs can be further checked in maintenance; no update to the running CR in this respect now. - Similar to SIB request, a paging information list is introduced for the intermediate relay UE to carry the paging information from the downstream remote UEs. Signalling details can be checked in the ASN.1 review as usual.

In this paper, we continue to discuss Paging forwarding issue unresolved as depicted above, and some ASN.1 issues raised in ASN.1 review comments collection document for SL Relay [2].

# 2 Discussion

### 2.1 Paging forwarding for RRC_CONNECTED Intermediate Relay UE

For an intermediate UE, there are some proposal to use CONNECTED intermediate UE to intercept paging requests for its children UE, as similar to SIB forwarding case But we think SIB forwarding and paging forwarding for intermediate relay UE are different in the following aspects:

- The SIB information is usually valid for a much longer time than the transient paging information. That means the SIB information is potentially useful for the intermediate relay UE even when intermediate relay UE does not request it right now. It will become handy if the intermediate relay UE later decides to request a SIB and finds that it has already have a stored copy of it;

- The paging information will result in RRC connection setup/resume procedure initiated by the remote UE. The paging forwarding in downstream and RRCSetupReq forwarding in upstream will happen sequentially in a very short period of time.  So there is no point to make those two paths different. If a CONNECTED intermediate relay UE decides to intercept paging for a child, there is a potential problem to ensure the path to the last relay UE is still viable and the upcoming RRC Setup procedure will succeed. This is because the all SI/Paging forward has been cut-short by this RRC_CONNECTED intermediate relay UE, and the condition of the upstream path segment from this UE to the last relay UE becomes unknown. Therefore, we think paging forwarding is better to be kept as same as connection setup path which is between the remote UE and last relay UE, so that the path can be properly probed via DL paging forwarding first;

- Moreover, the current SidelinkUEInformationNR only allows to report of paging requests of directly connected remote UEs. To support the dedicated delivery of indirect children, the ASN.1 Uu signaling for SUI message has to be updated, which results more specification impact;

- Finally, if we allow intermediate UE to intercept paging, but not to cancel the requests to the parent relay(s), then there will be duplicate paging messages received and some suppression mechanism has to be implemented by the intermediate relay UE, which adds unnecessary complexity.

Also, in current ASN.1 design for RemoteUEInformationSidelink PC5-RRC message, an intermediate relay UE separate its own paging request from its children’s paging request, those are carried by two different IE: sl-PagingIdentityRemoteUE and sl-PagingInfo-RemoteUE-List. From this perspective, the signaling to separate this paging forwarding behavior is handy.

Given the above considerations, we think it is more straightforward to forbit a RRC_CONNECTED intermediate relay UE to conduct paging interception for its children. Therefore, we propose:

Proposal 1 	Intermediate relay UE in RRC_CONENCTED state is not allowed to monitor paging on behalf of its child UE. Only the last relay UE is responsible for paging monitoring.

Note that the current RRC CR [3] does not contain any behaviour of RRC_CONNECTED intermediate relay UE relays a paging request (e.g., in RemoteUEInformationSidelink). It does not have any texts to let this UE to cancel prior requests sent in RemoteUEInformationSidelink, either. So, either way, this procedure is incomplete. We understand that the intermediate relay UE may enter RRC_CONNECTED for sending its own UL traffic, and the connected cell may not support SL relay at all, so at least in this case, if it receives a paging request from its child UE, it shall forward it. This kind of text is currently missing in the RRC CR [3]. The suggested change is to merge the IDLE/INACTVE UE behaviour with CONNECTED together for paging forwarding as there is no need to single out IDLE/INACTVE case.

Proposal 2 	Specify RemoteUEInformationSidelink transmission behaviour for RRC_CONNECTED intermediate relay UE in TS 38.331.

### 2.2 Paging/SI forwarding procedure for multi-hop relay

Note that in current RRC CR [3], the trigger of RemoteUEInformationSidelink procedures begins with the following text in 5.8.9.8.2

##### 5.8.9.8.2 Actions related to transmission of RemoteUEInformationSidelink message When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE or L2 Intermediate U2N Relay UE shall: <text omitted>

Compared to the legacy Rel-`17 spec, the only change is to add “L2 Intermediate U2N Relay UE”. There are several problems with such a casual change. which has been raised, as partially and briefly explained in [RIL:A500][RIL:X501]. Here, we elaborate the observed problems:

- It mixes the single-hop case and multi-hop case together in the same procedure. However, this is not suitable for multi-hop case in a even very high-level perspective. As a result, it will gives an wrong impression that all the text below in 5.8.9.8,.2 are same for single-hop and multi-hop case. However, they are different in several aspects:

- PosSIB forwarding is not supported yet for multi-hop relay case

- SFN-DFN offset forwarding is for single-hop only and for multi-hop case, it was discussed in RAN2#130 meeting, but the conclusion is that “For this WI, we do not specify forwarding of the SFN-DFN offset.  It can be addressed under TEI with the input of positioning experts. “. Therefore, we can assume that this SFN-DFN offset procedure is only applicable to single-hop case now.

- The “Entering RRC_IDLE or RRC_INACTIVE” condition is not applicable to Intermediate Relay UE. Because Intermediate relay UE entering RRC_IDLE/INACTIVE does not affect its capacity for “multi-hop” forwarding case, it only affects its behavior when acting as a “L2 U2N remote UE”, e.g, cancelling its own paging request. So, this triggering condition is also a single-hop only condition

- Multi-hop relay case has unique triggers for sending SIB/Paging requests upstream

- Triggered by child (direct or indirect children)’s request. We think this trigger should be highlighted as an distinctive triggering condition in 5.8.9.8.2 because it is related for a relay UE to re-generate a new RemoteUEInformationSidelink message, which is not covered by the 5.8.9.8.3 “Reception of RemoteUEInformationSidelink message by the L2 U2N /U2U Relay UE“.

- Triggered by the loss of one or more child(ren). This is also a unique case for multi-hop relay as the downstream link to a child may fail due to RLF or PC5 link release, there is no point to continue the SI/Paging forwarding for all the requests from this child and its children, this should be clearly specified. RRC rapporteur has proposed to use a very ambiguous NOTE in 5.8.9.8.3 [RIL:H451] to cover this, but we think this shall be captured in normative text in the right place. And this part should also be properly hooked to RLF procedure as suggested in [RIL:K002] and changes in 5.8.9.8.3 to be added to trigger the SI/Paging forwarding behaviour for intermediate relay UE.

Proposal 3 	Single-hop and Multi-hop SI/Paging forwading are to be separately specified in clarity in 5.8.9.8.2  for the transmission of RemoteUEInformationSidelink.

Proposal 4 	The triggering of the transmission of RemoteUEInformationSidelink is also described in respective subclauses when SL RLF occurs or when child UE’s request is received.

Given the above P1/P2/P3/P4, we provided a TP to be adopted in RRC specification in Annex.

Proposal 5 	RAN2 adopts the TP in Annex.

### 2.3 Sending Multiple paging record in one PC5-RRC message

Given that RAN2#130 [1] has already agreed the following:

Similar to SIB request, a paging information list is introduced for the intermediate relay UE to carry the paging information from the downstream remote UEs.  Signalling details can be checked in the ASN.1 review as usual.

It is not unreasonable to adopt a similar change in downstream direction, so a remote UE does not waste multiple PC5-RRC messages to send paging records separately towards the same child, as proposed in [RIL:O505]. This could help to reduce the RRC signalling overhead, especially for a last relay UE which may have quite a lot of potential children. Therefore, we think having multiple paging information in UuMessageTransferSidelink should be supported in Rel-19.

Proposal 6	In UuMessageTransferSidelnik, support a list of paging messages to be contained in ASN.1.

# 3 Conclusion

In this contribution, we discuss the remaining ASN.1 issues related to SI and Paging forwarding, and have the following proposals:

Proposal 1 	Intermediate relay UE in RRC_CONENCTED state is not allowed to monitor paging on behalf of its child UE. Only the last relay UE is responsible for paging monitoring.

Proposal 2 	Specify RemoteUEInformationSidelink transmission behaviour for RRC_CONNECTED intermediate relay UE in TS 38.331.

Proposal 3 	Single-hop and Multi-hop SI/Paging forwading are to be separately specified in clarity in 5.8.9.8.2  for the transmission of RemoteUEInformationSidelink.

Proposal 4 	The triggering of the transmission of RemoteUEInformationSidelink is also described in respective subclauses when SL RLF occurs or when child UE’s request is received.

Proposal 5 	RAN2 adopts the TP in Annex.

Proposal 6	In UuMessageTransferSidelnik, support a list of paging messages to be contained in ASN.1.

# 4 References

[1] Chairman’s notes, RAN2#131

[2] ASN.1 review comments document: “SLRelay Comments file v019.docx“, which is available at https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BMisc%5D/ASN1%20review/Rel-19%202025-09/38331/CR%20ASN1%20review/SLRelay

[3] R2-2506326, Rel-19 SL Relay RRC CR

# 5 Annex

First change

#### 5.8.9.3   Sidelink radio link failure related actions

The UE shall:

1> upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1> upon T400 expiry for a specific destination; or

1> upon indication from MAC entity that HARQ-based Sidelink RLF for a specific destination has been detected; or

1> upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination; or

1> upon indication of consistent sidelink LBT failures for all RB sets for a specific destination from MAC entity:

2> consider sidelink radio link failure to be detected for this destination;

2> release the DRBs (if any) of this destination, according to clause 5.8.9.1a.1;

2> release the SRBs of this destination, according to clause 5.8.9.1a.3;

2> release the PC5 Relay RLC channels of this destination if configured, in according to clause 5.8.9.7.1;

2> discard the NR sidelink communication related configuration of this destination;

2> reset the sidelink specific MAC of this destination, except for end-to-end PC5 connection in L2 U2U Relay operation;

2> consider the PC5-RRC connection is released for the destination;

2> indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2> if UE is in RRC_CONNECTED:

3> if the UE is acting as L2 U2N Remote UE for the destination:

4> if MP is configured, and neither MCG transmission nor indirect path transmission is suspended:

5> initiate the indirect path failure information procedure as specified in 5.7.3c;

4> else (i.e., MP is not configured, or MP is configured and MCG transmission or indirect path transmission is suspended):

5> initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3> else:

4> perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

2> if the UE is acting as L2 Intermediate U2N Relay UE for the destination and the destination is a child UE:

3> trigger the Remote UE information for NR sidelink communication procedure with its parent UE, as specified in 5.8.9.8.2;

Next change

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When L2 U2N remote UE entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink for L2 U2N Remote UE’s own SIB/posSIB/Paging or SFN-DFN offset request while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall:

1>	if the UE has SIB request information to provide (e.g. the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before) :

2>	include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not stored a valid version, in accordance with clause 5.2.2.2.1, of one or several posSIB(s) that the UE requires for a positioning operation, and the requested posSIB has not been indicated in RemoteUEInformationSidelink message to the parent L2 U2N Relay UE before, and the connected L2 U2N relay UE set posSIB-ForwardingSupported to supported:

2>	include sl-RequestedPosSIB-List in the RemoteUEInformationSidelink to indicate the requested posSIB(s);

1>	if the UE needs the SFN-DFN offset based on the request from upper layers and the connected L2 U2N relay UE set sfn-DFN-OffsetSupported to supported:

2>	set sl-SFN-DFN-OffsetRequested to true;

1>	if the UE has paging related information to provide (e.g. the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the L2 U2N Relay UE before), set sl-PagingInfo-RemoteUE/ sl-PagingInfo-RemoteUE-List as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

When L2 Intermediate U2N Relay UE receives new or updated SIB/Paging request from one or more child UE(s) according to 5.8.9.8.3, or PC5 link to a Child UE is no longer viable (e.g., due to SL RLF), the UE shall:

1>	if the UE is in RRC_IDLE or RRC_INACTIVE and there needs to be any variation of the requested SIB list:

2>	include sl-RequestedSIB-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has received paging related information from Child UE as specified in 5.8.9.8.3:

2> include the received paging information for Child UE(s) in the sl-PagingInfo-RemoteUE-List;

1>	if t a Child UE is no longer connected to the L2 U2N intermediate Relay UE (e.g. due to SL RLF):

2> update sl-PagingInfo-RemoteUE-List to excludethis Child UE;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

<text omitted>

Next change

##### 5.8.9.8.3	Reception of RemoteUEInformationSidelink message by the L2 U2N /U2U Relay UE

The L2 U2N Relay UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE:

2>	if the UE is in RRC_CONNECTED on an active BWP with common search space configured including pagingSearchSpace; or

2>	if the UE is in RRC_IDLE or RRC_INACTIVE:

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	monitor the Paging message at the L2 U2N Remote UE's paging occasion calculated according to sl-PagingIdentityRemoteUE and sl-PagingCycleRemoteUE included in sl-PagingInfo-RemoteUE;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

2>	else:

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	include the received sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message and perform Sidelink UE information transmission in accordance with 5.8.3;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	initiate transmission of the SidelinkUEInformationNR message to release the sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message in accordance with 5.8.3;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List:

2>	if the sl-RequestedSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List:

4>	perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2 or 5.8.9.8.2;

3>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedSIB-List is set to release:

3>	release received SIB request in sl-RequestedSIB-List;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedPosSIB-List:

2>	if the sl-RequestedPosSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of posSIB(s) indicated in sl-RequestedPosSIB-List:

4>	perform acquisition of the positioning system information indicated in sl-RequestedPosSIB-List in accordance with 5.2.2 or 5.8.9.8.2;

3>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedPosSIB-List is set to release:

3>	release received posSIB request in sl-RequestedPosSIB-List.

1>	if the RemoteUEInformationSidelink includes the connectionForMP:

2>	if the L2 U2N Relay UE is in RRC_IDLE:

3>	initiate an RRC connection establishment as specified in 5.3.3;

2>	else if the L2 U2N Relay UE is in RRC_INACTIVE:

3>	initiate an RRC connection resume as specified in 5.3.13;

If the L2 U2N Relay UE is a L2 Intermediate U2N Relay UE, the UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List and UE is in RRC_IDLE or RRC_INACTIVE:

2>	update the list of SIB requests

2) if the updated SIB request list is different from the one shared with its parent relay UE:

3>	trigger the Remote UE information for NR sidelink communication procedure with its parent UE, as specified in 5.8.9.8.2;

1>	if the RemoteUEInformationSidelink includes sl-PagingInfo-RemoteUE or sl-PagingInfo-RemoteUE-List:

2>	trigger the Remote UE information for NR sidelink communication procedure with its parent UE, as specified in 5.8.9.8.2.

The L2 U2U Relay UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-DestinationIdentityRemoteUE:

2>	consider the end-to-end PC5 connection release for the end-to-end PC5 connection between the L2 U2U Remote UE and the peer L2 U2U Remote UE identified by sl-DestinationIdentityRemoteUE;

2>	initiate the end-to-end PC5 connection failure/release related actions as specified in 5.8.9.3b;

End of changes