3GPP TSG RAN WG2 Meeting #131bis      	                                  R2-2507106
Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.19.1

Source:	Apple

Title:	DRX Adaptation for Voice Activity

WID/SID:	TEI19

Document for:	Discussion / Decision

# Introduction

Cellular voice calls have periods of silence that can be quite lengthy. Human speech has natural breaks between words and phrases; in addition, there are significant periods of silence before and after the speaker switches. The configuration for voice over cellular connections typically consists of a short DRX cycle to match the periodicity of voice packets when one of the users is actively speaking. This contribution proposes to adapt the DRX configuration to match silence durations when they occur.

# Discussion

Human conversations have natural breaks and silence periods. Speech traffic patterns have been studied extensively in the literature [1]. Speech consists of alternating talk spurts and silence periods. Analyzing voice packet traffic on VoLTE/VoNR calls across multiple carriers reveals that voice calls follow a roughly 40:40:20 distribution. That is, each of the participants in the voice call spends approximately 40% of the total call time talking, and the remaining 20% of the time there is silence.

Figure 1 shows a histogram of silence periods of user B in a typical voice call trace (call duration of ~20 mins) as seen at user A. As can be seen from the data, there are periods of silence that have durations that range from less than 100 ms to multiple seconds, and the fractions of silence periods with durations of 1s or more is quite significant.

Figure 1: Distribution of silence periods in a sample voice call trace

Voice codecs are adapted to handle the silence durations. The transmitting side continuously monitors to see if the speaker is silent. If silence is detected, it transmits a silence indication instead of transmitting an audio packet. This reduces the size of the payload to be carried end-to-end and simplifies the decoding behavior when there is silence.

In a voice call, there is a generally one speaker at any given time and one or more listeners. Figure 2 illustrates the sequences of talk spurts and silence periods in a two-party call. UE A is initially transmitting voice packets. When the transmitting side detects silence (of the speaker) it switches from transmission of voice packets to silence indications. The voice packets and silence indications have different periodicities (typically 40 or 80 ms for voice packets and 160 ms or 320 ms for silence indications). Switching between transmission of voice packets and silence indications can occur as needed determined by the speaker's voice pattern. During this period, UE B is generating and transmitting silence indications. At some point, the speaker switches from A to B, and a sequence of voice packets and silence indications can be transmitted by UE B, while UE A transmits silence indications.

Figure 2: Illustration of voice activity in two party call

DRX configuration for VoNR calls is typically adjusted to take into account periodicity of voice packets. It typically consists of the following parameter settings:

- DRX cycle of 40ms

- DRX on-Duration of 10ms

- DRX Inactivity timer of 10ms

The 40 ms DRX cycle (which is shorter than the typical DRX cycle of 320 ms for data calls) ensures that there is minimal delay in the play out of the voice data. However, this also means the UE wakes up much more often to monitor the DL. Due to silence periods, a significant fraction of the PDCCH on-durations is unnecessary, i.e., there is no DL data for the UE since the peer device is in silence and if the UE has not indicated any buffered UL data, the UE does not need to monitor the PDCCH for UL grants.

Figure 3 shows the fraction of silence in successive 1 second windows for a portion of the trace shown in Figure 1. Many of these windows have 100% silence (i.e., no DL data expected). This clearly indicates that DRX wakeups during such periods may not be needed.

Figure 3: Fraction of silence durations in successive 1 second windows

Observation 1: Due to silence periods in voice calls, a significant fraction of the DRX wakeups maybe unnecessary and wasteful.

The silence indications are a part of the codec functionality and are transmitted at the RTP layer (i.e., these indications are exchanged between the end-points of the voice call). They are not visible to the gNB. Security mechanisms such as header encryption are regularly employed, making it impossible for intermediate nodes (such as the gNB) to decode the silence indications, even if they were to use techniques such as deep packet inspection.

Observation 2: The silence indication cannot be recognized by the RAN node (it is only visible to the UE).

In order to leverage the information about silence at the peer device to reduce PDCCH monitoring, the following procedure can be considered:

- When the device receives a silence indication, it checks to see if it has any pending data to transmit.

- If there is no pending data to transmit, the UE transmits a "Peer Silence" MAC CE to the gNB. The MAC CE indicates to the network a certain number of DRX cycles for which the UE will not monitor the PDCCH (this should match the configured periodicity of the silence indications, so that the UE wakes up to receive the next silence indication).

- Network does not transmit to the UE for the indicated number of DRX cycles. If any DL data arrives for the UE, it is buffered until the next DRX wakeup.

Observation 3: An indication from the UE that the peer device is in silence can be used to optimize PDCCH monitoring when no DL data is expected and there is no UL data to transmit.

UE Assistance Information: The UAI framework in RRC is designed to adjust network scheduling behavior based on some temporary conditions at the UE. It allows the UE to signal constraints related to overheating, max BW limitations, max MIMO layer limitations, max CC limitations, DRX preference and in-device coexistence. The UAI information provided by the UE is strictly based on its own constraints; it does not take into account conditions at a peer device with which the UE is communicating. The underlying conditions on which UAI is based also evolve slowly. For example, conditions related to overheating, BW limitations, MIMO layer limitations, in-Device coexistence can last for many seconds to minutes. The network reaction to these conditions is not necessarily immediate. If the network performs a reconfiguration based on the UAI, the new configuration is typically maintained for the remainder of the connection (i.e., UAI is not designed for changing the configuration based on voice data patterns).

An indication that the peer device is in silence (peer-silence-indication) is dependent on activity at the peer device and is not something in the UE's control. It does not follow a predictable pattern. A fundamental difference between UAI and the peer-silence-indication is that while with UAI the UE is asking the network to modify its scheduling (delay/buffer data as needed), the peer-silence-indication is effectively informing the network that scheduling is not needed/expected for a certain amount of time.

Given the unpredictable nature of silence in voice calls, use of RRC messaging such as UAI would be impractical for indicating peer-silence. The peer-silence-indication can occur more frequently and overhead with RRC messaging would be large. What is needed is a simple indication that can be quickly inserted into MAC PDU that is to be transmitted. We believe that a MAC CE would be the right vehicle for the peer-silence-indication.

Proposal 1: Introduce a new peer-silence-indication MAC CE that the UE can use to indicate the number of on-Durations for which the UE will be in a Reduced Monitoring mode (i.e., skip PDCCH monitoring or use an alternate SSSG pattern for PDCCH monitoring).

Proposal 2: UE enters the Reduced Monitoring mode upon transmitting the peer-silence-indication MAC CE.

While in the reduced monitoring mode, the upper layers of the UE may have UL data for transmission. In such cases, the UE can follow the standard UL data arrival procedure: i.e., if there is no UL grant, trigger an SR. This will then bring the UE out of the reduced monitoring mode.

Observation 4: Standard UL data arrival procedures enable the UE to exit the reduced monitoring mode and return to normal operation.

Proposal 3: If there is UL data in the buffer in the reduced monitoring mode, the UE exits the reduced monitoring mode.

# Conclusions

In this contribution we have discussed the nature of silence patterns in voice calls and their relationship to DRX configurations. We find that device power consumption can be reduced by utilizing information about silence at the peer device. We propose to enable an indication from the UE to the network indicating peer silence and a reduced monitoring state to be used when the peer device is in silence and there is no UL data.

Observation 1: Due to silence periods in voice calls, a significant fraction of the DRX wakeups maybe unnecessary and wasteful.

Observation 2: The silence indication cannot be recognized by the RAN node (it is only visible to the UE).

Observation 3: An indication from the UE that the peer device is in silence can be used to optimize PDCCH monitoring when no DL data is expected and there is no UL data to transmit.

Proposal 1: Introduce a new peer-silence-indication MAC CE that the UE can use to indicate the number of on-Durations for which the UE will be in a Reduced Monitoring mode (i.e., skip PDCCH monitoring or use an alternate SSSG pattern for PDCCH monitoring).

Proposal 2: UE enters the Reduced Monitoring mode upon transmitting the peer-silence-indication MAC CE.

Observation 4: Standard UL data arrival procedures enable the UE to exit the reduced monitoring mode and return to normal operation.

Proposal 3: If there is UL data in the buffer in the reduced monitoring mode, the UE exits the reduced monitoring mode.

# References

- Menth M. et al, "Source Model for Speech Traffic Revisited," IEEE/ACM Transactions on Networking, Vol 17, No 4, Aug 2009.