3GPP TSG-RAN WG2 Meeting #131bis	R2-2507136

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	9.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on support of voice over NB-IoT-NTN via GEO

WID/SID:	IoT_NTN_Ph4 - Release 20

Document for:	Discussion and Decision

# 1	Introduction

At RAN #109 the work item on NB-IoT NTN Phase 4 [1] was agreed with the following objective:

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

To down-select between CP and UP solutions, SA2 and SA4 have sent questions to RAN2 in the LS[2] and LS[3] for further clarification. In this document, we discuss the questions from SA WGs and elaborate more on our view for CP/UP and IP/non-IP solutions.

# 2	Discussion

## 2.1	Questions from SA4 LS

In LS[2], SA4 asked below questions to RAN2:

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

For Q1: whether a packet overhead of 1 byte of MAC header is realistic

In current MAC specification 36.321[4], a MAC PDU for UL/DL SCH can be organized as Figure 6.1.2-3 where one MAC PDU may consist of a MAC header, zero or more MAC Service Data Units (MAC SDU), zero or more MAC control elements, and optionally padding; Correspondingly, the MAC PDU header may consist of one or more MAC PDU subheaders; each subheader corresponds to either a MAC SDU, a MAC control element or padding. Figure 6.1.2-1 and Figure 6.1.2-2 defines two types of minimum size of the MAC subheader.

Figure 6.1.2-3: Example of MAC PDU consisting of MAC header, MAC control elements, MAC SDUs and padding Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader with 7-bits and 15-bits L field Figure 6.1.2-2: R/F2/E/LCID MAC subheader

For voice packet transmission over NB-IoT, assuming the scheduled MAC TBS is for a voice packet transmission only (which means there is no multiplexing with other data, no multiplexing with other voice packet and no mulitplexing with MAC CE), the MAC PDU may consist of one MAC SDU (for voice packet), the MAC header and the possible padding.

- If the size of the allocated TBS is exactly the same as the size of the MAC SDU plus its MAC subheader (R/F2/E/LCID subheader),  there is no need for any padding or including the L field to indicate the MAC SDU length in the MAC subheader. In this case, the size of the MAC header is 1 byte.

- If the size of the allocated TBS is larger than the size of MAC SDU plus its MAC subheader (R/F2/E/LCID subheader), some padding bits should be added to the TBS. If the TBS room for padding is more than 2 bytes, due to the padding subheader is the last subheader, the size of the MAC subheader for MAC SDU should be 2 bytes (R/F2/E/LCID/F/L) to include the L length field for the variable MAC SDU length. And another MAC subheader for padding should be included as the last subheader with size of 1 byte (R/F2/E/LCID subheader). In this case, the size of the MAC header is 3 bytes. Alternatively, if the TBS room for padding is less than or equal to 2 bytes, the MAC subheader for MAC SDU can be placed as last subheader hence the subheader can be 1 byte (R/F2/E/LCID subheader). Accordingly, the required single-byte or two-byte padding will be filled with one of two MAC PDU subheader (R/F2/E/LCID subheader) at the begining of the MAC PDU. In this case, the size of the MAC header is 2 or 3 bytes.

Observation 1: According to current MAC spec, the size of MAC header for a voice packet can be 1 or 2 or 3 bytes, taking into account the requirement for padding where the allocated TBS may not exactly accomodate the voice packet.

In our understanding, it is too restricted on the UL/DL grants to exactly match the TBS with the voice packet size plus RAN protocol layer header, considering the possible voice codec rate change and talk-silent period switch in voice application layer, the high priority of MAC CE (e.g., BSR) to be transmitted from UE to NW and the overhead change in different layers (e.g., RTP header or ROHC header or MAC/RLC header). Therefore, a voice packet with only 1 byte of MAC header is a corner case. It is not a reasonable conclusion to say the MAC header for a voice packet is 1 byte following the current MAC specfication.

Proposal 1: RAN2 to respond to SA4 that, 1 byte of MAC header for a voice packet is not realistic. In most practical cases, the MAC header size for a voice packet is 2 bytes or 3 bytes based on current MAC specification.

For Q2: The header size in different layers for CP/UP and IP/non-IP solutions

In this section, we will discuss the header size for voice data/payload.

- UP IP based solution

In the TR23.700-19 [5], it illustrates the protocol stack for voice data in UP IP based solution as Figure 6.2.2-4.

Figure 6.2.2-4: Protocol stack for IMS voice data

In this option, a voice packet undergoes encapsulation through multiple protocol layers as it traverses from the application layer down to the physical layer at the UE. For RTP/UDP/IP headers, they are compressed via ROHC in PDCP layer. On top of that, the PDCP, RLC and MAC headers are added before the MAC PDU transmission.

- For size of RTP/UDP/IP header size with ROHC, it depends on the ROHC state ( e.g., Initialization & Refresh, First Oder or Second Order). Typically, 80% of ROHC headers size is less than or equal to 24 bits (3 Bytes) while 98% of ROHC header size is less than or equal to 40 bits (5 Bytes). So the ROHC header size for RTP/UDP/IP could be 3~5 bytes.

- For PDCP layer header,  in NB-IoT, short PDCP SN (7 bits) is supported for user plane PDCP Data PDU. Hence the PDCP header size is 1 byte.

- RLC UM should be used for the voice data transmisison where 5 bit SN length will be used for NB-IoT. Without considering RLC segmentation and concatenation, the size of the RLC header could be 1 byte.

- For  MAC layer header, as analyzed in Q1, the MAC header size could be 2~3 bytes.

Observation 2: For UP IP solution, the overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC) could be range from 7 to 10 bytes.

- UP non-IP based solution

According to TR23.700-19 [5], the protocol stack for voice data in UP non-IP solution is the same as the UP IP based solution except the removal of the UDP/IP layer. In this solution, the header size in PDCP, RLC and MAC are the same as those in the UP IP based solution. However, since there is no UDP/IP header, how to handle the RTP header should be further discussed. The RTP header (e.g., SN, PT, TS, etc) compression/de-compression between UE and P-GW were discussed in SA2, hence, RAN2 can leave the size of the RTP header to be decided by SA2.

Figure 1: UP non-IP solution

Observation 3: For UP non-IP solution, the packet overhead size at the PDCP, RLC and MAC layer is identical to that of the UP IP based solution.

Observation 4: For UP non-IP solution, the determination of RTP header size can be left to SA2.

- CP IP based solution

In the TR23.700-19 [5], it illustrates the protocol stack for voice data in CP IP based solution as Figure 6.6.1-3.

Figure 6.6.1-3: Protocol stack for IMS voice data

In this option, a voice packet is encapsulated with RTP/UDP/IP headers, compressed using ROHC, and then carried within a NAS message encapsulated in an RRC container before being transmitted over the radio interface through the RRC, RLC, MAC, and PHY layers.

- For ROHC, as analyzed in UP IP solution, the ROHC header size for RTP/UDP/IP could be 3~5 bytes.

- For the size of NAS layer header, the mechanism is designed and discussed in CT1 hence RAN2 can leave the question to CT1.

- For RRC layer, the NAS message may be transmitted in RRCConnectionSetup/DLInformationTransfer for DL and RRCConnectionSetupComplete/ULInformationTransfer for UL. Per ASN.1 encoder, the RRC layer header can be roughly 2~3 bytes depending on the message type and final RRC message octet alignment.

- In NB-IoT CP solution, only RLC AM mode is applicable to SRB1bis. If RLC AM is adopted in the CP IP solution, the SN length is 10 bits and the size of the RLC header could be 2 bytes. However, for voice traffic, in legacy RLC UM should be adopted to avoid unnecessary ARQ retransmission since it may introduce additional delay, jitter, and potential head‑of‑line blocking. Therefore, RAN2 may need to first discuss whether RLC UM should be supported for SRB.

- For  MAC layer header, as analyzed in Q1, the MAC header size could be 2~3 bytes.

Observation 5: For CP IP solution, the determination of NAS layer header size can be left to CT1.

Observation 6: For CP IP solution, RAN2 should discuss whether RLC UM should be introduced for SRB.

- CP non-IP based solution

According to TR23.700-19 [5], the protocol stack for voice data in CP non-IP solution is the same as the CP IP based solution except the removal of the UDP/IP layer. In this solution, the header size in NAS, RRC, RLC and MAC are the same as those in the CP IP based solution. However, since there is no UDP/IP header, how to handle the RTP header should be further discussed. The RTP header (e.g., SN, PT, TS, etc) compression/de-compression between UE and P-GW were discussed in SA2 hence RAN2 can leave the size of the RTP header to be decided by SA2.

Observation 7: For CP non-IP solution, the packet overhead size at the NAS, RRC, RLC and MAC layer is identical to that of the CP IP based solution.

Observation 8: For CP non-IP solution, the determination of RTP header size can be left to SA2.

Below table is a summary of the overall overhead for CP/UP and IP/non-IP solution.

Table 1: Overhead estimation for CP/UP and IP/non-IP solution

|  | UP IP | UP non-IP | CP IP | CP non-IP |
| --- | --- | --- | --- | --- |
| RTP | ROHC header size 3~5 Bytes | Up to SA2 | ROHC header size 3~5 Bytes | Up to SA2 |
| UDP |  | NA |  | NA |
| IP |  | NA |  | NA |
| PDCP | 1 byte | 1 byte | NA | NA |
| NAS | NA | NA | UP to CT1 | Up to CT1 |
| RRC | NA | NA | 2~3 bytes | 2~3 bytes |
| RLC | 1 byte (RLC UM) | 1 byte (RLC UM) | RAN2 to discuss if RLC UM should be supported. | RAN2 to discuss if RLC UM should be supported. |
| MAC | 2~3 bytes | 2~3 bytes | 2~3 bytes | 2~3 bytes |

Proposal 2: RAN2 to respond to SA4 the overall packet overhead size estimation for UP/CP and IP/Non-IP solutions, with the table 1 for reference.

## 2.2	Questions from SA2 LS

In LS[3], SA2 asked below questions to RAN2:

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20? Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

For Question 1: ROHC state fall back at the compressor

The eNB can configure the RoHC profiles to UE via PDCP-Config. However, the exact RoHC compression/de-compression mechanism is not defined by RAN2. Instead, RAN2 just refer to the RFC specifications defined by IETF and classify them into different profiles. Therefore, the criteria to trigger the ROHC state change is not in the scope of RAN2. RAN2 may not be the appropriate WG to reply question 1.

#### 36.306: 4.3.1.1A supportedROHC-Profiles-r13 This field defines which ROHC profiles from the list below are supported by the UE: - 0x0000 ROHC uncompressed (RFC 5795) - 0x0002 ROHC UDP (RFC 3095, RFC 4815) - 0x0003 ROHC ESP (RFC 3095, RFC 4815) - 0x0004 ROHC IP (RFC 3843, RFC 4815) - 0x0006 ROHC TCP (RFC 6846) - 0x0102 ROHCv2 UDP (RFC 5225) - 0x0103 ROHCv2 ESP (RFC 5225) - 0x0104 ROHCv2 IP (RFC 5225) A UE that supports one or more of the listed ROHC profiles shall support ROHC profile 0x0000 ROHC uncompressed (RFC 5795). This field is only applicable if the UE supports S1-U data transfer or User plane CIoT EPS Optimisation, see TS 36.331 [5], and any ue-Category-NB.

Proposal 3: RAN2 is not the appropriate WG to reply Question 1 regarding the criterion to trigger the ROHC state fall back at the compressor.

For Question 3: scheduling methods for different size of voice packets

To save the PDCCH resource, Semi-Persistence Scheduling (SPS) can be used for voice packets transmission, assuming a fixed traffic pattern. The exact SPS periodicity and TBS to be allocated for an SPS configuration may depend on the SA4 conclusion regarding voice frame length and codec rate for ULBC. Ideally, the TBS of each SPS occassion should be sufficient to accomodate the voice payload in UE’s buffer, eliminating the need to trigger dynamic scheduling to transmit any additional payload after each of SPS transmission.

However, many factors can affect the size of voice packets. Taking the UL as an example, these include talk/silence period switching, voice codec rate change, voice packet aggregation/bundling level change as well as the ROHC state change (if IP-based solution is selected). The SPS TBS should be designed in a way to accomodate most of the voice packets (rather than covering the worst case) to avoid unnecessary NW resource waste. Consequently, the SPS TBS may not always be sufficient to carry the entire payload in every SPS occassion. In such cases, the NW should issue dynamic scheduling (DS) to transmit the remaining payload.

Of course, for talk/silence period switching, if the talkspurt period and silent period have significantly different SPS periodicity and TBS requirements, the network could also configure multiple SPS configurations to match these varying needs, thereby avoiding the need for dynamic scheduling and minimizing additional PDCCH overhead. However, in some other cases, the voice packet size change may be transparent to the RAN layers. For example, a voice codec rate change may happen during the call where the rate negotiation happens between two UEs via Codec Mode Request (CMR) field in the RTP payload (e.g., as defined in RFC4867 for AMR). In such cases, dynamic scheduling should be used as a complementary mechanism to empty the residual data from the UE’s buffer. Therfore, in our understanding, both the SPS and DS mechanisms should anyway be supported.

Proposal 4: RAN2 to confirm that RAN scheduling can handle the dynamically changed voice packets sizes.

However, since the dynamic scheduling will consumes additional PDCCH resource compared to SPS, for voice call, it would be beneficial to limit the packet size variation to some extent thereby reducing the need for DS. Moreover, due to the high RTT in GSO secnario, the BSR-based DS may impact the packet transmission latency. Therefore, RAN2 can further discuss how to minimize the voice packets size variations and the impact caused by these variations.

Proposal 5: RAN2 further discuss how to minimize voice packets size variations to reduce the need for triggering dynamic scheduling, as well as how to mitigate the impact caused by these variations.

For Question 6: feasibility to support more than 2 DRBs in NB-IoT UE

According to the current specficiations, depending on its capability, a NB-IoT UE can support up to 2 DRBs. Technically, similar to normal LTE where the UE can support up to 15 DRBs, it could be feasible for NB-IoT UE to support more than 2 DRBs, provided they have the additional UE capabilities, However, this may need further input from UE vendors.

TS36.331: A NB-IoT UE only supports 0, 1 or 2 DRBs, depending on its capability. A NB-IoT UE that only supports the Control Plane CIoT EPS optimisation (see TS 24.301 [35]) or the Control Plane CIoT 5GS optimisation (see TS 24.501 [95]) does not need to support any DRBs and associated procedures. #### 36.306: 4.3.8.5 multipleDRB-r13 This field indicates whether the UE supports multiple DRBs. This field is only applicable if the UE supports S1-U data transfer or User plane CIoT EPS Optimisation as defined in TS 24.301 [28] or NG-U data transfer or User plane CIoT 5GS Optimisation as defined in TS 24.501 [39], and any ue-Category-NB. If a UE of this release supports multiple DRBs, the UE shall support two simultaneous DRBs.

Proposal 6: RAN2 to respond to SA2 that it is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20, with the introduction of additional UE capability.

For Question 9 and 10: Possibility to define a new SRB and configure it as RLC UM to carry voice media

According to the current specifications, in NB-IoT CP solution, only RLC AM mode is supported for SRB. However, if the voice traffic were to be carriered over an SRB, RLC UM should be adopted to avoid unnecessary ARQ retransmission since RLC AM could introduce additional delay, jitter, and potential head‑of‑line blocking. Furthermore, the NB-IoT CP solution currently defines only one SRB, which may be not sufficient to differentiate between IMS signalling and voice traffic in terms of their respective latency and reliablity requirements. Therefore, to support voice call over NB-IoT CP solution, a new SRB and RLC UM should be introduced. This kind of new function introduction may impact RAN2 specification a lot. On the other hand, the legacy VoLTE design, using separate DRBs with appropriate RLC modes, effectively support both IMS signalling and the voice traffic well. Extending the NB-IoT UP solution to support signalling and voice traffic in a similar manner as VoLTE would be straightforward and would entail only limited impact on the specifications.

Proposal 7: RAN2 to respond to SA2 that extending the NB-IoT UP solution with separate DRBs for IMS signalling and voice traffic is more suitable than adapting the NB-IoT CP solution, which would require defining a new SRB with RLC UM mode.

## 2.3	View on CP/UP and IP/non-IP solution

Based on above discussions, supporting voice call over NB-IoT via UP solution entails limited RAN2 specification impact compared to the CP solution. Furthermore, the CP solution offers no advantage over UP solution in terms of the size of overall packet overhead, while also requiring coordination with other other WG (e.g., CT1) to address the NAS header optimization.  Therefore,we think RAN2 should adopt the UP-based solution for simplicity.

For UP IP and non-IP solution, the key point is whether to introduce a new compression mechanism for RTP header while bypasses the UDP/IP layers. The feasiblity and complexity for such kind of new function have been extensively discussed in SA2. Therefore, the decision on whether to adopt IP or non-IP based solution can be left to SA2.

Proposal 8: The UP solution should be adopted to support voice over NB-IoT-NTN.

Proposal 9: RAN2 to inform SA2 to downselect between the UP IP and UP non-IP solutions.

# 3	Conclusion

This document has made the following observations and proposals:

Questions from SA4 LS

Observation 1: According to current MAC spec, the size of MAC header for a voice packet can be 1 or 2 or 3 bytes, taking into account the requirement for padding where the allocated TBS may not exactly accomodate the voice packet.

Proposal 1: RAN2 to respond to SA4 that, 1 byte of MAC header for a voice packet is not realistic. In most practical cases, the MAC header size for a voice packet is 2 bytes or 3 bytes based on current MAC specification.

Observation 2: For UP IP solution, the overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC) could be range from 7 to 10 bytes.

Observation 3: For UP non-IP solution, the packet overhead size at the PDCP, RLC and MAC layer is identical to that of the UP IP based solution.

Observation 4: For UP non-IP solution, the determination of RTP header size can be left to SA2.

Observation 5: For CP IP solution, the determination of NAS layer header size can be left to CT1.

Observation 6: For CP IP solution, RAN2 should discuss whether RLC UM should be introduced for SRB.

Observation 7: For CP non-IP solution, the packet overhead size at the NAS, RRC, RLC and MAC layer is identical to that of the CP IP based solution.

Observation 8: For CP non-IP solution, the determination of RTP header size can be left to SA2.

Proposal 2: RAN2 to respond to SA4 the overall packet overhead size estimation for UP/CP and IP/Non-IP solutions, with the table 1 for reference.

Table 1: Overhead estimation for CP/UP and IP/non-IP solution

|  | UP IP | UP non-IP | CP IP | CP non-IP |
| --- | --- | --- | --- | --- |
| RTP | ROHC header size 3~5 Bytes | Up to SA2 | ROHC header size 3~5 Bytes | Up to SA2 |
| UDP |  | NA |  | NA |
| IP |  | NA |  | NA |
| PDCP | 1 byte | 1 byte | NA | NA |
| NAS | NA | NA | UP to CT1 | Up to CT1 |
| RRC | NA | NA | 2~3 bytes | 2~3 bytes |
| RLC | 1 byte (RLC UM) | 1 byte (RLC UM) | RAN2 to discuss if RLC UM should be supported. | RAN2 to discuss if RLC UM should be supported. |
| MAC | 2~3 bytes | 2~3 bytes | 2~3 bytes | 2~3 bytes |

Questions from SA2 LS

Proposal 3: RAN2 is not the appropriate WG to reply Question 1 regarding the criterion to trigger the ROHC state fall back at the compressor.

Proposal 4: RAN2 to confirm that RAN scheduling can handle the dynamically changed voice packets sizes.

Proposal 5: RAN2 further discuss how to minimize voice packets size variations to reduce the need for triggering dynamic scheduling, as well as how to mitigate the impact caused by these variations.

Proposal 6: RAN2 to respond to SA2 that it is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20, with the introduction of additional UE capability.

Proposal 7: RAN2 to respond to SA2 that extending the NB-IoT UP solution with separate DRBs for IMS signalling and voice traffic is more suitable than adapting the NB-IoT CP solution, which would require defining a new SRB with RLC UM mode.

View on CP/UP and IP/non-IP solution

Proposal 8: The UP solution should be adopted to support voice over NB-IoT-NTN.

Proposal 9: RAN2 to inform SA2 to downselect between the UP IP and UP non-IP solutions.

# Reference

[1] RP-252473 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4

[2] S4-251584 LS on the RAN simulation assumptions for ULBC

[3] S2-2507636 LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

[4]	TS 36.321, Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification (Release 18)

[5] TR23.700-19 Study on Integration of satellite components in the 5G architecture; Phase 4