3GPP TSG-RAN WG2 Meeting #131bis						R2-2507144

Prague, Czech, October 13th – 17th, 2025

Agenda Item:	8.0.1

Source: 	OPPO

Title:  	Discussion on remaining issues of UE capability

Document for:	Discussion and Decision

- Introduction

This is to discuss the remaining issues for UE capability.

- Discussion

  - UECap #2: R4 46-4/5

| 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW and 1/2*NRB extension for dual-sided symmetric and single-sided case Supported cases as following ({ratio value on low side, ratio value on high side} ) : {1/2, 0}, {0, 1/2}, {1/2, 1/2} NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1 | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | This capability is applicable for single UL CC case | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46-5 | MPR reduction for single carrier with multiple values | Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW and 1/X*NRB extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios of {1/2, 1/4} Supported cases as following ({ratio value on low side, ratio value on high side}): {1/2, 0}, {0, 1/2}, {1/2, 1/2}, {1/4, 0}, {0,1/4}, {1/4, 1/4} NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1 | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | This capability is applicable for single UL CC case | Optional with capability signaling |

Currently, the two are implemented as separate capabilities

BandNR ::=                          SEQUENCE {

[...]

-- R4 46-4: MPR reduction for single carrier with single value

mpr-SingleCC-SingleValue-r19                                    ENUMERATED {supported}                                     OPTIONAL,

-- R4 46-5: MPR reduction for single carrier with multiple values

mpr-SingleCC-MultipleValue-r19                                  ENUMERATED {supported}                                     OPTIONAL,

[...]

}

Clearly, 46-4 is a subset of 46-5, in the sense that 46-4 supports a smaller number of MPR reduction combination compared to 46-5. So, there is no need for UE to simultaneously report the support of 46-4 and 46-5.

(UE Cap#2) R2 implement 46-4 and 46-5 as two enumerate values of a same capability. If R2 cannot converge, ask R4 whether there is a reason that UE has to report the support of both 46-4 and 46-5 other than 46-5 only.

  - UE Cap#3: R1 67-4

| 67. TEI19 [Simul_SRSCS] | 67-4 | Support of simultaneous SRS carrier switching [Simul_SRSCS] | 1.- Support simultaneous SRS carrier switches. Two SRS carrier switches are considered to be simultaneous if the SRS transmission (including RF retuning time) in both CCs overlap in time | 2-56 | Yes | N/A | Simultaneous SRS CS across multiple CC is not supported | Per band combination | N/A | N/A | N/A | For each target band, the UE can indicate with which other target bands in the band combination can SRS carrier switching be simultaneously triggered | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For which the R1 conclusion was

In legacy SRS carrier switching, the srs-SwitchingAffectedBandsListNR was to indicate during SRS carrier switch CC1 => CC2, whether CC3 would be affected. While here for simultaneous SRS carrier switching, the 67-4 is to indicate whether the SRS carrier switch of CC1 => CC2 and CC3 => CC4 can happen simultaneously.

Considering this collision, and since the yellow part above seems also imply a like architecture is sufficient, R1 needs to clarify before R2 reflect this in spec, i.e, whether 67-4 should be implemented per-band-pair1-per-band-pair2?

(UE Cap#3) R2 sends LS to R1, to ask whether 67-4 intends to be reported for the relationship of two band pairs.

  - UE Cap#4: R4 55-1

Based on the LS from R4, i.e., R4-2511863

- There are, however, low band CA switching to CA BCs whose traditional CA are already in the market. And, there are UEs supporting only low band CA switching, only traditional CA BCs, or both low band CA switching and traditional CA BCs for the said CA BCs.

- When specifying the UE requirements, RAN4 has assumed that the network supporting any release of the NR specification should be able to distinguish these UEs

So, the problem is how to implement 55-1 to differentiate the capability of the three cases, where the key issue is how to avoid NBC issue to legacy network due to a UE supporting only low-band CA switching. A safer way is to rely on the BC-list level differentiation.

In legacy, we have two BC list supportedBandCombinationList, and supportedBandCombinationList-UplinkTxSwitch, where

- Via the former one, the network may misunderstood a FDD-SDL BC as supporting the traditional CA

- While the latter one was specified to be limited to the case as follows, I.e., it was specified for a band pair with UL/Tx

| ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700 Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4]. <Text Removed> | BC | FD | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840 Indicates UE supports Rel-18 dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL as defined in TS 38.214 [12] and TS 38.101-1 [2]. The capability signalling comprises the following parameters: - bandIndexUL1-r18 and bandIndexUL2-r18 indicate the band pair on which UE supports dynamic UL Tx switching. bandIndexUL1/bandIndexUL2 xx refers to the xxth UL band entry in the band combination. UE shall indicate support of 2-layer UL MIMO in FeatureSet on both bands for 2Tx-2Tx switching, or indicate support of 2-layer UL MIMO on one band and 1-layer MIMO on the other band for 1Tx-2Tx switching, or indicate support of 1-layer UL MIMO on both bands for 1Tx-1Tx switching. <Text Removed> | BC | FD | N/A | FR1 only |

While now for low-band switching, it is for a FDD band (with UL/Tx) and a SDL band (without UL/Tx)

For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

So it seems legacy network would not take a low-band switching BC as a Tx-switching BC by mistake, if such BC is reported within supportedBandCombinationList-UplinkTxSwitch, if the checking of UL/Tx presence (or the duplex mode of the two bands) has been done properly.

Therefore, there seems no NBC concern if report the LBCA_SW BC in the supportedBandCombinationList-UplinkTxSwitch, but if any concern on NBC issue from network perspective, it can be solved by introducing another BC list.

(UE Cap#4) R2 discuss to solve the NBC issue for a BC supporting low-band CA switching but not traditional CA, e.g., by reporting such BC using supportedBandCombinationList-UplinkTxSwitch or a new BC list.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Proposal 1	(UE Cap#2) R2 implement 46-4 and 46-5 as two enumerate values of a same capability. If R2 cannot converge, ask R4 whether there is a reason that UE has to report the support of both 46-4 and 46-5 other than 46-5 only.

Proposal 2	(UE Cap#3) R2 sends LS to R1, to ask whether 67-4 intends to be reported for the relationship of two band pairs.

Proposal 3	(UE Cap#4) R2 discuss to solve the NBC issue for a BC supporting low-band CA switching but not traditional CA, e.g., by reporting such BC using supportedBandCombinationList-UplinkTxSwitch or a new BC list.

4. Reference

[1] xxx