3GPP TSG-RAN WG2 Meeting #131-bis					   R2-2507147

Prague, Czech, 13th – 17th October 2025

Agenda item:	10.2

Source:			MediaTek Inc.

Title:	On 6GR UE capability

Document for:	Discussion and Decision

- Introduction

In the 6G Radio SID [1], the 6GR UE capability related objectives are as follows:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

- Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

…

…

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

…

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

…

- 6GR core and performance requirements

…

- UE RF requirement aspects including band and band combination [RAN4]

…

    - Study UE RF capabilities considering different device types and implementations

…

In this contribution, we discuss on general aspects of signaling design for UE capability that 6GR system should consider from the start.

- Discussion

  - UE capability size explosion

According to the observations in the field [2], the Band Combination (BC) and the Feature Set (FS) reporting consume 80% or more the RAT capability container size.

As spectrum holding and network deployment evolved, the capability request filter criteria did not serve what the UE and the network really want to do well:

- The resolution (in band level) is so insufficient that numerous BCs (and the corresponding features) are not filtered, such as intra-band CA ones.

- The most gNBs usually look for the interested configurations rather than traversing all the reported BC and FS capability IEs.

We continuously expanded or complexed radio access capability signaling design to allow more flexibilities in meeting a wide range of 3GPP requirements. These requirements are not only from the UE capability advertisement needs, but also from the other areas, e.g., regulations and certifications. When tailoring a global SKU for the worldwide market demands, we saw a skyrocketing size growth over the number of the carrier components and of the BCs. Furthermore, the UE capability signaling design additionally faced the following challenges:

- An emerging trend in the size growth of the supported band list. (This is not good…)

- Endless atypical cases (from other WGs) to impact the common signaling design. (Like future-proofing never quite holds up!)

- Lack of capability envelope indication to absorb signaling overhead.

All the above haven’t even mentioned the new 6GR operating bands, such as FR3 ones, yet. The significant capability information size (causing incomplete reporting) even jeopardizes the mobility purpose of including more bands in the capability request filter, leads to the other observation where the network continually re-acquired the UE capability once after certain types of UE mobility occurred [2].

- BC capability reporting is the main aggressor of UE capability size explosion issue. The filter criteria provided was inadequate for the demands of the interoperability between the network and the UE.

To cater for the interoperability with legacy systems, multiple RAT containers could be requested and loaded at the same time (in a single RRC message transaction) in terms of efficiency. Also, RAN2 cannot tell what the future will bring, but the 6GR capability signaling design shall meet the interoperability requirements (whatever the other WGs ask) of delivering UE radio access capability information in sufficient detail. The new demands never stop coming. Therefore, uplink segmentation for UE capability reporting will probably be useful (regardless of the obvious drawbacks such as the procedure latency), however unfortunately it would not be a future-proof solution given that, again, the current filter criteria is not good enough for constraining the RAT container size under the interface limitation of IWK specification.

- UL segmentation as a last resort to workaround the L2 transport size limitation but could still hit the MTU limitation on the inter-CN interface.

Evidently, the capability request filter design should be revisited, and the provided criteria ought to be richer (to filter out more unnecessary UE capability reporting). This is the good start that RAN2 can work on without heavily relying on other WGs inputs.

To escape from the dilemma, we must manage to control the capability reporting size, let’s go back to the basics.

- RAN2 to study the UE capability transfer procedure and signaling design managing to ensure the complete 6GR RAT container can be reported without segmentation.

  - UE capability change

In 5G NR, thermal mitigation is a MUST to save the UE from damaging itself. However, there is no way to adjust some of UE capabilities with immediate effect during RRC_CONNECTED. The whole UAI mechanism was governed by the network. The UE cannot trigger upon the irresistible event. Even it is doable, it is not compulsory to the network.

The other existing 5G NR use cases for changing lower layer related UE capabilities are power savings, interference avoidance, MUSIM and so on. The exclusive RF/BB capability parameters associating with diverse purposiveness were deemed to be just UE preferences in the UAI mechanism.

- UAI had evolved into a broad framework with diverse parameters and lacks immediate effect transaction, making it unsuitable for urgent UE capability updates during RRC_CONNECTED.

Later better than never. The UE-initiated full range capability updating procedure could be taken as an alternative, however:

- The ongoing connection(s) must be dropped first, which impacts UX badly.

- It takes long to or never see the network re-acquiring the UE capability. (At risk for inconsistency between the network and the UE.)

- Even longer cycle to get back to the original state if the change is temporary.

As the recovery measure, the UE can still attempt to re-attach the network if there is unexpected interoperability failure between the UE, gNB, and AMF breaking the UE-initiated full range capability updating procedure (i.e., the network does not subsequently re-acquire the UE capability information). However, all the aforesaid situations in the field had been troublesome for quite a while since from 4G LTE era.

- The sole UE-initiated capability updating procedure was suitable only when the UE is in no hurry to do capability change without concerning UX.

Looking forward to the 6G, more use cases are anticipated because of the foreseeable trends from the verticals. For example, the immersive communication devices, sensing-based clients, and the AI-powered autonomous systems are getting more and more popular. Among them, there will be vital events with the need of temporarily or permanently changing RAN-centric capabilities that practically affect the network KPI and/or resolve UX issues.

For a unified 6GR air interface design, it is a clear desire to separate timely, operational capability change use cases from UAI, and integrate a UE-initiated and immediate-effect procedure within the connection management (CM) framework so the network can safely maintain, reconfigure, or even start the ongoing connections over again. The following design principles are considered as the start points and to be studied by RAN2:

- UE capability changed within a limited scope.

- Procedure acknowledgment and coordination that preserves connection continuity.

- Abuse prevention.

- Testing and operational readiness.

- RAN2 to study the basic RRC procedure that UE can use to initiate the immediate updating of its capabilities without leaving RRC_CONNECTED.

  - Essential inter-WGs/TSGs issues

In general, the whole CA framework (including the BC capability reporting) is highly related to the UE implementation and the network architecture, and to how the corresponding RF, BB capabilities are organized. We had the following views with observations (not an exhaustive list) and found these aspects are so crucial that RAN2 should study with close communication with other WGs, or TSGs in the beginning to make UE capability signaling design benefits from the aligned principle and synergy.

    - Envelope indication and data metrics

In terms of BC capability, we usually had three different approaches to signal the UE capability envelope and for the data metrics determination:

- Only per-FS/per-FSPC reporting.

- Reduced per-FS/per-FSPC reporting with per-BC envelope indication.

- Only per-BC reporting.

Our observations are that on one hand, the RF-centric capabilities are more complicated and used to be constrained by the practical RF/FE implementation considerations making no room for being indicated in coarse granularity sometimes. On the other hand, though the BB-centric capabilities could also have similar concerns (as RF-centric ones), most BB capabilities are comparatively easier to be reported in per-BC granularity as the solution to under-reporting, which is also another form of envelope indication. Therefore, the RAN4-related RF-centric capabilities should be prioritized and studied first. And RAN2 should develop a unified model or the design principles to accommodate both the cases in the 2nd and 3rd bullet.

How to balance between the envelope indication (capability signaling in coarse granularity) and the differentiation flexibility (capability signaling in fine granularity) is a key, and we need the working assumptions on such as channel bandwidth, MIMO layer aspects, etc. RAN4 and RAN1 conclusions/agreements are needed.

Moreover, the data metrics (involving more parameters such as HOM, SF, etc.) may need to be rediscussed when introducing new envelope indication or changing the current BC capability signaling design.

    - Forward compatibility impact by placeholder-like band/BC

There were NR operating band and band combinations submitted to 3GPP without any practical support by modern UE implementation and by technical foundation. For example, n65, and CA_n5A-n29A. Usually(ideally) such exercises are not problems because the BC signaling used to be forward compatible. However, our life is getting not that easy when more and more atypical things came out.

Take Rel-19 NR_LBCA_Sw as example, the BC configurations were defined in the specs for years without indicating it is infeasible for them to be supported by the practical RF/Antenna implementation. With the difficulties to confirm there was no implementation of the traditional CA for LB BCs in the market, this exercise impacted the future-proofness of BC capability signaling framework (could also impact the Release Independence characteristic) and led to even consider introducing the 3rd supported band combination list.

RAN2 can remind RAN4 of the severity, and the responsibility of being as the sole gatekeeper.

    - Contradictory philosophy

All RAN WGs shared the same understanding of the fallback rules for BC capability signaling. The common fallback rules work well with the “basket” practice in RAN4 works. However, there was a different approach in e.g. the worst-case-oriented Lower MSD capability. RAN4 agreed that Lower MSD capability for higher order combination is inherited from lower order fallback combinations. The intent to control the capability reporting size is understandable and appreciated, but it cannot be denied that practice introduced an opposite concept for interpreting the fallback capabilities and influenced the RAN4 contexts (and mindset) when discussing whether a certain feature is applicable to the higher order BCs.

The other example is how the fallback groups are organized in defining NR CA bandwidth classes. Ideally the fallback rule of bandwidths (and its classes) should be straightforward and simple (the fallback from the larger BW should be simply contiguous and sequential to the smaller ones) in terms of carrier aggregation, however, the current fallback group is defined to be artificially complicated.

We need to harmonize these aspects otherwise we should clarify the applicability (scope) for each.

    - DL and UL decoupling

We currently had 2 separated supported band combination lists in the UE-NR-Capability container, one is for the UL Tx switching, another one is for the ordinary CA (and DC if appliable). In Rel-19, we started specifying the techniques to support the LB CA with switching. Apparently, there could be duplicate information reported between the BCs and even between the BC lists (and among the associated FSs). From RAN2 point of view, we would like to exploit the size reduction opportunities of splitting the DL and UL part of a BC in UE capability reporting.

In addition to the pure BC/FS capability signaling design considerations, RAN4 could study and assess the required specs impacts if it is agreed that DL and UL decoupling could also reflect the new 6GR RF design philosophy and requirements considering of the following 5G NR facts (and the implications to the current CA framework) additionally:

- SRS antenna switching capabilities are counterintuitively reported in DL feature set IEs.

- Spectrum contiguousness differentiation of DL and UL parts (due to US CBRS SAS).

- The UL CA_n5-n8 operation without n5 reception of the FDD-FDD inter-band CA_n5-n8.

At least, the asymmetric BCS, simultaneous RxTx and the duplexing aspects should be revisited by RAN4 first.

Meanwhile RAN2 should focus on how to retain the essential UE capability signaling characteristics that the networks heavily relied on nowadays, e.g., each individual BC capability reporting is self-contained.

One thing in UL part is worthwhile to note that RAN2 should also collaborate with RAN4 and RAN1 to renew the whole UE power class capability signaling design because its reporting (including the related facilities such as duty cycle, NS value, PA architecture and misc. things) are intertwined across per-band and per-BC levels, and simply a mess now. The WF could be either we consider MIMO, Tx diversity, HPUE (for single band or for BC), regulations and device-specific designate (e.g., the PCs in FR2) things altogether for refactoring the power class signaling design, or we strive for getting rid of reporting the UE power class entirely if it is feasible.

    - Capability ID mechanism

Beyond the RAN scope, if we retune the focus to the whole UE capability retrieval framework, there is Capability ID mechanism that we think it would be useful for handling the uniform and identical UE capability from the massive communication verticals (other eligible device types are not precluded) in 6G era. Yet we don’t know what the 6G network architecture will look like, we need SA/CT views on the UE capability retrieval framework, and on whether the Capability ID mechanism should be considered as baseline or not, given that there are more and more outside-in requirements (such as open manufacturer information) to the 3GPP territory. It’s crucial for 6G to consider the synergy with outside 3GPP.

- RAN2 to conduct the essential study topics and scoping taking the five aspects (as the subclauses) in the section 2.3 of R2-2507147 as start points for consolidating the WAs and/or WFs across RAN WGs and TSGs.

- Conclusion

According to our internal investigation, for UE EUTRA capability, it took 5 years to reach the max. allowed L2 SDU size; For 5G one, it took only 1 year even though the fallbacks reporting is skipped by default in 5G UE capability signaling. There will be an immediate size explosion problem if we additionally introduce the 6GR operating bands without any CA framework simplification and procedure/signaling design refactoring.

RAN2 is responsible for delivering the clear message to other stakeholders: We must take the 6GR UE capability design very seriously otherwise it would be the day-1 showstopper.

In previous sections we made the following observations and proposals:

Observation 1	BC capability reporting is the main aggressor of UE capability size explosion issue. The filter criteria provided was inadequate for the demands of the interoperability between the network and the UE.

Observation 2	UL segmentation as a last resort to workaround the L2 transport size limitation but could still hit the MTU limitation on the inter-CN interface.

Observation 3	UAI had evolved into a broad framework with diverse parameters and lacks immediate effect transaction, making it unsuitable for urgent UE capability updates during RRC_CONNECTED.

Observation 4	The sole UE-initiated capability updating procedure was suitable only when the UE is in no hurry to do capability change without concerning UX.

Proposal 1	RAN2 to study the UE capability transfer procedure and signaling design managing to ensure the complete 6GR RAT container can be reported without segmentation.

Proposal 2	RAN2 to study the basic RRC procedure that UE can use to initiate the immediate updating of its capabilities without leaving RRC_CONNECTED.

Proposal 3	RAN2 to conduct the essential study topics and scoping taking the five aspects (as the subclauses) in the section 2.3 of R2-2507147 as start points for consolidating the WAs and/or WFs across RAN WGs and TSGs.

- References

- R2-251881	New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- 6GWS-250071	MediaTek Views on 6G Day-1 Radio Aspects