# 3GPP TSG-RAN2 Meeting #131bis	R2-2507173

# Prague, Czech Republic, Oct 13-17, 2025

Agenda Item:	9.2.2

Work Item:	Ambient_IoT_Solutions_Ph2

Source: 	Qualcomm Incorporated

Title: 	Rel-20 A-IoT: Topology 2 aspects

Document for:	Discussion/Decision

# Introduction

Revised WID for Ambient IoT Ph2 is in [1]. One of the objectives for RAN2 is listed as follows:

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: <<skip RAN1 parts>> RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

This document provides our initial views about RAN2 work needed for Rel-20 A-IoT. According to the guidelines by the chair, we limit it only to traffic types DO-DTT and DT (i.e. exclude aspects on DO-A, active devices etc.).

# Discussion

## Rel-20 passive devices

In terms of device types and scenarios, we think Rel-20 work is mostly focused on active devices and new scenarios compared to Rel-19. So, for passive devices, we think Rel-19 A-IoT radio interface would be reused as much as possible.

- Rel-20 work is not expected to enhance operations for passive devices.

So, we propose:

For Rel-20 passive A-IoT device, Rel-19 A-IoT radio interface is baseline.

## Resource configuration for A-IoT Reader

WID says the “Resource allocation to the UE Reader shall be via Uu RRC signaling”.

In general, Uu RRC signalling could be interpreted as both of the following being possible:

- Broadcast signalling such as SIB-based.

- Unicast/dedicated signalling.

On a quick look, it may seem ok to have SIB-provided “resource pools” for A-IoT readers to use. However, we should note that the WID also has explicitly captured “Coordination between UEs operating as UE readers” as “not supported”. In lack of such coordination, it would not be straightforward to avoid a scenario where multiple nearby readers may pick the same resources from the pool and result in interference/collision.

- Since no coordination is supported between the UEs operating as UE Readers, A-IoT resource allocation for UE readers by using SIB-provided resource pools is challenging.

Consequently, we have the following proposal to clarify the WID scope:

Resource allocation to the UE Reader shall be via dedicated Uu RRC signaling.

Now, there are various ways already possible in 5G to configure the resources using dedicated Uu RRC signaling. We think as a baseline, periodic configured resources are needed for A-IoT operation. In addition, one-time resources are also useful.

For resource allocation to the UE Reader, both periodic (e.g. RRC-based SPS-like or CG-like resources) and one-time resources configured by dedicated RRC are supported.

Resource allocation for T2 was briefly discussed during study item phase and the following was captured in TR 38.769 section 6.3.6:

<<skip>> For the radio resources allocation request, following alternatives are studied: - Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN. NOTE 2: The above two alternatives can co-exist. <<skip>>

In our view, it would be beneficial for the NW to be aware of the radio resources needed by a certain reader at a certain time for efficient operation. Based on the UE reader’s need, e.g. to perform an inventory or a command, the UE reader can request the radio resources from the gNB. This request can provide assistance to the gNB scheduler to efficiently allocate the radio resources for the UE reader for the A-IoT operation. In addition, the A-IoT resources that the UE reader is able to utilize may also depend on the deployment information where the reader may have better knowledge. For example, the tags deployed under a certain reader may be able to only work in a certain frequency band, and without such knowledge, gNB providing A-IoT resources autonomously in another band will basically make the A-IoT reader inoperable with the tags.

Figure 1 A-IoT resource configuration to UE reader

- A-IoT resource request from UE reader helps gNB for efficient resource allocation.

UE reader can request A-IoT resources from the gNB. The actual resource allocation is decided by the gNB.

As already mentioned above, coordination between different UEs operating as A-IoT reader is not supported. This would mean the gNB is responsible for allocating A-IoT resources for each UE reader under it. However, it is unclear whether each A-IoT reader will have their dedicated resources (thus guaranteeing no collision) or whether the gNBs can share the same (time/frequency) radio resources across different UEs operating as Reader at the same time.

RAN2 to discuss if gNB sharing the same radio resource across different UEs operating as reader is a possible scenario.

  - Uu and A-IoT coexistence

The WID states “Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band” is not supported. Considering that the gNB is in full control of the resource allocation for the UE readers within its coverage, at the first glance, it is natural expectation that the gNB should ensure time overlapping resources are not configured for a UE operating as a Reader for both Uu and A-IoT interfaces.

However, let’s consider the following scenario. Suppose the gNB has configured certain PRBs for certain number of slots for A-IoT operation. There is no guarantee that there will not be any UL or DL traffic for that UE during this time. If such traffic is delay tolerant, that could be less of an issue, but what if such traffic is critical, e.g. related to emergency call, other control information (e.g. periodic CSI, PHR reports). Making sure that allocated resources for Uu and A-IoT never overlap in time (in the same band) is a huge restriction for NW.

It is worth reclarifying that we are not arguing to change the WI objective or to allow time overlapping operation of Uu and A-IoT (in the same band). What we are saying, instead, is there may be situations where the gNB ends up configuring overlapping resources, but that does not necessarily mean there is time overlapping operation. Then the question is: should this even be allowed (for the NW to configure time overlapping resources for both Uu and A-IoT in the same band)? And if yes, how does the UE handle such situations? E.g. is it left up to UE, or some prioritization rules would need to be defined?

- Simultaneous (time overlapping) operation of Uu and A-IoT at the UE reader in the same band is not supported. However, that may not necessarily mean time overlapping resource allocation for Uu and A-IoT is not allowed.

RAN2 to discuss whether gNB ensures simultaneous (time overlapping) resources are not configured for operation of Uu and A-IoT at the UE reader in the same band.

RAN2 to discuss how UE handles the case if Uu and A-IoT allocated resources overlap on time in the same band.

Another aspect to be considered for the coexistence of Uu and A-IoT operation at the UE reader is the following. Consider a scenario where the Reader finished certain steps of an A-IoT session, but not all the steps. This could happen for various reasons such as the allocated resources were not enough to finish all the steps, there was higher priority interruption due to e.g. emergency call on the Uu side. Now, should the remaining steps of ongoing A-IoT session be allowed to continue in a future resource allocation? It seems reasonable to allow this as long as the future resources is within a reasonable time frame. This is illustrated in the figure below. We would encourage RAN2 to discuss this scenario.

f

t

A-IoT Resource Allocation X

Uu Resource Allocation

A-IoT Resource Allocation X+1

A-IoT Paging with Transaction ID1,

msg1, msg2… up to step M

A-IoT (same session or TX ID),

step M+1, step M+2 … last step.

Uu communication

Figure 2 Uu and A-IoT in the same band for a UE Reader

RAN2 to discuss whether one A-IoT session (e.g. inventory + command) can span across multiple resource allocations (e.g. some steps of A-IoT operation completed in resource X followed by remaining steps in resource X+1).

  - Handling of A-IoT traffic by the UE reader

According to the WID, the UE reader may collect A-IoT data from the device(s) while the UE reader is in RRC_CONNECTED. Then the UE reader needs to send the collected A-IoT data to the AIOTF.

Furthermore, according to the WID, “Specify RRC-based architecture, in coordination with SA2”, which would mean the A-IoT data is sent via RRC messages. That means SRB is used to transfer such A-IoT data.

Then one question is whether the UE reader needs to treat the A-IoT data any differently compared to the UE’s other control traffic. Given the traffic characteristics may be different (e.g. A-IoT data may be delay tolerant while control data may be delay sensitive), there may be need to define a new SRB. Alternatively, existing low priority SRBs (e.g., SRB4) can be reused for this purpose.

- The A-IoT data is transferred over SRB. The traffic characteristics and QoS requirements of A-IoT data may be different from UE’s other control data.

The A-IoT data is transferred by UE to network via RRC message(s) transferred over SRB. RAN2 to discuss if a new SRB is needed for this purpose to handle A-IoT data differently from other control data, or if an existing SRB is reused.

  - UE reader temporary out of connection

Note that UE reader’s temporary out of connection scenario was heavily discussed during the study item phase. And the following was captured for resource allocation in TR 38.769 section 6.3.6 regarding different RRC states and scenarios (highlights added):

For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): - Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration. - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations. NOTE 1: The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.) - Scenario 3: In RRC_ INACTIVE, <<skip>> - Scenario 4: In RRC_ IDLE, <<skip>> NOTE 2: The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios) <<skip>>

As also reconfirmed by the WID, there can be valid scenarios where the UE Reader is temporarily out of connection e.g. due to HO or RLF. E.g., the commands from the AIOTF arrive to the UE Reader at time t1 while it is in coverage; the UE reader starts to perform the actual inventory or command with the A-IoT devices (tags) at t2, but during this time before the A-IoT operation is complete (recall that A-IoT operations can take much longer than couple of milliseconds), the UE Reader suffers RLF. In such case, the UE reader should not be required to prematurely stop all the ongoing A-IoT operations as long as the previously provided resource allocation is still valid. In other words, merely going through temporary interruption in Uu should not require the Reader to abandon A-IoT resources already provided to it by the NW unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured and released the resource, etc.).

Then, after performing the A-IOT operation, the Reader may come back to the coverage at t3 and can successfully transmit the collected A-IoT data to the AIOTF.

Figure 3 UE Reader temporary out of connection

As explained above, and as captured in WID, UE Reader performing A-IoT operations with the A-IoT devices while suffering from temporary interruption in Uu such as HO or RLF is a valid scenario and are explicitly captured as being within the scope of the WI. It is also understood that the WID captures the following “A-IoT radio resource of the UE reader is only valid within the serving cell.” Putting the two together, the intent should be that the A-IoT radio resources of the UE reader is not considered immediately invalid while HO or RLF recovery is ongoing.

To make sure the UE reader doesn’t wonder around or stays in such temporary state for far too long, timer-based resource validity should be introduced.

In short, we prefer option 2 from the TR.

- UE reader going through a HO or RLF event shall not be the sole reason to consider the A-IoT resources as immediately invalid unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

- Timer based resource configuration validity is simple and fully under NW control.

Therefore, we propose:

The A-IoT resources configured by the serving cell are considered as being valid while the UE is ongoing HO or RLF event (i.e. until HO complete or end of RLF recovery procedure or until a validity timer expires).

To fulfil the above principle (that the temporary out of connection should not necessarily lead to dropping of all ongoing A-IoT operations), RAN2 should design solution(s) to enable uninterrupted A-IoT operation during HO or RLF of the UE reader. Details of such solutions can be discussed once the principle is agreed.

RAN2 aims to enable uninterrupted A-IOT operation during HO or RLF of UE reader. FFS how to achieve this.

# Summary

Based on the discussion above, we have following observations and proposals:

Observation 1.	Rel-20 work is not expected to enhance operations for passive devices.

Observation 2.	Since no coordination is supported between the UEs operating as UE Readers, A-IoT resource allocation for UE readers by using SIB-provided resource pools is challenging.

Observation 3.	A-IoT resource request from UE reader helps gNB for efficient resource allocation.

Observation 4.	Simultaneous (time overlapping) operation of Uu and A-IoT at the UE reader in the same band is not supported. However, that may not necessarily mean time overlapping resource allocation for Uu and A-IoT is not allowed.

Observation 5.	The A-IoT data is transferred over SRB. The traffic characteristics and QoS requirements of A-IoT data may be different from UE’s other control data.

Observation 6.	UE reader going through a HO or RLF event shall not be the sole reason to consider the A-IoT resources as immediately invalid unless the resources have become invalid for other reason (e.g. based on timer or NW reconfigured or explicitly released the resource, etc.).

Observation 7.	Timer based resource configuration validity is simple and fully under NW control.

Proposal 1:	For Rel-20 passive A-IoT device, Rel-19 A-IoT radio interface is baseline.

Proposal 2:	Resource allocation to the UE Reader shall be via dedicated Uu RRC signaling.

Proposal 3:	For resource allocation to the UE Reader, both periodic (e.g. RRC-based SPS-like or CG-like resources) and one-time resources configured by dedicated RRC are supported.

Proposal 4:	UE reader can request A-IoT resources from the gNB. The actual resource allocation is decided by the gNB.

Proposal 5:	RAN2 to discuss if gNB sharing the same radio resource across different UEs operating as reader is a possible scenario.

Proposal 6:	RAN2 to discuss whether gNB ensures simultaneous (time overlapping) resources are not configured for operation of Uu and A-IoT at the UE reader in the same band.

Proposal 7:	RAN2 to discuss how UE handles the case if Uu and A-IoT allocated resources overlap on time in the same band.

Proposal 8:	RAN2 to discuss whether one A-IoT session (e.g. inventory + command) can span across multiple resource allocations (e.g. some steps of A-IoT operation completed in resource X followed by remaining steps in resource X+1).

Proposal 9:	The A-IoT data is transferred by UE to network via RRC message(s) transferred over SRB. RAN2 to discuss if a new SRB is needed for this purpose to handle A-IoT data differently from other control data, or if an existing SRB is reused.

Proposal 10:	The A-IoT resources configured by the serving cell are considered as being valid while the UE is ongoing HO or RLF event (i.e. until HO complete or end of RLF recovery procedure or until a validity timer expires).

Proposal 11:	RAN2 aims to enable uninterrupted A-IOT operation during HO or RLF of UE reader. FFS how to achieve this.

# References

[1] RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, RAN#109, Sep 2025

[2] TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Release 19

[3] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18