3GPP TSG-RAN WG2 Meeting #131bis	R2-2507181

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.1.2.2

Source:	Nokia

Title:	On Simplifying Procedures and ASN.1 for AI/ML

WID/SID:	NR_AIML_air-core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc proposes changes to the applicability determination procedure, the provision and indication of preferred data collection candidates, and the configuration for NW-side data collection.

# 2	Discussion

## 2.1		Applicability reporting

The applicability report is transmitted using the following ASN.1 definition as of the latest RRC CR.

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..maxNrofServingCells) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19                  ServCellIndex,

applicabilityInfoReportList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports-r19)) OF ApplicabilityInfoReport-r19     OPTIONAL,

...

}

ApplicabilityInfoReport-r19 ::=    SEQUENCE {

applicabilityInfoReportId-r19          CHOICE {

csi-ReportConfigId-r19                     CSI-ReportConfigId,

applicabilitySetId-r19                     ApplicabilitySetConfigId-r19,

spare2                                     NULL,

spare1                                     NULL

},

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19             ENUMERATED {true}                                                          OPTIONAL,

...

}

The report thus consists of a Sequence of Sequences. The outer Sequence, ApplicabilityReportList, is essentially a list of cells and the inner Sequence, applicabilityInfoReportList, is a list of applicability reports for the cell indicated in the outer Sequence. The applicabilityCellId is redundant information at least for CSI-ReportConfig, which includes a field called carrier, which points to the serving cell index of the cell transmitting the RS. Since many fields have been copied from CSI-ReportConfig to ApplicabilitySetConfig, it is natural to include carrier and eliminate this level of hierarchy.

Observation 1: The field applicabilityCellId in ApplicabilityInfoReport is redundant for CSI-ReportConfig and the field which makes it redundant, carrier, could be included in ApplicabilitySetConfig, where to many parameters were copied from CSI-ReportConfig,  instead of inside ApplicabilityReport.

The changes have been manually applied to the same ASN.1 below for illustration.

ApplicabilityReportList-r19 ::=   SEQUENCE (SIZE (1..maxNrofServingCellsmaxNrofApplicabilityReports-r19) OF ApplicabilityReport-r19

ApplicabilityReport-r19 ::=       SEQUENCE {

applicabilityCellId-r19                  ServCellIndex,

applicabilityInfoReportList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilityReports-r19)) OF ApplicabilityInfoReport-r19     OPTIONAL,

...

}

ApplicabilityInfoReport-r19 ::=    SEQUENCE {

applicabilityInfoReportId-r19          CHOICE {

csi-ReportConfigId-r19                          CSI-ReportConfigId,

applicabilitySetId-r19 [RIL]: N027 AIML         ApplicabilitySetConfigId-r19,

spare2                                          NULL,

spare1                                          NULL

},

applicabilityStatus-r19                        ENUMERATED {applicable, inapplicable},

releaseConfigurationPreference-r19             ENUMERATED {true}                                                          OPTIONAL,

...

}

Proposal 1: Eliminate a level of hierarchy from ApplicabilityReportList by replacing the contents of the existing ApplicabliityReport-r19 with those of ApplicabilityInfoReport-r19.

If the previous proposal is agreed, because the procedures loop over cell ID, the procedures can be simplified by removing the loop over serving cells.

Observation 2: Removing applicabilityCellId and its associated layer of hierarchy, the procedures can be simplified by eliminating the loop over serving cells.

Additionally, we think that certain wording can be collapsed for simplicity. First, the procedure starts with a chain of 4 conditions combined with OR logic. The (numbers) refer to lines in the excerpt of Section 5.3.5 from the latest review file for TS 38.331. The first (1) and third (3) address new and previously configured CSI-ReportConfig, while the second (2) and fourth (4) address new and previously configured ApplicabilitySetConfig. We think the first and third and the second and fourth could be combined and treated as loops instead of introductions to the two loops later on.

Note that all of the configurations currently considered are only ever configured on the PCell since the reports from our use cases are sent to the PCell, which hosts the uplink channel.

The following numbered excerpt from Section 5.3.5, RRC Reconfiguration, of the latest RRC CR is provided as a reference. The second loop, which addresses the sets of inference-related parameters was omitted for brevity.

- 2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or

- 2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

- 2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or

- 2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

- 3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

- 4>	set the applicabilityCellId to the serving cell index of the cell;

- 4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

- 5>	include an entry in the applicabilityInfoReportList and set the content as follows:

- 6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

- 6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

- 6>	if the applicabilityStatus is set to inapplicable:

- 7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

Lines 1 and 3 could be combined with lines 5 and 7 to create the first loop (1-6) and lines 2 and 4 could be combined with lines 5 and 7 to create the second loop (7-11) as shown below.

- 2>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

- 3>	include an entry in the applicabilityInfoReportList and set the content as follows:

- 4>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

- 4>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

- 4>	if the applicabilityStatus is set to inapplicable:

- 5>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

- 2>	for each entry within applicabilitySetConfigList, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

- 3>	include an entry in the applicabilityInfoReportList and set the content as follows:

- 4>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

- 4>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

- 4>	if the applicabilityStatus is set to inapplicable:

- 5>	if the UE prefers to release the concerned ApplicabilitySettConfig, include releaseConfigurationPreference;

Observation 3: Removing the loops over cell IDs and combining the conditional statements with the for loops, two levels of hierarchy can be eliminated from the applicability reporting procedure.

Proposal 2: Remove the loop over cell IDs from the applicability reporting procedure of Section 5.3.5 to match the ASN.1 changes from Proposal 1.

Proposal 3: Combine the if statements for new and existing CSI-ReportConfigs in the applicability reporting procedure of Section 5.3.5 and combine the result with the loop over the same configurations to reduce the level of hierarchy by one level.

Proposal 4: If Proposal 2 and Proposal 3 are agreed, adopt the text proposal of Annex A.

Sets of inference-related parameters are signalled as ApplicabilitySetConfigs in a list contained in ApplicabilityConfig-r19, which configures the SCellIndex. As with the ApplicabilityReport, we think that the additional hierarchy is unnecessary and that the SCellIndex, or carrier should be included in the ApplicabilitySetConfig directly. Then, one list of ApplicabilitySetConfigs can be signalled in applicabilityConfigList in ApplicabilityReportConfig directly, removing another layer of hierarchy.

ApplicabilityReportConfig-r19 ::= SEQUENCE {

reportApplicabilityUAI-r19     ENUMERATED {true} [RIL]: E042, AIML                                                                   OPTIONAL, -- Need R

applicabilitySetConfigList-r19    SEQUENCE (SIZE (1..maxNrofServingCellsApplicabilitySets-r19)) OF ApplicabilitySetConfig-r19                  OPTIONAL, --

...

}

ApplicabilityConfig-r19 ::= SEQUENCE {

applicabilityConfigCellId-r19       ServCellIndex                                                                      OPTIONAL, -- Need R[RIL]: B205, AIML

applicabilitySetConfigList-r19      SEQUENCE (SIZE (1..maxNrofApplicabilitySets-r19)) OF ApplicabilitySetConfig-r19    OPTIONAL, -- Need R[RIL]: C084, AIML

...

}

ApplicabilitySetConfig-r19 ::= SEQUENCE {

applicabilitySetConfigId-r19                ApplicabilitySetConfigId-r19                            OPTIONAL,   -- Need R

carrier                                     ServCellIndex,

resourcesForChannelMeasurement              CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

Proposal 5: Rename applicabilityConfigList-r19 in ApplicabilityReportConfig-r19 to applicabilitySetConfigList-r19, storing a SEQUENCE of ApplicabilitySetConfig-r19. Add a new field called carrier to ApplicabilitySetConfig-r19 with the type ServCellIndex to refer to the correct resources.

## 2.2	Data collection configuration candidates

The indication of preferred data collection configuration candidates is performed differently depending on whether to indicate a preference or to stop the indication of a preference, and there is a start command ambiguous in nature.

The ASN.1 of DataCollectionPreference-r19 is copied below for reference.

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStart-r19                          ENUMERATED {start}                   OPTIONAL,

dataCollectionPreferredConfigurationList-r19     SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19      OPTIONAL,

dataCollectionStopConfigurationList-r19          SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19               OPTIONAL,

...

}

The field dataCollectionStart-r19 is defined in the field descriptions as indicating “the UE’s preference to be configured with radio resource for UE-side data collection”. However, the UE also indicates its preference to be configured with radio resources for UE-side data collection by indicating one or more preferred dataCollectionCandidateConfigId-r19. Therefore, there is no apparent purpose to dataCollectionStart-r19and it should thus be removed.

Proposal 6: Remove dataCollectionStart-r19 from DataCollectionPreference-r19.

The field dataCollectionStopConfigurationList-r19 is a list of CSI-ReportConfigIds, grouped by serving cell. It is assumed that the UE can associate a UE-side data collection configuration from a UE-side data collection candidate configuration, so there is no technical issue with having a different ID for the start list and the stop list. However, there is an ambiguity in behaviour if the UE does not remove the corresponding DataCollectionCandidateConfigId from the dataCollectionPreferredConfigurationList. And if the UE is able to map between the two IDs, the gNB certainly can. Therefore, we propose to simplify the preference indication to a single list of preferences, wherein the exclusion of a previous preference means to stop and the inclusion of a new preference means to start.

The related ASN.1 and proposed changes are shown below.

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStart-r19                          ENUMERATED {start}                   OPTIONAL,

dataCollectionPreferredConfigurationList-r19     SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19      OPTIONAL,

dataCollectionStopConfigurationList-r19          SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19               OPTIONAL,

...

}

Proposal 7: Simplify the UE-side data collection preference indication to a single list of dataCollectionPreferredConfigurationList-r19 wherein the exclusion of a previously included preference indicates “stop”, the inclusion of a previously excluded preference indicates “start”, and the inclusion of a previously included or the exclusion of a previously excluded preference indicates no change.

Lastly, we find the grouping by cell ID to be unnecessary. The UE-side data collection configurations map to CSI-ReportConfig, which includes the field carrier, which is the serving cell index. Because fields have already been copied from CSI-ReportConfig, and carrier is part of CSI-ReportConfig, we find it reasonable to copy it into DataCollectionCandidateConfigParameters.

The related ASN.1 and proposed changes are shown below.

DataCollectionPreferenceConfig-r19 ::= SEQUENCE {

dataCollectionCandidateConfigList-r19   SEQUENCE (SIZE (1..maxNrofServingCellsmaxNrofDataCollectionCandidateConfigs-r19)) OF DataCollectionCandidateConfig-r19   OPTIONAL, -- Need R

...

}

DataCollectionCandidateConfig-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19                  ServCellIndex,

dataCollectionCandidateConfigParameterList-r19   SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19  OPTIONAL, -- Need R[RIL]: X004 AIML

...

}

DataCollectionCandidateConfigParameters-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19             ServCellIndex,

dataCollectionCandidateConfigId-r19         DataCollectionCandidateConfigId-r19,

resourcesForChannelMeasurement              CSI-ResourceConfigId                                     OPTIONAL, -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                     OPTIONAL, -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                         OPTIONAL, -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                         OPTIONAL, -- Need R

...

}

Proposal 8: Add a field called carrier, with the type ServCellIndex to DataCollectionCandidateConfigParameters-r19 and remove the additional hierarchy resulting from the grouping of UE-side data collection candidate configurations.

# 3	Conclusion

This document has made the following observations:

Observation 1: The field applicabilityCellId in ApplicabilityInfoReport is redundant for CSI-ReportConfig and the field which makes it redundant, carrier, could be included in ApplicabilitySetConfig, where to many parameters were copied from CSI-ReportConfig,  instead of inside ApplicabilityReport.

Observation 2: Removing applicabilityCellId and its associated layer of hierarchy, the procedures can be simplified by eliminating the loop over serving cells.

Observation 3: Removing the loops over cell IDs and combining the conditional statements with the for loops, two levels of hierarchy can be eliminated from the applicability reporting procedure.

And proposed the following:

Proposal 1: Eliminate a level of hierarchy from ApplicabilityReportList by replacing the contents of the existing ApplicabliityReport-r19 with those of ApplicabilityInfoReport-r19.

Proposal 2: Remove the loop over cell IDs from the applicability reporting procedure of Section 5.3.5 to match the ASN.1 changes from Proposal 1.

Proposal 3: Combine the if statements for new and existing CSI-ReportConfigs in the applicability reporting procedure of Section 5.3.5 and combine the result with the loop over the same configurations to reduce the level of hierarchy by one level.

Proposal 4: If Proposal 2 and Proposal 3 are agreed, adopt the text proposal of Annex A.

Proposal 5: Rename applicabilityConfigList-r19 in ApplicabilityReportConfig-r19 to applicabilitySetConfigList-r19, storing a SEQUENCE of ApplicabilitySetConfig-r19. Add a new field called carrier to ApplicabilitySetConfig-r19 with the type ServCellIndex to refer to the correct resources.

Proposal 6: Remove dataCollectionStart-r19 from DataCollectionPreference-r19.

Proposal 7: Simplify the UE-side data collection preference indication to a single list of dataCollectionPreferredConfigurationList-r19 wherein the exclusion of a previously included preference indicates “stop”, the inclusion of a previously excluded preference indicates “start”, and the inclusion of a previously included or the exclusion of a previously excluded preference indicates no change.

Proposal 8: Add a field called carrier, with the type ServCellIndex to DataCollectionCandidateConfigParameters-r19 and remove the additional hierarchy resulting from the grouping of UE-side data collection candidate configurations.

# Annex A	Text proposal for 5.3.5 on Applicability Reporting

2>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

3>	include an entry in the applicabilityInfoReportList and set the content as follows:

4>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

4>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

4>	if the applicabilityStatus is set to inapplicable:

5>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

2>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

3>	include an entry in the applicabilityInfoReportList and set the content as follows:

4>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

4>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

4>	if the applicabilityStatus is set to inapplicable:

5>	if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;