3GPP TSG RAN WG2 Meeting #131bis                                                         R2-2507186

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	10.3.1

Source:	Ofinno

Title:	Overview of User Plane

Document for:	Discussion and decision

# Introduction

This contribution outlines our initial views of User Plane (UP) according to the guidelines provided in the 6G SID for RAN [1], as highlighted below:

- Overview of User Plane architecture

  - Limitations observed in 5G UP architecture.

  - 6G UP architecture design principle.

- Overview of User Plane functions

  - 5G main functions of SDAP, lessons from 5G, and expectations in 6G.

  - 5G main functions of PDCP, lessons from 5G, and expectations in 6G.

  - 5G main functions of RLC, lessons from 5G, and expectations in 6G.

  - 5G main functions of MAC, lessons from 5G, and expectations in 6G.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. (3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signaling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

# Discussion

## Overview of User Plane Architecture

Figure 1: Limitations observed in 5G UP architecture

The 5G UP was designed with a layered structure consisting of SDAP, PDCP, RLC, and MAC sublayers. While this architecture has served a wide variety of services, several limitations have been observed:

- Complexity and overhead: Multiple sublayers, each with their own headers, add latency and processing cost. This is especially burdensome for low-power or reduced-capability devices (e.g., RedCap, LPWA, A-IoT).

- No scalability across device types: 6G must cater to both ultra-low complexity devices (kbps-level, long battery life) and multi-Gbps advanced devices (e.g., XR). A uniform UP design is too heavy for simple devices while lacking flexibility for high-end use cases.

- Specific service type inefficiency: Some service types (e.g., AI, sensing) gain little from existing UP functions yet still incur their overhead. Since the UP is optimized around IP traffic, handling non-traditional data such as sensing signals or AI feature vectors leads to resource waste and higher latency.

- Cross-layer functions lack of integration: Functions such as PDCP duplication, RLC ARQ, and MAC HARQ are all for reliability increasement, but they are tightly bound to fixed layers, creating redundancy and inefficiency. For instance, retransmissions are split between RLC and MAC, often leading to suboptimal performance.

For 6G, the UP should be lean, modular, and flexible, with a design that enables:

- Baseline essential functionality for all device types.

- Add-on functional modules that can be composed based on device/service requirements.

- Cross-layer coordinated/integrated UP functions to increase the efficiency.

Study and identify essential UP functions for all device/service types.

Study modular/flexible/scalable UP functions for diverse device/service types.

Study cross-layer coordinated/integrated UP functions.

## Overview of User Plane Functions

### Overview of SDAP Functions

5G main functions of SDAP [2]

Figure 2: 5G main functions of SDAP

- Mapping between QoS flows and RBs: For DL and UL, SDAP maps QoS flows to DRBs.

- Marking QoS flow IDs in packets: SDAP marks the QoS flow ID (QFI) in both DL and UL: packets, which is essential for QoS handling. This is done by adding an SDAP header to the data packet when configured.

- Reflective QoS Flow to DRB Mapping: For UL data, SDAP supports a reflective mapping mechanism. This allows a UE to monitor the QoS flow to DRB mapping used in the DL and apply the same mapping rule for the UL. This function is performed at the UE if a DL SDAP header is configured.

Lessons from 5G and expectations in 6G

In 5G, SDAP mainly classifies traffic based on QoS flows. In 6G, traffic may not fit neatly into existing QoS categories (e.g., AI training vs inference, digital twin state updates, XR interactive frames). RAN2 should study how it can support service-aware QoS flow mapping that goes beyond delay/jitter and explicitly distinguishes traffic classes like AI, sensing, or XR.

5G QoS mapping is largely static (configured via RRC/NAS). For example, static SDAP-to-DRB mapping struggles to accommodate emerging multi-modal service flows. In 6G, XR traffic/AI-driven RAN control may dynamically re-map flows. RAN2 should explore mechanisms for flexible QoS flow mapping policies.

6G will feature tighter TN/NTN integration and multi-TRP operation. SDAP (or an evolved equivalent) may need to support cross-link QoS flow mapping or distributed QoS enforcement across multiple access paths. RAN2 should investigate how SDAP (or an evolved equivalent) can support distributed bearer mapping and ensure QoS consistency across multiple access legs.

For SDAP (or an evolved equivalent) functions, study at least:

- Service-aware QoS flow mapping for distinguishing traffic classes.

- Dynamic/flexible QoS flow mapping for multi-modal/AI traffics.

- Distributed QoS flow mapping to ensure QoS consistency across multiple access legs.

### Overview of PDCP Functions

5G main functions of PDCP[3]

Figure 3: 5G main functions of PDCP

- Security functions:

  - Ciphering and deciphering: PDCP performs encryption and decryption for both user plane data (on DRBs) and control plane data (on SRBs), if configured. This applies to the MAC-I and the data part of the PDCP Data PDU.

  - Integrity protection and verification: This function provides integrity for control plane data on SRBs and can be configured for user plane data on DRBs.

- Header compression: PDCP can compress and decompress packet headers. The protocols supported include:

  - ROHC (RObust Header Compression) for IP packets.

  - EHC (Ethernet Header Compression) for Ethernet packets.

  - UDC (Uplink Data Compression), which is a DEFLATE-based compression for uplink user data.

- Reordering and in-order delivery: The receiving PDCP entity can reorder received packets to ensure they are delivered in sequence to the upper layers. A reordering timer is used to detect lost packets.

- Duplicate discarding: PDCP is responsible for discarding duplicate packets received from lower layers.

- SDU/PDU set Discard: PDCP supports a timer-based SDU discard function, where SDUs that have been buffered for too long are discarded. A PDU set discard operation was introduced in Rel-19 for XR traffic. The UE may discard all packets in a PDU set when one PDU belonging to this PDU set is discarded due to discard timer expiry.

- Routing and PDCP duplication:

  - For split bearers in DC or DAPS bearers, the transmitting PDCP entity performs routing, deciding which path (e.g., MCG or SCG) to send the data on.

  - PDCP also supports packet duplication, where the same PDCP PDU is sent over multiple RLC entities to increase reliability, a feature particularly useful for URLLC services.

- Status reporting: PDCP can generate and process status reports to handle retransmissions and confirm successful delivery of data.

- Data recovery: PDCP supports a data recovery procedure, for instance, after a handover, to retransmit PDCP Data PDUs that were not successfully delivered by the source cell.

Lessons from 5G and expectations in 6G

5G PDCP security provides confidentiality (ciphering) and authenticity (integrity protection). In 5G, ciphering applies to DRBs/SRBs; integrity is mandatory for SRBs, optional for DRB. Data confidentiality and integrity are baseline requirements for mobile networks. 6G will handle highly sensitive services (e.g., industrial automation, immersive XR, AI/ML model distribution, critical IoT). However, not all devices/services need the same level of security (e.g., low-power IoT vs mission-critical XR). Furthermore, application-layer encryption (end-to-end) may overlap with PDCP security. SA3 is studying security for the 6G System. RAN2 can study scalable security profiles (full, partial, lightweight), and study per-service/per-flow security differentiation.

5G header compression/decompression reduces overhead by compressing repetitive header fields. 5G supports ROHC (IP), EHC (Ethernet), UDC (uplink data). Without compression, header overhead consumes significant air interface resources. 6G will carry new types of traffic (AI/ML updates, XR scene graphs, sensing data). 5G schemes are not optimized for these. 6G research can explore service-specific compression profiles, e.g., tailored for XR, AI, and sensing applications. Moreover, cross-layer compression schemes, where PHY/MAC layer information enhances PDCP layer compression, warrant further study.

In 5G, PDCP has a timer-based discard of SDU/PDU set that exceed latency budget. In 6G, the static discard timers may not meet all service needs. 6G should study adaptive SDU/PDU (set) discard mechanisms.

For PDCP (or an evolved equivalent) functions, study at least:

- Security on scalable security profiles and per-service/per-flow security differentiation.

- Service-specific header compression and cross-layer compression schemes.

- Adaptive SDU/PDU (set) discard mechanisms.

### Overview of RLC Functions

5G main functions of RLC [4]

Figure 4: 5G main functions of RLC

- Data transfer modes: The RLC sublayer supports three distinct modes for data transfer:

  - Transparent Mode (TM): Used for logical channels like BCCH, PCCH, and CCCH. It involves no segmentation and adds no RLC headers; it simply passes RLC SDUs to lower layers.

  - Unacknowledged Mode (UM): Used for DTCH, MCCH, MTCH, SCCH, and STCH. It supports segmentation and reassembly of RLC SDUs but does not provide acknowledged delivery.

  - Acknowledged Mode (AM): Used for DCCH, DTCH, SCCH, and STCH. It is the most complex mode, providing reliable data delivery.

- Error correction through ARQ: In AM, RLC performs ARQ. The receiving RLC entity can detect lost PDUs and request retransmissions from its peer entity by sending a STATUS PDU. This is a key function for ensuring reliable data delivery.

- Segmentation, re-segmentation, and reassembly:

  - For UM and AM, the RLC sublayer segments RLC SDUs into smaller RLC PDUs to fit the transmission opportunities indicated by the MAC layer.

  - The receiving entity reassembles these segments back into the original RLC SDU. A reassembly timer is used to detect the loss of PDUs.

  - In AM, RLC can also re-segment an already transmitted RLC SDU segment into smaller segments for retransmission.

- Duplicate detection: In AM, the receiving RLC entity is responsible for detecting and discarding duplicate RLC PDUs.

- RLC SDU discard: Both UM and AM support the discarding of RLC SDUs, which can be triggered by PDCP.

Lessons from 5G and expectations in 6G

In 5G, RLC defines three rigidly separated transfer modes (TM, UM, AM), each optimized for a narrow set of use cases. The problem is that services don’t always map neatly into these categories. In short, 5G modes are coarse-grained: the choice is “no reliability,” “full reliability,” or “transparent.” There is no in-between or tunable mechanism for services with intermediate reliability/latency needs. 6G will support a wider range of service categories than 5G. It has heterogeneous reliability, latency, and energy requirements. RAN2 can study a framework (e.g., configurable/dynamic transfer mode) where UEs and gNBs can negotiate reliability levels.

RLC ARQ complements MAC HARQ by providing reliability at longer timescales. HARQ ensures rapid retransmission but is limited by PHY/MAC timing constraints and HARQ process numbers. ARQ in AM mode ensures more reliable delivery when HARQ cannot recover errors, especially in challenging conditions like poor radio links, mobility, or interference. Future ARQ design in 6G should be service-aware and adaptive. For example, study QoS-aware ARQ policies: aggressive retransmissions for ultra-reliable services (URLLC/AI control), relaxed or early discard for XR and conversational AI (where stale packets reduce QoE). RAN2 can also investigate predictive ARQ, using AI-based channel prediction to minimize unnecessary retransmissions. Furthermore, the cross-Layer coordination/integration between MAC HARQ and RLC ARQ to avoid redundant retransmissions can be studied. For example, a unified retransmission budget shared across HARQ and ARQ, so that the system optimizes total retransmissions instead of each layer working in isolation.

For RLC (or an evolved equivalent) functions, study at least:

- Configurable/dynamic transfer mode.

- QoS-aware ARQ policies and AI-driven/predictive ARQ.

- Cross-layer coordination/integration between ARQ and HARQ.

### Overview of MAC Functions

5G main functions of MAC [5]

Figure 5: 5G main functions of MAC

- Random Access (RA) procedure: MAC initiates and manages the RA procedure, which is triggered for different events, e.g., initial access, re-establishment, handover, and when UL synchronization is lost, etc.

- Scheduling and resource allocation:

  - MAC handles logical channel prioritization (LCP), deciding which data to send first based on priority and token-bucket algorithm. Logical channel mapping restrictions can also be configured to control which resources a logical channel can use.

  - MAC supports Semi-Persistent Scheduling (SPS) for downlink and Configured Grants (CG) (Type 1 and Type 2) for uplink, allowing for pre-allocated resources without dynamic grants.

- Data multiplexing: MAC multiplexes MAC SDUs from various logical channels into TBs to be sent to the physical layer. In the reverse direction, it demultiplexes TBs into MAC SDUs for the correct logical channels.

- Error correction through HARQ: MAC performs HARQ for error correction on the DL-SCH and UL-SCH. This involves managing HARQ processes for retransmissions.

- Information reporting: MAC triggers Scheduling Request (SR), Buffer Status Reports (BSR), Delay Status Report (DSR), Power Headroom Reports (PHR), and Timing Advance Reporting (TAR) from the UE to assist the network.

- Power saving: MAC manages DRX to save UE power, controlling when the UE monitors the PDCCH.

- BWP/Cell adaptation: MAC preforms BWP switch and SCell activation/deactivation to adapt the resources in frequency/spatial domain.

Lessons from 5G and expectations in 6G

The RA procedure in 5G can suffer from high collision rates in massive device scenarios and long access delays in NTN or aerial networks. 6G should study RA enhancements on preamble selection (e.g., AI-assisted) and service-aware RA mechanisms (e.g., faster RA for XR, energy-optimized RA for IoT). 6G must support more heterogeneous traffic types (AI/ML updates, XR interactive frames, sensing data).

In 6G, new service types may require more flexible mapping frameworks, e.g., how to map logical channels to upper layer/lower layer channels. RAN2 should study dynamic multiplexing strategies optimized for latency-critical vs bulk traffic and study adaptive CG/SPS parameters/configurations.

In 6G, RAN2 should study AI-driven dynamic scheduling for increasing resource allocation accuracy. RAN2 can also investigate predictive HARQ, e.g., using AI-based channel prediction to minimize unnecessary retransmissions, to enhance reliability performance. There is also a need to study how scheduling interacts with multi-link resource allocation, e.g., including TN/NTN spectrum sharing. The information/status reports (e.g., SR/BSR/DSR/PHR/TAR, etc.) should be smarter and less overhead-intensive. RAN2 should study predictive or AI-driven reporting, e.g., where UEs report only when thresholds are exceeded or based on predicted traffic patterns. 6G should also study AI-driven DRX adaptation (predicting traffic to optimize sleep/wake cycles), service-aware power saving (e.g., XR vs IoT), and coordinated multiple DRX configurations across multi-TRP/Carrier/Cell/RATs to reach the optimal power saving gain.

For MAC (or an evolved equivalent) functions, study at least:

- RA enhancements on preamble selection and service-aware RA.

- Flexible logical channel mapping, dynamic multiplexing strategies, multi-link resource allocation, and adaptive CG/SPS parameters.

- AI-driven/predictive RA, scheduling/HARQ and information/status reporting.

- AI-driven/predictive DRX adaptation and coordinated multiple DRX configurations across multi-TRP/Carrier/Cell/RATs.

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	Study and identify essential UP functions for all device/service types.

Proposal 2	Study modular/flexible/scalable UP functions for diverse device/service types.

Proposal 3	Study cross-layer coordinated/integrated UP functions.

Proposal 4	For SDAP (or an evolved equivalent) functions, study at least:

- Service-aware QoS flow mapping for distinguishing traffic classes.

- Dynamic/flexible QoS flow mapping for multi-modal/AI traffics.

- Distributed QoS flow mapping to ensure QoS consistency across multiple access legs.

Proposal 5	For PDCP (or an evolved equivalent) functions, study at least:

- Security on scalable security profiles and per-service/per-flow security differentiation.

- Service-specific header compression and cross-layer compression schemes.

- Adaptive SDU/PDU (set) discard mechanisms.

Proposal 6	For RLC (or an evolved equivalent) functions, study at least:

- Configurable/dynamic transfer mode.

- QoS-aware ARQ policies and AI-driven/predictive ARQ.

- Cross-layer coordination/integration between ARQ and HARQ.

Proposal 7	For MAC (or an evolved equivalent) functions, study at least:

- RA enhancements on preamble selection and service-aware RA.

- Flexible logical channel mapping, dynamic multiplexing strategies, multi-link resource allocation, and adaptive CG/SPS parameters.

- AI-driven/predictive RA, scheduling/HARQ and information/status reporting.

- AI-driven/predictive DRX adaptation and coordinated multiple DRX configurations across multi-TRP/Carrier/Cell/RATs.

# Reference

3GPP RP-252476, “Revised SID: Study on 6G Radio (NTT DOCOMO, CMCC, AT&T, Vodafone).”

3GPP TS 37.324: “NR; Service Data Adaptation Protocol (SDAP) specification.”

3GPP TS 38.323: “NR; Packet Data Convergence Protocol (PDCP) specification.”

3GPP TS 38.322: “NR; Radio Link Control (RLC) protocol specification.”

3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification.”