3GPP TSG RAN WG2 Meeting #131bis                                                       R2-2507188

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	10.3.3

Source:	Ofinno

Title:	Overview of Common User Plane and Control Plane

Document for:	Discussion and decision

# Introduction

This contribution outlines our initial views on the following topics according to the guidelines provided in the 6G SID for RAN [1], as highlighted below:

- Energy saving

  - UE power saving schemes in 5G, lessons learned from 5G, and expectations in 6G

  - Network energy saving schemes in 5G, lessons learned 5G, and expectations in 6G

- Data transfer design

  - 5G data transfer, lessons learned from 5G, and expectations in 6G

- AI/ML framework

  - 5G AI/ML framework for the air interface and what 5G AI/ML framework can be leveraged to 6G

- Access stratum security

  - SA3 6G security study, lessons learned from AS security in 5G, expectations on AS security in 6G

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. (3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signaling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc.) [RAN2, RAN3] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etch… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etic…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

# Discussion

## Energy saving

### UE Power Saving Schemes in 5G

According to [2][3], UE power saving mechanisms aim to reduce battery consumption by adapting UE behavior based on traffic patterns and network configurations. The table below summarizes 5G key UE power saving schemes.

| 5G key UE power saving schemes |  |
| --- | --- |
| Discontinuous Reception (DRX) | This is a fundamental power-saving feature where the UE does not have to continuously monitor the PDCCH. DRX is defined by parameters like DRX on-duration timer, DRX inactivity-timer, and DRX cycle. The total time the UE monitors PDCCH is called Active Time. DRX can be configured in two DRX groups for different sets of Serving Cells. |
| Bandwidth Adaptation (BA) | The UE can be configured with one or more BWPs, which are subsets of the total cell bandwidth. Only one BWP is activated at a time, allowing the UE to operate on a narrower bandwidth and save power, especially during periods of low activity. Switching between BWPs is triggered by RRC signaling, DCI, an BWP inactivity timer, or RA initiation. |
| DCI with CRC scrambled by PS-RNTI (DCP) | To further reduce power consumption during DRX, the UE can be configured to monitor a DCP before its DRX on-duration. If the DCP indicates a specific value (or the UE does not detect the DCP), the UE can skip monitoring the PDCCH during the next on-duration. The DCP is only configured on the PCell and/or PSCell. |
| Cross-Slot Scheduling (CSS) | The cross-slot scheduling allows the gNB to schedule PDSCH or PUSCH in a slot later than the slot carrying the scheduling DCI. This provides the UE with "micro-sleep" opportunities between PDCCH decoding and data reception/transmission, thus saving UE power. |
| SCell Activation/Deactivation | In Carrier Aggregation (CA), Secondary Cells (SCells) can be activated or deactivated via a MAC CE. When an SCell is deactivated, the UE does not need to monitor its PDCCH/PDSCH or perform CQI measurements for the SCell. |
| Dormant BWP | The primary goal of a dormant BWP (one of the UE's dedicated BWPs) is to allow for quick SCell activation while enabling UE power saving. When the active BWP of an activated SCell is the dormant BWP, the UE alters its behavior to conserve power. Specifically, the UE stops monitoring the PDCCH and ceases transmissions on the PDSCH, PUCCH, and SRS on that SCell. |
| Adaptation of MIMO Layers | UE power saving can be achieved by adapting the maximum number of downlink MIMO layers, for example, by switching to a BWP configured with fewer layers. |
| UE Assistance Information (UAI) | The UE can send assistance information to the network indicating its preference for certain power-saving configurations, such as changes to DRX parameters, a reduced number of component carriers, or fewer MIMO layers. It can also indicate overheating issues. |
| Paging DRX | In RRC_IDLE and RRC_INACTIVE, the UE monitors for paging messages only during specific Paging Occasions (POs) within a DRX cycle, allowing it to sleep at other times. |
| Extended DRX (eDRX) | The eDRX feature allows for much longer DRX cycles, up to nearly three hours (10485.76 seconds), significantly reducing power consumption for UEs with infrequent data traffic. |
| Paging Early Indication (PEI) | To reduce power consumed by monitoring paging for other UEs, a group of UEs sharing a PO can be divided into subgroups. A UE only needs to monitor its PO if its subgroup is indicated in a PEI broadcast before the PO. |
| Radio Resource Management (RRM) Measurement Relaxation | UEs in RRC_IDLE and RRC_INACTIVE can relax RRM measurements of neighboring cells when they meet certain criteria, such as low mobility or not being at the cell edge. This relaxation is also available for RLM/BFD in RRC_CONNECTED state. |
| Tracking Reference Signal (TRS) | The network can provide configurations for TRS in SIB17. This may allow UEs in RRC_IDLE/RRC_INACTIVE to sleep longer before waking up for their paging occasion. |
| Low-Power Wake-Up Signal and Receiver (LP-WUS/WUR) | This involves a secondary, LP-WUR that continuously or discontinuously monitors for a specific LP-WUS. This allows the UE's main radio to remain in an "ultra-deep sleep" state until woken up by the LP-WUS, offering substantial power savings compared to legacy DRX. |

Lessons Learned from UE Power Saving in 5G

- Fragmented feature introduction limited effectiveness: Features like DRX/eDRX/DCP, dormant BWP, and LP-WUS were introduced incrementally across different releases, leading to lack of holistic coordination. Backward compatibility limited flexibility in optimizing signaling and reference signals.

- Trade-off between power saving and latency/QoS: Aggressive DRX or eDRX cycles save battery but risk higher latency for paging or data recovery. UEs often had to choose between user experience (low delay) and battery savings.

- Complexity and signaling overhead: Multiple timers (e.g., DRX timers, BWP timers, SCell timers) created tuning challenges. Frequent switching between Active Time/BWPs/SCells could consume more energy than saved if not optimized.

- Limited coordination between UE and network: UE Assistance Information (UAI) was underutilized. Lack of AI-driven or traffic-aware adaptation limited efficiency.

Expectations on UE power saving in 6G

- Day-1 power saving framework: Unlike 5G’s fragmented path, 6G should integrate UE energy efficiency features from the first release. Define baseline UE energy saving states and procedures applicable across device types.

- Scalability across device types: Ensure solutions scalable across smartphones, wearables, XR devices, IoT sensors, etc. Study minimum common framework + add-on device-type-specific features.

- Unified Low-power modes: Study whether DRX/eDRX/DCP, dormant BWP, and LP-WUS can be unified into a single deep sleep framework, reducing signaling overhead.

- Traffic-Aware adaptation: Study network-assisted AI/ML-driven DRX and BWP/Carrier/SCell adaptation based on traffic prediction, mobility state, UE capability, and so on. Consider dynamic power-saving adaptation (e.g., fewer MIMO layers when low throughput is expected).

- Cross-Layer and multi-domain: Study joint operation of time-domain (e.g., DRX), frequency-domain (e.g., BWP), spatial-domain (e.g., MIMO adaptation), and processing-domain power saving techniques. Aim for a unified signaling and control framework.

### Network Energy Saving Schemes in 5G

According to [4], network energy saving schemes focus on reducing the operational expenses of the network by, for example, switching off cell capacity boosters when not needed. The table below summarizes 5G key Network Energy Saving (NES) schemes:

| 5G key NES schemes |  |
| --- | --- |
| Cell DTX/DRX | The network can configure a periodic pattern of active and non-active periods for a cell. During non-active periods, the gNB reduces its downlink transmission and uplink reception activities, saving energy. UEs are informed of this pattern and adapt their monitoring and transmission behavior accordingly. A dedicated RNTI, cellDTRX-RNTI, is used for L1 signaling related to this feature. |
| On-Demand SSB (OD-SSB) On-Demand SIB1 (OD-SIB1) | Instead of broadcasting SSBs periodically, the gNB could transmit them only when needed, such as upon a UE wake-up trigger or initial access request. This would allow cells in low-traffic or dormant states to suppress unnecessary SSB transmissions, saving network energy. Similar to OD-SSB, OD-SIB1 could be provided only when a UE requests it, after detecting synchronization. This reduces periodic broadcast overhead. |
| Conditional Handover (CHO) for NES | A UE can be configured with CHO to move away from a cell that is entering an energy-saving state (e.g., activating cell DTX/DRX). |
| Camping restrictions for NES | A cell entering an energy-saving mode can restrict access to only UEs capable of handling that mode, using standard barring mechanisms for other UEs. |
| SSB-less SCell | To save energy from broadcasting Synchronization Signal Blocks (SSB), an SCell can be configured without an SSB. The UE can obtain timing and synchronization from another serving cell. |

Lessons Learned from NES in 5G

- Cell DTX/DRX integration gaps: Cell DTX/DRX provided substantial BS-side energy saving, but coordination with UE DRX was limited. Inconsistent support across UEs created interoperability issues. Latency/QoS trade-offs were not fully optimized, particularly for low-latency services.

- OD-SSB/OD-SIB1 limitation: In 5G, the OD-SSB feature is limited to the SCell case which really limits the potential energy saving as well as the network still needs to transmit SSB in an “always on” fashion for all PCells. OD-SIB1 in 5G is also limited to the case of an anchor cell and an NES cell (i.e., cell supporting OD-SIB1 still needs another cell to help). As there can be many instances where the SIB1 is transmitted and no UEs actually receive it, this is wasted overhead and power from the network point of view.

- Conditional handover for NES was underused: CHO was effective in theory for moving UEs away from cells entering NES mode, but adoption was slow due to signaling complexity and UE readiness. Highlighted the need for predictive, proactive UE mobility management to align with network sleep cycles.

Expectations on NES in 6G

- Unified power saving states: Study whether network and UE sleep modes (e.g., Cell DTX/DRX and UE DRX) can be jointly designed as part of a common 6G power state model, avoiding fragmented approaches from 5G.

- Extend the support of OD-SSB/OD-SIB1: Study the support of OD-SSB/OD-SIB1 for more cases (e.g., including PCell, on synch raster, standalone cell, etc)

- Mobility-aware power saving: Enhance CHO by introducing predictive and proactive UE mobility management, e.g., early notification of cell energy state transitions. Study AI/ML-assisted handover triggers to reduce unnecessary wake-up and measurement overhead.

### 6G Study on Energy Saving

Study unified and scalable UE power saving framework by considering at least:

- Define baseline UE power saving states/procedures by considering scalability across device types.

- Unify existing UE power saving schemes (e.g., DRX, DCP, dormant BWP, LP-WUS, etc.) into a single power saving framework.

Study UE and NW energy saving coordination by considering at least:

- Procedures to better align UE DRX operation with network Cell DTX/DRX cycles,

- Joint operation of time-domain (e.g., DRX), frequency-domain (e.g., BWP), spatial-domain (e.g., MIMO adaptation) and processing-domain power saving techniques.

- More cases for supporting OD-SSB/OD-SIB1 (e.g., PCell, on synch raster, standalone cell, etc).

Study AI-driven/predictive energy saving by considering at least:

- AI-driven/predictive traffic-aware adaptation to dynamically adjust DRX, BWP, MIMO layers, and other parameters based on traffic patterns, mobility state, UE preferences, etc.

- AI-driven/predictive mobility-aware mechanisms for power saving.

## Data Transfer Design

In 5G, data transfer is supported by a layered protocol architecture:

- RRC provides the control for bearer/channel configuration.

- SDAP for QoS flow mapping.

- PDCP for header compression, ciphering, integrity, and reordering.

- RLC for segmentation, reassembly, and ARQ.

- MAC for multiplexing, HARQ, and scheduling.

These were designed to support a wide variety of data traffic types in 5G. For example:

- eMBB (high throughput, streaming, bulk transfers).

- URLLC (low latency, high reliability).

- mMTC (small, infrequent packets from massive devices).

- RedCap (low-complexity, moderate data).

- Sidelink/V2X traffic.

Lessons Learned from data transfer in 5G

- Complexity and overhead: Multiple protocol layers (SDAP/PDCP/RLC/MAC) introduced redundancy and high signaling overhead, especially for small and sporadic data (e.g., IoT, sensor traffic).

- Inefficiency for short/bursty traffic: Small data transmissions required full bearer setup, causing latency and energy inefficiency (hence solutions like SDT were introduced).

- Fragmentation across services: URLLC, eMBB, and mMTC requirements were partly addressed but not seamlessly integrated into a single efficient framework.

- QoS limitations: Mapping to QoS flows worked for broadband, but fine-grained adaptation (e.g., for AI-driven applications or time-sensitive networking) was difficult.

Expectations on data transfer in 6G

- Native support for various data patterns

  - Lightweight transfer for small, infrequent data (sensor/IoT), e.g., without full bearer establishment.

  - High-throughput optimized transfer for XR/AI, e.g., with parallelized encoding/decoding and efficient scheduling.

  - Time-sensitive flows with bounded latency/jitter, e.g., preventing from obsolete data.

  - The transfer of various data categories, including AI/ML and sensing data, e.g., new radio bearer configurations, such as a bearer for data collection that terminates within the RAN.

- Facilitating Data-as-a-Service (DaaS)

  - A primary goal of DaaS is to create new business opportunities and revenue streams by monetizing network data, such as offering "AI & Data as a Service" to third parties.

  - A potential 6G architecture could incorporate a multi-purpose plane (evolved UP/CP or a new data plane) operating alongside the traditional control and user planes to facilitate DaaS.

  - The system could introduce specialized entities such as data handling, processing, and storage functions to enable this framework.

  - The user plane can be evolved to support DaaS through greater flexibility and programmability, which would allow for the dynamic generation of traffic forwarding policies.

  - The control plane would be responsible of orchestration of DaaS, which would include policy management, data subscriptions, and the exposure of data services.

- Data transfer across diverse device types: Ensure seamless support for NTN devices, RedCap/LPWA devices, XR headsets, AI agents, etc.

- Dynamic and AI-driven adaptability: Protocols that self-adjust depending on traffic patterns prediction (short vs. continuous, periodic vs. bursty).

Study native support for various data patterns (e.g., small/infrequent data, high-throughput data, time-sensitive data), DaaS, and data transfer across diverse device types (e.g., NTN devices, RedCap/LPWA devices, XR headsets, AI agents).

Study dynamic and AI-driven/predictive adaptability on data transfer (e.g., protocols that self-adjust depending on traffic patterns prediction).

## AI/ML Framework

5G AI/ML framework for the air interface, as detailed in technical report TR 38.843 [5], serves as a foundational element for discussions on 6G. This framework provides a structured approach to integrating AI/ML algorithms into RAN operations and encompasses several key components, including Life Cycle Management (LCM), model management, and specific procedures for data collection and performance monitoring. The table below summarizes 5G AI/ML framework:

| 5G AI/ML framework - LCM |  |
| --- | --- |
| Model Generation | This stage involves the offline processes of training, validating, and testing AI/ML models. |
| Model Inference | This is the process of using a trained model to generate outputs from a set of inputs. The 5G framework assumes NW-sided model inference is not visible to the UE, while for UE-sided models, specific configurations are used for inference configuration |
| Data Collection | The framework supports both UE-side and NW-side data collection. For UE-sided models, data collection configurations are often defined within use-case discussions, while for NW-sided models, extensions to the Minimization of Drive Tests (MDT) framework are considered. |
| Performance Monitoring | For UE-sided models, UE-assisted performance monitoring is supported to ensure models operate effectively post-deployment. For NW-sided models, monitoring is handled by NW implementation. |
| Functionality-Based LCM | This manages AI/ML operations without requiring the network/UE to know the specifics of the model. This approach indirectly supports model selection, activation, deactivation, switching, and fallback to non-AI solutions. The process involves: - UE reporting its AI/ML capabilities. - NW providing configurations based on the reported capabilities to enable inference. - UEs reporting the applicability of these configurations, especially when network conditions change. |
| 5G AI/ML framework - Models |  |
| One-Sided Model | The entire AI/ML model inference is performed at either the UE (UE-sided model) or the network (NW-sided model). Model management for these is typically left to the implementation of the respective side (UE or NW). |
| Two-Sided Models | This involves a paired set of models where inference is performed jointly across both the UE and the network. This architecture is central to the AI-based CSI compression use case, which will be specified in Release 20 of 5G-Advanced. |

What 5G AI/ML frameworks can be leveraged to 6G

- Model generation & inference: Keep the same stage definitions (training offline, inference at UE/NW/both). Extend in 6G to allow for continuous/online adaptation and broader support for two-sided models beyond CSI compression.

- Data collection: Reuse UE-side configs and NW-side MDT-based collection. Extend to a common AI/ML data collection profile (privacy-aware, use-case independent).

- Performance monitoring: Retain UE-assisted monitoring (for UE models) and NW-side monitoring. Extend with standardized applicability/drift reports when model performance degrades.

- Lifecycle management (LCM): Keep the 5G principle of functionality-based LCM (NW configures functions, not models). Extend with energy/latency-aware policies, pairing/versioning for two-sided models, and online update hooks.

- Model architectures: Continue with one-sided (UE or NW) and two-sided inference. Extend two-sided model pairing/coordination as a generic framework (not limited to CSI).

Study a general AI/ML framework for 6G that unifies mechanisms defined in 5G while extending them to support new requirements by considering at least:

- Lifecycle management (LCM): Maintain the functionality-based approach from 5G but extend with energy/latency-aware policies.

- Data collection & monitoring: Define a common AI/ML data collection profile (e.g., privacy-aware, use-case independent).

- Model architectures: Generalize two-sided model pairing into a generic framework applicable across use cases, while continuing support for UE- or NW-only inference.

## Access Stratum Security

Based on the SA3 SID on security for the 6G system [6], the 6G security study aims to define a comprehensive security and privacy architecture for 6G mobile networks, recognizing that the next generation of communication technology requires a much higher level of security from its inception. The justification for this study stems from rapid technological advancements in spectrum, chipsets, and computing platforms, evolving communication needs, and new security challenges posed by emerging technologies such as threats from quantum computing, cloud and virtualization of network functions, and the widespread adoption of AI/ML. The primary objective of the study is to define a security and privacy architecture and its associated procedures for the 6G system, supporting both new and improved services while creating lean and streamlined standards. The scope includes RAN architecture security.

Lessons Learned from AS security in 5G

In 5G, PDCP provides ciphering/integrity for SRB/DRB payloads, but MAC data that are below PDCP are not cryptographically protected. Their confidentiality relies only on physical-layer scrambling and the practical difficulty of decoding; their authenticity/integrity relies on higher-layer state coherence rather than per-PDU cryptographic checks. As a result, information carried or implied at MAC can be observed or influenced by an active adversary earlier and with lower cost than breaching PDCP. Unprotected MAC data, for example, LTM cell switch command/SR/BSR/DSR/PHR/DRX/TA reveal, mobility, traffic load and wake/sleep patterns; they can be used to infer UE activity or to induce counterproductive behavior.

Expectations on AS security in 6G

6G should make the MAC layer trustworthy. RAN 2 can study and identify most sensitive MAC procedures/MAC CEs/MAC payloads first (e.g., for TA and DRX) and considering privacy for usage reports (e.g., BSR/DSR/PHR). RAN 2 can also study on attaching a tag to MAC message so the network and UE know they’re genuine and fresh (no spoof/replay).

Study MAC security mechanisms and identify the sensitive MAC procedures/MAC CEs/MAC payloads.

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	Study unified and scalable UE power saving framework by considering at least:

- Define baseline UE power saving states/procedures by considering scalability across device types.

- Unify existing UE power saving schemes (e.g., DRX, DCP, dormant BWP, LP-WUS, etc.) into a single power saving framework.

Proposal 2	Study UE and NW energy saving coordination by considering at least:

- Procedures to better align UE DRX operation with network Cell DTX/DRX cycles,

- Joint operation of time-domain (e.g., DRX), frequency-domain (e.g., BWP), spatial-domain (e.g., MIMO adaptation) and processing-domain power saving techniques.

- More cases for supporting OD-SSB/OD-SIB1 (e.g., PCell, on synch raster, standalone cell, etc).

Proposal 3	Study AI-driven/predictive energy saving by considering at least:

- AI-driven/predictive traffic-aware adaptation to dynamically adjust DRX, BWP, MIMO layers, and other parameters based on traffic patterns, mobility state, UE preferences, etc.

- AI-driven/predictive mobility-aware mechanisms for power saving.

Proposal 4	Study native support for various data patterns (e.g., small/infrequent data, high-throughput data, time-sensitive data), DaaS, and data transfer across diverse device types (e.g., NTN devices, RedCap/LPWA devices, XR headsets, AI agents).

Proposal 5	Study dynamic and AI-driven/predictive adaptability on data transfer (e.g., protocols that self-adjust depending on traffic patterns prediction).

Proposal 6	Study a general AI/ML framework for 6G that unifies mechanisms defined in 5G while extending them to support new requirements by considering at least:

- Lifecycle management (LCM): Maintain the functionality-based approach from 5G but extend with energy/latency-aware policies.

- Data collection & monitoring: Define a common AI/ML data collection profile (e.g., privacy-aware, use-case independent).

- Model architectures: Generalize two-sided model pairing into a generic framework applicable across use cases, while continuing support for UE- or NW-only inference.

Proposal 7	Study MAC security mechanisms and identify the sensitive MAC procedures/MAC CEs/MAC payloads.

# Reference

3GPP RP-252476, “Revised SID: Study on 6G Radio (NTT DOCOMO, CMCC, AT&T, Vodafone).”

3GPP TR 38.840: “Study on User Equipment (UE) power saving in NR.”

3GPP TR 38.869: “Study on low-power wake up signal and receiver for NR.”

3GPP TR 38.864: “Study on network energy savings for NR.”

3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.”

3GPP S3-252973, “Study on Security for the 6G System (Nokia).”