3GPP TSG-RAN WG2 Meeting #131bis	R2-2507206

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		10.4

Source:		Futurewei

Title:		Discussions on 6G Mobility

Document for:		Discussion and Decision

# Introduction

During RAN#108 meeting, a new study item on “6G Radio” was approved [1]. The SID is further revised in RAN#109. According to the revised SID [2], the following key objectives of the study are related to “mobility for 6GR”:

The detailed objectives of the study are: ………. - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] ………. - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

In this paper, we discuss mobility-related issues that remain unresolved in 5G NR and propose alternative solutions to address them in the context of 6GR.

# Discussions

## Unified mobility handover (HO)

In 5G NR, several mobility HO mechanisms have been standardized over different releases, typically introduced incrementally in the form of patches, as illustrated in Figure 1. While these mechanisms aim to support various deployment scenarios, by specifying them separately, it significantly increases signaling overhead and complicates the implementation of mobility HO management. For instance, up to Release 17, HO mechanisms such as basic L3 HO and L3 conditional HO (CHO) relied on layer 3 (L3) measurements, with serving cell changes typically triggered by RRC signaling (except in CHO) for the switching of the PCell (or PSCell). In addition, different measurement events were defined in the RRM framework to trigger L3-based measurement reporting or CHO, tailored for different scenarios and requirements. Starting from Release 18, however, 5G NR introduced layer 1/2–based mobility HO, also known as lower-layer triggered mobility (LTM), in which serving cell changes are executed via L1/L2 signaling. This approach can significantly reduce HO latency mainly through early downlink and uplink synchronizations before the HO execution phase and by replacing the RRC-signaling based trigger with the L1/L2-signaling based trigger. Nevertheless, in LTM (including CLTM), UE’s measurement reporting is based on the CSI reporting framework of the serving cell, and the associated triggering events are also defined within that framework, even reported by MAC layer for event triggered reporting. In summary, the coexistence of multiple mobility HO mechanisms in 5G NR, with different measurement resources, reporting frameworks, triggering signals, and event definitions, has led to a fragmented mobility HO landscape that results in big challenges for practical deployment in 5G NR.

Figure 1: Different mobility HO mechanisms in 5G NR

Therefore, based on the above analysis and discussion, we have the following observation:

Observation 1: In 5G NR, several HO mechanisms have been standardized over different releases. While these mechanisms aim to support various deployment scenarios, the fact that they were specified separately has significantly increased signaling overhead and complicated the implementation of mobility HO management.

Overall, the mobility mechanisms specified in 5G NR can be broadly classified into two types of handover (HO), distinguished by the method used to trigger cell switching. The first type is UE-triggered mobility HO, which includes CHO, CLTM, and their enhancements. The second type is network-triggered mobility HO, which encompasses basic L3 HO, LTM, and their enhancements. In our view, having multiple mechanisms for configuring RRM measurements and for RRM measurement reporting for each of these two types of HO is unnecessary in 6GR, as they introduce additional signaling overhead and complexity.

As aforementioned, up to Release 17, HO mechanisms primarily relied on layer 3 (L3) measurements, with serving cell changes typically triggered by RRC signaling (except in CHO and its enhancements). However, beginning in Release 18, 5G NR introduced LTM, where serving cell changes are triggered by L1/L2 signaling based on L1 measurements within the serving cell’s CSI reporting framework. Moreover, in the current 5G NR standard, measurement resources in the RRM framework and the CSI reporting framework of the serving cell are configured and measured separately. This separated configuration and measurement significantly increases signaling overhead and complexity in HO measurement reporting from both UE and NW perspectives, which should be avoided in 6GR.

Regarding HO triggering signaling in 5G NR, both RRC message (e.g., RRC Reconfiguration for L3 HO) and MAC CE (e.g., for LTM) are used to trigger PCell (or PSCell) switching during HO, except in CHO and its enhancements. In our view, a unified HO triggering mechanism, for example, whether through a lightweight RRC message, a MAC CE, or even a DCI, should be considered in 6GR to avoid redundant designs.

In 5G NR, many events have been defined for different mobility HO mechanisms [3]. For example, basic L3 HO with event-triggered measurement reporting specified six events (A1–A6), while L3 CHO introduced three conditional events (CondEvent A3, A4, and A5). For LTM with event-triggered reporting, four events (eventLTM2–5) were defined, and for CLTM, at least two events (e.g., CondEvent A3- and A5-based) have been agreed. This proliferation of events increases signaling overhead and implementation complexity for both UE-triggered HO and network-triggered HO. In our view, for 6GR, event definitions should also be streamlined, with only two sets of events (such as, one for UE-triggered HO and one for network-triggered HO) being sufficient.

Therefore, based on the above analysis and discussion, a unified framework for mobility in 6GR is desirable. Hence, we have the following proposal:

Proposal 1: RAN2 to study a unified mobility HO design including HO mechanisms, measurement resources, measurement report framework, HO triggering signaling, and event definitions in 6GR.

## Mobility between 5G NR and 6GR

5G/6G Inter-Radio Access Technology (RAT) mobility refers to the process of handing over a user’s connection or performing cell (re)selection between different RATs, i.e., from 5G NR to 6G R or vice versa, as users move across heterogeneous network coverages. This procedure is critical for maintaining service continuity, particularly during the coexistence and migration phases of 6G deployment, where partial rollouts may result in coverage gaps. A similar challenge was encountered in the context of 4G LTE/5G NR inter-RAT mobility during the early coexistence and migration phases of 5G. However, due to the dual-RAT measurement requirement in 4G LTE/5G NR inter-RAT mobility, the measurement complexity and efficiency remain insufficiently addressed in 5G NR, as outlined below:

- Energy efficiency and measurement complexity in RRC Idle/Inactive states: Energy efficiency in Idle/Inactive state for inter-RAT mobility between 4G LTE and 5G NR remains a significant issue, as UEs must periodically monitor both LTE CRS (continuous, narrowband) and NR SSBs (bursty, wideband) to support cell (re)selection. This dual-RAT measurement requirement leads to frequent RF activation over large bandwidths, causing substantial power consumption and measurement complexity for UEs. While mechanisms such as TRS-based measurements, and RAN-area registration reduce signaling overhead, they do not fully mitigate the energy cost of dual-RAT scanning based on two types of RSs with different features. Consequently, inter-RAT idle/inactive mobility still imposes non-negligible battery drain, particularly in heterogeneous NSA deployments and areas with fragmented LTE/NR coverage.

- Measurement complexity and efficiency in RRC Connected state: Similarly, measurement complexity and inefficiency in inter-RAT mobility between 4G-LTE and 5G-NR arises from the heterogeneous nature of their RSs with different features in RRC connected state. LTE employs always-on CRS, whereas NR relies on bursty, wideband SSBs, requiring UEs to perform dual measurements with different periodicities and bandwidth demands. This not only necessitates measurement gaps, reducing data throughput, but also increases UE power consumption and complicates the network’s ability to harmonize reporting for mobility decisions. In addition, in inter-RAT Event B2 [3] between LTE and NR, UE must simultaneously monitor the serving NR cell and the neighbor LTE cell using different RSs and measurement metrics. On the NR side, the serving cell quality is measured either from SSB (RSRP/RSRQ/SINR) or configured CSI-RS, while on the LTE side, the neighbor quality is measured from CRS (RSRP/RSRQ/RS-SINR), which is always present in LTE downlink. The event is triggered only when the serving NR measurement drops below a configured threshold and the LTE CRS measurement rises above another threshold, both holding for the configured TTT with hysteresis. However, this dual-RAT monitoring introduces measurement complexity and extra UE power consumption: NR SSB spans wide bandwidth (3.6–7.2 MHz in FR1, larger in FR2) and requires wide RF/baseband activation, while LTE CRS is sparse but continuous, forcing the UE to keep additional RF chains active to scan another RAT. As a result, continuous B2-based monitoring increases UE energy drains and raises complexity in scheduling measurements across asynchronous NR and LTE carriers.

In 5G-NR/6GR inter-RAT mobility, a harmonized (or optimized) mechanism for measurement based on dual-RAT scanning, such as in RS density, periodicity and/or types, needs to be considered to increase energy efficiency and decrease measurement complexity in RRC Idle/Inactive/Connected states.

Therefore, based on the above analysis and discussion, we have the following observation and proposal:

Observation 2: Due to the dual-RAT measurement and different RS features in 4G LTE/5G NR inter-RAT mobility, the measurement complexity and efficiency remain insufficiently addressed in 5G NR, including RRC Idle/Inactive states as well as RRC connected state.

Proposal 2: RAN2 to study a harmonized mechanism with more efficiency in terms of RS density, periodicity and types for measurements for inter-RAT mobility in 6GR.

## Bandwidth bottleneck of mobility HO with CA

In 5G NR, the mobility HO process is managed only by the PCell (or PSCell), while SCell(s) are just taken as additional resources by UE and NW. Thus, when CA is configured in both source and target cells in 5G NR, SCell(s) are deactivated before HO triggering and reactivated in the target cell after HO completion, as illustrated in Figure 2. With the legacy mobility HO procedure under CA in 5G NR, wide bandwidth cannot be guaranteed because only a single cell/carrier (i.e., PCell/PSCell) remains active during the HO. This limitation can significantly degrade the performance of high-data-rate and low-latency services that rely on CA (multiple carriers), particularly in emerging 6G-oriented applications such as XR, immersive communication, holographic services, etc. Although a new WID for mobility HO enhancement [4] was initiated in Release 20 trying to reduce the latency of activating SCell(s) in the target cell, this enhancement does not fully resolve the underlying issues. In our view, a more forward-looking solution for mobility HO, such as executing HO simultaneously on both the PCell/PSCell and the SCell(s), can be considered in the context of 6GR.

Figure 2: Mobility HO with CA in 5G NR

Therefore, based on the above analysis and discussion, we have the following observation and proposal:

Observation 3: With the legacy mobility HO procedure under CA in 5G NR, wide bandwidth cannot be guaranteed because only a single cell/carrier (i.e., PCell/PSCell) remains active during the HO. This limitation can significantly degrade the performance of high-data-rate and low-latency services that rely on CA (multiple carriers), particularly in emerging 6G-oriented applications such as XR, immersive communication, holographic services, etc.

Proposal 3: RAN2 to study a solution for mobility HO with CA in the context of 6GR, such as executing HO simultaneously on both the PCell/PSCell and the SCell(s).

## UE-based timing advance (TA) for mobility HO

In 5G NR, UE-based TA measurement was introduced [5] for LTM in Release-18. In this mechanism, UE estimates the TA value by combining the Rx timing difference between the current serving cell and the candidate cell with the TA value of the serving cell. This approach works reliably only when the serving and candidate cells are synchronized or when their timing difference is within a very small range (such as less than the cyclic prefix of an OFDM symbol). In scenarios with large timing misalignments, however, the mechanism fails to operate effectively. This limitation significantly restricts the deployment flexibility of UE-based TA measurement for LTM in 5G NR.

Therefore, based on the above analysis and discussion, we have the following observation and proposal:

Observation 4: With large timing misalignments between serving cell and candidate cell, UE-based TA measurement fails to operate effectively, which significantly restricts the deployment flexibility of UE-based TA measurement for LTM in 5G.

Proposal 4: RAN2 to study the impact of large timing misalignments between serving cell and candidate cell for UE-based TA measurement for LTM in the context of 6GR.

# Conclusions

Observation 1: In 5G NR, several HO mechanisms have been standardized over different releases. While these mechanisms aim to support various deployment scenarios, the fact that they were specified separately has significantly increased signaling overhead and complicated the implementation of mobility HO management.

Observation 2: Due to the dual-RAT measurement and different RS features in 4G LTE/5G NR inter-RAT mobility, the measurement complexity and efficiency remain insufficiently addressed in 5G NR, including RRC Idle/Inactive states as well as RRC connected state.

Observation 3: With the legacy mobility HO procedure under CA in 5G NR, wide bandwidth cannot be guaranteed because only a single cell/carrier (i.e., PCell/PSCell) remains active during the HO. This limitation can significantly degrade the performance of high-data-rate and low-latency services that rely on CA (multiple carriers), particularly in emerging 6G-oriented applications such as XR, immersive communication, holographic services, etc.

Observation 4: With large timing misalignments between serving cell and candidate cell, UE-based TA measurement fails to operate effectively, which significantly restricts the deployment flexibility of UE-based TA measurement for LTM in 5G.

Based on these observations, we have the following proposals:

Proposal 1: RAN2 to study a unified mobility HO design including HO mechanisms, measurement resources, measurement report framework, HO triggering signaling, and event definitions in 6GR.

Proposal 2: RAN2 to study a harmonized mechanism with more efficiency in terms of RS density, periodicity and types for measurements for inter-RAT mobility in 6GR.

Proposal 3: RAN2 to study a solution for mobility HO with CA in the context of 6GR, such as executing HO simultaneously on both the PCell/PSCell and the SCell(s).

Proposal 4: RAN2 to study the impact of large timing misalignments between serving cell and candidate cell for UE-based TA measurement for LTM in the context of 6GR.

# References

RP-251881	 New SID: Study on 6G Radio,	NTT DOCOMO, RAN#108.

RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

3GPP TS 38.331 V18.6.0, NR: Radio Resource Control (RRC) protocol specification.

RP-251865	 New WI: NR mobility enhancements Phase 5, CATT, RAN#108

Final_Minutes_report_RAN1#113_v100.zip.