3GPP TSG-RAN WG2 Meeting #131bis	R2-2507220

# Prague, Czech Republic, Oct.13th-17th

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5519 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on validation of reported idle/inactive and reselection measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - When setting the valid NR reselection measurement results according to 5.7.10.3, the UE has to set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig, not that of measIdleValidityDuration. - For the validation of idle/inactive measurements as part of early measurement report, the UE sets validityStatus to the value of measIdleValidityDuration per each reported measurement i.e. per cell in both E-UTRA and NR according to 5.7.10.3. However, it is missing to define the field validityStatus within the IE MeasResultsPerCellIdleEUTRA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace measIdleValidityDuration with measReselectionValidityDuration in 5.7.10.3 so that the UE sets the validityStatus properly for NR reselection measurement reporting. - Add a new field validityStatus-r18 within the IE MeasResultsPerCellIdleEUTRA to allow UE to set validityStatus for reporting valid E-UTRA measurement results. Impact Analysis Impacted 5G architecture options: NR SA, NE-DC, NR-DC Impacted functionality: Validation of reported idle/inactive and reselection measurements Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue but network has no idea how UE checked the reported measurement validity and might configure CA/DC wrongly. 2. If the UE is implemented according to the CR and the network is not, ther. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE has no way to set validityStatus for IMR as measIdleValidityDuration is not defined in VarMeasReselectionConfig. - When transmitting the UEInformationResponse message to the network, the UE does not include validityStatus in the measResultIdleEUTRA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.10.3, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.7.10.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measIdleValidityDuration is included in VarEnhMeasIdleConfig;

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

3>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig;

2>	else:

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

3>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

1>	if the reselectionMeasurementReq is included in the UEInformationRequest:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig;

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig for each reported measurement;

3> else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measReselectionValidityDuration in VarMeasReselectionConfig;

2>	else:

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig;

3>	else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from the VarLogMeasReport starting from the entries logged first, and for each entry of the logMeasInfoList that is included, include all information stored in the corresponding logMeasInfoList entry in VarLogMeasReport;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

4>	if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableBT;

4>	if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableWLAN;

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report; or

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the registered SNPN identity is included in snpn-IdentityList stored in VarRA-Report:

2>	set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report;

2>	discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

2>	else if the UE is capable of cross-RAT RLF reporting as defined in TS 38.306 [26] and has radio link failure information or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	set timeSinceFailure in VarRLF-Report of TS 36.331 [10] to the time that elapsed since the last radio link failure or handover failure in EUTRA;

3>	set failedPCellId-EUTRA in the rlf-Report in the UEInformationResponse message to indicate the PCell in which RLF was detected or the source PCell of the failed handover in the VarRLF-Report of TS 36.331 [10];

3>	set the measResult-RLF-Report-EUTRA in the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report of TS 36.331 [10];

3>	discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

1>	if connEstFailReportReq is set to true and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	if the UE supports multiple CEF report:

3>	for each connEstFailReport in the connEstFailReportList in VarConnEstFailReportList:

4>	set timeSinceFailure to the time that elapsed since the associated connection establishment failure or connection resume failure in NR;

2>	for each connEstFailReport in the connEstFailReportList in the UEInformationResponse message, set the value to the value of connEstFailReport in VarConnEstFailReport in VarConnEstFailReportList;

2>	discard the connEstFailReport from VarConnEstFailReport and VarConnEstFailReportList upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the mobilityHistoryReportReq is set to true:

2>	include the mobilityHistoryReport and set it to include visitedCellInfoList from VarMobilityHistoryReport;

2>	include in the mobilityHistoryReport an entry for the current PCell, possibly after removing the oldest entry if required, and set its fields as follows:

3>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PCell:

3>	set field timeSpent to the time spent in the current PCell;

3>	if the UE supports PSCell mobility history information and if visitedPSCellInfoList is present in VarMobilityHistoryReport:

4>	for the newest entry of the PCell in the mobilityHistoryReport, include visitedPSCellInfoList from VarMobilityHistoryReport;

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since last PSCell release since connected to the current PCell in RRC_CONNECTED;

3>	else if the UE supports PSCell mobility history information:

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since connected to the current PCell in RRC_CONNECTED;

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the RPLMN is included in the plmn-IdentityList stored in VarSuccessHO-Report; or

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	if the successHO-Report in the VarSuccessHO-Report concerns a mobilityFromNRCommand:

3>	set timeSinceSHR in VarSuccessHO-Report to the time that elapsed since the execution of the MobilityFromNRCommand associated to the successful handover report in the VarSuccessHO-Report;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessPSCell-Report:

2>	set the successPSCell-Report in the UEInformationResponse message to the value of successPSCell-Report in the VarSuccessPSCell-Report;

2>	discard the VarSuccessPSCell-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the coarseLocationRequest is set to true:

2>	include coarseLocationInfo, if available;

1>	if the flightPathInfoReq is included in the UEInformationRequest and the UE has (updated) flight path information available, set the flightPathInfoReport in the UEInformationResponse message as follows:

2>	include the list of up to maxWayPointNumber waypoints, if any, along the flight path;

2>	if the includeTimeStamp is set to true, for each included waypoint:

3>	if available, set the field timestamp to the time when UE intends to arrive at the waypoint;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

Next change

6.3.2	Radio resource control information elements

–	MeasResultIdleEUTRA

The IE MeasResultIdleEUTRA covers the E-UTRA measurement results performed in RRC_IDLE and RRC_INACTIVE.

MeasResultIdleEUTRA information element

-- ASN1START

-- TAG-MEASRESULTIDLEEUTRA-START

MeasResultIdleEUTRA-r16 ::= SEQUENCE {

measResultsPerCarrierListIdleEUTRA-r16   SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleEUTRA-r16,

...

}

MeasResultsPerCarrierIdleEUTRA-r16 ::=  SEQUENCE {

carrierFreqEUTRA-r16                    ARFCN-ValueEUTRA,

measResultsPerCellListIdleEUTRA-r16     SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleEUTRA-r16,

...

}

MeasResultsPerCellIdleEUTRA-r16 ::=     SEQUENCE {

eutra-PhysCellId-r16                    EUTRA-PhysCellId,

measIdleResultEUTRA-r16                 SEQUENCE {

rsrp-ResultEUTRA-r16                     RSRP-RangeEUTRA                                                     OPTIONAL,

rsrq-ResultEUTRA-r16                     RSRQ-RangeEUTRA-r16                                                 OPTIONAL

},

...,

[[

validityStatus-r18						MeasurementValidityDuration-r18													 OPTIONAL

]]

}

-- TAG-MEASRESULTIDLEEUTRA-STOP

-- ASN1STOP

MeasResultIdleEUTRA field descriptions

carrierFreqEUTRA Indicates the E-UTRA carrier frequency.

eutra-PhysCellId Indicates the physical cell identity of an E-UTRA cell.

measIdleResultEUTRA Idle/inactive measurement results for an E-UTRA cell.

measResultsPerCarrierListIdleEUTRA List of idle/inactive measured results for the maximum number of reported E-UTRA carriers.

measResultsPerCellListIdleEUTRA List of idle/inactive measured results for the maximum number of reported best cells for a given E-UTRA carrier.

End of change