3GPP TSG-RAN WG2 Meeting#131bis   		                                                        R2-2507241

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	10.3.1

Source:	ETRI

Title:	Discussion on User Plane Design for 6GR

Document for:	Discussion and Decision

# Introduction

In last RAN#108 meeting, followings are agreed for radio interface protocol architecture and procedures for 6GR [1].

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] a) User Plane architecture and protocol design [RAN2] b) Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2] d) Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] e) Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this paper, we discuss serveral considerations for user plane design in 6GR and present our perspectives.

# Discussions

Supporting Immersive Communication (IC)

Immersive communication is one of the usage scenarios presented in the ITU-R IMT-2030 framework and requires significantly higher data rates to enable multisensory and ultra-realistic experiences, including XR and holographic communication. It is supported by composite traffic that exhibits multi-modality data traffic characteristics.

Multi-modality data traffic is a combination of different data types, such as text, audio, images, and video. This kind of traffic has unique characteristics compared to single-modality traffic as below:

- High Bitrate: The total volume of data can be very large, especially when it includes high-resolution images or video, requiring a high bitrate.

- Dependency: Different data modalities are often semantically linked. For example, a video's audio track must be synchronized with the video, so a delay or loss in one modality affects the quality of the others.

- Variability: The requirements for real-time delivery and the size of the data can vary depending on the data type. For instance, voice data requires real-time transmission, while image data may be less sensitive to delay.

- Heterogeneity: Each data modality has its own unique characteristics in terms of compression methods, coding schemes, and transmission protocols.

Therefore, when processing multi-modality data traffic, the following points should be considered:

- Resource Management and Allocation: It is essential to efficiently distribute the limited wireless bandwidth. Differentiated Services are needed to prioritize real-time data like voice and video over data that is less sensitive to delay, like text and images.

- Quality of Service (QoS): A comprehensive QoS policy is required to meet the demands of each modality. Accordingly, as each data modality has distinct QoS requirements, a mechanism is needed to ensure them while ensuring transmission efficiency.

- Synchronization: Maintaining temporal synchronization between different modalities is critical. When video and audio data are combined, a lack of synchronization can significantly degrade the user experience. Synchronization mechanisms must be applied to prevent this.

- Energy Efficiency: When processing multi-modal data on various types of mobile devices, it is important to minimize energy consumption during both transmission and processing is important.

Observation 1. Among IMT-2030 usage scenarios, immersive communication (IC) requires the simultaneous handling of multi-modal data traffic with diverse characteristics within strict time constraints.

Proposal 1. Study a packet handling mechanism capable of efficiently processing composite traffic of multiple data streams for multi-modality services, while meeting the required timing constraints and ensuring energy efficiency.

Supporting an AI-Native RAN

In 5G, AI/ML was primarily employed as an add-on feature in the upper layers of the protocols. In contrast, 6G aims to achieve performance optimization and enable new services by natively embedding AI (AI-Native) into the network architecture and protocols from the outset. The user plane protocols are responsible for the actual transmission of user data, scheduling, error control, and encryption. Therefore, in 6G, exchanging data from these protocols with AI modules becomes essential for the following reasons:

- Real-time Performance Optimization: By analyzing user plane data, AI modules can instantly detect variations in channel conditions or occurrences of traffic congestion, thereby make optimal decisions for functions such as scheduling, beam management, and resource allocation. For instance, industrial IoT services require predictable and stable communication. Therefore, AI leverages user plane information to learn traffic flow patterns, thus enabling proactive resource reservation.

- Network Energy Efficiency Enhancement: By analyzing real-time user plane throughput and buffer status information, AI can reduce power consumption by powering down base station components (e.g., power amplifiers) and dynamically placing entire cells into sleep mode during low-traffic periods. Accordingly, interworking between AI and user plane protocols is indispensable.

- Context Awareness and Prediction: AI modules can analyze the type, volume, and temporal variation of traffic in the user plane to infer traffic characteristics. This capability enables AI to predict traffic demand and proactively allocate network resources. Thus, close cooperation between AI and the user plane protocols is essential.

Observation 2. 6G is expected to enable native integration of AI within the RAN. Therefore, the 6G RAN will require mechanisms for data exchange between the user plane and AI-related functions (e.g., LCM).

Proposal 2. Study the necessity of data exchange between the user plane and AI-related functions in the 6G RAN, and identify the required data types along with feasible interface mechanisms.

Issues of User Plane Protocol Architecture

The 5G NR UP protocol consists of four upper layers (SDAP, PDCP, RLC, MAC) and the Physical Layer (PHY). In the 6G mobile communication, this fixed multi-layered structure can give rise to several challenges:

- Challenges in Meeting 6G Service Requirements: This multi-layered protocol structure makes it difficult to efficiently accommodate the diverse and stringent requirements of 6G. In particular, for services that should satisfy multiple usage scenarios simultaneously, the need for header processing and framing across several protocol layers increases processing time and computational complexity, ultimately leading to potential failures in meeting performance requirements. Moreover, for certain services (e.g., IoT data), functions such as sequencing in the RLC or header compression in the PDCP may be unnecessary. Nevertheless, as data still traverses these layers, this incurs additional processing latency and header overhead, resulting in inefficiency.

- Functional Imbalance and Redundancy Across Protocol Layers: With respect to imbalance, the SDAP layer, which is responsible for QoS flow handling, performs a relatively simple compared to other user plane protocols and could potentially be integrated into another protocol layer. With respect to redundancy, the RLC’s ARQ (Automatic Repeat reQuest) and the MAC’s HARQ (Hybrid ARQ) both handle error recovery through retransmissions. Since these two functions partially overlap, integrating them into a single, more efficient mechanism may be considered.

- Challenges in Integrating AI/ML Functions: In 6G, the goal is an AI-Native RAN (Radio Access Network), which requires protocol behavior itself to be optimized by AI/ML. However, the 5G NR protocol layer structure makes this difficult due to the strict separation of layer interfaces and the absence of defined interfaces with AI-related modules.

Observation 3. Due to the multi-layer (four-layers) protocol architecture in 5G NR, the system incurs overhead from header processing and framing, as well as functional imbalances and redundancies across the layers. In addition, the 5G NR protocol architecture cannot effectively support an AI-native RAN.

Proposal 3. Study a simple and lightweight user plane protocol architecture consisting of two or thee layers for an AI-native 6G RAN.

Limitations of RLC ARQ

The RLC protocol of 5G NR enhances reliability by recovering data transmission errors over the radio link through the ARQ function. However, the conventional ARQ scheme of 5G RLC poses several fundamental challenges in ultra-low latency and hyper-reliability service scenarios envisioned for 6G:

- Increased Latency: ARQ involves multiple steps to detect and request retransmissions when packet loss occurs. RLC employs a timer to identify sequence errors or missing packets, and the waiting time until this timer expires constitutes a major source of delay. Another source of latency is the feedback process. the receiver must generate and transmit a Negative Acknowledgment (NACK), which the transmitter must then process before initiating retransmission. This round-trip procedure introduces delays ranging from several to tens of milliseconds, which are intolerable for HRLLC services.

- Rigid Operational Parameters and Complexity: The operational parameters of RLC ARQ (e.g., maximum retransmission attempts, timer values) are statically configured, typically based on the assigned QoS class. As a result, the protocol lacks the flexibility to adapt to rapidly changing channel conditions or variations in data urgency within the same QoS class. Moreover, RLC ARQ requires complex operations such as Sequence Number (SN) management and Status Report processing, which consume CPU resources and complicate the flexible functional split between the Centralized Unit (CU) and Distributed Unit (DU).

- Inefficient Duplication with Lower-Layer HARQ: 5G NR also employs HARQ in the MAC/PHY layers, which provides a faster and more efficient retransmission mechanism. HARQ operates within a short Transmission Time Interval (TTI) and leverages physical-layer coding techniques to recover errors without discarding the packet. Since RLC ARQ is triggered only upon HARQ failure, a highly reliable HARQ design often makes RLC ARQ redundant, adding unnecessary complexity and delay overhead.

Observation 4. 5G NR ARQ is inefficient in handling traffic that requires both low latency and high data rates, due to excessive retransmission delays, the complexity of RLC window maintenance, and redundant functionalities.

Proposal 4. Study a new retransmission mechanism with either ARQ-less or simple ARQ for error recovery.

Limitations of Logical Channel Prioritization (LCP)

Logical Channel Prioritization (LCP) mechanism is a critical component in 5G NR for radio resource allocation and QoS assurance for traffic. However, it exhibits several limitations, when faced with the advanced QoS requirements of 6G:

- Starvation of Low Priority Logical Channels: LCP relies on Priority and Prioritized Bit Rate (PBR) parameters. If high-priority channels continuously receive data, low-priority channels may have limited transmission opportunities, leading to a 'starvation' effect and increased overall transmission latency.

- Difficulty in Meeting HRLLC Requirements: The sequential nature of the LCP procedure (which involves data moving from RLC to MAC and constructing the MAC PDU) introduces a time cost. This acts as a bottleneck for achieving low latency targets required by HRLLC. In addition, in 5G NR, after the LCP procedure is performed for all logical channels, a single transport block (TB) is generated and transmitted, meaning that new, higher-priority data cannot be transmitted when the TB is retransmitted. The Preemption Indication (PI) scheme, introduced in 5G NR to enable immediate URLLC transmission, degrades the QoE of eMBB traffic and generates signaling overhead. Accordingly, novel mechanisms for transmitting HRLLC data in coexistence with IC traffic should be investigated.

- Static Priority Parameters: The priority values used in LCP are statically configured via RRC signaling. While adequate at service initiation, they fail to reflect the real-time user experience or the urgency of control information.

Observation 5. The 5G NR Logical Channel Prioritization (LCP) mechanism is inadequate to meet the 6G requirements. Therefore, an new priority handling and enhanced LCP mechanism that enables more fine-grained and dynamic configuration is required.

Proposal 5. Study a new priorty handling mechanism and enhanced LCP in the MAC.

Issues in User Plane Security Mechanisms

In the 5G NR user plane, data security functions such as ciphering and integrity protection are primarily concentrated in the PDCP. This architecture presents the following limitations in the 6G environment:

- Fixed Security Termination Point: When PDCP is located in the CU as part of the functional split, data handled in the DU below the PDCP layer may be remained in plaintext, creating potential vulnerabilities.

- Security Vulnerabilities in the MAC Layer: The MAC layer is responsible for critical functions such as scheduling, HARQ, and power control. However, it lacks ciphering and integrity protection for the control data, rendering it vulnerable to attacks such as the forgery or modification of control channel information. For instance, if an attacker forges or alters Timing Advance (TA) values, synchronization may be disrupted, potentially causing interference across transmisions. Similarly, manipulation of the Buffer Status Report (BSR) can undermine scheduling by either triggering resource allocation without actual traffic, leading to resource waste, or blocking resource allocation despite high traffic, resulting in service degradation.

Observation 6. In 6G, security requirements are more stringent than in 5G, necessitating the introduction of more robust security mechanisms into the user plane.

Proposal 6. Study a new security mechanism for MAC CE transmission.

# Conclusions

Based on the discussion in the contribution, we have the following observation and proposals:

Observation 1. Among IMT-2030 usage scenarios, immersive communication (IC) requires the simultaneous handling of multi-modal data traffic with diverse characteristics within strict time constraints.

Observation 2. 6G is expected to enable native integration of AI within the RAN. Therefore, the 6G RAN will require mechanisms for data exchange between the user plane and AI-related functions (e.g., LCM).

Observation 3. Due to the multi-layer (four-layers) protocol architecture in 5G NR, the system incurs overhead from header processing and framing, as well as functional imbalances and redundancies across the layers. In addition, the 5G NR protocol architecture cannot effectively support an AI-native RAN.

Observation 4. 5G NR ARQ is inefficient in handling traffic that requires both low latency and high data rates, due to excessive retransmission delays, the complexity of RLC window maintenance, and redundant functionalities.

Observation 5. The 5G NR Logical Channel Prioritization (LCP) mechanism is inadequate to meet the 6G requirements. Therefore, an new priority handling and enhanced LCP mechanism that enables more fine-grained and dynamic configuration is required.

Observation 6. In 6G, security requirements are more stringent than in 5G, necessitating the introduction of more robust security mechanisms into the user plane.

Proposal 1. Study a packet handling mechanism capable of efficiently processing composite traffic of multiple data streams for multi-modality services, while meeting the required timing constraints and ensuring energy efficiency.

Proposal 2. Study the necessity of data exchange between the user plane and AI-related functions in the 6G RAN, and identify the required data types along with feasible interface mechanisms.

Proposal 3. Study a simple and lightweight user plane protocol architecture consisting of two or thee layers for an AI-native 6G RAN.

Proposal 4. Study a new retransmission mechanism with either ARQ-less or simple ARQ for error recovery.

Proposal 5. Study a new priorty handling mechanism and enhanced LCP in the MAC.

Proposal 6. Study a new security mechanism for MAC CE transmission.

# References

- RP-251881, “New SID: Study on 6G Radio,” June. 2025

- TR 38.914, Study on 6G Scenarios and requirements