3GPP TSG-RAN WG2 Meeting#131bis                               	R2-2507242

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	8.9.3

Source: 	Google

Title: 	Issues on early termination of CB-Msg3-EDT without RRC message

Document for:	Discussion and Decision

# Introduction

In the RAN2#130 and RAN2#131 meetings [1][2], it was agreeed that a CB-Msg4 with neither an RRC message nor a C-RNTI (but with a contention resolution identity) is allowed as a complete response to the CB-Msg3 in the CP solution. The intention is to save the RRC message overhead in the situation when the network intends to terminate a CB-Msg-EDT procedure and keep the UE in RRC_IDLE if no further data are expected.

Agreement in RAN2#130: - A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution. Agreement in RAN2#131: - Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4 without an RRC message, the UE considers UL transmission as successful but the UE does not consider the CB-Msg3-EDT procedure has ended and continues monitoring PDCCH. - Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including both an RRC message and a C-RNTI, the UE behaves as if it received an empty EarlyDataComplete message, terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

To make it easier for the network to decide whether to early terminate a CB-Msg3-EDT procedure upon receiving a CB-Msg3, RAN2 further agreed that a UE can indicate (in CB-Msg3) to the network whether DL data following the UL data in CB-Msg3 is expected or not, using the existing Release Assistance Indication (RAI) mechanism.

Agreement in RAN2#131: - Based on NW indication, it shall be possible for the UE to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not. RAI will be reused for this.

The above agreements and the corresponding amendments in the running CRs work well for eMTC UEs, but may not be applicable for the NB-IoT UEs as it is unclear whether NB-IoT UEs can transmit the “DCQR and AS RAI MAC CE” in CB-Msg3. Several proposals and a corresponding TP are proposed in this paper, with the intention to make NB-IoT UEs benefit from the early termination of a CB-Msg3-EDT procedure.

# Discussion

## RAI for NB-IoT UEs

According to TS 36.331 [3], if the network indicates rai-ActivationEnh in SIB2-NB, NB-IoT UEs are allowed to report the AS RAI using the “DCQR and AS RAI MAC CE”, when connected to EPC.

SystemInformationBlockType2-NB field descriptions

[irrelevant parts omitted]

rai-ActivationEnh Indicates whether the UE is allowed to report the AS Release Assistance Indication using the DCQR and AS RAI MAC CE as specified in TS 36.321 [6] when connected to EPC.

However, according to TS 36.321 [4], the transmission of the “DCQR and AS RAI MAC CE” by an NB-IoT UE seems to be limited to the “regular DCQR” scenarios where Msg3 is not involved. Hence, it is not clear whether an NB-IoT UE can include a “DCQR and AS RAI MAC CE” in a CB-Msg3 and use it to inform the network of whether DL data following the UL data in CB-Msg3 is expected or not, which potentially make the RAN2#131 agreement on RAI ineffective for NB-IoT UEs.

## 5.25 Transmission of Downlink Channel Quality Report The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported. If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur: - DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR"; - for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR". If any type of DCQR has been triggered: - start performing DL channel quality measurements according to TS 36.133 [9]. If "Regular DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - cancel the triggered "Regular DCQR". If "Msg3 DCQR" has been triggered: - if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI: - if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization: - instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19; - else if the uplink grant is not for EDT: - if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1; - cancel the triggered "Msg3 DCQR".

NB-IoT UEs are not allowed to transmit DCQR using “DCQR and AS RAI MAC CE” in CB-Msg3, which may hinder these UEs from informing the network of whether DL data following the UL data in CB-Msg3 is expected or not (i.e., may conflict with the RAN2#131 agreement).

To allow NB-IoT UEs to benefit from the early termination of the CB-Msg3-EDT procedure (e.g., for power saving), these UEs must be able to signal to the network whether or not subsequent DL data is expected after the UL data carried in CB-Msg3. Without this explicit indication, the network lacks the necessary information to safely terminate a CB-Msg3-EDT procedure in an RRC-less manner. To enable this crucial signaling, a straightforward clarification or amendment is required in sub-clause 5.4.8 (Access Stratum Release Assistance Indication) of TS 36.321. The accompanying Annex provides a TP that can be used a potential baseline amendment for the TS 36.321 running CR.

NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB.

If P1 is agreed, consider the TP in Annex as the baseline for amending the current 36.321 running CR.

In addition, although RAN2 has agreed that an NB-IoT UE cannot include the CQI report in CB-Msg3 for non-anchor carriers, the NB-IoT UE should be allowed to include a “DCQR and AS RAI MAC CE” in CB-Msg3 and transmit it in a non-anchor carrier, for the purpose of indicating whether DL data following the UL data in CB-Msg3 is expected or not. In this case, the UE shall set the ‘Quality Report’ field to the value ‘noMeasurement’ in the “DCQR and AS RAI MAC CE”, to avoid any confusion.

NB-IoT UEs shall set ‘Quality Report’ to the value ‘noMeasurement’ in the “DCQR and AS RAI MAC CE”, when transmitting the “DCQR and AS RAI MAC CE” in a CB-Msg3 and in a non-anchor carrier.

## UE capability on the early termination of CB-Msg3-EDT

The introduction of the early termination of the CB-Msg-EDT procedure implements two key changes in the Rel-19 specifications. The first change is that the UE MAC needs to indicate the successful completion of a CB-Msg3-EDT procedure to the upper layers upon receiving a CB-Msg4 containing  neither a C-RNTI nor an RRC message, and the sceond change is that the UE RRC receiving the indication needs to perform the action in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. These changes are clearly deviated from the legacy UE behaviours in a random access procedure or in an EDT procedure. In RAN2#131, it was agreed that the UE shall be able to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not, using the existing RAI scheme. Only with this new indication from the UE, the network receiving a CB-Msg3 is able to decide whether to terminate a CB-Msg3-EDT in a RRC-less manner and move UE back to idle. Therefore, we think the new feature (i.e., terminating an EDT procedure without an RRC message) is tightly connected to UE’s RAI capability.

Terminating (successfully) an EDT procedure without an RRC message is a new procedure/feature introduced in Rel-19, and is tightly connected to UE’s RAI capability.

Without particularly mentioning this new feature (i.e., terminating an EDT procedure without an RRC message) in the specification for UE capability (TS 36.306 [5]), it implies that the UE is mandated to implement it. However, we believe it is only required for a UE capable of sending an RAI indication to implement the new feature, as the network may never terminate a CB-Msg3-EDT procedure in a RRC-less manner without knowing whether there is subsequent DL/UL data following the CB-Msg3. Therefore, we respectfully ask RAN2 to discuss first whether the support for terminating an EDT procedure without an RRC message is a mandatory or optional feature for the UE supporting CB-Msg3-EDT.

RAN2 to discuss whether the support for terminating (successfully) a CB-Msg3-EDT without an RRC message is a mandatory or an optional feature for the UE supporting CB-Msg3-EDT.

If RAN2 concludes that the support for terminating CB-Msg3-EDT without an RRC message is an optional feature, TS 36.306 [5] needs to capture this optional feature with or without a UE radio access capability parameter. Our preference is to reuse the existing rai-SupportEnh-r16 parameter and make it also indicate the support for terminating CB-Msg3-EDT without an RRC message. This would further tighten these two dependent features, introduce no additional signaling, and have the minimal spec impact. Another possibility is to introduce a new UE radio access capability parameter for indicating the support for terminating CB-Msg3-EDT without an RRC message, making it possible for the UE to indicate the RAI support and the support for terminating CB-Msg3-EDT without an RRC message independently and separately. Yet another possibility is to introduce a new optional feature without UE radio access capability parameter used to mark the support for terminating CB-Msg3-EDT without an RRC message.

If the support for the terminating CB-Msg3-EDT without an RRC message is an optional feature according to P4, RAN2 to discuss how to capture this optionality in TS 36.306, among the following options: A) the existing rai-SupportEnh-r16 also indicates the support for terminating CB-Msg3-EDT without an RRC message; B) a new UE radio access capability parameter is introduced to indicate the support for terminating CB-Msg3-EDT without an RRC message; C) a new optional feature without UE radio access capability parameter is introduced to mark the support for terminating CB-Msg3-EDT without an RRC message.

# Conclusion

In this paper, we discuss the leftover issues on the transmission of RAI in a CB-Msg3-EDT procedure. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

RAI for NB-IoT UEs

Observation 1	NB-IoT UEs are not allowed to transmit DCQR using “DCQR and AS RAI MAC CE” in CB-Msg3, which may hinder these UEs from informing the network of whether DL data following the UL data in CB-Msg3 is expected or not (i.e., may conflict with the RAN2#131 agreement).

Proposal 1	NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB.

Proposal 2	If P1 is agreed, consider the TP in Annex as the baseline for amending the current 36.321 running CR.

Proposal 3	NB-IoT UEs shall set ‘Quality Report’ to the value ‘noMeasurement’ in the “DCQR and AS RAI MAC CE”, when transmitting the “DCQR and AS RAI MAC CE” in a CB-Msg3 and in a non-anchor carrier.

UE capability on the early termination of CB-Msg3-EDT

Observation 2	Terminating (successfully) an EDT procedure without an RRC message is a new procedure/feature introduced in Rel-19, and is tightly connected to UE’s RAI capability.

Proposal 4	RAN2 to discuss whether the support for terminating (successfully) a CB-Msg3-EDT without an RRC message is a mandatory or an optional feature for the UE supporting CB-Msg3-EDT.

Proposal 5	If the support for the terminating CB-Msg3-EDT without an RRC message is an optional feature according to P4, RAN2 to discuss how to capture this optionality in TS 36.306, among the following options: A) the existing rai-SupportEnh-r16 also indicates the support for terminating CB-Msg3-EDT without an RRC message; B) a new UE radio access capability parameter is introduced to indicate the support for terminating CB-Msg3-EDT without an RRC message; C) a new optional feature without UE radio access capability parameter is introduced to mark the support for terminating CB-Msg3-EDT without an RRC message.

# Reference

R2-2505002, Report of 3GPP TSG RAN WG2 meeting #130, ETSI MCC.

Draft Report of 3GPP TSG RAN WG2 meeting #131, ETSI MCC.

3GPP TS 36.331 v18.6.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

3GPP TS 36.321 v18.4.0, Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC); Protocol specification.

3GPP TS 36.306 v18.5.0, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities.

# Annex: TP for enabling AS RAI in CB-Msg3

--------------------------------------------- TP for TS 36.321 [4] ---------------------------------------------------

5.4.8	Access Stratum Release Assistance Indication

Access Stratum Release Assistance Indication is used to provide the serving eNB with information whether subsequent DL or UL transmission is expected. AS RAI uses the DCQR and AS RAI MAC Control Element. Upper layers trigger AS RAI.

For EDT and transmission using PUR, if AS RAI is triggered by upper layers but is not included in the resulting MAC PDU with the MAC SDU as a result of logical channel prioritization, AS RAI is cancelled, for other transmissions if AS RAI is not included in the resulting MAC PDU as a result of logical channel prioritization, AS RAI may be cancelled.

If rai-Activation is configured and a buffer size of zero bytes has been triggered for the BSR and no subsequent DL and UL data transmission is expected, and if rai-ActivationEnh is enabled and applicable as specified in TS 36.331 [8], it is up to UE to send BSR MAC control element or DCQR and AS RAI MAC control element.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, transmission of AS RAI in CB-Msg3 is configured by upper layers in rai-ActivationEnh, in which case AS RAI is referred below to as "CB-Msg3 AS RAI"

If "CB-Msg3 AS RAI" has been triggered:

-	if an uplink grant has been selected by the UE for CB-Msg3 transmission:

-	if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

- --------------------------------------------- End of TP ---------------------------------------------------