3GPP TSG-RAN WG2 Meeting #131bis	R2-2507250

Prague, Czech Republik, 13th – 17th October 2025

Agenda item:		10.3.1

Source:		Lenovo

Title:		6G Radio protocol architecture - User Plane Aspects

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss User Plane (UP) aspects of the 6G Radio Protocol Architecture. The discussion is divided into two parts. In the first part, we examine the 5G User Plane protocol stack and highlight some of the inefficiencies and limitations of its current protocol design. In the second part, we identify potential areas of investigation for 6G, focusing on the essential functions that a 6G User Plane should support.

# Discussion

### 5G NR Protocol stack UP

The 5G NR protocol stack was initially developed based on the protocol stack designed for LTE, with modifications to accommodate the specific key performance indicators (KPIs) of 5G. This protocol stack has evolved, with new optimizations and features added across various releases to support a wide range of services and their diverse service requirements. A notable issue with this approach is that the continual addition of new functions and optimizations has resulted in the duplication of functionalities across different protocol layers, leading to an overall complex protocol behaviour and inherent inefficiencies. In current Rel-19 XR enhancements WI, optimizations are introduced addressing some of those inefficiencies, e.g. enhancements targeting the inefficient ARQ retransmission protocol behaviour stemming from the independent receiving operations implemented in PDCP and RLC layer as further discussed below.

For 6G, the aim should be therefore to streamline the protocol stack compared to 5G, by removing functionalities which are found to be inefficient/unnecessary or redundant.

Observation 1: The 5G protocol stack has evolved with new optimizations and features added across various releases resulting in the duplication of functionalities across different protocol layers, leading to an overall complex protocol behaviour and inherent inefficiencies. For 6G, the aim should be therefore to streamline the protocol stack compared to 5G, by removing functionalities which are found to be inefficient/unnecessary or redundant.

One area worth exploring is for example the consolidation of the various data recovery mechanisms supported in 5G on different protocol layers, e.g. HARQ on PHY/MAC as well as ARQ on RLC layer, a data recovery mechanism on PDCP as well as higher layer, e.g. TCP layer, retransmission mechanisms. It will be beneficial to investigate whether some simplification or streamlining of the recovery mechanisms supported on the AS can be done. For example, by increasing the reliability of the HARQ protocol and also considering the additional reliability offered by AI/ML expected to be available from day one the recovery mechanism on RLC and PDCP can be simplified or even merged into one recovery procedure.

One further issue with the current protocol design in 5G – as for example analysed in Rel-18/19 XR WI – is the fact that the RLC and PDCP receiver windows, may become unsynchronized. At present, the PDCP and RLC Tx/Rx windows in the user plane protocol stack function independently from each other. On the receiving side, the PDCP layer maintains a reordering/receiving window to receive PDCP PDUs and deliver them (in-sequence) to higher layers. While in RLC, the RLC receiving entity maintains a reception window to receive RLC PDUs and transmit them to PDCP entity. In PDCP, the reordering window is controlled by a t-Reordering timer that is configured by RRC. When this timer expires, it causes the reordering window to slide forward, i.e., update the lower bound of the reordering window. If a packet is received outside of this receiving window, it is discarded by the PDCP receiving entity. In RLC AM Mode, the receiving entity slides its reception window when the lowest packet in the window (i.e., packet with SN that matches the lower bound of the RLC AM reception window) has been completely received and an acknowledgement has been sent for the same. As these two layers function independently, the reception window and reordering window may not be updated at the same time. If the PDCP reordering window moves forward while the RLC window is not updated, the RLC layer might transmit packets to the PDCP layer that are outside of the reordering window leading to discard of such packets. For example, figure 1 shows the reordering window in PDCP and reception window in RLC AM mode wherein the lower bounds of both windows are expecting PDU with SN = 0 (COUNT = 0 in case of PDCP wherein COUNT = [HFN, SN]). As per figure 1, the RLC receiving entity has correctly and fully received packets numbered 0, 2 and 3, and hence transmitted this to PDCP layer. PDCP has also received packets numbered 0, 2 and 3, with a missing packet numbered 1. Hence the PDCP receiving entity starts the t-Reordering timer when it receives packet 2 before 1. While in RLC, the t-reassembly timer is started since one or more segments of packet 1 are not received yet. When the t-Reordering timer expires, the PDCP reordering window updates its lower bound to the next packet that has not been consecutively received. For example, in figure 2, packet 4 is the next packet that has not been consecutively received after the t-Reordering expired. Hence, the lower bound of the PDCP reordering window is moved to packet 4, whereas in RLC, the t-reassembly only triggers a status report upon expiry and the reception window does not update unless the status report contains an ACK for packet 1. Since the RLC receiving entity has not received packet 1, it still tries to recover the packet by means of (re)transmissions. Once the packet is recovered by the RLC, it transmits this to PDCP which will discard the packet as it is outside of the reordering window. These redundant transmissions are not only a waste of resources but can also add unwanted latency on the user plane.

Figure 1: PDCP and RLC Reception                           Figure 2: PDCP and RLC Reception after t-Reordering expiry

In Rel-19 solutions were introduced to remove some of those inefficiencies.

Another issue with the 5G protocol stack design is that at most one complete PDCP SDU or PDCP PDU, e.g. since no concatenation is supported on PDCP layer, can be carried in a RLC PDU. Even though there is a one-to-one mapping between PDCP PDU and RLC PDU, the PDCP layer as well as the RLC layer assigns its own sequence number (SN), which increases the general Layer 2 overhead as well as the processing complexity. However, PDCP concatenation—a mechanism that combines multiple PDCP SDUs into a single PDCP PDU—offers several benefits: the number of L2 headers is significantly reduced as the number of concatenated SDUs increases, lowering overall L2 overhead. Furthermore, the processing efficiency of integrity protection/verification and (de)ciphering of UP can be significantly improved by ‘concatenation’ of small PDCP PDUs e.g., TCP ACKs.

In 5G NR, a large number of different PDU formats are specified for Layer 2 protocols (e.g., MAC PDUs, RLC PDUs), addressing many specific exceptions. The header design philosophy largely aimed at optimizing overhead “down to the last bit”. While this approach minimizes signaling overhead, it has resulted in complex and fragmented header structures in the specifications.

For 6G, such complexity should be avoided. Instead, Layer 2 header formats should be designed with simplicity, extensibility, and implementation efficiency in mind, even if this entails a marginal increase in overhead. This would improve protocol clarity, ease of implementation, and long-term maintainability of the specifications.

Observation 2:  The Layer 2 PDU format/header design philosophy largely aimed at optimizing overhead “down to the last bit”. While this approach minimizes signaling overhead, it has resulted in complex and fragmented format/header structures in the specifications. For 6G, such complexity should be avoided.

Furthermore, the 5G protocol stack was generally designed in an information-agnostic way where neither content of incoming data packets nor their attributes are considered when allocating radio resources, e.g. UE simply selects the next packet in the queue/buffer for transmission, i.e., UE performs selection in an information-agnostic manner. It should be noted that the enhancements specified in recent releases (Rel-18/19) for the efficient support of XR services, introduce application awareness in RAN only to a very limited extend.

For 6G, it would be desirable that application awareness is natively supported in the RAN, to help the network utilise the resources more efficiently and improve the quality of experience (QoE). The 6G protocol stack should be designed to support application-optimized communication, e.g. semantics and context of the data should be considered for resource allocation given that different data packets of a service contribute differently to the user experience. In general, the 6G protocol stack should facilitate better integration and collaboration between applications and the radio access network, enabling more flexible and efficient service delivery.

Observation 3: 5G protocol stack was generally designed in an information agnostic way, e.g. neither content of incoming data packet nor the contribution to application layer performance are considered during radio resource allocation. To some very limited extend application-layer awareness in AS was introduced for XR services.

To support 6G services and applications, and to meet the requirements of 6G higher KPIs, we also see the need to enhance the resource allocation schemes in particular for the uplink. The goal should be to minimize the overall uplink scheduling delay and to enable a more flexible logical channel prioritization/ transport block generation procedure, which can adjust to the changing significance of a data packet from the perspective of user experience.

Given that downlink/uplink resource signalling up to 5G NR supports only a single modulation and coding scheme (MCS) per transport block, even for allocations covering a wide bandwidth, resource allocation in 6G should allow for the assignment of multiple MCS for data transmissions. This would enable link adaptation to take advantage of frequency diversity gains across frequency ranges with varying signal-to-interference-plus-noise ratios (SINR).

Random Access procedure

In 5G initially only the 4-step Random Access procedure was supported. The RACH was designed for beamforming and massive MIMO, e.g. beam-aware RACH, multiple preamble sets. In order to improve a latency of the random access the 2-step random access procedure was introduced in Rel-16.

The partitioning in the 2-step RACH procedure may require partitioning the entire set of RACH resources into two pools, with each pool dedicated to the corresponding RACH procedure (i.e., 2-step or 4-step RACH).

In 3GPP Rel-16, the gNB may broadcast the two pools using a RACH-ConfigCommonTwoStepRA information element (IE), which may be further subdivided via a GroupB-ConfiguredTwoStepRA-r16 IE contained in the RACH-ConfigCommonTwoStepRA IE. The UE may then indicate whether it is using a 2-step or 4-step RACH procedure in message A (MsgA) for 2-step RACH procedure, or in an Msg1 for 4-step RACH, respectively, by selecting and transmitting a RACH resource from the corresponding pool of resources.

Contention-free radio access (CFRA) resources and CBRA resources may be further partitioned. For example, CFRA resources may be dedicated to a given UE (e.g., in RRC Connected mode with an assigned cell radio network temporary identifier (C-RNTI)), while the UE may need to contend for RACH resources in the contention-based pool. For CBRA partitioning, different UEs may select the same CBRA resource, and/or may interfere with other UEs, causing an undesirable RACH collision.

RACH partitioning was introduced in Rel-17/18 to enable the network to identify each feature/Feature combination based on the preamble/RO used by the UE, e.g. RACH resources may be further partitioned to support initial access of some 3GPP Rel-17 features or feature combinations, such as:

| Feature | RACH indication / reason for partitioning |
| --- | --- |
| CovEnh | To indicate the need for coverage enhancement (e.g., for request of Msg3 repetition) to the network |
| Slicing | To indicate the need for prioritization and isolation of a slice (including RACH isolation) to the network |
| RedCap | To indicate the reduced capabilities of the UE to the network so that the network can adapt subsequent transmissions (as compared to a regular UE) |
| SDT | To indicate the small data transmission (SDT) procedure, thus to request a larger Msg3 size or MSGA size (as compared to regular Msg3/MSGA size for non-SDT/legacy resume) |

Over successive releases, the Random Access (RACH) procedure has been subject to multiple enhancements to address new use cases and performance requirements. While each enhancement was introduced with the intent of improving efficiency, latency, or coverage, the cumulative effect has been a significant increase in procedural and configurational complexity.

This evolution has not only increased the behavioral complexity on the UE side—where different RACH modes and configurations must be supported depending on capability and scenario—but also on the network side, where the gNB is required to manage heterogeneous RACH resources and procedures simultaneously.

As a result, the specification text covering Random Access has expanded considerably across releases, with multiple branching conditions, exceptions, and optional behaviors. This proliferation of procedures has made the handling of RACH resources more difficult, and the overall specification framework more extensive and less straightforward to interpret.

### 6G design principles / areas for investigation

##### Functionalities and mapping to protocol layers

In current discussions on protocol stack architecture, the debate often begins with identifying which functionalities are supported by which protocol layers. This approach risks prematurely constraining solutions and may lead to inefficient allocation of functions across layers. Starting from protocol layers without a prior consolidated list of required functionalities may cause:

- Overlaps in functionality assignment across layers.

- Gaps where certain required functionalities are not addressed.

- Layer-driven rather than requirement-driven design decisions.

RAN2 discussions should follow a two-step order:

- Define and consolidate required functionalities of the protocol stack

  - Examples: reliability, retransmission, QoS differentiation, security, mobility support, service mapping, etc.

  - Ensure requirements are technology-neutral and do not presuppose specific layer implementations.

- Map functionalities to protocol layers

  - Once the functionality set is agreed, allocate each to the most suitable layer (e.g., PDCP, RLC, MAC, RRC, SDAP).

  - Verify that each functionality is uniquely covered, and no duplication or gaps remain.

Proposal 1: It is proposed that RAN2 first agree on the complete set of required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.

##### Lower layer security

The 5G NR Access Stratum (AS) security architecture protects communication between the UE and gNB. It is built on mutual authentication and key management established in the Non-Access Stratum (NAS), after which AS security keys are derived. Two main functions are applied: integrity protection to ensure authentic and unaltered reception and ciphering to protect confidentiality of both signaling and user plane data. Security contexts are refreshed to mitigate replay and key compromise risks. The security is performed in PDCP layer, which sits below RRC and user plane SDAP layer.

Protection of MAC layer control information has been an important discussion in 5G especially around the work for L1/ L2 mobility where some MAC CEs have been designed and specified carrying information centered around UE mobility.

Also, other lower layer signalling e.g., in PUCCH, PDCCH, Paging messages and system information etc. are transmitted in open. Some of the information carried in MAC CEs as well as in physical layer signalling can be used to compromise user privacy, including identity and others may be used to launch a DoS attack (Denial of Service). While a DoS attack (Denial of Service) may not alone lead to issues around data privacy and authentication, nevertheless, should be minimized to avoid bad user experience. A prominent example is as already mentioned LTM where in the 3GPP Rel-19 a study was carried out on the security aspects of Mobility (LTM), where the solutions and key issues are capture in the living document in S3-252053 [1]. The issue is further discussed and extended from the MAC CE for the LTM feature to other contents of the MAC CE in [2], [3], [4], proposing that solutions should be developed to protect the UE from attackers, spoofing or modifying the MAC CE in various conditions.

For 6G, we propose that 3GPP investigate potential lower-layer security issues. As a first step, SA3 should analyze and assess the problem. If SA3 concludes that a solution is necessary, RAN2 should then be tasked with developing the appropriate solution.

##### Configuration adaptability

In 5G systems, radio and protocol parameters are configured to (semi-)static values based on worst-case network assumptions. Any subsequent adjustment typically requires an RRC Reconfiguration procedure, which is signaling-intensive and incurs a non-negligible delay. This results in sub-optimal performance, particularly in scenarios with highly dynamic conditions.

For 6G, it would be beneficial to introduce mechanisms that allow the UE to autonomously adapt selected parameters within network-defined bounds, thereby reducing reliance on frequent RRC reconfigurations. Such adaptation could leverage AI/ML models executed at the UE, enabling adaptation of parameters intelligently to meet key performance indicator (KPI) targets.

There may be different categories of parameters /configurations as follows:

- Fully Network-Controlled Parameters

  - UE shall strictly follow the configuration provided by the network.

- UE-Adjustable Parameters with Network Notification

  - UE may adjust parameter values within network-defined ranges, with mandatory reporting of the applied value to the network.

- UE-Autonomous Parameters

  - UE may autonomously adapt parameter values, e.g. based on predefined rules or conditions, without the need to inform the network. Examples of those category of parameters could be e.g. RLC timers (t-Reassembly), LCH priority, PBR value

This framework would enable reduced signaling overhead through fewer RRC reconfiguration procedures and a faster adaptation to local and dynamic conditions resulting in an enhanced QoS tailoring to diverse UE applications.

Proposal 2: RAN2 should study solutions which enable UE to adapt NW parameter/configurations, e.g. RLC timer or LCH priority, intelligently within some limited ranges which are under network control.

##### Application awareness

The 5G protocol stack has been designed in an information-agnostic manner where neither the content of data packets nor their inter-relationships are considered when allocating radio resources. Packets are transmitted in buffer order without considering application-level semantics or QoE contribution. While Rel-18/19 introduced the concept of an application layer data unit (PDU set) to enable limited application awareness in the RAN, the uplink (UL) remains constrained as the gNB is for example not aware of metadata associated with a PDU. Furthermore, inter-dependencies across PDUs or bearers, or AL-FEC information are currently not exploited for resource allocation at the RAN. Different data units of the same service are treated equally despite unequal contribution to user experience.

For 6G, it would be desirable that application awareness is natively supported in the RAN, to help the network utilise the resources more efficiently and improve the quality of experience (QoE). The 6G protocol stack should be designed to support application-optimized communication, e.g. semantics and context of the data should be considered for resource allocation given that different data packets of a service contribute differently to the user experience. Mechanisms should be introduced for the application layer to convey metadata to the RAN, e.g., packet priority, dependency, relevance, or semantic weight. The metadata should be available to both UE and gNB in the AS for uplink and downlink resource optimization. RAN scheduling decisions should incorporate semantic importance of PDUs.  Retransmission, HARQ, and resource allocation strategies should be adapted based on application relevance. In general, the 6G protocol stack should facilitate better integration and collaboration between applications and the radio access network, enabling more flexible and efficient service delivery, e.g. define standardized interfaces between application and RAN layers to enable seamless metadata exchange.

Proposal 3: RAN2 should study support of native application-awareness at the Access Stratum.

##### Data recovery mechanism

5G uses the following L2 protocol layer to enable error recovery: MAC, RLC and PDCP each of them targeting a different retransmission mechanism in terms of latency and reliability. 
MAC/PHY which employs a HARQ protocol enables fast retransmissions. Due to a wrong link adaption the selected MCS may not be suitable for the channel conditions which will lead to residual HARQ errors. The main reason for residual HARQ errors is furthermore errors of the HARQ feedback signalling, e.g. ACK->NACK errors or NACK->ACK errors on PUCCH or PDCCH misdetection and DTX-> ACK errors. To recover from such errors RLC ARQ can be used, e.g RLC AM. 
On PDCP layer retransmissions of lost data is supported at certain events like handover, e.g. when HARQ and RLC are reset.

Improving HARQ feedback reliability and cross-layer interactions between HARQ and ARQ enable a simplified and faster recovery mechanism of residual HARQ errors. When HARQ protocol at the transmitter side informs RLC Tx entity about HARQ failures the RLC recovery delay can be reduced by triggering local RLC retransmission, e.g. no need to rely on gap detection on the RLC receiver side and subsequent Status reporting, e.g. RLC Status Report.

It should be noted that the RLC ARQ procedure specified in 5G NR is a slow data recovery mechanism which may not be suitable for latency critical services requiring at the same time a high reliability.

Another aspect to be considered is the enablement of a faster advancement of the RLC transmitting window in order to avoid window stalling. In 5G NR, the RLC Tx window advancement relies on the reception of RLC Status Reports (SR) from the receiving side. The legacy RLC Status Report provides a comprehensive view of the reception state across the RLC window, including both acknowledgments for successfully received PDUs as well as indications of missing PDUs requiring retransmission.

A limitation of the current RLC Status PDU format, however, is that among the PDUs acknowledged, all remaining PDUs within the window are implicitly negatively acknowledged. To prevent premature NACK feedback, the t-Reassembly timer is employed, allowing lower-layer retransmissions (e.g. HARQ) to complete. While this mechanism avoids unnecessary retransmission triggers, it also delays the reporting of positive acknowledgments. Consequently, the advancement of the RLC Tx window is held back, potentially resulting in transmission window stalling.

Proposal 4: RAN2 to study streamlining of different data recovery protocols in AS by e.g. considering increased HARQ feedback reliability, HARQ/ARQ interactions.

##### LCP enhancements

The currently specified Logical Channel Prioritization (LCP) procedure in 5G NR is quite in-flexible: all PDUs belonging to a given logical channel (LCH) / radio bearer are treated uniformly from a QoS and scheduling perspective because the LCH priority is statically configured. In practice, packets multiplexed within the same LCH can have very different contributions to user experience and their relative importance may change over time (for example due to delay evolution or application-layer significance). XR-related enhancements introduced priority adaptation based on delay status (e.g. elevating an LCH’s priority when delay-critical data is pending). However, these enhancements operate only at the inter-LCH level: when an LCH’s priority is elevated, all packets of that LCH are elevated, regardless of whether an individual packet is delay-critical or not. In other words, intra-LCH prioritization is not supported.

Further, current UL resource allocation and prioritization do not consider other packet-level factors that affect user experience, such as:

- Per-packet delay urgency (time-varying deadlines within an LCH);

- Per-packet importance (relative contribution to application-level QoE, which may change dynamically);

- Inter-packet dependencies (e.g. coding/decoding or rendering dependencies between packets within or across LCHs).

These limitations can cause suboptimal scheduling and transport block formation: delay-critical or high-value packets may be delayed behind lower-value packets from the same LCH, or resources may be allocated inefficiently because static LCH priorities do not reflect packet-level urgency.

In 6G RAN2 should study a more flexible LCP and transport block generation framework that supports a fine-grained, e.g. per-packet, prioritization and time-variant priority adaptation. The framework should LCP procedure to adjust the prioritization to the changing significance of a data packet from the perspective of user experience thereby improving resource efficiency and end-user experience.

Proposal 5: RAN2 to discuss enhancements for the Uplink prioritization /LCP procedure compared to NR LCP, e.g. adjust to the changing significance of a data packet from the perspective of user experience

##### UL scheduling latency

Low latency is critical for the perceived Quality of experience of a service by the user. The dynamic UL scheduling procedure specified for 5G NR has an inherent delay due to the multi-step approach, e.g. SR followed by BSR and subsequent UL grants tailored to the reported buffer status. So far, two approaches can been mainly used to mitigate this latency:

- usage of configured UL grants

- configuration of a small periodicity for D-SR on PUCCH.

The configuration of frequent SR occasions enables a reduced time for requesting UL resource, but on the other hand PUCCH resources are capacity limited. CG UL grants are suitable for semi-periodic traffic, with constant data sizes. Since the MCS is selected for all the CG-UL grant occasions, the scheduler typically selects a conservative MCS in order to cater for changing channel conditions. This inherently reduces the resource efficiency.

Both solutions have limitations: CG-UL lacks adaptability for bursty or variable traffic, while frequent SR occasions are not scalable due to PUCCH capacity constraints.

For 6G, RAN2 should study mechanisms to further reduce UL scheduling latency. One candidate is to enable contention-based UL transmissions, where limited UL resources are available for immediate use without requiring a dedicated grant. Such a mechanism could allow, for example, the immediate transmission of buffer status or delay status information, thereby reducing UL latency compared to the procedures in 5G NR.

Also, an AI/ML-based or AI/ML assisted uplink resource allocation procedure should be studied for 6G. For example, knowledge about the upcoming data traffic and its statistics in advance, such as time of occurrence, volume, and QoS of data, can be beneficial in scheduling decisions and support services that require low-latency transmissions. Predictions at the network-side using some assistance (UE status information) from the UE can be applicable for a less volatile scenario. However, the performance is likely to be limited by the uplink data knowledge available at the NW-side. This may lead to signaling of additional information from the UE to the NW (compared to the legacy uplink reporting, such as SR/BSR) to accomplish prediction outcomes with high accuracy. In contrast, necessary measures for uplink scheduling decisions can be performed closer to the edge, i.e., at the UE-side, which can be reported to the network well in advance by exploiting the knowledge and experience of the device.

Proposal 6: RAN2 to study mechanisms for 6G aiming to the reduce the UL scheduling latency, e.g. support of contention-based UL transmissions for certain data (e.g. uplink control information) or AI-based resource allocation procedure.

##### PDCP concatenation

In 5G NR concatenation of PDCP SDUs is not supported. Even though there is a one-to-one mapping between PDCP PDU and RLC PDU, the PDCP layer as well as the RLC layer assigns its own sequence number (SN), which increases the general Layer 2 overhead as well as the processing complexity.

Supporting concatenation at PDCP layer offers several notable benefits that can be further studied for 6G:

- The number of L2 headers is significantly reduced as the number of concatenated SDUs increases, lowering overall overhead. Currently, each IP packet has headers of each PDCP, RLC and MAC layer in 5G.

- The processing efficiency of integrity protection/verification and (de)ciphering of UP can be significantly improved by ‘concatenation’ of small PDCP PDUs e.g., TCP ACKs.

- It enables full utilization of the data processing capacity of hardware accelerators designed for 9000-byte payloads, maximizing hardware efficiency.

Proposal 7: RAN2 to study the concatenation of SDUs at the PDCP layer, e.g. PDCP concatenation.

# Conclusion

In this contribution, we discuss User Plane (UP) aspects of the 6G Radio Protocol Architecture. We have the following observations and proposals:

Observation 1: The 5G protocol stack has evolved with new optimizations and features added across various releases resulting in the duplication of functionalities across different protocol layers, leading to an overall complex protocol behaviour and inherent inefficiencies. For 6G, the aim should be therefore to streamline the protocol stack compared to 5G, by removing functionalities which are found to be inefficient/unnecessary or redundant.

Observation 2:  The Layer 2 PDU format/header design philosophy largely aimed at optimizing overhead “down to the last bit”. While this approach minimizes signaling overhead, it has resulted in complex and fragmented format/header structures in the specifications. For 6G, such complexity should be avoided.

Observation 3: 5G protocol stack was generally designed in an information agnostic way, e.g. neither content of incoming data packet nor the contribution to application layer performance are considered during radio resource allocation. To some very limited extend application-layer awareness in AS was introduced for XR services.

Proposal 1: It is proposed that RAN2 first agree on the complete set of required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.

Proposal 2: RAN2 should study solutions which enable UE to adapt NW parameter/configurations, e.g. RLC timer, LCH priority, intelligently within some limited ranges which are under network control.

Proposal 3: RAN2 should study support of native application-awareness at the Access Stratum.

Proposal 4: RAN2 to study streamlining of different data recovery protocols in AS by e.g. considering increased HARQ feedback reliability, HARQ/ARQ interactions.

Proposal 5: RAN2 to discuss enhancements for the Uplink prioritization /LCP procedure compared to NR LCP, e.g. adjust to the changing significance of a data packet from the perspective of user experience

Proposal 6: RAN2 to study mechanisms for 6G aiming to the reduce the UL scheduling latency, e.g. support of contention-based UL transmissions for certain data (e.g. uplink control information) or AI-based resource allocation procedure.

Proposal 7: RAN2 to study the concatenation of SDUs at the PDCP layer, e.g. PDCP concatenation.

# References

[1] 	3GPP S3-252053 “Living document on NR mobility enhancement”, Samsung

[2] 	3GPP S3-251958 “New SID on MAC layer security”, ZTE Corporation

[3] 	3GPP S3-252190 “Security consideration for MAC CE and lower layers”,OPPO

[4] 	3GPP S3-252192 “Study on Security Aspects of Lower Layers”, OPPO