3GPP TSG-RAN WG2 Meeting #131bis	R2-2507254

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on two SMTC periodicities

WID/SID:	NR_NTN_Ph3-Core – Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#131, it was discussed whether to support the configuration of two different SMTC periodicities with different offsets. It was agreed that RAN2 would support to configure 2 different periodicities, but due to concerns raised on RAN4 progress on this, it was amended that in case RAN4 could not complete the work in time this agreement could be reverted. There were no clear consensus on how to revert the agreement and still complete the work item, UE capabilities etc. This agreement was sent to RAN4 in an LS.

- RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. - Send an LS to RAN4 on latest RAN2 agreements about SMTC enhancements

In this contribution we present our views regarding the configuration of two different SMTC periodicities per frequency layer.

# 2	Discussion

The discussion in RAN2 was based on an LS received from RAN4 in RAN2#131 mentioning that the RAN4 requirements assume that the periodicity of the SSB is no shorter than the periodicity of the configured SMTC.

- For the SMTC enhancement on configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements: From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but that requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Note: RAN4 has not estimated the full impact during RAN4#115 meeting. There is a concern on the workload on to support those agreements in Rel-19 timeframe from RAN4 requirements perspective.

As discussed in the RAN2 meeting, RAN4 may not be able to finish the work on the dual SMTC periodicities within reasonable time. If RAN4 cannot finish the RRM requirements design within this Work Item, the current version of the agreement states that RAN2 will ‘somehow’ limit the configuration to one periodicity only. Depending on the time spent by RAN4, such NBC may be a bad solution in case some operators has already planned deployment using the current spec as baseline.

Observation 1: RAN2 to be cautious on agreeing to void the dual SMTC periodicities in an NBC manner.

If the network is only allowed to configure one SMTC periodicity, but two different SSB periodicities are present in the deployment, we see two possible scenarios, as illustrated in Figure 1:

Figure 1 - Possible scenarios when only one SMTC periodicity is configured for neighbour cells with different SSB periodicity

In Scenario A, the UE is configured with a SMTC periodicity corresponding to the largest SSB periodicity. In this case, for all the deployments in which SSBs with different periodicities are configured among the neighbor cells, for example cells that support DL-CE and cells that do not support DL-CE, the network is forced to configure measurements with the largest periodicity. Here we note that all the measurement delays are defined as a function of the SMTC periodicity, then by forcing the network to configure a SMTC periodicity with 160 ms, all the SSB based measurements will be delayed at least by a factor of 4.

Observation 2: if the network can only configure one SMTC periodicity, and it configures the largest SSB periodicity, all the RRM measurements will be delayed.

In Scenario B, the UE is configured with a SMTC periodicity corresponding to the shortest SSB periodicity. In this case, there will be some SMTC occasions that will not overlap with the SSBs of some of the cells, i.e. measurements of Cell 1 in the example in Figure 1, which is against the current assumption of RAN4.

Observation 3: If the network configures only one SMTC periodicity, according to the shortest SSB, the UE may still be doing the measurements as configured. The measurement requirements are not applicable to cell 2, as they currently assume that the SMTC periodicity is equal to or longer than the SSB periodicity.

Observation 4: In practice, limiting the network to configure only one SMTC periodicity will lead to the situation in which the network is forced to always select SMTC periodicity of 160 ms for all neighbor cells even when only a single DL-CE cell is measured.

An alternative solution is to maintain the RAN2 agreement to enable the network to configure two SMTC periodicities irrespectively of RAN4 conclusion. In such case, even if RAN4 cannot complete the requirements in time, RAN2 should make a note that the UE in stage 2 spec stressing that the measurement requirements are not applicable i.e. that it is up to UE implementation how

Proposal 1: In case RAN4 cannot complete requirements in time, instead of limiting the configuration to one periodicity only, do not revert RAN2 solution but add a note to stage 2 specification mentioning that for 2 configured SMTCs the RAN4 requirements may not apply and time to complete measurements will be up to UE implementation.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: RAN2 to be cautious on agreeing to void the dual SMTC periodicities in an NBC manner.

Observation 2: if the network can only configure one SMTC periodicity, and it configures the largest SSB periodicity, all the RRM measurements will be delayed.

Observation 3: If the network configures only one SMTC periodicity, according to the shortest SSB, the UE may still be doing the measurements as configured. The measurement requirements are not applicable to cell 2, as they currently assume that the SMTC periodicity is equal to or longer than the SSB periodicity.

Observation 4: In practice, limiting the network to configure only one SMTC periodicity will lead to the situation in which the network is forced to always select SMTC periodicity of 160 ms for all neighbor cells even when only a single DL-CE cell is measured.

Proposal 1: In case RAN4 cannot complete requirements in time, instead of limiting the configuration to one periodicity only, do not revert RAN2 solution but add a note to stage 2 specification mentioning that for 2 configured SMTCs the RAN4 requirements may not apply and time to complete measurements will be up to UE implementation.