3GPP TSG-RAN WG2 Meeting #131bis	R2-2507255

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.11.2

Source:	Samsung

Title:	Discussion on the remaining MAC open issues

Document for:	Discussion & Decision

Introduction

In this paper, we share our views for the remaining MAC open issues.

Discussion

[MAC-2] How to clarify "the preambleReceivedTargetPower used in Msg3 transmission power derivation is that configured for legacy RO, in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured."

The current RRC IE structure in the RRC CR [1] for preambleReceivedTargetPower can be illustrated as:

where the fields in red and blue are parent fields of the preambleReceivedTargetPower for legacy RO and SBFD RO, respectively. We can note that all the fields in blue are with a common field name, sbfd-RACH-DualConfig-r19, while the fields in red can be either rach-ConfigCommon (without suffix) or rach-ConfigCommon-r17, based on the applied feature combination.

For MAC-2, there are four options that we have identified for clarification:

- Option 1: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon or the rach-ConfigCommon-r17". This option might lead to ambiguity, as it could imply that both rach-ConfigCommon and rach-ConfigCommon-r17 are valid parents, leaving the UE to choose one of them.

- Option 2: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon". This option can be mis-interpreted as only to choose rach-ConfigCommon (without suffix), and precluding rach-ConfigCommon-r17.

- Option 3: "indicate the preambleReceivedTargetPower not included in the sbfd-RACH-DualConfig". This option may not clearly indicate which parent IE/field UE should choose, but minimizes the risk of further mis-interpretation.

- Option 4: "indicate the preambleReceivedTargetPower configured for the first PRACH occasions". Instead of specifying specific parent IE or field, this option explicitly captures the intent of the correction, focusing on the expected UE behavior.

[MAC-2] Proposal 1: RAN2 to prioritize Options 3 and 4, over Options 1 and 2, to minimize the risk of further mis-interpretation, where

- Option 1: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon or the rach-ConfigCommon-r17".

- Option 2: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon".

- Option 3: "indicate the preambleReceivedTargetPower not included in the sbfd-RACH-DualConfig".

- Option 4: "indicate the preambleReceivedTargetPower configured for the first PRACH occasions".

[MAC-3] Whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

MAC-3 is for ensuring preamble transmission power continuity during RO type switching. The TP from Nokia is to compensate the difference between the two preambleReceivedTargetPower values for legacy RO and SBFD RO, when performing RO type switching.

However, we think the preamble transmission power difference introduced by RO type switching, can be easily controlled by sensible network configurations on the preambleReceivedTargetPower parameters, which can ensure the difference can be confined within a reasonable range, given that the other part of the difference introduced in power ramping is already compensated.

[MAC-3] Proposal 2: RAN2 to not support introducing further compensation on preamble received target power between legacy RO and SBFD RO for ensuring preamble transmission power continuity during RO type switching, which can be left to sensible network configuration.

Conclusion

Based on the discussion above, we have the following proposals:

[MAC-2] Proposal 1: RAN2 to prioritize Options 3 and 4, over Options 1 and 2, to minimize the risk of further mis-interpretation, where

- Option 1: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon or the rach-ConfigCommon-r17".

- Option 2: "indicate the preambleReceivedTargetPower included in the rach-ConfigCommon".

- Option 3: "indicate the preambleReceivedTargetPower not included in the sbfd-RACH-DualConfig".

- Option 4: "indicate the preambleReceivedTargetPower configured for the first PRACH occasions".

[MAC-3] Proposal 2: RAN2 to not support introducing further compensation on preamble received target power between legacy RO and SBFD RO for ensuring preamble transmission power continuity during RO type switching, which can be left to sensible network configuration.

Reference

[1] R2-2506605 Introduction of Rel-19 Evolution of NR duplex operation (SBFD), CR#5414, Huawei, HiSilicon.