3GPP TSG-RAN WG2 Meeting #131bis	R2-2507268

Prague, Czech Republic, 13th – 17rd Oct. 2025

Agenda Item:	10.3.3

Source:	ITRI

Title:	Discussion on the radio protocols for transfer of various type of data

Document for:	Discussion

1 Introduction

6G aims to provide the best user experience for diverse devices with different services. However, based on the lessons learned from 5G, a one-size-fits-all RAN architecture for the support of different data transmission requirements results in a complex RRC design and less efficiency. In order to achieve a scalable and simple 6G radio architecture for the transfer of various type of data, RAN2 is suggested to study a lean control plane design based on a modular user plane for 6G.

2 Discussion

6G aims to provide the best user experience for diverse devices with different services. RAN1 had discussed the diverse devices in August, and agreed to have consensus in the RAN plenary. About the issue, we have the same view with Nokia [1] to specify a scalable Technology Stack that maximizes commonalities between the needs of different devices so that each device segment has the volume potential to become commercially successful. For example, a low-end UE may be an IoT sensor with battery power, a mainstream UE may be a wearable device with mobile AI applications, and a high-end UE may be an automated driving vehicle with local AI model. The transmission requirements of various traffic types are different. For example, a video traffic needs a stringent latency and a high data rate transmission, an image traffic needs a stringent latency and a reliable transmission, and a raw sensor data needs a stringent latency and a secured transmission. How to transfer various type of data (including AI/ML data, sensing, etc.) for diverse devices is the goal of 6G user plane.

Observation 1: To support diverse device types optimized for different purposes is the goal of 6G user plane.

5G uses a one-size-fits-all RAN architecture to support the data transmission requirements of different services, including eMBB, URLLC, and mIoT. This results in a complex RRC design and is inefficient. Introducing new control-plane services is challenging because they often impact various network entities. 5G control plane suffered the growing complexity, heavy signalling and many optional capabilities; however, it results in less efficiency. From the 6G workshop in March, almost every company agree to have a lean, streamlind, and market-relevant 6G.

Observation 2: Lean, streamlined, and market-relevant are critical keys to increase value of 6G standards.

On the journey towards 6G, we share the same view with Qualcomm [2]. By adopting a user plane-first design strategy for network services and leveraging the design principles from the past, a more scalable, simple and efficient 6G network will be created. We think that a modular UP design may be suitable to provide native support for various device types with different purposes. We can study the concept of a module, for example, a function-specific module, a protocol module, a configuration module, a radio processing unit, etc. The details also need to be elaborated, such as scheduling and resource allocation. At the same time, the modular UP design necessitates a more efficient and modular CP to function seamlessly. A modular RRC design will allow each component function to operate independently for specific aspects of UP operation. The above discussion implied that a lean control plane design based on a modular user plane is the key to achieve a scalable and simple 6G radio architecture.

Proposal: In order to transfer various type of data, RAN2 is suggested to study a lean control plane design based on a modular user plane for 6G.

3 Conclusion

Based on the discussion in the previous section, we have the following observations and proposal:

Observation 1: To support diverse device types optimized for different purposes is the goal of 6G user plane.

Observation 2: Lean, streamlined, and market-relevant are critical keys to increase value of 6G standards.

Proposal: In order to transfer various type of data, RAN2 is suggested to study a lean control plane design based on a modular user plane for 6G.

- References

- Nokia, 6G radio protocols: Architecting for tomorrow’s diverse connectivity needs, 26 February 2025

- Qualcomm, 6G foundry: Rethinking the control plane, 20 March 2024