3GPP TSG-RAN WG2 Meeting #131bis 	R2-2507269

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	9.2.2

Source:	Panasonic

Title:	Considerations for Deployment Scenario 2 with Topology 2

Document for:	Discussion

# Introduction

Recently, RAN plenary has approved for Rel-20 a new work item on A-IoT phase 2. One important task led by RAN2 is to specify support for Deployment Scenario 2 with Topology 2 (D2T2, see RT 38.848) with intermediate UE as Reader in Rel-20. See following objective 2 from WID [1].

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 …(omitted) RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 …(omitted) RAN4 …(omitted)

In this paper, we discuss some initial considerations for supporting D2T2.

# Discussion

## Resource allocation to UE reader

As defined by WID, the resource allocation to the UE reader shall be via Uu RRC signalling, and the resource of the UE reader is only valid within the serving cell.

The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid. Regarding the resource validities, the following options have been identified during the Rel-19 study item phase [2]:

- Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration. - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations. NOTE 1: The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.) - Scenario 3: In RRC_ INACTIVE, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: - Option 1: UE receives the resources configuration in NR Uu dedicated RRC signaling, before moving to RRC_INACTIVE. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration (the UE may need to resume or be paged by the network back to RRC_CONNECTED before that). - Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, before moving to RRC_INACTIVE, which configures a time period in which the corresponding resource can be used. The UE reader can consider that the resources remain valid for that time period. - Scenario 4: In RRC_ IDLE, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface. - If this scenario 4 is supported, the options of scenario 3 can be the candidates. - The support of scenario 4 (UE reader performing A-IoT operations in RRC_IDLE) is down-prioritized. It can be considered in WI phase, if feasible with minimum specification impact.

Among all scenarios defined above, Scenarios 3 and 4 are not relevant for Rel-20 according to WID. We can then focus on Scenarios 1 and 2 since both are applied to RRC_CONNECTED UE reader, and discuss the options under each scenario.

For Scenario 1 (RRC_CONNECTED), option 1 considers the resources as valid until the network releases them explicitly by RRC reconfiguration. Option 2 further introduces a time period to regulate the validity of the allocated resources. In our view, it is beneficial to specify both options in Rel-20. Option 1 can be a baseline operation. Option 2 has advantage of saving the signaling overhead for releasing the resources.

For Scenario 2 (RRC_CONNECTED with temporary out of connection), whether/how to use the allocated resources during RLF and handover need to be considered according to WID. Since the RLF and handover only affects the Uu link between UE reader and gNB, it makes sense that UE reader can continue to use the already allocated resources for A-IoT operation between UE reader and A-IoT devices. In this sense, option 1 for scenario 2 is not so good for resource utilization. On the other hand, the gNB should be still in control over how long the allocated resource can be valid to avoid resource conflict. Using a timer to regulate the validity of resources is a good option. As for the open question during study item phase “whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period”, we think the same one as option 2 for scenario 1 can be sufficient.

Proposal 1. RAN2 to agree to specify the following schemes that were previously studied in TR 38.769 to determine the validity of A-IoT radio resources assigned to the UE reader:

- Scenario 1: options 1 and 2

- Scenario 2: option 2(using the same time period as option 2 for scenario 1).

## Reader ID

Since Rel-19 deployment considers only D1T1 where reader is fixed base station, it is feasible to place BS reader well separated in order to ensure no overlapping multi-reader scenario. However, Rel-20 deployment supports D2T2 where UE is a reader. Since UE reader can be moving around, it is impractical to assume that the coverage of different readers is always non-overlapping. In other words, it could happen that one A-IoT device receives paging messages to perform A-IoT operation from different readers.

For inventory service, it is sufficient for device to respond to one (“best”) reader and ignore the paging from other readers to save the energy and signaling overhead. In Rel-19, transaction ID is introduced to avoid duplicated responses. More specifically, for CBRA, if device sees in the paging message the same transaction ID as the one that device has successfully responded before, device will not respond in order to avoid duplicated responses. In Rel-19, how to generate transaction ID is up to reader implementation (i.e. not specified).

In multi-reader scenario, if transaction ID is generated by reader implementation as Rel-19, it is likely different transaction IDs will be used by different readers even if the paging is associated with the same CN request. If the device cannot determine whether the different transaction IDs are resulted from different services or different readers (but same service request), it is effectively requested that the device always needs to respond to multiple readers even for the same service request.

On the other hand, the length of transaction ID is 6-bit per Rel-19 design. This gives some room to incorporate reader identity. We suggest to use part of transaction ID, e.g. 2 bits, for reader identification purpose. It should be clearly specified which part of transaction ID is related to service identification and which part for reader identification. With this separation, device can skip the redundant responses if the transaction ID is related to the same service request.

Proposal 2. RAN2 to agree to specify the composition of transaction ID: which part is for reader identification and which part is for service identification.

Furthermore, since A-IoT operation does not have a “serving and non-serving reader” concept per Rel-19 design, some rule needs to be defined for device to know the device-reader association for multi-reader scenario in Rel-20. With such association, device will respond only to the corresponding reader until such association is changed. For example, the association should be regulated by a timer to avoid ping-pong. Before timer expires, the device only responds to the paging from the same associated reader, and ignore the paging from other readers regardless of the same or different service request.

Proposal 3. RAN2 to discuss the need to introduce device-reader association, and whether/how device-reader association is kept for a certain time.

# Conclusion

In this paper, we have discussed some initial considerations for supporting D2T2. The proposals are as follows.

Proposal 1. RAN2 to agree to specify the following schemes that were previously studied in TR 38.769 to determine the validity of A-IoT radio resources assigned to the UE reader:

- Scenario 1: options 1 and 2

- Scenario 2: option 2(using the same time period as option 2 for scenario 1).

Proposal 2. RAN2 to agree to specify the composition of transaction ID: which part is for reader identification and which part is for service identification.

Proposal 3. RAN2 to discuss the need to introduce device-reader association, and whether/how device-reader association is kept for a certain time.

# Reference

[1] RP-252894, Rel-20 A-IoT phase 2 WID (revised)

[2] TR38.769, v2.0.1, Dec. 2024