GPP TSG-RAN WG2#131bis   									           R2-2507306

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.9.3

Source: 	NEC

Title:	        CB-EDT relevant MAC open issues

Document for:	 Discussion and Decision

# Introduction

In this contribution, we will discuss several MAC issues relevant to CB-EDT:

- Backoff values indicated by BI field in CB-Msg4

- DSA window selection

- RV of CB-Msg replicas

# Discussion

MAC open issue 1: backoff indication

BI has been always a 4bits field. As specified in tables 7.2-1 and 7.2-2 MAC specification, different value set are defined for NB-IOT and other cases, and the unit is millisecond. This is the situation before CB-EDT was introduced.

Table 7.2-1: Backoff Parameter values.

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 20 |
| 3 | 30 |
| 4 | 40 |
| 5 | 60 |
| 6 | 80 |
| 7 | 120 |
| 8 | 160 |
| 9 | 240 |
| 10 | 320 |
| 11 | 480 |
| 12 | 960 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Table 7.2-2: Backoff Parameter values for NB-IoT.

| Index | Backoff Parameter value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | 256 |
| 2 | 512 |
| 3 | 1024 |
| 4 | 2048 |
| 5 | 4096 |
| 6 | 8192 |
| 7 | 16384 |
| 8 | 32768 |
| 9 | 65536 |
| 10 | 131072 |
| 11 | 262144 |
| 12 | 524288 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

When cb-Msg3-TxWindow is configured, the UE first selects the next CB-Msg3 window and then randomly chooses PUSCH resources within it for msg3 transmission. Therefore, having two BI values within the same Tx window periodicity does not create a distinction. For example, with window periodicity set as n32, i.e. 320ms, any BI value less than 320 (with BI index from 0-10) will not make a difference, because all UEs who failed to receive response will re-attempt CB-msg3 transmission in the very next msg3 transmission window.

Moreover, we notice that none of current BI values (all equal or less than 960ms) defined in table 7.2-1 is effective for 1280ms window periodicity.

cb-Msg3-TxWindow-r19			SEQUENCE {

windowSize-r19						ENUMERATED {n1, n2, n4, n8, n16, n32, n64,

n128},

windowPeriodicity-r19				ENUMERATED {n1, n2, n4, n8, n16, n32, n64,

n128}				OPTIONAL	--Need OP

cb-Msg3-TxWindow CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. When windowPeriodicity is absent, the window periodicity uses the same value as windowSize. Value n1 corresponds to 10ms, n2 corresponds to 20ms and so on.

To make the BI values are meaningful and adaptive, it can be defined as number of CB-Msg3 transmission window periodicity. Without introducing a new table, 4bit BI field could indicate maximum BI value of 16 periods. If bigger value is preferred, a new table using periodicity as unit could be introduced for CB-Msg3 case covering both eMTC and NB-IOT.

If we still use ms as BI unit, at least we need to add bigger BI values which can be used for 1280ms CB-Msg3 transmission window size.

Proposal 1: When cb-Msg3-TxWindow is configured, it is necessary to review the BI values indicated by the 4-bit BI field and consider the following options:

- Option A: The Backoff field indicates the number of DSA window periods

  - A1: without a table, 4bit BI field indicates the number of DSA window periods directly

  - A2: with a new table using periodicity as unit for both eMTC CB-Msg3 and NB-IOT CB-Msg3.

- Option B: Larger backoff values in ms are introduced

  - B1: by utilizing the reserved code points in the current BI parameter values tables.

  - B2: two new BI tables with bigger values in ms are defined for eMTC CB-Msg3 and NB-IOT CB-Msg3 respectively.

MAC open issue 2: DSA window selection

In the current specification [1], when CB-Msg3-EDT procedure is triggered, the UE selects DSA window as below:

Before the CB-Msg3 transmission, the MAC entity shall select cb-Msg3-NumOfReplicas UL grants for CB-Msg3 transmission as follows: - select the next upcoming CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level; - randomly select cb-Msg3-NumOfReplicas PUSCH resources in the time domain within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by cb-Msg3-TimeResource associated with the selected enhanced coverage level; - randomly select a frequency domain PUSCH resource for each selected time domain resource provided by prb-AllocationInfoSet (npusch-SubCarrierSetList for NB-IoT) associated with the selected enhanced coverage level.

For NB-IoT, the following values for CB-Msg3 window periodicity can be configured:

cb-Msg3-TxWindow-NB-r19			SEQUENCE {

windowSize-NB-r19			ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024},

windowPeriodicity-NB-r19	ENUMERATED {n8, n16, n32, n64, n128, n256, n512， n1024}

Taking window periodicity value n512 as an example, when CB-EDT procedure is triggered between CB-Msg3 occasion 1 and CB-Msg3 occasion 2, the UE shall perform CB-Msg3 transmission in the next DSA window according to the current specification, which means that the UE shall perform CB-Msg3 transmission after 5120ms as shown in Figure 1. The latency is very large.

Fig. 1 DSA window selection

Observation 1: For DSA window selection, when the next DSA window is selected, latency of the CB-Msg3-EDT procedure may be very large.

To reduce the possible latency, a simple way is that the UE will first determine the indexes of the replica occasions randomly when CB-Msg3-EDT procedure is triggered. If the determined first replica is after the triggering timing, the UE will transmit CB-Msg3 since the replica occasion. Taking Fig. 2 as an example, the UE determines the indexes of replica occasions as 3, 5, and 6, when the CB-EDT procedure is triggered. As the timing of the first occasion (occasion 3) is after the triggering time, the UE transmits CB-Msg3 since occasion 3. Taking Fig. 3 as an example, the UE determines the indexes of replica occasions as 1, 3, and 5, when the CB-Msg3 EDT procedure is triggered. As the timing of the first occasion (occasion 1) is before the triggering time, the UE transmits CB-Msg3 since occasion 1 in the N+1 DSA window.

Using this procedure, the UE could transmit CB-Msg3 timely manner without any unnecessary latency.

Fig. 2 Determined replica occasion instance

Fig. 3 Another determined replica occasion instance

Proposal 2: Multiple replica occasions are randomly selected first. If the selected first replica occasion corresponding to the current DSA window is after triggering time, the current DSA window is selected. Otherwise, the next upcoming DSA window is selected.

MAC open issue 3: RV

In RAN2 #130 [3], the agreement regarding RV0 was reached as following:

Agreements: 1. HARQ process 0 is used to transmit all the CB-Msg3 replicas in the transmission window (RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window)

- The content for the bracket above, i.e. RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window, is missing, which should be captured in the current specification. The corresponding TP is provided in TP #3 of the Annex.

Proposal 3: RV for selected multiple replica occasions should be captured in TS 36.321 and the corresponding TP is provided in TP #3.

# Conclusion

This contribution provides our observation and proposals for UL capacity enhancement as listed below:

Proposal 1: When cb-Msg3-TxWindow is configured, it is necessary to review the BI values indicated by the 4-bit BI field and consider the following options:

- Option A: The Backoff field indicates the number of DSA window periods

  - A1: without a table, 4bit BI field indicates the number of DSA window periods directly

  - A2: with a new table using periodicity as unit for both eMTC CB-Msg3 and NB-IOT CB-Msg3.

- Option B: Larger backoff values in ms are introduced

  - B1: by utilizing the reserved code points in the current BI parameter values tables.

  - B2: two new BI tables with bigger values in ms are defined for eMTC CB-Msg3 and NB-IOT CB-Msg3 respectively.

Observation 1: For DSA window selection, when the next DSA window is selected, latency of the CB-EDT procedure may be very large.

Proposal 2: Multiple replica occasions are randomly selected first. If the selected first replica occasion corresponding to the current DSA window is after triggering time, the current DSA window is selected. Otherwise, the next upcoming DSA window is selected.

Proposal 3: RV for selected multiple replica occasions should be captured in TS 36.321 and the corresponding TP is provided in TP #3.

# References

[1] R2-2506493 MAC CR1591 for R19 IoT NTN.

[2] Remaining MAC CR open issues in Rel-19 IoT NTN.

[3] Draft_RAN2_130_Meeting_Report_v1.

# Annex

## TP #1 for proposal 1 option A 1, BI issue

### 5.1x CB-Msg3-EDT Procedure

#### 5.1x.1	CB-Msg3-EDT initialization

The CB-Msg3-EDT procedure is initiated by the RRC sublayer and can only be performed in a non-terrestrial network. If the UE is an NB-IoT UE, the CB-Msg3-EDT procedure is performed on the anchor carrier or one of the non-anchor carriers for which CB-Msg3-EDT resource has been configured in system information.

skip

#### 5.1x.3	CB-Msg4 reception

Once all the CB-Msg3s of a CB-Msg3 transmission window are transmitted, the MAC entity shall monitor the PDCCH of the SpCell, while CB-Msg3ResponseTimer is running, for CB-Msg4(s) identified by the CB-RNTI defined below.

skip

After the CB-Msg3 transmission, the MAC entity shall:

-	start the CB-Msg3ResponseTimer at the subframe that contains the end of the selected CB-Msg3 transmission window plus UE-eNB RTT;

-	monitor the PDCCH while CB-Msg3ResponseTimer is running;

-	if notification of a reception of a PDCCH transmission is received from lower layers; and

-	if the CB-Msg4 is successfully decoded:

-	if the CB-Msg4 contains a CB Backoff Indicator subheader:

-	set the CB-Msg3-EDT backoff parameter value as indicated by the BI field of the CB Backoff Indicator subheader

-	else:

-	set the CB-Msg3-EDT backoff parameter value to 0.

-	if the CB-Msg4 contains one or more MAC CMRs; and

-	if there is a UE Contention Resolution Identity in a MAC CMR that matches the first 48 bits of the CCCH SDU transmitted in CB-Msg3(s):

-	consider CMR reception successful;

-	discard the CB-RNTI;

-	stop CB-Msg3ResponseTimer if running;

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	if the corresponding CMR contains a Timing Advance Command field:

-	process the received Timing Advance Command (see clause 5.2).

-	start the timeAlignmentTimer associated with this TAG;

-	if the corresponding CMR contains a C-RNTI field:

-	set the C-RNTI to the value of the C-RNTI field.

-	finish the disassembly and demultiplexing of the MAC PDU;

-	consider CB-Msg3 transmission successfully completed;

-	if the C-RNTI field is absent in the corresponding CMR and no corresponding MAC SDU is present in CB-Msg4:

-	indicate the successful completion of CB-Msg3-EDT procedure to the upper layers.

NOTE:	The UE can receive CB-Msg4 after the CB-Msg3ResponseTimer expired if the corresponding PDCCH is received before the CB-Msg3ResponseTimer expired.

-	if the CB-Msg3ResponseTimer expires:

-	when MAC entity has finished decoding of all received CB-Msg4 and if CMR reception is not successful:

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	increment CB_MSG3_TRANSMISSION_COUNTER_CE by 1;

-	if CB_MSG3_TRANSMISSION_COUNTER_CE = cb-Msg3-MaxAttemptNum for the corresponding enhanced coverage level + 1:

-	discard the CB-RNTI;

-	consider the CB-Msg3 transmission unsuccessfully completed;

-	indicate the transmission using CB-Msg3-EDT has failed to the upper layers.

-	else:

-	based on the CB-Msg3-EDT backoff parameter, select a random backoff number according to a uniform distribution between 0 and the CB-Msg3-EDT backoff Parameter value;

-	delay the subsequent CB-Msg3 transmission by the backoff times of CB-Msg3 transmission window periodicity corresponding to the selected enhanced coverage level;

-	proceed to the CB-Msg3 transmission (see clause 5.1x.2).

## TP #2 for proposal 2, CB-Msg3 Tx window selection issue

#### 5.1x.2	CB-Msg3 transmission

Before the CB-Msg3 transmission, the MAC entity shall select cb-Msg3-NumOfReplicas UL grants for CB-Msg3 transmission as follows:

-	select the next upcoming CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level;

-	randomly select cb-Msg3-NumOfReplicas PUSCH resources in the time domain within the selected CB-Msg3 transmission window from the CB-Msg3 resources provided by cb-Msg3-TimeResource associated with the selected enhanced coverage level;

-	randomly select a frequency domain PUSCH resource for each selected time domain resource provided by prb-AllocationInfoSet (npusch-SubCarrierSetList for NB-IoT) associated with the selected enhanced coverage level.

-	if the time domain position of the selected first PUSCH resource is after triggering time of CB-Msg3-EDT procedure, the current CB-Msg3 transmission window provided by the cb-Msg3-TxWindow associated with the selected enhanced coverage level is selected; otherwise, the next upcoming CB-Msg3 transmission window is selected.

## TP #3 for proposal, RV issue

#### 5.4.2.2	HARQ process

Each HARQ process is associated with a HARQ buffer.

For synchronous HARQ, each HARQ process shall maintain a state variable CURRENT_TX_NB, which indicates the number of transmissions that have taken place for the MAC PDU currently in the buffer, and a state variable HARQ_FEEDBACK, which indicates the HARQ feedback for the MAC PDU currently in the buffer. When the HARQ process is established, CURRENT_TX_NB shall be initialized to 0.

The sequence of redundancy versions is 0, 2, 3, 1. The variable CURRENT_IRV is an index into the sequence of redundancy versions. This variable is up-dated modulo 4. For serving cells configured with pusch-EnhancementsConfig, BL UEs or UEs in enhanced coverage see clause 8.6.1 in TS 36.213 [2] for the sequence of redundancy versions and redundancy version determination. For NB-IoT UEs see clause 16.5.1.2 in TS 36.213 [2] for the sequence of redundancy versions and redundancy version determination. For an SPS configuration with totalNumberPUSCH-SPS-STTI-UL-Repetitions or totalNumberPUSCH-SPS-UL-Repetitions (TS 36.331 [8]), the redundancy version for each transmission within a bundle are determined by rv-SPS-STTI-UL-Repetitions or rv-SPS-UL-Repetitions in the SPS configuration (TS 36.331 [8]).

For NB-IoT UEs, BL UEs or UEs in enhanced coverage for UL_REPETITION_NUMBER for Mode B operation, the same redundancy version is used multiple times before cycling to the next redundancy version as specified in clauses 16.5.1.2, 8.6.1 and 7.1.7.1 in TS 36.213 [2].

For NB-IoT UEs, BL UEs or UEs in CE mode A, the redunary version 0 is used to transmit the first repetition of each CB-Msg3 replica within the CB-Msg3 transmission window for CB-Msg3-EDT procedure.