3GPP TSG-RAN WG2 Meeting #131bis	                                                                                          R2-2507352

Prague, Czech Republic, Oct. 13th-17th

Title:                         Paging monitoring in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses paging monitoring in CONNECTED state.

# Discussion

Paging monitoring in CONNECTED state

For paging, since the paging is cell specific and can be applicable for all RRC state. For CONNECTED state UE, it is possible to monitor paging DRX cycle for SI notification if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace. There are two options for paging DRX handling in CONNECTED state.

- Option 1: UE in CONNECTED state monitors paging as legacy. That LP-WUS is not used for paging DRX in CONNECTED state.

- Option 2: IDLE/Inactive LP-WUS can be used for the UE in CONNECTED state for paging monitoring. In this option, UE needs to monitor LP-WUS as in IDLE/Inactive state.

Option 2 has more power saving, but UE needs to support monitor two different LP-WUS signalling. For paging for SI update, gNB needs to send paging using both LP-WUS and legacy methods since there are LP-WUS UE and legacy UE in the network. Considering UE may have different capability to monitor two different LP-WUS signalling, then it should be left UE implementation to Proposal 9	It is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

If the UE is in CONNECTED state, it is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring.

- Clarify in stage 2 that it is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

# Conclusion

- Clarify in stage 2 that it is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)