3GPP TSG-RAN WG2 Meeting #131bis		R2-2507354

Prague, Czech Republic, Oct. 13th-17th

Source:	Qualcomm Incorporated, vivo, Samsung, Xiaomi, OPPO

Title:		Last relay UE capability

Agenda Item:		8.13.4

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses leftover issue for multi-hop U2N relay UE capabilities.

# Discussion

In last RAN2 meeting, there was discussion on relay UE capability, and whether remote operation is a prerequisite for relay operation can be discussed in maintenance.

Agreements:

RAN2 confirm that UEs without Uu interface support are not in scope of this WI.

In Rel-19, the new band combination capability for multi-hop relay discovery is not needed.

In Rel-19, the new path switch capability for multi-hop relay is not needed.

In Rel-19, the L3 relay capability for multi-hop relay is not needed.

Keep relayUE-MH-Operation-L2 for both last relay UE and intermediate relay UE, while at least for intermediate relay UE, the remoteUE-MH-Operation-L2 is also indicated.  Whether remote operation is a prerequisite for relay operation can be discussed in maintenance.

During the discussion in the last RAN2 meeting, some companies argued that the last relay UE needs to support remote operation for support the case that the last relay UE role is changed to intermediate relay UE.

However, one typical scenario of multi-hop U2N relay deployment is to address the coverage in the basement of a house. In this scenario, the last relay UE could be fixed in a location which has good Uu coverage. That means the last relay UE could be a FWA which provides the Uu coverage extention for the whole house including basement. And also for public safety scenario, it is also possible to fix a FWA only implementing last relay operation in a location, to provide coverage for the coverage hole the Uu signal cannot cover. Such relay UE does not need to implement remote UE operation.

In order to support such deployment flexibility, it should allow that there is a type of UE which only support relay operation. That means, remote operation is NOT a prerequisite for relay operation.

If operaters want the UE also support the scenario that the last relay UE can also act as an intermediate relay UE, operatoers can request the UE venders to support remote operation in the last relay UE.

- Remote operation is not a prerequisite for the last relay UE, and keep the current specification as it is.

# Conclusion

- Remote operation is not a prerequisite for the last relay UE, and keep the current specification as it is.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet