3GPP TSG RAN WG2 #131bis	R2-2507359

Prague, Czechia, October 13 – 17, 2025

Agenda Item:	8.9.4
Source:	Toyota ITC
Title:	Discussion on S&F Idle Mode Procedures

Document for:	Discussion and Decision

# Introduction

The RAN plenary has approved a work item for Internet of Things (IoT) Non-Terrestrial Networks (NTN) [1]. One area of focus is the store-and-forward operation, where data can be transmitted to a satellite for storage if the feeder link is unavailable. The stored data can then be offloaded to a ground station once the feeder link is restored. This document discusses paging monitoring improvements and cell selection aspects of this feature.

# Discussion

## Paging monitoring

The store-and-forward feature enables a satellite to operate in real-time mode for two-way communication or in store-and-forward mode for one-way traffic from IoT devices to satellite storage. Several companies have raised concerns about the energy consumption involved in monitoring paging channels when satellites switch to store-and-forward mode. To conserve energy, it is recommended that User Equipment (UE) refrain from monitoring the paging channel in idle mode because no mobile-terminated traffic is expected.

Although the energy-saving benefits are evident, several challenges remain:

- Mobile-terminated data may arrive before mode switching, but paging can occur afterward. If the UE stops monitoring the paging channel immediately after the mode switch, it risks missing the delivery of this data.

- Disabling paging channel monitoring makes the UE unreachable. Once turned off, there is no guarantee it can be reactivated during store-and-forward mode. This complicates updating of system information during this operating mode, since the UE cannot receive system information change notifications on the paging channel.

- The work item is already closed, and the ASN.1 review is ongoing, making it difficult to introduce new signaling mechanisms during the maintenance phase.

Therefore, it is proposed to consider changes with minimal impact. The primary issue to address is identifying the trigger for turning paging monitoring off and on. Two possible triggers for disabling paging monitoring are:

- Option 1: Paging monitoring is automatically turned off when the operating mode switches to store-and-forward. After a specified number of paging occasions following the mode switch, the UE stops monitoring the paging channel.

- Option 2: A specific indicator is added as an extension to a system information block. Upon receiving this indicator, the UE immediately stops monitoring the paging channel.

In both cases, paging monitoring is reactivated when the system transitions back to real-time mode. These two approaches ensure that any pending mobile-terminated data can be delivered to the UE after switching to store-and-forward mode. With option 1, the UE monitors several paging occasions immediately after mode switching, allowing it to detect any pending mobile-terminated data. In option 2, paging monitoring is disabled via an indicator only after the pending data has been delivered. However, option 2 requires ASN.1 modifications, while option 1 can be implemented through procedural changes alone. Therefore, option 1 is the preferred solution.

The challenge of updating system information is similar for both options because once paging channel monitoring is stopped, the UE becomes unreachable until the operating mode switches back to real-time. Although, in theory, an indicator within a system information block could signal the need to resume paging monitoring, the UE may not detect system information changes since these updates are notified via paging messages.

To enable the network to update system information after mode switching, the UE behavior could be specified to require a sufficiently large number of paging occasions monitoring immediately following the mode switch. This would ensure the UE remains reachable and monitors the paging channel at least during the initial system information modification periods after switching operating mode.

Proposal 1: The UE is required to monitor a fixed number of paging occasions after switching from real-time to store-and-forward mode.

## Cell reselection

At RAN2-131, the S&F monitoring list was discussed. SA2 agreed that the satellite list is a tool which is “used by the UE assist with saving power and determining which satellites to synchronise data with”, see [2].

A note in the 23.401 CR reads: “When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.”.

As currently specified in TS 36.304, the UE reselects to a cell based on the RSRP/RSRQ ranking. Therefore, the UE reselects to the best ranked cell based on RSRP/RSRQ conditions, regardless if the cell corresponds to a satellite included in the S&F Monitoring list or not.

During RAN2-131, the following proposal was discussed and not agreed:

Proposal 1 from [3]: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the S&F Monitoring list.

One of the concerns was to prevent reselection to a normal cell. This could lead to added interferences if the UE is not camping on the best ranked cell, as the UE would transmit data towards a cell which is not the best ranked cell, increasing e.g. power and retransmissions. Note that in idle mode, the UE transmits registration update messages to the network at each expiry of Registration Update timer, and at change of Registration Area. Interferences happen if the cell that is reselected to is too much lower compared to the best cell.

Observation 1: Reselecting to a cell that is not the best cell from the reselection cell ranking may produce negative interferences if the cell that is reselected to is too much lower compared to the best cell.

Therefore, to mitigate interference issues , one way forward is to agree on the Proposal 1 from RAN2-131, with the additional condition that in the cell ranking, the cell reselected to shall not be lower than the best cell of a delta value.

Proposal 2: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the S&F Monitoring list, if the following condition is fulfilled: in the cell ranking, the cell reselected to shall not be lower than the best cell of a delta value.

# Conclusion

This document discusses paging monitoring improvements and cell selection aspects of the store-and-forward feature. The proposals are summarized as follows:

Proposal 1: The UE is required to monitor a fixed number of paging occasions after switching from real-time to store-and-forward mode.

Proposal 2: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the S&F Monitoring list, if the following condition is fulfilled: in the cell ranking, the cell reselected to shall not be lower than the best cell of a delta value.

# Reference

- RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

- S2-2502513, "23.401 CR on Support of S&F Monitoring List in S&F Mode".

- R2-2505228, "Discussion on cell reselection enhancement based on the S&F monitoring list", CATT, Samsung, Google, Huawei, Thales, Sateliot.