3GPP TSG-RAN WG2#131bis											   R2-2507362

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	9.7.2

Source: 	NEC

Title:	        Voice over GSO based on NB-IOT

Document for:	 Discussion

- Introduction

In RANP#108, a WID on enhancement of NB-IoT NTN to support IMS voice call over GSO [1] was approved. The objective of study is as follows:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

An LS from SA2 in [2] was received, which ask some questions for evaluating alternative solutions. The details are as follows:

1. Overall Description: Under the study item FS_5GSAT_Ph4_ARC SA2 is studying the support for IMS voice over NB-IoT NTN connected to EPC. In the context of this study item SA2 is investigating solutions in TR 23.700-019 that could potentially use IP or non-IP type (i.e. with and without IP/UDP header) for the (voice) traffic and would like to ask some questions in order to help SA2 to evaluate the alternative solutions. When IP PDN connection is used, in order to achieve the target KPIs for support of IMS voice over NB-IoT NTN connected to EPC, it is expected that the support for Robust Header Compression (ROHC) will likely be required. ROHC is supported between UE and eNB when "user plane" DRBs are used to transport the data or between the UE and MME when "control plane CIoT" and SRBs are used to transport the data. In order for SA2 to be able to evaluate the viability of using ROHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 has the following questions: Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20? Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

In this contribution, we will discuss two solutions, i.e. CP and UP solutions, and reply to some questions from SA2 LS.

- Discussion

2.1	CP-CIOT vs. UP-CIOT

Currently CP CIOT feature is supposed to support very infrequent and small number of data packet transmission, it is a simple feature with restrictions. Following is initial analysis on the needed changes, faced restriction and some advantages/disadvantages to support voice call service based on CP-CIOT procedure:

- Currently, there is no differentiation between the different data types (i.e. IP, non-IP or SMS) in the AS: to support voice service, we probably better differentiate the voice call relevant signalling and data from other NAS/AS signalling and differentiate voice call relevant data and signalling themselves. This requires changes a) NAS PDU shall be marked with data types b) new (signalling) bearers shall be introduced, and RRC shall map NAS PDU containing voice call relevant data and signalling into these new (signalling) radio bearer and treat them differently. Moreover, different from SRB1bis, these new (signalling) radio bearer may be configured with UM RLC mode.

- RRC connection reconfiguration is not supported: to support voice call, all necessary radio resource configurations for voice service shall be blindly configured during RRC setup stage. These configurations include extra radio bearers and SPS (if necessary). And once these are configured, it cannot be changed during the connection session.

- AS security is not activated: But NAS security should be in place to protect the communication.

- Since everything is configured at the RRC connection setup stage, it potentially has shorter call set-up delay

- As for SRB1bis, the data packets will probably bypass PDCP layer, use simple RLC UM mode, in summary AS user plane handling will be simple.

UP CIOT feature is like NR RRC inactive mode: A RRC connection suspend procedure is used at RRC connection release, the eNB request the UE to retain the UE AS context including UE capability in RRC_IDLE. A RRC connection resume procedure is used at transition from RRC_IDLE to RRC_CONNECTED where previously stored information in the UE as well as in the eNB is utilised to resume the RRC connection:

- Currently maximum 2 DRB can be configured for UP CIOT, this number needs to increase to support voice call.

- AS security is in place: no need of NAS security handling at MME.

- Multiplexing of CCCH and DTCH in the transition from RRC_IDLE to RRC CONNECTED is not supported. This means call requests can only be sent at earliest together with RRC connection resume complete message. Hence longer cell setup delay is expected.

- RRC reconfiguration is possible, which provides certain flexibility.

No matter voice over GSO is based on CP-CIOT or UP-CIOT, the essential enhancements are common for both:

- Better to have dedicated radio bearers (either SRB or DRB) for voice call relevant data and signalling. These radio bearers are either being configured during RRC setup stage, suspended/resumed during RRC connection resuming stage or re-configured after RRC connection is resumed.

- Better to have SPS in both downlink and uplink for more efficient scheduling.

- Security of the communication is done through either AS PDCP layer or NAS MME

In our understanding, CP-CIOT and UP-CIOT solution provides different levels of adaptation, simplicity and call set-up delay. CP-CIOT is less adaptive due to no RRC reconfiguration is allowed, but it keeps the thing simple for a low-end device, and potentially fast call setup. Less adaptation means less radio resource efficiency. On the other side, the UP-CIOT based solution is more adaptive (with reconfiguration). However, it may have long call setup delay and more complicated implementations. Which one among adaptivity/resource efficiency, simplicity and short call set-up delay is what we want most?

Proposal 1: Both CP-CIOT based and UP-CIOT based solution are possible with enhancements, we propose to down select one of these two, by considering/weighting on adaptation, simplicity and delay.

In addition to above-mentioned CP-IoT and UP-CIoT solutions, in which both SIP signaling and voice media are assumed to be carried either via the CP plane or the UP-plane, SA2 working group has also discussed a combined CP/UP bearer solution. The high-level solution principles are described in TS23.700-19 as follows:

- This solution addresses Key Issue #1 (Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC), by using one CP bearer and one UP bearer in IMS PDN connection.

- This solution utilizes the benefits of both CP and UP for separate transmission of IMS signalling and voice data, by combining a CP bearer and a UP bearer in a IMS PDN connection. The solution only uses one dynamically allocated radio UP bearer for GEO call, so the remaining radio UP bearer (s) can be used for other services during the voice call, if the NB-IoT UE is assumed to support up to 2 UP radio bearers.

As stated above, the combined CP/UP bearer solution can take advantages of both the CP-CIoT and UP-CIoT approaches, while also inheriting their respective disadvantages. In our understanding, the initial analysis summarized above can also be applied to the combined CP/UP bearer solution. In other words, we believe that the combined CP/UP bearer solution is covered by the discussions regarding the objective “Down-select between CP and UP solutions for voice support” in the current WID [1].

Proposal 2: RAN2 to confirm the combined CP/UP bearer solution is covered by the discussions regarding the objective “Down-select between CP and UP solutions for voice support” in the current WID.

2.2 SA2 LSin

Q1: Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the ROHC state fall back at the compressor when using ROHC?

As mentioned in LS, ROHC can fallback based on multiple consecutive packets that are lost or erroneously decompressed. For example, when UO-0 header with 4 bits is used in SO state, the UE can enter FO state once 16 consecutive packets are lost. From a theoretical analysis perspective, it is reasonable. In our understanding, it may be further observed whether the ROHC state fallback can occur from some analysis of actual network. For example, it may be analysed based on vendor’s log information.

Besides, we also observe that voice frame bundling can increase the probability of ROHC state fallback. Hence, if voice frame bundling is adopted, this factor should be considered for ROHC state fallback.

Q3: Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

According to the LS, we can see that the different packet sizes are from voice packet, silence packet and ROHC state change. For voice packet and silence packet, two SPSs of different sizes may be configured and dynamically activated/deactivated by DCI based on reported BSR or received MAC PDU with padding. For example, when SPS of large size is used for voice packet, once silence periodicity starts, the network may know packet change based on the received padding. In this case, the network may activate SPS of small size with long periodicity and deactivate SPS of large size with short periodicity. For ROHC state change, the network can know variation of data volume based on reported BSR. Then, the network may dynamically schedule UE based on BSR, or SPS of the maximum size is configured.

Proposal 3: For voice packet and silence packet, two separate SPSs may be activated/de-activated dynamically by DCI.

Proposal 4: For packet size change incurred by ROHC state change, SPS of a maximum size or dynamic scheduling may be used to accommodate packet size change.

Q6: Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

In the current specification, for NB-IoT, up to two DRBs may be configured. Then, in order to support IMS voice, IMS signalling, and simultaneous other services, it is feasible to extend the DRB number.

Proposal 5: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT.

Q9: Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

In the current specification, for CP-CIoT, SRB1bis can be used for data transmission. Then, for IMS voice, if CP solution is used, introducing new SRB to transmit IMS voice is feasible. For example, one SRB is for IMS signalling and the other SRB is used for IMS voice.

Proposal 6: It is possible to define new SRB for IMS voice.

Q10: If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

Under the current specification, RLC AM is the only applicable RLC mode for SRBs. Then, for new introduced SRB for IMS voice, RLC UM mode may be configured to avoid introducing latency due to retransmissions and reordering of RLC AM mode. And similar to SRB1bis, new SRB does not need PDCP entity.

Proposal 7: It is feasible to support and configure RLC UM for SRB that carries IMS voice.

- Conclusion

This contribution provides our views for voice over GSO based on NB-IoT NTN as listed below:

Proposal 1: Both CP-CIOT based and UP-CIOT based solution are possible with enhancements, we propose to down select one of these two, by considering/weighting on adaptation, simplicity and delay.

Proposal 2: RAN2 to confirm the combined CP/UP bearer solution is covered by the discussions regarding the objective “Down-select between CP and UP solutions for voice support” in the current WID.

Proposal 3: For voice packet and silence packet, two separate SPSs may be activated/de-activated dynamically by DCI.

Proposal 4: For packet size change incurred by ROHC state change, SPS of a maximum size or dynamic scheduling may be used to accommodate packet size change.

Proposal 5: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT.

Proposal 6: It is possible to define new SRB for IMS voice.

Proposal 7: It is feasible to support and configure RLC UM for SRB that carries IMS voice.

- References

[1] RP-252473 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4

[2] S2-2507636 LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC NTT Docomo