3GPP TSG-RAN WG2 #131bis	R2-2507371

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		10.2	General aspects

Title:		Overall framework for 6G from higher layer perspective

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#109, the revised SID has been approved, which includes many aspects as RAN2 objectives [1]. In this contribution, we generally discuss the various aspects to be studied in RAN2 under the SI. Some more details are also discussed in other contributions for the other sub-agendas.

# 2. Discussion

2.1	Overall framework

As per the SID, RAN2 should study a single technology framework based on a stand-alone (SA) architecture to support multiple services and satisfy the multiple usage scenarios and deployment scenarios for 6G. Although the non-stand-alone (NSA) architecture (e.g. 5G anchor/master with 6G secondary in Dual Connectivity) is still on the table for study, it was decided for RAN plenary to make decision by September 2026 whether to expand WG SI scope to cover additional migration option(s) including the NSA. RAN2 should wait for that and firstly focus on the SA.

For the multiple services support, the single technology framework based on the SA architecture should support them. RAN2 should take the wide range of services (with wide range of data rate, QoS requirement, etc.) into account from Day 1 for future proof. Exceptionally, for the service which requires distinctive design work, like MBS, sidelink communication, we can decide to support or not case by case, or deprioritized from the Day1.

Proposal 1: RAN2 aim to support the multiple services by the single technology framework based on the SA architecture, with some exceptions which require distinctive design, e.g. MBS, sildelink.

For the multiple usage scenarios and deployment scenarios, there can be some types of nodes or cells:

- Coverage node/cell

- Capacity node/cell

- Coverage extension node/cell (e.g. relay, NCR, IAB)

- Wider coverage node/cell (i.e. NTN)

Coverage node/cell should support essential services as well as any other services to be available from Day1. In addition, capacity node/cell should be considered with assuming that the overlayed deployment scenario would become so common. It would be good to study introducing a node/cell type with zero (or much less than 5G) always-on signal, where the corresponding cell can be adaptively waken-up for offloading. One possible way to wake-up the node/cell is that the node monitors, by keeping the UL Rx on, an wake-up signal from the UE. Note that the zero always-on signal in a cell is subject to RAN1 study basically, but RAN2 should be able to study its possibility from higher layer perspective.

Due to non-uniform cell coverages as well as less flexible site locations, coverage extension node/cell (e.g. relay, NCR, IAB) may be necessary in the commercial system. Unlike the coverage node/cell and capacity node/cell, the coverage extension node/cell may be available by implementation (e.g. normal relay) with required performance requirement. RAN2 may need to study this type of node/cell, for example, to decide which one (if necessary) is to specify based on simplicity and efficiency evaluation. However, it can be deprioritized at least.

Yet another type is the wider coverage node/cell and the main target of this type is the NTN. The SI objective intends to study the NTN to introduce it from Day1. We will discuss the details later.

Proposal 2: RAN2 aim to support the different types of deployment scenarios including coverage node/cell, capacity node/cell, and wider coverage node/cell (NTN) from Day1.

Proposal 3: RAN2 study a feasibility of a new node type with zero always-on signal but uplink Rx is on for being adaptively waken-up

Regarding the overall RAN architecture, RAN3 will lead the discussion including the RAN internal functional split and related procedures. On the other hand, the potential RAN internal functional split may impact on radio interface, e.g. procedure, protocol design. For example, if the existence of the RAN split architecture (e.g. CU and DU) may impact on the radio protocol design, RAN2 should be able to discuss before progress in RAN3.

Proposal 4: RAN2 basically waits for RAN3 progress on RAN internal functional split. If the existence of RAN internal functional split would impact on radio protocol, RAN2 should be able to initiate the corresponding study.

For Radio interface protocol architectures and procedures, we discuss in other contributions for the other sub-agendas.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] < omit > - Physical Layer structure for 6GR, < omit > - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] < omit > - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

2.2	UE capability framework

In the legacy systems, UE capability framework is introduced for the network to retrieve the UE capabilities when the network needs them. Normally the UE capability reporting will be initiated after the UE has performed initial access to the network (e.g., RRC setup and AS security activation). The main drawback in this legacy framework is to send the huge size of capability message over the air. Therefore, in the NR, some mechanisms are introduced to mitigate this problem, for example:

- RRC segmentation for UE capability signaling

- Feature Set, Feature Set Combination

- RACS (ID-based capability retrieval)

- Filtering (e.g. band/band combination filter)

On one hand, these mechanisms would be useful for 6G as well. On the other hand, it would be good to study a new fundamental framework as well as other signaling reduction mechanisms.

Firstly, it would be good to confirm the fundamental framework of the UE capability handling in the 5G NR will be also applied to the 6G, i.e. UE reports its capability to the RAN node, the RAN node stores and transfers them to the CN node, and then the CN node stores them. The stored UE capability in the CN will be retrieved by the RAN node upon mobility or when the UE performs the RRC setup again. This framework is working well and it is very difficult to drastically change from this.

Regarding the capability introduction guideline, we observed that in the NR, sometimes there are many UE capabilities for the same goal, i.e. a feature under the same WI in the same release. Segmented capabilities complicate the UE capability signalling and configuration signalling, and it also complicates the network implementation for configuration decision. We take the XR as example, which is not so big feature but introduce many separate capabilities. For example, in Rel19 XR, to support XR service with low latency and high reliability requirement, we introduced ~10 capability bits. In extreme case, we have two independent capability bits for giving up out-dated packet retransmission at UE transmission operation and for giving up receiving an outdated packet at UE receiving side. These two functions as such, which highly relevant to the same target but could be enabled independently, would not well justify two independent capabilities. Therefore, it would be good to study a possibility that the UE capabilities for the same goal are not too segmented.

Proposal 5: RAN2 study a fundamental UE capability framework, which aims to avoid or reduce many segmentations for the same goal (i.e. a feature under the same WI in the same release).

Proposal 5bis: RAN2 consider a guideline that if two functions could be configured/enabled independently but target for the same goal in the same WI, it should not motivate two capability bits

To achieve less segmented capabilities, it may be good to study a kind of use case specific capability reporting. For example, we may define a use case specific capability set or introduce a use case specific filtering, where the network may give a filter for the use case which the UE is requesting to achieve.

Proposal 6: RAN2 study a possibility of use case specific capability set or use case specific capability filtering.

# 3. Conclusion

In this contribution we discussed the various aspects to be studied in RAN2 under the SI. We reached the following proposals.

Overall framework

Proposal 1: RAN2 aim to support the multiple services by the single technology framework based on the SA architecture, with some exceptions which require distinctive design, e.g. MBS, sildelink.

Proposal 2: RAN2 aim to support the different types of deployment scenarios including coverage node/cell, capacity node/cell, and wider coverage node/cell (NTN) from Day1.

Proposal 3: RAN2 study a feasibility of a new node type with zero always-on signal but uplink Rx is on for being adaptively waken-up

Proposal 4: RAN2 basically waits for RAN3 progress on RAN internal functional split. If the existence of RAN internal functional split would impact on radio protocol, RAN2 should be able to initiate the corresponding study.

UE capability framework

Proposal 5: RAN2 study a fundamental UE capability framework, which aims to avoid or reduce many segmentations for the same goal (i.e. a feature under the same WI in the same release).

Proposal 5bis: RAN2 consider a guideline that if two functions could be configured/enabled independently but target for the same goal in the same WI, it should not motivate two capability bits.

Proposal 6: RAN2 study a possibility of use case specific capability set or use case specific capability filtering.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone