3GPP TSG-RAN WG2 #131bis	R2-2507372

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		10.3.1	User plane

Title:		Overview of 6G User Plane protocol architecture

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#109, the revised SID includes many aspects as RAN2 objectives [1]. In this contribution, we discuss the overall aspects of the User Plane protocol architecture.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

# 2. Discussion

2.1	Overall protocol architecture

2.1.1	User Plane functions

The 5G NR Layer 2 protocol stack for the user plane is organized into SDAP (Service Data Adaptation Protocol), PDCP (Packet Data Convergence Protocol), RLC (Radio Link Control), and MAC (Medium Access Control) sublayers. Each sublayer contains and executes a set of distinct functions. Data packets are sequentially processed by each sublayer and their respective function blocks. The table 1 roughly shows the essential functions of each sublayer. Overall, in NR communication, the MAC layer ensures the basic requirements for user plane data transmission and reception, while also supporting advanced functionalities. In comparison, the SDAP, PDCP and RLC sublayers handle more specialized functions.

As these sublayers are well specified, the existing radio protocol architecture comprised of multiple sublayers is considered to remain for the 6G user plane protocol as the starting point. Then, RAN2 should study possible improvement or evolved mechanisms on top of that. For example, there are some questions: eliminating ARQ in favour of HARQ only for error correction, achieving power saving with unified DRX and DCP, determining whether in-order delivery should be handled by the PDCP or RLC layer, and whether security should be shifted to the MAC layer to protect MAC signalling as well. However, 6G must continue to support error correction, power saving, in-order delivery, security, and all above listed functions as in 5G.

Proposal 1: RAN2 start with the current set of user plane functions from 5G for 6G.

Table 1. Essential functions in L2 sublayers of 5G NR

| L2 sub-layer | Essential functions (partly) | Remarks |
| --- | --- | --- |
| SDAP | Mapping between QoS flows and data radio bearers, enabling QoS-aware data handling. | Can be transparent in case of RRC configured mapping |
| PDCP | Security functions including ciphering (encryption, decryption) and integrity protection, Robust Header Compression (ROHC), out-of-order delivery, etc. | UP IP is mandatory in 5G (from Rel-16) |
| RLC | Three modes: Unacknowledged Mode (UM), Acknowledged Mode (AM), and Transparent Mode (TM). Segmentation and reassembly of data (in AM, ensures in-order delivery of packets), Automatic Repeat Request (ARQ) | From 4G to 5G, concatenation is moved and merged with MAC multiplexing |
| MAC | Fundamental role in packet transmission and reception including resource scheduling, logical channel management (e.g. logical channel prioritization (LCP)) and resource requests, Hybrid ARQ (HARQ) retransmission management. Supports advanced features such as Discontinuous Reception (DRX), Carrier Aggregation (CA), Bandwidth part (BWP) adaptation | Increasing the important role of MAC signaling motivates to study security protection of MAC signaling (per SA3 requirement study) |

2.1.2	Requirements for User Plane

For improving the UP design, there can be three aspects for further study, which are minimizing the inter-layer interaction, adaptability and being lean. Note that some of these may be co-exist, while some may be not.

Minimizing inter-layer interaction is generally considered in protocol stack design. In NR, however, interaction between layers has increased. For instance, the date packet discard timer is managed at PDCP layer but is also utilized for DSR reporting in MAC layer and for polling and/or ARQ retransmission in RLC layer. As a result, discard timer-related indications are exchanged among these sublayers with greater frequency in NR. In the context of 6G, reducing inter-layer interactions is a potential objective.

Looking ahead to 6G, adaptability is an important consideration for Layer 2 protocol stacks, which is vital to support a wide range of services and devices—ranging from very low to very high data rates, short to long delays, lossless to best-effort delivery, and from small battery to power connected devices. Addressing as many combinations of requirements as possible is desirable. For example, prior to Rel-19 NR, RLC UM supports services requiring low reliability and short latency, while RLC AM supports services demanding higher reliability and longer latency. With NR Rel-19, enhanced RLC AM is introduced to handle use cases such as XR, which require both high reliability and short latency. However, supporting these diverse requirements may increase UE complexity, especially when traditional traffic types (e.g., VoIP) coexist with high-demand services.

In addition to or alternatively to the adaptability, a lean protocol stack is also improving the UP design, e.g. reduce processing time/latency, simplified protocol stack. In the 5G (especially at later releases), some advanced features have been specified, such as two-step RACH, EDT, enhanced RLC AM. If these advanced features are popular/widely used, they could be included in 6G from the start. This could replace the original feature and reduce number of options. For example, RAN2 could discuss if it is possible to remove ARQ or shift ARQ down close to HARQ, move reordering up to PDCP, eventually remove RLC sublayer. Potentially a more powerful HARQ would be required. In example, RAN2 study a possible lean 6G User Plane protocol stack by:

- Avoiding duplicated function in different sublayers

- Adopting commonly applicable advanced features (e.g. two step RACH, small data transmission, enhanced RLC AM) from Day 1

- Considering remove/merge RLC sublayer into other sublayers.

Proposal 2: RAN2 identify the requirements for the User Plane protocol, e.g. minimizing the inter-layer interaction, adaptability and/or lean design.

2.1.3	Modelling options for L2 protocols

To make L2 User Plane (UP) protocol stack adaptive, modular designs—similar to the UM, TM, and AM modes in the RLC sublayer—could be considered for other sublayers as well, allowing different modes to serve various service, device, or bearer types. In a slightly different way, multiple modules could be defined, and each module covers across multiple sublayers (as shown in the middle figure below)

Alternatively, sublayers could be maintained as they are in the current MAC layer, integrating multiple functions with configurable parameters and enable/disable options to support diverse requirements, which may be called as an integrated user plane protocol stack (the examples are shown in the left side figure below).

A mixed of modelling is also possible (as in the right-side figure).

The advantages and disadvantages of these approaches (or something others) can be further studied in RAN2.

To make the L2 UP protocol stack adaptive, for a particular UE-NB connection, it should be possible to select a subset of specified modules if modular design is used, or it should be possible to disable/enable certain functions if integrated design is agreed.

Figure:  Examples of user plane protocol stack structures

Proposal 3: RAN2 study the 6G user plane protocol stack structure options including:

- Modular stack/sublayers with selectable modes (as in 5G RLC),

- Integrated user plane protocol stack with configurable (including disabling) functions (as in 5G MAC)

2.2	Support of carrier aggregation and BWP

One of the most important functions in the legacy system, e.g. 5G NR is the carrier aggregation (CA). The new concept of more flexible frequency resource utilization is the Bandwidth Part (BWP). Both are essential in the NR. For 6G, to supper high data rate, or to adapt to moderate data rate of UE or and to save UE power consumption, it is necessary to continue support spectrum aggregation and bandwidth part (or the like).

In the NR, spectrum resource is modelled into carrier, and each carrier form a cell with or without SSB signal. For a given UE, it could be configured with multiple carriers, i.e. carrier aggregation, or configured to work on a smaller BWP for power saving. Both carrier aggregation and BWP operation are visible only up to MAC layer, invisible to SDAP/PDCP/RLC.

In 6G, it depends on whether (discontinuous or continuous) spectrum resource owned by a 6G NB will be identified as carrier/cell, or just purely used as schedulable resource in frequency domain, in later case, it may be invisible to MAC layer as well. Modelling of frequency resource should be first discussed then we will discuss how to support it in L2.

Proposal 4: RAN2 study frequency/spectrum utilization using the CA /BWP as the baseline, which should be invisible to RLC sublayer and up. FFS if it is visible to MAC, with close coordination with RAN1

# 3. Conclusion

In this contribution we discussed the overall aspects of the User Plane protocol architecture and made the following proposals.

Overall protocol architecture

Proposal 1: RAN2 start with the current set of user plane functions from 5G for 6G.

Proposal 2: RAN2 identify the requirements for the User Plane protocol, e.g. minimizing the inter-layer interaction, adaptability and/or lean design.

Proposal 3: RAN2 study the 6G user plane protocol stack structure options including:

- Modular stack/sublayers with selectable modes (as in 5G RLC),

- Integrated user plane protocol stack with configurable (including disabling) functions (as in 5G MAC)

Carrier aggregation and BWP

Proposal 4: RAN2 study frequency/spectrum utilization using the CA /BWP as the baseline, which should be invisible to RLC sublayer and up. FFS if it is visible to MAC, with close coordination with RAN1

# References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone