3GPP TSG-RAN WG2 #131bis	R2-2507374

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		10.3.3	Common User plane and Control plane

Title:		Transfer of various data and AIML framework

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#109, the revised SID has been approved, which includes many aspects as RAN2 objectives [1]. In this contribution, we discuss the transfer of various type of data (including AI/ML data, sensing, etc.) and general AI/ML framework. We provide some high-level views for them.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

# 2. Discussion

2.1	Various type data transfer

2.1.1	New Data Types for data transfer design

Within the 6G Radio study item, one of the objectives is to study data transfer design to support various types of data (e.g. AI/ML, Sensing, etc). The aspects related to data transfer of new types of data may be considered as neither UP nor CP data.

The new data types may include the AIML data collected for AIML training, AIML inference, or performance monitoring, Sensing data collected from the sensing node, and the data to be collected for other purposes. It may also include the data to be transmitted from the network to the UE side, for example, the AIML model transfer, data set sharing and model parameter configuration. These data may be characterized by large amount of data, with diverse latency requirements. For instance, the latency requirement is low for the data transfer supporting AIML training but may be stringent for AIML inference, and middle for AIML performance monitoring. For sensing based object tracking, a large amount of real time sensing data transmission may be required to transfer from the sensing node (e.g. a UE) to the network in order to calculate the real-time position of the object. It would be essential to clarify which type of the data should be considered before the actual study for data transfer design.

Proposal-1: RAN2 to clarify which type of the data should be considered before the actual study for data transfer design.

2.1.2	Protocol aspect of the data transfer design

With the assumption that the expected data transfer will not be simply carried by User Plane protocol architecture. We would like to discuss the feasibility of legacy Control Plane protocol architecture for the data transfer to support the new data types.

As studied in Rel-19 AI PHY work item for 5G NR for UE Side data collection, there are following constricts for adopting legacy CP-based approach to support the data transfer between the UE and network for collecting the AIML data:

• UE memory allocation. The collected data may need to be stored in the access stratum (AS) buffer for the CP-based data transfer via AS, which may require a large AS buffer comparing with legacy design.

• Segmentation for UE side data collection. A single RRC message can contain maximum about 9KB data in legacy design. Thus RRC or higher layer segmentation is needed to transfer more data that can fit in a single RRC message. The legacy RRC message only supports up to 16 segments in UL, which amounts to a maximum of 144KB of collected data that can be transmitted.

• Continuity of the collected data reporting. In case the collected data are not yet fully transferred before a handover, Xn / NG-AP like signaling enhancements may be required for the continuity of the data reporting. Similarly, it could be further discussed the case of collected data not yet fully transmitted before a radio link failure, or before transitions to RRC IDLE/INACTIVE mode.

During Rel-19 time frame for 5G NR, RAN2 was not able to assess the above issues and conclude a CP based solution to support the data transfer for the AIML data collected for UE-side model training. However, there was some discussion on the possibility to study a new protocol layer above RRC layer to handle such data transfer. A new protocol layer above RRC layer may resolve the issues identified during AIML study. For example, it may be able to support the data segmentation for the logged data stored within the UE, which may reduce the requirements to segment the data at RRC layer and avoid the specification impact for RRC layer.

Meanwhile, in Rel-20 6G Radio study, it should be possible to have a clean design from scratch without the need to consider backward compatibility, to support the data transfer functionality. In this regard, a new protocol architecture can be considered specific for data transfer. RAN2 should be make decision in this aspect during the study.

Proposal-2: RAN2 to study a new protocol architecture specific for data transfer design to support various new types of data.

2.1.3	Termination of the data transfer

The legacy User Plane (e.g., supported by 5G NR) is terminated between UE and external data network (e.g. an application server) across 5G Core network (5GC), as governed by PDU session. The legacy UP protocol stack was designed to support IP based data transmission and managed by QoS flow based granularity. It should be noted that, the consumer of the data carried by the UP is located in the public data network.

However it should be highlighted that the abovementioned new data types may be only collected for the 3GPP network internally, and sometimes it is only terminated at the RAN (e.g. 6G Base Station). In this case, the IP based data transmission and QoS flow based management may be not that essential anymore. In case of UE-Base Station termination of the data transfer, the QoS management may be simply governed by the 6G Base Station.

Regarding the mechanism to enable the transfer of diverse data types (e.g., AI related data, Sensing data, positioning data) across diverse sources, we may call it “Data Plane” for at least discussion purpose. The source or destination of data plane may be UE, RAN node, or CN function module (e.g., AI Function, Sensing Function, LMF).

Proposal-3: RAN2 to study a new “Data Plane” concept for the transfer of various new types of data including a termination point of those data, QoS management.

In 5G NR, NG-RAN and 5GC ensure quality of service by mapping packets to appropriate QoS flows and DRBs. This process follows a two-step mapping: from IP flows to QoS flows (NAS) and from QoS flows to DRBs (AS). A similar two-step mapping may also be applied to data plane traffic to ensure its QoS.

However, unlike the UP, the termination point of Data Plane traffic may be in either the RAN or the CN, and it remains unclear whether such traffic shall still be managed at the granularity of QoS flows. In any case, mapping Data Plane traffic to data flows (e.g., new data-plane-specific flows) falls within the scope of RAN3 and SA2, while RAN2 can focus on mapping data flows to radio bearers. From the RAN’s perspective, Control Plane data is carried over SRBs, while User Plane data is carried over DRBs. For Data Plane traffic, RAN2 should study whether to reuse existing DRBs or introduce a new Data-Plane-specific radio bearer.

Proposal-4: RAN2 to study “Data Plane” traffic management, including a possible mapping to a radio bearer.

2.1.4 Security for Data Transfer Plane

The security key for “Data Plane” may reuse the same security key as 6G User Plane. When the sensing data is carried by PDCP layer, the sensing data is subject to ciphering like normal data. Or alternatively, the security key for “Data Plane” may use a different security key from 6G User Plane. A specific sensing data ciphering is used (e.g. a specific ciphering key is derived and used for new type of data transmission).  As the key derivation architecture is managed by SA3 Work Group, SA3 may need to be informed during the study if a security key for “Data Plane” is required.

Proposal-5: RAN2 to collaborate with SA3 for the security aspects for “Data Plane” via a liaison when appropriate.

2.2	AI/ML general framework

We discuss high-level aspects of the AI/ML general framework. In 5G NR, the AI/ML functions (e.g., air interface, mobility and NG-RAN) has been introduced as the use case specific manner. Even though those AI/ML functions are useful, the framework is largely independent. With this way, there are need to design per use case, when a new use case is to be added. Normally this approach makes the system complicated.

For 6G, it is expected that many types of AI/ML applications and services. However, there will still be some different targets in the high-level. For example, one type of AI/ML is achieved within RAN domain, which may be similar to use cases supported in the 5G NR. While, the other type of AI/ML is achieved in wider range or in the CN domain, which may be similar to use cases supported in the 5GC. On one hand, it would not be good to divergent framework like use case specific like 5G. On the other hand, it would not be well justified to make a single framework for all types of the AI/ML to be expected for 6G. Therefore, we expect that RAN domain specific single framework would be beneficial with necessary coordination with the CN side.

Proposal-6: RAN2 to aim to support a single framework for RAN domain specific AI/ML applications and services in 6G.

One possible approach to achieve the single framework for RAN domain specific AI/ML may be to introduce a new AI/ML node (or entity) specific to AI/ML for air interface and mobility which can be located in RAN domain. Such the new AI/ML node will manage the AI/ML Life Cycle Management (LCM) including data collection, model training/monitoring, and/or inference operation. The new AI/ML node may also manage AI computing/storage resource scheduling, if introduced in 6G.

Proposal-7: RAN2 to study a possibility of introducing a new AI/ML node/entity in RAN domain to enable the single framework for RAN domain specific AI/ML.

Note that if the new AI/ML node/entity is to be introduced in the RAN domain, this should also manage a part of the transfer of various data types discussed in 2.1 above.

# 3. Conclusion

In this contribution we discussed the transfer of various type of data (including AI/ML data, sensing, etc.) and general AI/ML framework. We made the following proposals.

Various type data transfer

Proposal-1: RAN2 to clarify which type of the data should be considered before the actual study for data transfer design.

Proposal-2: RAN2 to study a new protocol architecture specific for data transfer design to support various new types of data.

Proposal-3: RAN2 to study a new “Data Plane” concept for the transfer of various new types of data including a termination point of those data, QoS management.

Proposal-4: RAN2 to study “Data Plane” traffic management, including a possible mapping to a radio bearer.

Proposal-5: RAN2 to collaborate with SA3 for the security aspects for “Data Plane” via a liaison when appropriate.

AI/ML general framework

Proposal-6: RAN2 to aim to support a single framework for RAN domain specific AI/ML applications and services in 6G.

Proposal-7: RAN2 to study a possibility of introducing a new AI/ML node/entity in RAN domain to enable the single framework for RAN domain specific AI/ML.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] TR38.843