3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507379

Prague, Czech Republic, 13th - 17th October 2025

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 MAC issues for LTM

Document for:  Discussion and Decision

# 1	Introduction

The Rel-19 Mobility Enhancement WI was finalized in the previous meeting. The overall procedure for Rel-19 LTM enhancement including inter-CU LTM, L1 event-triggered measurement reporting, and intra-CU conditional LTM, has been captured in the current CRs for RRC and MAC.

However, several MAC open issues are still remaining. This contribution provides input to address these outstanding MAC issues.

# 2	Discussion

## 2.1	TA timer for C-LTM

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | How to handle the CLTM TAT of target cell when receiving an LTM Cell switch Command MAC CE with valid TA? Which is related the previous MAC-14 open issue. | Issue Type: How to address it: |

According to the agreement on the MAC-14 open issue, it has been supported that if a UE receives a network-triggered LTM cell switch command, it should follow Rel-18 behavior to initiate/start the pTAG TA timer in the target cell, regardless of whether a valid CLTM TA value is available. Consequently, the existing CLTM TA value for the target cell can be discarded, and the corresponding CLTM TA timer should be stopped, as there is no longer a need to maintain it.

Proposal 1: When a UE receives an LTM Cell Switch Command MAC CE containing a valid TA, it should stop the CLTM TA timer of the target cell.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When current serving cell is an LTM candidate cell, whether UE maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE? | Issue Type: How to address it: |
| ASUSTeK | If the source cell is an LTM candidate cell, upon Cell switch, does the UE store the valid TA and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell (e.g., if the CLTM TAT for the Source Cell is not running at the time upon Cell switch)? |  |
| Nokia | Although the TAs of other candidates are maintained, the TA of the previous serving cell, which may also be a candidate, is discarded as it is maintained by the servingCell TAT and not the candidate TAT. For example, the corresponding candidate TAT may not have been started or it may have not been restarted upon reception of a serving cell TAC MAC CE. To address the issue: FFS whether to store the previous serving cell TA for the corresponding candidate and remaining TAT time in the CLTM TAT of the corresponding candidate upon cell switch or, as discussed above by vivo, maintain and update the cooresponding CLTM TAT based on reception of legacy TAC MAC CE for the serving cell. The former approach allows maintaining only one timer for the cell, the legacy TAT when it is serving and the candidate TAT when not. |  |

Some companies have raised the following issue: if the current serving cell (i.e., the source cell) is also configured as a candidate cell for LTM cell switch, should the UE store the TA value of the previous serving cell and maintain the corresponding TA timer? Two options have been considered: One is to store the TA value of the serving cell and keep the TA timer running; the other is not to store it.

Agreements on C-LTM 1. Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

Based on prior agreements, if the source cell is considered as one of the LTM candidate cells (other than the target cell), the UE should maintain the TA value and keep the early CLTM TA timer running. In such a case, if the source cell is included as a candidate cell, it should follow the same rules as other LTM candidate cells, i.e., the UE should maintain the TA value of the current serving cell and keep the corresponding early TA timer running. This approach reduces the overhead associated with the early TA acquisition procedure in subsequent LTM operations. Therefore, we prefer this option.

The alternative option is to discard the TA value and TA timer of the previous serving cell. In this case, the new serving cell (the target cell) may need to perform early uplink synchronization again to obtain the TA value(s) and restart the TA timer(s) in subsequent LTM. This would introduce additional overhead compared to the first option.

Proposal 2: RAN2 should discuss whether the UE should store the valid TA value and keep the CLTM TA timer running for the previous serving cell, if it is configured as one of the LTM candidate cells.

(Preferred) Option-1: The UE maintains the TA value and keeps the early CLTM TA timer running.

Option-2: The UE discards the TA value and TA timer of the previous serving cell.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| ASUSTeK | In the last meeting, it was agreed that if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released. Since the NW does not know which TA is released, whether the UE needs to report currently stored TAs to the NW to avoid unnecessary RA for early UL sync? | Issue Type: How to address it: |

In the previous meeting, it was agreed that if a UE receives more TAs than it can support, it is up to UE implementation to decide which TA(s) to release. In such cases, the network may not be aware of which TA has been released. However, there is no need for the UE to report the currently stored TAs, as this would introduce signaling overhead. To minimize spec impact, especially toward the end of Rel-19 discussions, it can be left to network implementation to avoid unnecessary RACH procedures for early uplink synchronization.

Proposal 3: It should be up to network implementation to avoid unnecessary RACH procedure for early UL synchronization, in cases where the network does not know which TA has been released.

## 2.2	TS 38.331 procedure text issues

### 2.2.1	Configuration

The following text is included:

### 5.x.1	Introduction

The network may configure an RRC_CONNECTED UE to perform L1 beam level measurements for LTM candidate cell(s) and/or serving cell. The network may configure the UE to report them in accordance with the event triggered L1 measurement configuration. The measurement report is used for indicating to serving gNB of the L1 measurement results from the serving cell and/or candidate cell(s). The measurement configuration is provided by means of RRC dedicated signalling.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	SS/PBCH block(s) resource indicator (SSBRI).

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	CSI-RS resource indicator (CRI).

The RRC configures the following parameters in the LTM-CSI-ReportConfig for event triggered L1 measurement and corresponding reporting procedure:

-	LTM-CSI-ReportConfig for the event-triggered measurement report;

-	eventTriggered for the event-triggered measurement report;

-	eventLTM2, eventLTM3, eventLTM4, eventLTM5: events for the event-triggered measurement report;

-	timeToTrigger: time during which an entering/leaving condition needs to be consistently satisfied for reporting event triggered L1 measurement report or for cell switch execution to be met;

-	ltm-CandidateReportConfigList: List of report configurations for LTM candidate IDs;

-	ltm-EventTriggeredPeriodicReport: whether the event triggered L1 measurement report is sent periodically if an LTM event is triggered;

-	reportOnLeave: whether the event triggered L1 measurement report shall be triggered when the leaving condition for an event is satisfied;

-	ltm-EventTriggeredReportReportContent: the content of the event triggered L1 measurement report.

According to the above text, it seems that and LTM-CSI-ReportConfig includes another LTM-CSI-ReportConfig. This is clearly wrong. What is missing is ltm-ResourcesForChannelMeasurement, which indicates the list of RSs of LTM candidate cells that may be applicable for the event.

Also, ltm-CandidateReportConfigList is defined as:

LTM-CandidateReportConfig-r19 ::=            SEQUENCE {

ltm-CandidateReportConfigId-r19                LTM-CandidateId-r18,

candidateSpecificOffset-r19                    MeasTriggerQuantityOffset                              OPTIONAL, -- Cond notEventLTM2

...

}

Rhis is actually as list of candidate cells, and a cell-specific offset.

Therefore, we propose the following modifications:

### 5.x.1	Introduction

The network may configure an RRC_CONNECTED UE to perform L1 beam level measurements for LTM candidate cell(s) and/or serving cell and report the corresponding measurement results in accordance with the event triggered L1 measurement configuration. The measurement report is used for indicating to serving gNB of the L1 measurement results from the serving cell and/or candidate cell(s). The measurement configuration is provided by means of RRC dedicated signalling.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	SS/PBCH block(s) resource indicator (SSBRI).

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	CSI-RS resource indicator (CRI).

RRC configures the following parameters in an LTM-CSI-ReportConfig for each event triggered L1 measurement report:

-	ltm-ResourcesForChannelMeasurement: indicates the LTM resource config containing the RS(s) of LTM candidate cell(s) that may be measured for the event;

-	eventTriggered for the event-triggered measurement report;

-	eventLTM2, eventLTM3, eventLTM4, eventLTM5: events for the event-triggered measurement report;

-	timeToTrigger: time during which an entering/leaving condition needs to be consistently satisfied for reporting event triggered L1 measurement report or for cell switch execution to be met;

-	ltm-CandidateReportConfigList: List of LTM candidates  that may trigger the report and, optionally, associated cell-specific offset;

-	ltm-EventTriggeredPeriodicReport: whether the event triggered L1 measurement report is sent periodically if an LTM event is triggered;

-	reportOnLeave: whether the event triggered L1 measurement report shall be triggered when the leaving condition for an event is satisfied;

-	ltm-EventTriggeredReportReportContent: the content of the event triggered L1 measurement report.

Proposal 4: Correct the description of RRC parameters as in the in TP in 2.2.1.

### 2.2.2	UE variables

UE variables are defined as follows:

#### 5.x.3.1	General

The UE maintains the following UE variables for event triggered L1 measurement and report procedure:

-	MR_LIST: includes the list of event triggered L1 measurement report information, including RS resource index of LTM candidate cell(s), L1 measurement result, and type of reporting RS(s) as defined in 6.1.3.x, for which the L1 measurement report triggering conditions have been met for TTT. Each entry in the list is associated with a ltm-CSI-ReportConfigId;

-	MR_SENT_COUNTER: represents the number of event triggered L1 measurement report performed by UE if the triggering condition for the corresponding event is met for TTT for each ltm-CSI-ReportConfigId;

-	BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report;

-	BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report;

-	BEAM_REPORTED_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have not been met for TTT.

From this text, it is not obvious that the 4 last variables are per item in the first variable, the "type of reporting RS(s) as defined in 6.1.3.x" is the same like the 3 last variables, and the text for variables is difficult to read.

We suggest the following update:

#### 5.x.3.1	General

The UE maintains the following UE variables for event triggered L1 measurement and report procedure:

-	MR_LIST: the list of ltm-CSI-ReportConfigId for which the L1 measurement report entering or leaving condition has been met for TTT for at least one applicable RS (of an LTM candidate cell), and for each ltm-CSI-ReportConfigId:

-	the RS resource index and L1 measurement result of each measured beam;

-	the MR_SENT_COUNTER: the number of reports sent;

-	BEAM_ENTERING_LIST: the reference signaling resource index of applicable RSs of LTM candidate cell(s) for which the L1 measurement report entering conditions have been met for TTT;

-	BEAM_LEAVING_LIST: the reference signaling resource index of applicable RS of LTM candidate cell(s)  which have been reported in the (Truncated) L1 measurement report MAC CE, and for which the L1 measurement report leaving conditions has been met for TTT for the triggered L1 measurement report;

-	BEAM_REPORTED_LIST: the reference signaling resource index of applicable RS of LTM candidate cell(s) which have been reported in the (Truncated) L1 measurement report MAC CE, and for which the L1 measurement report leaving condition has not been met for TTT.

Proposal 5: Correct the description of UE variables as in the TP in 2.2.2.

### 2.2.3	Measurement report triggering

The procedure is specified as:

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

[...]

However, there is a single ltm-CSI-ReportConfigId in an LTM-CSI-ReportConfig.

The MAC rapporteur has suggested the following instead:

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the serving cell:

[...]

However:

-	it remains unclear for which serving cell;

-	LTM-CSI-ReportConfig configured per serving cell is normally for periodic/semi-persistent/aperiodic reporting on PUCCH or PUSCH i.e., not event-triggered reporting.

The event triggered reports are not related to any particular serving cell. If they would be included in ServingCellConfig, there could be several reports configured in different serving cells with the same ID, which would result in ambiguous reports.

The following would be clearer:

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId of the MAC entity:

[...]

Proposal 6: The list of L1 event-triggered reports is per MAC entity (so per CG) i.e., not per serving cell like Rel-18 LTM reports. Adopt the corresponding MAC TP in 2.2.3.

The procedure further includes:

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>		if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4> consider only the current beam of serving cell, i.e. the beam corresponds to the RS configured in the indicated TCI State or the RS QCLed with the RS configured in the indicated TCI State indicated by TCI State in the serving cell as defined in clause 5.1.5 in TS 38.214, with the same RS type as the beam of LTM candidate cell, i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable;

3>		if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5> consider any beam of LTM candidate cell (except the serving cell) configured in ltm-CandidateReportConfigList, i.e. the RSs configured in LTM-CSI-ResourceConfig which is associated with this ltm-CSI-ReportConfigId, to be applicable;

4>	else:

5> consider any beam of LTM candidate cell (except the serving cell), i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable;

2>	if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event:

3>	if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

According to the MAC rapporteur:

The UE behaviour under this highlighted “2>” is for event triggered report, as there is a condition on “there is ltm-EventTriggeredReportContent configuration”, while the other “2>” doesn’t apply the corresponding UE behaviour, we don’t see any issue here.

In our understanding:

-	for all event-triggered reports, the yellow highlighted condition is fulfilled;

-	for L1 conditional event, 5.x.3.1 should not be executed at all;

-	5.y.2 entirely covers triggering for L1 conditional events.

Proposal 7: Remove "2> if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration" from 5.x.3.1.

We also notice that:

-	"beam" is used first and then "RS" is used, which is inconsistent;

- 	"serving cell" is used, while this only applies to the SpCell;

-	the text is unnecessarily verbose as e.g., "configured in LTM-CSI-ReportConfig associated with this ltm-CSI-ReportConfig" could be shortened to "in the associated LTM-CSI-ReportConfig" without any ambiguity, since in the context of this procedure, this is the only possible association.

-	the text refers to "serving cell", which is not correct, as it should only be the SpCell

-	the bullet on "QCLed" RS is unreadable, due to a combination of issues:

-	unclear "RS type": in LTM-CSI-ResourceConfig, ltm-NZP-CSI-RS-ResourceSet is used if present, ltm-SSB-resourceSet is used otherwise;

-	English mistakes ("the beam corresponds", missing "which" or "that", "which associated", missing "is"),

-	confusing ordering ("in the indicated TCI state indicated by TCI state");

-	verbose style (see above).

In our understanding, the RS of indicated TCI states can only be CSI-RS, so it is only when there is no NZP CSI-RS in the LTM-CSI-ResourceConfig that the UE needs to consider the SSB QCLed with the RS of the indicated TCI state.

Therefore, we propose the following corrections:

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>		if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet:

5> consider the RS configured in the indicated TCI State of the SpCell to be applicable;

4>	else:

5>	consider the SSB QCLed with the RS configured in the indicated TCI State of the SpCell to be applicable;

3>		if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5> consider any RS in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell which is listed in ltm-CandidateReportConfigList to be applicable;

4>	else:

5> consider any RS in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell to be applicable;

Proposal 8: Adopt the TP in 2.2.3 for the definition of applicable RSs (not beam), and replace "applicable beam" with "applicable RS" in 5.x.3.1.

There are also a number of procedural issues:

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

[...]

2>	if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event:

3>	if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

3>	include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> if the beam is in BEAM_LEAVING_LIST:

4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId;

3>	initiate the measurement reporting procedure, as specified in 5.x.4.

If the condition 3> is not fulfilled, there is no BEARM_ENTERING_LIST and no BEAM_REPORTED_LIST, then the meaning of "which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST" in the above bullet 2> is unclear.

Also:

-	the 2> bullet can select multiple beams, the a bullet 3> below is "if the beam", which implies there is only one;

-	"for this ltm-CSI-ReportConfigId" is sometimes added after BEAM_ENTERING/REPORTED/LEAVING_LIST, sometimes not added, without any clear reason. As it was clarified that these lists are per event, and the first bullet identifies one event, there is no ambiguity if the is removed everywhere, and readability is improved.

So we suggest the following changes:

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig:

[...]

2>	if the entry condition for the ltm-CSI-ReportConfigId is fulfilled during TTT for one or more applicable RSs:

3>	if the MR_LIST does not include a measurement reporting entry for the ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

3>	for each applicable RS for which the entry condition is fulfilled during TTT and that is not in the BEAM_REPORTED_LIST:

4>	include the SSBRI or CRI of the RS(s) in the BEAM_ENTERING_LIST;

3> for each applicable RS for which the entry condition is fulfilled during TTT and that is in the BEAM_LEAVING_LIST:

4> remove the RS from the BEAM_LEAVING_LIST;

4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId;

3>	initiate the measurement reporting procedure, as specified in 5.x.4.

Proposal 9: Adopt the TP in 2.2.3 for the logic of checking whether an event is in the MR_LIST first, and only after whether each RS that meets the entry condition for TTT is in one of the BEAM lists for the event (and not the opposite order).

The intention of the yellow highlighted bullet is to exclude ltm-CSI-ReportConfigIds that are for execution conditions. However, for such ltm-CSI-ReportConfigIds,

### 2.2.4	Measurement reporting

The measurement reporting procedure includes:

For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall:

1>	if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.x according to the measurement report information in the MR_LIST;

3> if reportAmount is configured in the LTM-EventTriggeredPeriodicReport by RRC:

4> if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled:

5>	set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId;

4> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId by 1;

4> stop the periodical reporting timer, if running;

4> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount:

5> restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

3> include the SSBRI or CRI in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId is empty:

4>	remove the measurement reporting entry within the MR_LIST for this ltm-CSI-ReportConfigId;

4>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running;

3> cancel the triggered L1 measurement report;

Accordingly, when a L1 measurement report is triggered, if a non-truncated MAC CE is transmitted, the L1 measurement report is cancelled.

If periodical reporting is configured, the following will be executed:

2>	upon expiry of the periodical reporting timer for this ltm-CSI-ReportConfigId:

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.x.4

In that case, the following condition is not met, because that L1 measurement has been cancelled:

1>	if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled:

This means that there will not be any additional report, which contradicts with the expected behaviour, so we suggested the following changes:

For each ltm-CSI-ReportConfigId, the MAC entity shall:

1>	if L1 measurement reporting is has been triggered for the ltm-CSI-ReportConfig as specified in 5.x.3:

[...]

3> consider L1 measurement report not to be triggered for the ltm-CSI-ReportConfig;

The rapporteur's reply to this suggestion was:

[Rapp]: This was discussed before, while the condition “and not cancelled” is needed due to the below note:

NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by leaving condition for the same RS(s), all the corresponding measurement reports are cancelled. When a measurement report is triggered by leaving condition for one or more RS(s), and included in the BEAM_LEAVING_LIST, another measurement report is triggered by entry condition for the same RS(s), all the corresponding measurement reports are cancelled.

This note has some readability issue that should be fixed, but it does not really require keeping the "and not cancelled" which is problematic for periodic reporting, as it could also be updated as:

NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, and while it is still triggered, another measurement report is triggered by leaving condition for the same list of RS(s), the UE considers all the corresponding measurement report as not triggered. When a measurement report is triggered by leaving condition for one or more RS(s), and while it is still triggered, another measurement report is triggered by entry condition for the same list RS(s), the UE considers all the corresponding measurement report as not triggered.

Nevertheless, this text looks like a UE requirement, it would be better placed as normative text (not a note) and "the UE shall consider".

Proposal 10: Adopt the TPs in 2.2.4 to replace "cancel" with "consider as not triggered", and remove "and not cancelled", to avoid cancelling the periodical reports.

### 2.2.4	Event Triggered L1 Measurement Report MAC CE

The description of this MAC CE includes:

-	Typei: This field indicates the type of the RS i of LTM candidate cell included in the event triggered L1 measurement report. The field is set to 00 to indicate the RS(s) that has satisfied the entry condition of the event associated with the report ID for TTT, triggers this measurement report MAC CE, and included in the BEAM_ENTERING_LIST; it is set to 01 to indicate the RS(s) that has satisfied the leaving condition of the event associated with the report ID for TTT, triggers this measurement report MAC CE, and included in the BEAM_LEAVING_LIST; it is set to 10 to indicate the RS(s) has been reported in the (Truncated) L1 measurement report MAC CE, and included in the BEAM_REPORTED_LIST associated with the report ID as specified in clause 5.x.3; it is set to 11 to indicate the RS(s) not satisfying the event for TTT, if configured by network by allowReportAnyBeam specified in TS 38.331 [5], i.e. the RS(s) with Type set to neither 00, 01, nor 10. The RS(s) not satisfying the event for TTT is selected based on the decending order of measured quantity. The RS(s) included in the truncated event triggered L1 measurement report MAC CE are selected based on a decreasing order of the priority for the type of beam: 00, 01, 10, 11. If the (truncated) event triggered L1 measurement report MAC CE cannot accommodate all RS(s) with the same priority, the RS(s) are selected based on a decreasing order of measured quality. The length of the field is 2 bits;

"type of RS", or "RS type" already has a meaning since Rel-15: it is whether an RS in an SSB or a CSI-RS, so it is confusing to call something completely different "RS type".

Basically, the meaning of this field is which BEAM list the RS is in when the report is triggered, so this field could be called e.g., "Listed in" and the description could be reworded accordingly:

-	Listed ini: This field indicates the list (BEAM_ENTERING_LIST, BEAM_REPORTED_LIST, BEAM_LEAVING_LIST or none them) that contains the RS i of LTM candidate cell included in the event triggered L1 measurement report. The field is set to 00 to indicate the BEAM_ENTERING_LIST; it is set to 01 to indicate the BEAM_LEAVING_LIST; it is set to 10 to indicate the BEAM_REPORTED_LIST; it is set to 11 to indicate none of those lists, if allowReportAnyBeam specified in TS 38.331 [5] is configured. The RS(s) not satisfying the event for TTT are selected based on the descending order of measured quantity. The RS(s) included in the truncated event triggered L1 measurement report MAC CE are selected based on a decreasing order of the priority for the beam lists: 00, 01, 10, 11. If the (truncated) event triggered L1 measurement report MAC CE cannot accommodate all RS(s) with the same priority, the RS(s) are selected based on a decreasing order of measured quality. The length of the field is 2 bits;

Proposal 11: To avoid confusion with "RS type" (i.e., SSB or CSI-RS), do not use "RS type" to refer to which list (BEAM_ENTERING_LIST, BEAM_REPORTED_LIST, BEAM_LEAVING_LIST or none them) a reported RS belongs to. Consider the TP in 2.2.4 for correction.

# 3	Conclusion

In this paper, we have addressed the objective regarding current MAC open issues for LTM. There are the following observations and proposals:

TA timer for C-LTM

Proposal 1: When a UE receives an LTM Cell Switch Command MAC CE containing a valid TA, it should stop the CLTM TA timer of the target cell.

Proposal 2: RAN2 should discuss whether the UE should store the valid TA value and keep the CLTM TA timer running for the previous serving cell, if it is configured as one of the LTM candidate cells.

(Preferred) Option-1: The UE maintains the TA value and keeps the early CLTM TA timer running.

Option-2: The UE discards the TA value and TA timer of the previous serving cell.

Proposal 3: It should be up to network implementation to avoid unnecessary RACH procedure for early UL synchronization, in cases where the network does not know which TA has been released.

TS 38.331 procedure text issues

Proposal 4: Correct the description of RRC parameters as in the in TP in 2.2.1.

Proposal 5: Correct the description of UE variables as in the TP in 2.2.2.

Proposal 6: The list of L1 event-triggered reports is per MAC entity (so per CG) i.e., not per serving cell like Rel-18 LTM reports. Adopt the corresponding MAC TP in 2.2.3.

Proposal 7: Remove "2> if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration" from 5.x.3.1.

Proposal 8: Adopt the TP in 2.2.3 for the definition of applicable RSs (not beam), and replace "applicable beam" with "applicable RS" in 5.x.3.1.

Proposal 9: Adopt the TP in 2.2.3 for the logic of checking whether an event is in the MR_LIST first, and only after whether each RS that meets the entry condition for TTT is in one of the BEAM lists for the event (and not the opposite order).

Proposal 10: Adopt the TPs in 2.2.4 to replace "cancel" with "consider as not triggered", and remove "and not cancelled", to avoid cancelling the periodical reports.

Proposal 11: To avoid confusion with "RS type" (i.e., SSB or CSI-RS), do not use "RS type" to refer to which list (BEAM_ENTERING_LIST, BEAM_REPORTED_LIST, BEAM_LEAVING_LIST or none them) a reported RS belongs to. Consider the TP in 2.2.4 for correction.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #129-bis chairman notes;

- R2-250xxxx, List of MAC open issues for R19 mobility