3GPP TSG-RAN WG2 Meeting #131bis	R2-2507438

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Open issues for idle mode operation

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this document we discuss the remaining open issues related to paging relaxation for SF mode operation.

# 2	Discussion

## 2.1 Paging monitoring relaxation based on NAS configured Satellite list.

When the satellite is configured with split MME architecture, the MME on board will reject the NAS registration or any NAS procedure if it does not have ground connectivity to fetch the UE context from MME-on-ground. In such case the MME on board will send NAS reject message including the satellite list where the UE context may be available. In case if the NAS procedure is successful also, the MME may include the satellite list where the MT data is available.  The specific text in 23.401 is given here for reference.

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to Store and Forward Satellite operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE's EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. The MME may provide S&F Monitoring List to the UE as part of detach procedure. NOTE 4: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with a satellite of the same PLMN that is operating in S&F Mode. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. - The MME may indicate to the UE an Estimated S&F UL Delivery Time in a NAS accept messages (i.e. Attach Accept, TAU Accept or Service Accept messages). NOTE 8: The Estimated S&F UL Delivery Time is an estimate of the time required to deliver the data or signalling sent by the UE to the ground. The Estimated S&F UL Delivery Time is associated with the satellite that provides it and how UE uses this information is left for UE implementation.

Observation 1: Based on the procedure text given in TS23.401, we can deduce that UE can attempt MT data reception only in the NAS configured satellite list.

However, the procedure also allows the possibility for MME to update the NAS configured satellite list to UE if needed based on the deployment changes. In such cases it may need to send paging messages to trigger the NAS procedure to update this list.  Even though paging for MT data will not be possible in the satellites outside the NAS configured list possibility of paging cannot ruled out.

The UE may also need to monitor the paging for any system information changes in this satellite eventhough MT data reception is not possible. To avoid paging monitoring for system information changes, the UE may need to acquire all the SIBs prior to MO data transmission.

Observation 2: MME initiated paging /Paging for system information modification is possible in the satellites outside the NAS configured satellite list for NAS procedures to update the satellite list.

Based on the above observations, we propose that UE cannot pause paging monitoring in the satellites outside the NAS configured list. Moreover, the NOTE in TS23.401 also indicate that the paging monitoring is left to UE implementation in this scenario.

Proposal 1: UE based paging monitoring relaxation based on NAS configured satellite list is not supported.

As part of earlier RAN2 discussions some companies indicated that paging monitoring should be applicable only if the cell is operating in store and forward mode.  We also agree to this view and RAN2 to clarify that paging monitoring relaxation is only applicable when the cell is operating in SF mode.

Proposal 2: RAN2 to confirm that paging monitoring relaxation is only applicable when the cell is operating in SF mode only. UE switches to regular paging monitoring when the SF mode is changed to normal mode.

## 2.2 Paging Monitoring Relaxation based on Pending MT Data in SF mode

In RA2-131bis, it is agreed to further discuss on possible enhancements for paging monitoring for the scenario where cell is operating SF mode and does not have any MT data to send trigger paging. For such cases introduction of indication in system information whether paging can be skipped or not is sufficient to address the basic scenario. However when the network wants to modify this parameter it needs to trigger system information modification procedure via paging. In this case the UE need to re-acquire the SIB to know the update which is only this indication. This additional system information acquisition can be avoided if the paging message also include flag to skip the paging. In this case the UE need not re-acquire the system information as the same information will be reflected in the new system information.

Observation 3:  Whenever paging monitoring is enabled or disabled via system information UE need to re-acquire the SIB.

Proposal 3: Paging message also includes new indication similar to system information for activation/deactivation of paging monitoring.

# 3	Conclusion

In this paper we analyse  the remaining open issues for paging monitoring relaxation for SF operation and make following observations and proposals.

Paging monitoring relaxation based on NAS Configured List

Observation 1: Based on the procedure text given in TS23.401, we can deduce that UE can attempt MT data reception only in the NAS configured satellite list.

Observation 2: MME initiated paging for NAS procedure /Paging for system information modification is possible in the satellites outside the NAS configured satellite list.

Proposal 1: UE based paging monitoring relaxation based on NAS configured satellite list is not supported.

Proposal 2: RAN2 to confirm that paging monitoring relaxation is only applicable when the cell is operating in SF mode only. UE switches to regular paging monitoring when the SF mode is changed to normal mode.

Paging monitoring relaxation based on MT data availability in SF mode

Observation 3:  Whenever paging monitoring is enabled or disabled via system information UE need to re-acquire the SIB.

Proposal 3: Paging message also includes new indication similar to system information for activation/deactivation of paging monitoring.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements