3GPP TSG-RAN WG2 Meeting #131bis                                    	R2-2507447

Prague, Czech Republic, 13th - 17th October 2025

Agenda item:	9.7.1

Source:	Qualcomm Incorporated

Title:	Discussion on SA4 and SA2 LS replies on voice over NB-IoT

Document for:	Discussion and Decision

- Introduction

RAN2 LS from SA4 in [1] asking questions on CP/UP solutions and whether 1 byte MAC header is realistic. RAN2 also received second LS from SA4 in [2] asking feedback on SPS periodicities such as 120ms and 240ms. Additionally, RAN2 received LS from SA2 in [3] with several questions specially regarding potential RAN2 impacts of UP solutions and CP solutions. In this document, we provide details for the response to the LSs.

- Discussion

## SA4 LS [1]:

3GPP SA4 has an ongoing feasibility study on Ultra Low Bitrate Speech Codec (FS_ULBC) based on the approved study item description in SP-250378 including the following objective:

- Study GEO channel characteristics and derive service-related dependencies, e.g. bitrates, mouth-to-ear delay or loss/delay/jitter profiles.

Followings are the actions:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In our view, first RAN2 and SA2 in coordination should discuss the UP/CP solutions and make decisions. In addition, RAN2 should also discuss whether any optimization for AS layer overhead reduction would be necessary. All this would need time as we expect to have good discussions on the subject. If no agreement could be reached, we suggest responding to the LS with status/progress at RAN2.

However, as per current specification support, i.e., without any AS layer optimization, it should be possible to provide answers on the AS layer protocol overhead size. From RAN2 perspective, in addition of PDCP, RLC and MAC, RoHC overhead for UP solution and RRC overhead for CP solution should also be taken into account.

1 byte MAC header:

In TS 36.321 Figure 6.1.6-2, 1 byte R/R/E/LCID MAC subheader format is already supported given the MAC header is the only MAC subheader in the PDU, i.e., field E = 0. If the MAC PDU includes only one voice data and it fits in the TBS size allocated by the network then length field does not need to be included.

This scenario is possible when codec rate is negotiated with the network. Using fixed size of headers across protocol layers, network may be able to configure the SPS with a TBS size that fits one MAC PDU carrying voice packet. In this case, 1 byte MAC header can be considered.

There can also be the case where UE is allocated with X byte TBS considering 4 byte of RoHC header (UO-1 header type + 2 byte UDP checksum). Suppose the RLC PDU size for voice traffic is (X-1) byte when RoHC header format UO-1 is used, in this case, one byte of MAC header is sufficient. Suppose the RLC PDU size for voice traffic is (X-2) byte when RoHC header format UO-0 is used, in this case, two bytes of MAC header is needed.

However, we should note that for a single voice data to be sent via UL-SCH as shown in Table 1, the most common scenario would be total MAC header of 3 or more than 3 bytes in a given TBS (i.e., configured UL grant). This means if 1 byte MAC header is not realistic, then it can be argued that total 2 bytes of MAC header is also equally not realistic. But currently 2 bytes of MAC header is used in determining TBS size and codec rate.

if 1 byte MAC header is not realistic, then it can be argued that total 2 bytes of MAC header is also equally not realistic in a TBS size. But currently 2 bytes of MAC header is used in determining TBS size and codec rate.

Table 1 Likelihood of total MAC header in a TBS size

| TBS size (available UL grant size) | Single voice packet size (total RLC PDU) | Total MAC header | Likelihood |
| --- | --- | --- | --- |
| X byte | (X-1) byte | 1 byte Fields E = 0, L present | low |
| X byte | (X-2) byte | 2 byte 1 byte Padding + field E = 0 | low |
| X byte | < (X-2) byte | > 2 byte Fields E = 1, L present | High |

Clarify to SA4 that total MAC header of 1 byte is feasible in case UE has only one PDU that fits in the available TBS.

UP solution overhead:

When RoHC is used for RTP/UDP/IP traffic, there can be different modes of operation and different types of RoHC headers used. The commonly used mode is the Bidirectional optimistic mode, i.e., O-mode. In this mode, commonly used headers are UO-0 (1 byte for voice packet in SO state), UO-1 (2 byte for SID packet in SO/FO state) and UOR-2 type header (8 to 13 bytes) to synchronize with decompressor at FO state.

We observe that the silent period is very common during voice call, which means it is very common to use UO-1 (of size 2 bytes) header for mainly two reasons, first to adjust the inter arrival time without changing TS_STRIDE and second to mark the beginning of the talk spurt. Therefore, the average header to be used for RoHC for the resource scheduling perspective (which in turns translates to PHY data rate calculation) should be 4 bytes, not 3 bytes what has been assumed by SA4. It is because RoHC header includes 2 bytes of UDP checksum and 2 bytes of UO-1 header.

For UP solution, clarify to SA4 that 4 bytes of RoHC header (not 3 bytes) is more reasonable to consider.

CP solution overhead:

For CP solution, PDCP is bypassed and RRC carries the NAS PDU as string. However, today the message to carry NAS PDU includes an OCTET string without definitive length indicator. Since size of NAS PDU can vary, the OCTECT strings requires 1 byte of overhead. This means the RRC layer adds 2 bytes of overhead.

However, for CP solution, it is likely that a non-IP solution is used. This means when codec rate is negotiated, the size of voice packet will remain same and hence the size of NAS PDU. This means it is possible that RRC message carrying a fixed size NAS PDU will have just 1 byte of total overhead, i.e., OCTET STRING (SIZE(FixedZize)) can be defined in the RRC message.

For CP solution, indicate that RRC layer adds 2 bytes of overhead. If the size of NAS PDU carrying voice packet is fixed based on codec rate, the RRC layer adds just 1 byte of overhead.

To SA4, RAN2 agree to draft LS reply provided in [4].

## SA4 LS [2]

RAN2 also received second LS from SA4 with the following action.

ACTION:

SA4 kindly asks RAN2 to confirm whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, and provide feedback, if any.

Once initialized at SFN start and Subframe start, the SPS occasion occurs periodically. If the SPS periodicity divides the 10240, then every H-SFN the SPS occasion pattern remains same, i.e., if SPS occasion is SFN 1, subframe 1 in H-SFN 1, then every H-SFN after SFN wraparound, the SPS occasion falls on SFN 1, Subframe 1. This helps network to manage resource efficiently avoiding any potential collisions.

If the SPS periodicity does not divide the 10240, then the SPS occasion pattern changes every SFN wrap around as shown in Figure 1.

Figure 1 An example of SPS occasion pattern change with 120ms of SPS periodicity

Such periodicities like 120ms and 240ms add complexity to UE and Network to maintain the start frame and number of SPS occasions after SFN wrap around until it is released. After SFN wrap around, the SPS occasion may overlap with other scheduling such as SIB1 and PSS/SSS which results in postponement of the SPS occasion. This may cause collision of UL SPS occasion among multiple UEs. Therefore, we think periodicities such as 120ms and 240ms should be avoided.

Send LS reply to SA4 that RAN2 recommends avoiding periodicities such as 120ms and 240ms and agree draft LS in [5].

## SA2 LS [3]

SA2 has asked use to answer few questions help them make decision on down selection for CP vs UP solutions.

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

In our understanding the RoHC protocol is outside of 3GPP scope and RoHC state change depends on use of different operating modes. The use of RoHC modes is up to implementation and is not mandated by 3GPP specification. Therefore, we can only answer based on what we observe in the RoHC implementation based on RoHC protocol.

As specified in RFC 3095, the decisions about RoHC state fall back are taken by the compressor on the basis of:

- variations in packet headers

- negative feedback from decompressor (Negative ACKs -- NACKs)

- periodic timeouts (when operating in unidirectional mode, i.e., over simplex channels or when feedback is not enabled)

The commonly used mode for RoHC is Bidirectional Optimistic mode (O mode). In O mode, RoHC header type (i.e., UO-0 header type) can carry only 4 bits LSB of RTP SN and 3 bits CRC SN. When more than 16 consecutive packets are lost between compressor and decompressor, this may cause the LSBs of the RTP SN in compressed packets to be interpreted wrongly, because the decompressor has not moved the interpretation interval for lack of input (e.g., a kind of context damage). This can lead to CRC check failure (i.e., decompression failure). As per section 5.3.2.2.3 in RFC 3095, when the CRC check of a decompressed packet fails, context damage may be assumed and a NACK is sent. This will trigger the RoHC state fall back at compressor.

However, due to GEO channel condition, local clock error (in case of time-based decompression) and wraparound issue of compressed RTP SN and/or compressed RTP timestamp, decompression failure can occur for any packet arriving at the decompressor. This can result in RoHC NACK feedback and ROHC state fall back at compressor.

This issue resulting from the channel condition (i.e., link budget, NLOS) should be further investigated by RAN1.

For Q1, respond to the Question 1 that RoHC implementation is out of 3GPP scope and RAN2 observation is that the decompression failure of one or more packets can result in ROHC state fall back at compressor.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

Due to nature of very long RTT over GEO channel, UL scheduling based on scheduling requests or buffer status report would be very inefficient. This incurs large scheduling delays and older voice packets may need to be discarded as they become obsolete. Retransmission may be disabled over GEO channel.

NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to periodic UL voice traffic and DL voice traffic. Network may not be able to allocate further UL resources to UE for transmitting RoHC feedback and other data such TA report etc.

Figure 2 For HD NB-IoT, UL and DL voice traffic alone can overwhelm the resources making hard for ROHC feedback channel.

As shown in figure 2, when the UE’s voice packet size exceeds the TBS size allocated for voice traffic with negotiated codec rate and considering the 4 bytes of RoHC header, the RLC layer would need to segment the UL data. RLC segmentation adds further complexity and larger overhead. Due to this, the segmented voice packet needs to be sent which will delay the transmission of the next arrived voice packet. It is to note that NB-IoT operates in half duplex mode and resources are already exhausted for allocating resources to UL voice traffic and DL voice traffic at the same time. For a TBS size of 144 bits, a minimum of 64 repetitions of NPUSCH transmission may be needed at worst coverage level. Network may not be able to allocate further UL resource to UE. This can further impact the network’s ability to create a feedback channel for RoHC and configure other resources such as TA report, BSR etc.

Eventually the older voice packets would need to be dropped to make space for the new fresh voice packets. Such voice packet drops will further aggravate the user experience of voice call. In short, RoHC leads to dynamically changing packet size which can be very challenging to handle from resource scheduling point of view for NB-IoT UEs operating in HD mode.

For Q3, respond in the LS that NB-IoT over GEO channel cannot handle the voice packets of different sizes changing dynamically without significant performance impact on voice service.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

For supporting voice traffic over NB-IoT, a new enhancement or signaling design would be inevitable. As part of such new signaling design, it is feasible to extend DRB-ToAddModList-NB to add more than 2 DRBs if needed.  it is also feasible to extend SRB-ToAddModList-NB to add a new SRB (e.g., SRB2 or SRB2bis) which is dedicated to carry voice traffic using ULInformationTransfer-NB RRC message.

For Q6 and Q7, respond in the LS that it is feasible to support more than 2 DRBs or define a new SRB.

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

SRB1bis is used for CP solutions. It is specified that SRB1bis uses the same configuration as SRB1. SRB1bis is used for RRC messages for NAS messages prior to the activation of security, i.e., bypassing PDCP layer. RLC AM is specified as the only applicable RLC mode for SRB1 and SRB1bis.

Similarly, it is possible for a new SRB, um-Uni-Directional-UL-r15 i.e., in RLC UM mode is configured explicitly or RLC UM is specified as a default configuration implicitly.

For Q10, respond in the LS that it is possible for a new SRB, um-Uni-Directional-UL-r15 i.e., in RLC UM mode is configured explicitly or RLC UM is specified as a default configuration implicitly.

To SA2, RAN2 agree the draft LS reply provided in [6].

- Conclusion

Following proposals are made:

Observation 1.	if 1 byte MAC header is not realistic, then it can be argued that total 2 bytes of MAC header is also equally not realistic in a TBS size. But currently 2 bytes of MAC header is used in determining TBS size and codec rate.

Proposal 1	Clarify to SA4 that total MAC header of 1 byte is feasible in case UE has only one PDU that fits in the available TBS.

Proposal 2	For UP solution, clarify to SA4 that 4 bytes of RoHC header (not 3 bytes) is more reasonable to consider.

Proposal 3	For CP solution, indicate that RRC layer adds 2 bytes of overhead. If the size of NAS PDU carrying voice packet is fixed based on codec rate, the RRC layer adds just 1 byte of overhead.

Proposal 4	To SA4, RAN2 agree to draft LS reply provided in [4].

Proposal 5	Send LS reply to SA4 that RAN2 recommends avoiding periodicities such as 120ms and 240ms and agree draft LS in [5].

Proposal 6	For Q1, respond to the Question 1 that RoHC implementation is out of 3GPP scope and RAN2 observation is that the decompression failure of one or more packets can result in ROHC state fall back at compressor.

Proposal 7	For Q3, respond in the LS that NB-IoT over GEO channel cannot handle the voice packets of different sizes changing dynamically without significant performance impact on voice service.

Proposal 8	For Q6 and Q7, respond in the LS that it is feasible to support more than 2 DRBs or define a new SRB.

Proposal 9	For Q10, respond in the LS that it is possible for a new SRB, um-Uni-Directional-UL-r15 i.e., in RLC UM mode is configured explicitly or RLC UM is specified as a default configuration implicitly.

Proposal 10	To SA2, RAN2 agree the draft LS reply provided in [6].

- References

[1] R2-2505068, “LS on the RAN simulation assumptions for ULBC”, Bengaluru, India, 25th - 29thAugust 2025.

[2] R2-2506754, “LS on bundling period and SPS for ULBC”, Prague, Czech Republic, 13th - 17th October 2025.

[3] R2-2506747, “LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC”, Prague, Czech Republic, 13th - 17th October 2025.

[4] R2-2507444, “Reply LS on the RAN simulation assumptions for ULBC”, Prague, Czech Republic, 13th - 17th October 2025.

[5] R2-2507446, “Reply LS on bundling period and SPS for ULBC”, Prague, Czech Republic, 13th - 17th October 2025.

[6] R2-2507445, “Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC”, Prague, Czech Republic, 13th - 17th October 2025.