3GPP TSG-RAN WG2 Meeting #131bis	R2-2507492

Prague, Czech Republic, 13th - 17th Oct, 2025

Agenda Item:	8.13.2

Source: 	Huawei, HiSilicon

Title: 	Remaining issues for Multi-hop Relay

Document for:	Discussion and Decision

# Introduction

RAN2#131 meeting agreed to complete the Rel-19 WI for the multi-hop relay.

Rel-19 relay WI is complete from RAN2 point of view (pending closure of the CR email discussions).

This paper discusses remaining issues that need to be addressed  to ensure reliable operation of  Rel-19 multi-hop relay. The  details are provided below.

# Discussion

## 2.1 Remaining issues for control plane

### 2.1.1 Error handling for SRAP

In single-hop relay scenario, the relay UE and remote UE do error handling by comparing the bearer ID and local ID information and the mapping information, except the following case for the relay UE:

- the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [2] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID

The above error handling is for path switching case when an IDLE/INACTVIE relay UE is selected and the remote UE sends the SRB1 message via the IDLE/INACTVIE relay UE, and the mapping configuration on the relay UE side cannot be ready.

Figure 1. Connection establishment for multi-path relay

Figure 1 shows the connection establishment procedure for multi-path relay. In step 1 and step 2, the remote UE selects a proper path and establishes the PC5 connection with the first relay UE. The first relay UE triggers the SUI message in step 3. After receiving the SUI message form the first relay UE, which indicates the newly connected remote UE identity, the gNB need configure the SRB0 mapping information for all relay UEs. In the meantime, the remote UE sends the SRB0 message to the first relay UE without SRPA header. Therefore, the first relay UE will not do the error handling for the SRB0 message, and the first relay UE will forward after receiving the SRB0 mapping information and contain the bearer ID and local ID in the SRAP header.

To be more effective, the gNB can configure all the relay UEs in parallel in steps 4-1, 4-2, and 4-3, since each relay UE is configured through the RRC reconfiguration procedure of itself. The configuration latency is uncontrollable since there is several hops between the relay UE and the gNB, so is the order of mapping configuration for the relay UEs.

As a result, when the first relay UE forward the SRAP PDU which carries the SRB0 message to the intermediate relay UE, the SRB0 mapping configuration may be not available at the intermediate relay UE. In this case, if the intermediate relay UE do the error handling, it will discard this SRB0 message from the remote UE, which can cause the connection establishment failure. Therefore, we believe for the first SRB0 message from a remote UE, the relay UE should also be forbidden from perform the error handling.

For the SRB1 message forwarding, the similar problem exists in steps 7 to 9. For example, the remote UE will respond the RRCSetupComplete message after receiving the SRB1 mapping configuration and send the SRAP PDU carrying SRB1 message in the UL link. It is possible that the SRB1 mapping configuration is not ready at the relay UE side. As a result, the SRB1 massage will be discard if the relay UE do the error handling in this case. Similarly, the error handling for the first SRB1 message from a remote UE should also be forbidden.

Observation 1. When the intermediate relay UE receives the SRB0/SRB1 message form the child UE, the SRB mapping configuration may be not yet be ready.

Observation 1: When an intermediate relay UE receives the SRB0/SRB1 message from a child UE, the SRB mapping configuration may not yet be ready.”

We had proposed this case to be handled in the email discussion [Post131][411][Relay] Rel-19 relay 38.351 CR .  The rapporteur suggested that since the intermediate relay UE is assumed to be in RRC connected state, the network would provide the necessary configurations for the Remote UE’s SRB0/SRB1. Therefore, no special handling appears to be needed for this case, while inputs from other companies are still welcomed.

If the network provides the necessary configuration for the remote UE, it should be noted that uplink SRB0 and SRB1 message is not fully controlled by the network. For the UL SRB0, when the first relay UE receives the SRB0 mapping information, it will forward the SRB0 message. To avoid incorrect discarding, gNB should configure the SRB0 mapping at the first relay UE only after the SRB0 mapping configuration isready at all the parent relay UEs. This implies that RRC configuration across  all relay UE(s) cannot be performed in parallel, which may result in expiration of the T300 timer.

To be more effective, we suggest we should allow the parallel configuration of all the relay UEswhile ensuring  appropriate error handling for SRB0 and SRB1 in multi-hop relay.

Proposal 1. The relay UE should not perform error handling for the first SRB0 and the first SRB1 messages received form a child UE.

## 2.2 Remaining issues for Discovery

### 2.2.1 Discovery Model B with no PC5 Link in multi hop scenario

In the previous meeting there were discussions on Discovery Model B, specifically for scenarios where no PC5 link is established between a parent Relay UE and the Remote UE. It was agreed that in Discovery Model B, when there is no established PC5 link between the Remote UE and the Intermediate Relay UE, the Intermediate Relay UE should forward the solicitation message only if the PC5 RSRP between the Remote UE (or First Relay UE) and the Intermediate Relay UE is above a defined threshold.

One possible scenario involves a UE that is capable of functioning as an intermediate relay UE (i.e., it supports the Rel-19 multi-hop relay feature through the path gNB <-> relay UE2 <-> the current intermediate relay UE as shown in Figure 2), but is not currently operating as a relay in a multi-hop configuration (i.e., it is now connected through the single-hop relay UE, the relay UE1, as illustrated in Figure 2). In this figure, the UE is capable of acting as an intermediate relay UE but is currently connected to a Rel-17 single-hop relay. In such a scenario, if a multi-hop remote UE attempts to discover a valid multi-hop path, the intermediate relay UE can participate in Model B discovery by forwarding the discovery message by evaluate the AS conditions [2]. This enables the remote UE to identify potential relay paths.

Figure 2. Intermediate Relay UE participating in Model B multi-hop relay discovery.

Observation 2: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

In above case in Figure 2, we should allow the remote UE to select the multi-hop relaying path through the candidate intermediate relay UE, since the remote UE may not find any other available candidate path. In this case, when the remote UE selects a path through am intermediate relay UE, the intermediate relay UE should be allowed to change the path form the single-hop relaying to multi-hop relaying.

Proposal 2: When the remote UE selects a path through an intermediate relay UE, the intermediate relay UE should be allowed to release the legacy single-hop relaying path and establish the candidate multi-hop relaying path.

2.2.2 Conditions for stopping discovery procedure

In multi-hop relay, each relay UE along the path can perform discovery messaging and accept the relaying service requests form one or more remote UEs. However, a relay UE may decide to stop performing discovery procedure, for example, if serving too many Remote UEs creates excessive traffic burden or if its battery level is too low to provide additional relaying service. In such cases, the relay UE should also indicate the its child UEs (both directly or indirectly connected) to stop performing discovery messaging.

Figure 3. Last Relay UE in overload situation indicating child UEs to stop performing discovery.

Proposal 3: If a relay UE decides to stop performing discovery and does not wish to provide relaying service for additional Remote UEs, it should indicate to all its child UEs to also stop discovery.

# Conclusion

In this paper, we discuss the remaining issues and have the following observation and proposals.

Error handling for SRAP

Observation 1. When the intermediate relay UE receives the SRB0/SRB1 message form the child UE, the SRB mapping configuration may be not yet be ready.

Proposal 1. The relay UE should not perform error handling for the first SRB0 and the first SRB1 messages received form a child UE.

Discovery Model B with no PC5 Link in multi hop scenario

Observation 2: The scenario where there is no established PC5 link between the last relay UE and the intermediate relay UE, and the intermediate relay UE sends/forwards the discovery solicitation message received from a nearby Remote UE before establishing itself as a Remote UE could happen when A UE that is capable of functioning as an intermediate relay UE, but not currently operating as a relay in a multi-hop configuration, participates in Model B discovery by forwarding the discovery Solicitation message.

Proposal 2: When the remote UE selects a path through an intermediate relay UE, the intermediate relay UE should be allowed to release the legacy single-hop relaying path and establish the candidate multi-hop relaying path.

Conditions for stopping discovery procedure

Proposal 3: If a relay UE decides to stop performing discovery and does not wish to provide relaying service for additional Remote UEs, it should indicate to all its child UEs to also stop discovery .

# References

[1] RAN 2 # 131 - Report from session on positioning and sidelink relay

[2] R2-2506574, Introduction of NR sidelink multi-hop relay, 38.331 CR