3GPP TSG-RAN WG2 Meeting #131bis	R2-2507511

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.2

Source:	Hanbat National University

Title:	Initial Considerations for 6GR Access Technology

Document for:	Discussion and Decision

# Introduction

In the SID of Study on 6G Radio [1], the following objective to study of radio interface protocol architecture and procedures for 6GR is defined.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we provide the general aspect of architecture and deployment scenarios for 6GR.

# Support of AI-native RAN architecture

The evolution toward 6G mobile communication systems requires RAN to go beyond conventional connectivity functions and act as an intelligent, autonomous, and adaptive system. Future services such as immersive communication, ultra-reliable low-latency control, global-scale IoT, and integration with satellite networks demand AI/ML-native design principles within RAN.

While NWDAF in 5GC provides network data analytics, it is NOT sufficient to address RAN-specific issues, e.g., interference, dynamic channel conditions, mobility, and localized congestion. Therefore, it is essential to define a AI-native intelligent architecture that integrates with UE and next generation RAN to enable distributed, cooperative, and adaptive intelligence.

The AI/ML framework for air interface was discussed in [2], however, it has a limitation to fully support AI-native functions in RAN-specific scenarios. Hence, in this contribution, we identify the AI-native functions to support various intelligent RAN management.

## AI-native RAN function

Traditional rule-based RAN management cannot adequately address the dynamic and heterogeneous requirements expected in 6G networks. Rapid traffic fluctuations, high mobility, and multi-cell interference expose the limitations of static algorithms, leading to degraded QoS and inefficient resource utilization. To overcome these challenges, AI-native RAN functions are required to enable predictive and adaptive control of scheduling, handover, interference management, and energy optimization.

Proposal 1: AI-native RAN includes followed functions to support intelligent RAN management.

- Analytic and Intelligence Function Manager (AIFM): orchestration, model management, inference control.

- Local Model Training (LMT): localized model learning.

- Global Model Training (GMT): federated learning hub, deployed either inside RAN nodes or external servers.

- Inference Engine (INF): real-time decision-making using local or global models.

- Data Processing Unit (DPU): handles measurement reports, KPIs, and training data.

## UE capability in AI-native RAN

The AI-native RAN provides system-level optimization, certain limitations remain in latency, personalization, and scalability in UE-perspective, hence, the embedding AI-native functions in the UE addresses these challenges. Local inference enables immediate decision-making for ultra-low latency services, avoiding round-trip delays between the UE and RAN. On-device training allows personalized optimization based on unique user conditions while preserving privacy by keeping raw data local. Furthermore, UE-side intelligence reduces signalling overhead by reporting only model updates or performance indicators instead of raw measurements. Finally, UEs with local intelligence enhance system resilience, maintaining autonomous operation even under weak or intermittent connectivity.

Proposal 2: UE in AI-native RAN includes followed functions to support intelligent RAN management.

- Analytic and Intelligence Function Manager (AIFM): orchestration, model management, inference control.

- Local Model Training (LMT): on-device localized model learning.

- Inference Engine (INF): real-time decision-making using local or global models.

- Data Processing Unit (DPU): handles measurement reports, KPIs, and training data.

To reduce UE power consumption and computational burden, LMT or/and INF functions of UE can be removed. These minimize UE overhead and signalling while still enabling service adaptation, though at the cost of reduced personalization and increased dependency on the RAN.

Proposal 3: Reduced UE Capability

In this deployment, to reduce UE power consumption and computational burden, LMT or/and INF functions of UE can be removed, while the RAN includes all functions with external GMT.

## AI-native RAN deployment option

To support heterogeneous device capabilities, multiple architecture options are envisioned for the AI-native RAN. These options define the distribution of AI functions, i.e., Analytic and Intelligence Function Manager (AIFM), Local Model Training (LMT), Global Model Training (GMT), and Inference (INF).

If the GMT is located outside the RAN, e.g., at the core network or an edge server, while both UE and RAN include LMT and INF modules. UEs and RAN perform local training and inference, with model updates aggregated at the external GMT.

We also consider that the GMT can be located in some RAN nodes while others have only LMT. Here, RAN performs federated aggregation from GMT-enabled nodes, which then distribute global models back into other nodes. This configuration reduces reliance on external servers and supports scalability, making it appropriate for large deployments where certain node act as regional AI hubs. Hence, if the GMT is located in some RAN nodes, while other nodes have only LMT. The node which has GMT acts as a regional AI hub, then the federated training with other nodes is performed.

To support fully distributed federated model training, the GMT can be removed in RAN node. In this deployment, all nodes in RAN have only LMT, which performs local training and exchanges model parameters between nodes. While it may suffer from slower convergence and requires robust synchronization mechanisms, this approach maximizes resilience and reduces dependency on centralized training module. In this deployment, all nodes of RAN have only LMT without GMT. The node only performs local training and exchanging model parameters between nodes. The system achieves model consistency via distributed parameter synchronization rather than centralized aggregation.

Proposal 4: RAN2 is encouraged to study AI-native RAN deployment, including the following:

- Centralized Training Deployment

  - In this deployment, the GMT is located outside the RAN, e.g., at the core network or an edge server, while both UE and RAN include LMT and INF modules. UEs and RAN perform local training and inference, with model updates aggregated at the external GMT.

- Semi-Distributed Training Deployment

  - In this deployment, the GMT is located in some RAN nodes, while other nodes have only LMT. The node which has GMT acts as a regional AI hub, then the federated training with other nodes is performed.

- Fully Distributed Training Deployment

  - In this deployment, all nodes of RAN have only LMT without GMT. The node only performs local training and exchanging model parameters between nodes. The system achieves model consistency via distributed parameter synchronization rather than centralized aggregation.

# General Aspects of ISAC in 6G RAN

Integrated Sensing and Communication (ISAC) is recognized as a key new capability in 6G, allowing the RAN to simultaneously provide communication services and environment sensing. In Release-19, initial studies laid the foundation for ISAC: SA1’s feasibility study (TR 22.837) identified diverse use cases (from intruder detection to health monitoring) [3], and RAN1 investigated channel models for sensing targets like UAVs, vehicles, and humans [4]. Building on these, the Release-20 6G RAN study (6GR) is now considering higher-layer protocol and architectural support for ISAC [1]. While early discussions often highlighted UAV detection as a representative example especially in NR ISAC, it is important to broaden the perspective to ensure 6G RAN design accommodates a wide range of ISAC applications. This section focuses on general and architectural aspects of ISAC suitable for RAN2’s scope, i.e. higher-layer procedures, deployment scenarios, and coordination mechanisms.

## Broader ISAC use cases

To fully realize ISAC’s potential, 6G should target a broad set of use cases beyond a single scenario like UAV sensing, which may include:

- Traffic monitoring: Roadside or urban cell sites using 6G signals to monitor vehicular traffic flow, detect accidents, or identify pedestrians and obstacles in smart cities. This can enhance transportation management and safety by providing real-time situational awareness to traffic control systems.

- Indoor occupancy sensing: Small cells or access points detecting human presence and movement inside buildings (offices, homes, malls) via radio signals. Such capability could enable smart building automation and security (intruder alerts) without dedicated sensors.

- Road condition detection: Using reflections from 6G signals to assess road surface conditions (e.g. wet, icy, or debris on roads). For instance, a base station could detect anomalous reflections indicative of hazards on a highway, supplementing vehicular sensors and improving automotive safety.

- Gesture-based user interface: Leveraging high-frequency 6G radios (e.g. mmWave or sub-THz) to interpret fine hand or body gestures as control inputs. ISAC technology could detect and interpret human gestures, enabling hands-free control of devices in smart homes or industrial settings.

Supporting such a variety of applications in a unified RAN requires flexible architecture and procedures that are agnostic to the specific object or phenomenon being sensed.

Proposal 5: RAN2 is encouraged to include and examine use cases beyond UAV detection such as traffic monitoring, indoor occupancy sensing, road condition detection, and gesture-based interfaces.

## Potential deployment scenarios

Corresponding to the above use cases, several deployment scenarios can be envisioned for 6G ISAC:

Urban infrastructure sensing: 6G base stations deployed on city infrastructure (e.g. street lamps, traffic lights, smart poles) perform environmental sensing as a service. In dense urban areas, network nodes could scan for traffic density, detect incidents, or monitor public spaces for safety, all while providing communication services. The network may aggregate sensing data city-wide to support required operations such as traffic management and emergency response.

Smart transportation corridors: Along highways or railway lines, 6G RAN nodes (including road-side units or vehicular repeaters) integrate sensing to assist transportation. For example, along a smart highway, a series of 6G TRPs could jointly detect hazardous road conditions or coordinate with connected vehicles to warn of obstacles and enable cooperative perception. This scenario emphasizes reliability and low latency. The RAN is required to quickly share sensed information (e.g. a detected object on the road) with vehicles or control centers.

Indoor hotspot and user-centric sensing: In venues like homes, shopping malls, airports, industrial campuses, 6G small cells and devices can create a local sensing mesh. An indoor gNB might track the number of occupants in each room for climate control, or user devices might use ISAC features for AR/VR applications (e.g. mapping a user’s surroundings, gesture recognition). This scenario highlights personalized and on-demand sensing, potentially initiated by UE and coordinated with the network to manage interference and data reporting.

RAN2 should ensure the architecture can flexibly accommodate these scenarios, for instance by allowing network slicing or QoS differentiation for sensing services, and by providing a means to deliver sensing results either to the core network or directly to UEs.

Proposal 6: RAN2 is encouraged to outline several reference deployment scenarios in the study (e.g. urban infrastructure sensing with network-centric processing, smart transportation corridors with vehicular involvement, and indoor hotspot sensing for user devices).

## High-level architectural and coordination aspects

From an architecture perspective, a key challenge for ISAC is managing the dual use of radio resources for communication and sensing. RAN2’s protocols are required to enable coordinated resource management so that sensing functions achieve the needed accuracy without unduly degrading communications. One important consideration is interference management through intelligent scheduling and muting strategies:

- TRP-Coordinated Muting: To reliably detect objects, a sensing receiver (e.g. a base station listening for echoes) needs a quiet radio environment during the sensing interval. In a dense network, neighboring TRPs’ normal transmissions can act as strong interference that masks the weak reflected signals. Therefore, the concept of TRP-coordinated muting can be studied, i.e., neighboring transmission points temporarily reduce or mute their communications signals in certain time-frequency resources to create an interference-suppressed window for sensing by a given TRP.

- Cooperative Sensing and Interference Suppression: Beyond simply muting interference, 6G RAN can exploit cooperative schemes where multiple TRPs work together for sensing. In a cooperative muting approach, a group of TRPs may alternate roles, e.g., one acts as the sensing transmitter while others mute and possibly serve as additional receivers (a form of multi-static sensing). It allows coverage of larger areas and diverse angles, and can improve resilience against interference by design. RAN2 might consider enhancements on the existing RRC signaling to facilitate such cooperation. In addition, various sensing deployment configurations (monostatic vs. bi-static, single-TRP vs. multi-TRP) should be supported, which implies flexibility in RAN protocols to handle different roles (transmit, receive, mute) for different nodes during sensing operations.

In all cases, the emphasis is on high-level coordination to define how and when sensing is initiated and coordinated. This could include, for example, RRC messages to configure a UE or TRP for sensing mode, new MAC scheduling policies that treat sensing tasks with appropriate priority. Architectural considerations also include how sensing information is handled within the RAN.

Overall, these general considerations aim to guide RAN2 in specifying an ISAC-ready architecture: one that can flexibly support new sensing use cases, deploy in various scenarios, and coordinate multi-cell resources to balance communication and sensing needs.

Proposal 7: RAN2 should investigate procedures for interference management tailored to ISAC and the feasibility of cooperative sensing operations, where multiple TRPs collaborate by sharing sensing roles or measurements.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: AI-native RAN includes followed functions to support intelligent RAN management.

- Analytic and Intelligence Function Manager (AIFM): orchestration, model management, inference control.

- Local Model Training (LMT): localized model learning.

- Global Model Training (GMT): federated learning hub, deployed either inside RAN nodes or external servers.

- Inference Engine (INF): real-time decision-making using local or global models.

- Data Processing Unit (DPU): handles measurement reports, KPIs, and training data.

Proposal 2: UE in AI-native RAN includes followed functions to support intelligent RAN management.

- Analytic and Intelligence Function Manager (AIFM): orchestration, model management, inference control.

- Local Model Training (LMT): on-device localized model learning.

- Inference Engine (INF): real-time decision-making using local or global models.

- Data Processing Unit (DPU): handles measurement reports, KPIs, and training data.

Proposal 3: Reduced UE Capability

In this deployment, to reduce UE power consumption and computational burden, LMT or/and INF functions of UE can be removed, while the RAN includes all functions with external GMT.

Proposal 4: RAN2 is encouraged to study AI-native RAN deployment, including the following:

- Centralized Training Deployment

  - In this deployment, the GMT is located outside the RAN, e.g., at the core network or an edge server, while both UE and RAN include LMT and INF modules. UEs and RAN perform local training and inference, with model updates aggregated at the external GMT.

- Semi-Distributed Training Deployment

  - In this deployment, the GMT is located in some RAN nodes, while other nodes have only LMT. The node which has GMT acts as a regional AI hub, then the federated training with other nodes is performed.

- Fully Distributed Training Deployment

  - In this deployment, all nodes of RAN have only LMT without GMT. The node only performs local training and exchanging model parameters between nodes. The system achieves model consistency via distributed parameter synchronization rather than centralized aggregation.

Proposal 5: RAN2 is encouraged to include and examine use cases beyond UAV detection such as traffic monitoring, indoor occupancy sensing, road condition detection, and gesture-based interfaces.

Proposal 6: RAN2 is encouraged to outline several reference deployment scenarios in the study (e.g. urban infrastructure sensing with network-centric processing, smart transportation corridors with vehicular involvement, and indoor hotspot sensing for user devices).

Proposal 7: RAN2 should investigate procedures for interference management tailored to ISAC and the feasibility of cooperative sensing operations, where multiple TRPs collaborate by sharing sensing roles or measurements.

# References

[1] RP-251881	New SID: Study on 6G Radio.

[2] TR 38.843 AI/ML for NR air interface.

[3] TR 22.837 Study on Integrated Sensing and Communication

[4] TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz