3GPP TSG-RAN WG2 Meeting #131bis                                 R2-2507513

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	9.2.2

Source:	TCL

Title:	Discussion on Topology 2 for Ambient IoT

Document for:	Discussion and Decision

1	Introduction

In the RAN#109 meeting, a new work item (WI) about Ambient IoT (Internet of Things) in NR was revised [1]. One of the main objectives of the study relates to scenario 2 with topology 2:

2. Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

For RAN2, the work is subject to the following constraints:

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we will share our views on the following aspects related to topology 2, including resource allocation and UE reader selection.

2 Discussion

In coordination with SA2, the RAN plenary has also updated the objective in the revised WID, as highlighted in yellow as following.

2. Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - Specify RRC-based architecture, in coordination with SA2. - The following are not supported:

Therefore, RAN2 should only consider the RRC-based solution to enable topology 2 for A-IoT in Rel-20.

For A-IoT topology 2, only the RRC-based solution is considered in Rel-20.

In Rel-19, for TP2 in Ambient IoT, it was agreed to configure dedicated resources to the UE Reader via dedicated signalling, as captured in TR [2] below.

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling.

Moreover, two alternatives for the radio resources were also discussed in Rel-19 [2]:

- Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

Two alternatives for the radio resources were also discussed in Rel-19:

- -	Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- -	Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

In our views, Alternative 1 facilitates a more efficient and precise resource allocation by the BS, using for the UE reader performing the A-IoT operations. This is because the UE Reader has a more direct understanding of the A-IoT interface conditions. For example, even if the quality of Uu interface between the UE reader and the BS is good, poor A-IoT interface conditions between the UE reader and the A-IoT devices may still result in a higher inventory failure rate, leading to longer completion time of the overall inventory procedure. Therefore, Alternative 1 allows it to autonomously decide when to initiate a resource request and the size of the required resources based on its actual situations. As for Alternative 2, gNB can only roughly estimate the required resources based on the received the assistant information carried by a service request from CN.

Hence, from our perspective, it is recommended that Alternative 1 should be studied during Rel-20.

For the A-IoT radio resource allocation from BS to UE reader, RAN2 should focus on the Alternative 1 (UE reader requests A-IoT radio resources to the gNB) in Rel-20.

For TP2, it is beneficial that some assistance information is provided to BS, which can help BS assign the appropriate resources, improve resource efficiency. In our understanding, the assistance information may include A-IoT service type, proximate number of A-IoT devices, A-IoT device type, etc. etc. For example, if the A-IoT service types (e.g., inventory, command) are provided to BS, BS may know it requires the operation duration period, which is good for the BS to allocate sufficient resources. In another instance, the SID [2] has also approved three types of A-IoT devices with different capabilities, some of them may require different frequency bands to send messages over A-IoT interface. For example, Device 2, which transmits the D2R message and receives the R2D message, may need different bands.

It is suggested that RAN2 study the potential assistance information provided to the BS, with the aim of improving the efficiency of resource allocation.

During the Rel-19 discussion, RRC based solution, NAS based solution and UP based solution were identified to support Topology 2 for conveying A-IoT upper layer information, as summarized below.

- RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

The new WID [1] specifies an exclusively RRC-based architecture, for which the corresponding protocol stack is depicted in Figure 1.

Figure 1: RRC based solution of Topology 2

In this solution, the A-IoT-related message is carried in RRC signalling between the A-IoT-enabled UE reader and the A-IoT-enabled gNB. Take an example for the UL data transmission, the UE reader receives the A-IoT data from A-IoT device and forwards the A-IoT data to the gNB via RRC signaling, then the gNB transfers the A-IoT data to the A-IoT CN functions via XXAP. It can be observed that the gNB has the capability to decode the RRC message and XXAP message.

Based on the above discussion, the following Figure 2 is given to describe the stage 2 procedure for A-IoT inventory procedure in Topology 2 for RRC based solution.

Figure 2: Potential signalling procedure for A-IoT Inventory in Topology 2 (RRC-based solution)

1.	The A-IoT CN sends an Inventory request message to the A-IoT enabled gNB over XXAP protocol.

2.	The A-IoT enabled gNB forwards the Inventory request to the A-IoT enabled UE by RRC message.

3. A-IoT enabled UE sends a A-IoT radio resource allocation request message to A-IoT enabled gNB, the message may include the number of AIoT devices.

4. A-IoT enabled gNB sends a A-IoT radio resource allocation response message to A-IoT enabled gNB

5. The A-IoT enabled UE performs inventory procedure in the new A-IoT interface using the allocated radio resource.

6. The A-IoT enabled UE sends the inventory report to the A-IoT-enabled gNB, which is received from A-IoT devices.

7. The A-IoT-enabled gNB sends the inventory report to the A-IoT-enabled CN.

For RRC-based solution, consider the above potential A-IoT Inventory in Topology 2 in Rel-20.

In the SA2 discussion, it has agreed that the CN function has capability to select the UE reader based on area/location information. While from RAN2 perspective, it is assumed that, at least for the RRC-based solution, there may be radio related reasons for BS involvement in UE Reader selection.

For RRC-based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

Therefore, considering that the CN-based approach to UE reader selection may prove insufficient due to the CN's inability to ascertain in a timely manner whether a UE is currently suitable to function as a reader. Hence, we think that UE reader selection with BS involvement should be considered during the WI phase.

In Rel-20, UE reader selection with BS involvement should be studied.

3	Conclusion

Based on the discussion in the previous sections we propose the following:

- For A-IoT topology 2, only the RRC-based solution is considered in Rel-20.

- Two alternatives for the radio resources were also discussed in Rel-19:

- -	Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

- -	Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

For RRC-based solution, there may be radio related reasons for the BS to be involved in the UE reader selection.

- For the A-IoT radio resource allocation from BS to UE reader, RAN2 should focus on the Alternative 1 (UE reader requests A-IoT radio resources to the gNB) in Rel-20.

It is suggested that RAN2 should study the potential assistance information provided to the BS, with the aim of improving the efficiency of resource allocation.

For RRC-based solution, consider the above potential A-IoT Inventory in Topology 2 in Rel-20.

In Rel-20, UE reader selection with BS involvement should be studied.

References

RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2.

TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR.