3GPP TSG-RAN WG2 Meeting #131bis									R2-2507530

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		8.4.3

Title: 	LP-WUS MAC Issues

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will discuss MAC issues on LP-WUS and give our proposals.

- Discussion

After RAN2#131 meeting, the following option issue is proposed[1]:

| Company+issue # (e.g. Apple 001) | Description of open issues and potential resolution | Rapporteur comment |
| --- | --- | --- |
| Eri-001 | RAN2 agreed: For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism. For certain configurations, where the LP-WUS monitoring occasions are sparse, e.g. smaller equal to the C-DRX cycle length, the UE becomes “temporarily” unreachable when it misses 2 or 3 consecutive LP-WUS occasions. The NW should be allowed to configure the number of consecutive LOs the UE is allowed to miss due to collisions before the UE needs to start the timer. | According to RAN2 agreement, this issue can be further discussed. Issue#1: Revisit the UE operation in LP-WUS collision for Option 1-2 Description: The revisit needs to be based on critical issues in the current RAN2 agreement. |

Considering that gNB will send PDCCH for radio resource scheduling with an offset after sending LP-WUS, if UE becomes “temporarily” unreachable when it misses 2 or 3 consecutive LP-WUS occasions, the UE will not monitor PDCCH for radio resource scheduling, and the radio resource will be wasted, which will affect the network capacity.

Observation 1:  If UE becomes “temporarily” unreachable when it misses 2 or 3 consecutive LP-WUS occasions, the UE may miss PDCCH for radio resource scheduling, which will affect the network capacity.

For this issue, based on the LP-WUS MO number (e.g. lpwus-NumOfMo12-r19) configured and proper Lp-WUS periodicity (e.g. lpwus-Mo12), we can assume that UE can always be reachable when it monitors Lp-WUS.

Proposal 1(Eri-001): RAN2 Confirms that UE can always be reachable, whether it monitors LP-WUS or monitors PDCCH directly.

After RAN2#131 meeting, the following option issue is proposed[1]:

| OPPO-001 | For a UE configured with both secondary DRX group and LP-WUS option 1-1 on PCell, there may be a case that primary DRX group is in short DRX cycle and secondary DRX group is in long DRX cycle. It is unclear whether UE monitors LP-WUS in this case. We need to discuss and clarify the UE behaviors. | According to RAN2#129bis agreement, for Option 1-1 UE doesnot monitor LP-WUS when short DRX cycle is used. The agreement should be also applicable for the dual DRX group with LP-WUS Option 1-1 configuration, i.e. UE doesnot monitor LP-WUS when short DRX cycle is used in either DRX group. => For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. (RAN2#129bis agreement) Proposal: Confirm UE doesnot monitor LP-WUS when short DRX cycle is used in either DRX group when secondary DRX group is configured. |
| --- | --- | --- |

As rapporteur points out that  For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. (RAN2#129bis agreement). And it has been agreed that LP-WUS is configured per cell group. Thus, LP-WUS should not been monitored when at least one DRX group in the cell group is using short DRX cycle.

Proposal 2(OPPO-001): RAN2 confirms that UE does not monitor LP-WUS in a cell group when it is using short DRX cycle for at least one DRX group in the cell group.

In the current TS 38.300, it has been captured that ”the UE does not monitor LP-WUS when short DRX cycle is used” as follows, which does not restrict to one ”cell group” and may lead some misunderstanding, and should be reworded.

A UE configured with DRX in RRC_CONNECTED can be configured with LP-WUS. LP-WUS is monitored outside active-time. If LP-WUS is detected, the UE shall start the on-duration timer or LP-WUS PDCCH monitoring timer to start PDCCH monitoring and enter active-time: - If the UE is configured to start on-duration timer after LP-WUS reception, the UE monitors LP-WUS at occasion(s) at a configured offset before the on-duration, and the UE does not monitor LP-WUS when short DRX cycle is used. If the UE is unable to monitor the LP-WUS occasion, it shall start the on-duration timer. ...

Proposal 2a(OPPO-001): Update TS 38.300 to capture the Proposal 2 as follows:

A UE configured with DRX in RRC_CONNECTED can be configured with LP-WUS. LP-WUS is monitored outside active-time. If LP-WUS is detected, the UE shall start the on-duration timer or LP-WUS PDCCH monitoring timer to start PDCCH monitoring and enter active-time: - If the UE is configured to start on-duration timer after LP-WUS reception, the UE monitors LP-WUS at occasion(s) at a configured offset before the on-duration, and the UE does not monitor LP-WUS in a cell cell group when short DRX cycle is used in at least one DRX group of the cell group . If the UE is unable to monitor the LP-WUS occasion, it shall start the on-duration timer. ...

After RAN2#131 meeting, the following option issue is proposed[1]:

| Vivo 003 | Whether/How to capture option 1-1/1-2 description in MAC or stage-2 specification In the first version of MAC running CR, there was some description for “option 1-1” and “option 1-2”. And in the later version, it was removed based on companies’ comments. In the current MAC specification, option 1-2 is differentiated by the condition “if lpwus-PDCCH-MonitoringTimer is configured”. But the present condition for parameter “lpwus-PDCCH-MonitoringTimer” is “this field is mandatory present for option 1-2”. This leads to mutually dependent circular conditions. Besides, there are other configurations with the condition of option 1-1/option 1-2. To resolve above problem and simplify the description in RRC, we request to discuss how to capture the description in MAC or stage-2 for “option 1-1” “option 1-2” or some similar description. | The options 1-1/1-2 description was removed from the MAC CR according to majority view. I understand that the current stage-2 38.300 CR already includes descriptions of option 1-1/1-2 in section 11, but it does not explicitly mention options 1-1/1-2. (see below) we can discuss the necessity of modifications based on the description of stage-2. The necessity to explicitly describe “option 1-1/1-2” can be discussed based on the 38.300 description. Proposal: Further discuss the necessity to explicitly describe “option1-1/1-2” based on 38.300 description. |
| --- | --- | --- |

For this issue, we have the same understanding as rapporteur that the terminology of option 1-1 and Option 1-2 should be defined in TS 38.300 so that the people that does not participate the LP_WUS discussion can understand the specification easily. Because that the terminology of option 1-1 and Option 1-2 are used in many specifications (e.g. both the RAN1 and RAN2 specification), the definition in stage 2 is a straightforward way to capture this issue.

Proposal 3(Vivo 003): The terminologies of LP-WUS Option 1-1 and Option 1-2 are defined in TS 38.300.

- Conclusion

The following observation and proposal are given based on the above analysis:

Observation 1:  If UE becomes “temporarily” unreachable when it misses 2 or 3 consecutive LP-WUS occasions, the UE may miss PDCCH for radio resource scheduling the radio resource, which will affect the network capacity.

Proposal 1(Eri-001): RAN2 Confirms that UE can always be reachable, whether it monitors LP-WUS or monitors PDCCH directly.

Proposal 2(OPPO-001): RAN2 confirms that UE does not monitor LP-WUS in a cell group when it is using short DRX cycle for at least one DRX group in the cell group.

Proposal 2a(OPPO-001): Update TS 38.300 to capture the Proposal 2 as follows:

A UE configured with DRX in RRC_CONNECTED can be configured with LP-WUS. LP-WUS is monitored outside active-time. If LP-WUS is detected, the UE shall start the on-duration timer or LP-WUS PDCCH monitoring timer to start PDCCH monitoring and enter active-time: - If the UE is configured to start on-duration timer after LP-WUS reception, the UE monitors LP-WUS at occasion(s) at a configured offset before the on-duration, and the UE does not monitor LP-WUS in a cell cell group when short DRX cycle is used in at least one DRX group of the cell group . If the UE is unable to monitor the LP-WUS occasion, it shall start the on-duration timer. ...

Proposal 3(Vivo 003): The terminologies of LP-WUS Option 1-1 and Option 1-2 are defined in TS 38.300.

- References

- R2-25xxxxx_Open issues on Rel-19 LPWUS 38.321 CR_v06_Eri