3GPP TSG-RAN WG2 Meeting #131bis	R2-2507539

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.12.2

Source: 	ASUSTeK, Ofinno, Ericsson

Title:  	Discussion on MIMO MAC open issues

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#131, there were discussions regarding UEI report configuration resource release upon TAT expiry:

R2-2506035 Discussion on MAC open issues for MIMO phase 5 ASUSTeK discussion Rel-19 38.321 NR_MIMO_Ph5-Core Noted Proposal 1: RAN2 to confirm that based on the current MAC spec, the UE releases PUCCH resources and clears type-1 CG PUSCH resources configured in a UEI report configuration of a SCell if the TAT of the sTAG of the SCell is expired, regardless of the Cell on which the PUCCH and PUSCH resources are configured.

In report for MIMO MAC open issues [3], a related open issue is also listed (Issue-1) for discussion.

In this contribution, we’d like to further clarify the identified issue, provide analysis on solutions on the table and try to reach a common understanding for UE behavior regarding releasing UEI report resources across TAGs upon STAG TAT expiry.

In addition, in the current MAC spec for DRX, after transmitting a UEIRI on PUCCH for CSI reporting mode-A, the UE stays in DRX active time before receiving a corresponding PDCCH scheduling the mode-A CSI report:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]).

and an open issue was also added in email discussion [217] [3] as [Issue-10].

In this contribution, we’d like to discuss missing cases for considering DRX active time condition regarding mode-A CSI reporting.

- Issue-1

2-1.  Background, previous agreement and identified issue

In the current running RRC CR for MIMO ph5[1], the UEI report is configured as a CSI-ReportConfig with CSI-ReportUE-IBR included in a Serving Cell configuration, containing:

1. a type-1 CG PUSCH resource for CSI report transmission, if the transmission mode for the UEI report is mode-B, and

2. a PUCCH resource via PUCCH-ResourceId, BWP-Id, and pucch-Cell-r19 indicating on which Cell the PUCCH resource is (i.e., PUCCH SCell or PCell) for UEI report indicator (UEIRI) transmission:

(38.331 running CR) #### – CSI-ReportConfig The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included (in this case, the cell on which the report is sent is determined by the received DCI). The IE CSI-ReportConfig is also used to configure UE initiated CSI reporting. See TS 38.214 [19], clause 5.2.1. CSI-ReportConfig information element … CSI-ReportUE-IBR-r19 ::= SEQUENCE { … reportTransmissionMode-r19 CHOICE { modeA-r19 NULL, modeB-r19 SEQUENCE { pusch-ResourceOfModeB-r19 SEQUENCE { configuredGrantConfigIndex-r19 ConfiguredGrantConfigIndex-r16, ul-BWP-Id-r19 BWP-Id, servCellIndex-r19 ServCellIndex }, … OPTIONAL, -- Need R pucch-Resource-r19 SEQUENCE { periodicityAndOffset CHOICE { … }, resource PUCCH-ResourceId, ul-BWP-Id-r19 BWP-Id, pucch-Cell-r19 ENUMERATED {spCell, pucch-Scell} } } … CSI-ReportUE-IBR field descriptions. pucch-Resource Indicates the periodic PUCCH resource for the UE initiated CSI reporting indicator for both mode-A and mode-B UE initated CSI reporting: - to request dynamically scheduled PUSCH to carry UE-initiated/event-driven beam report for mode-A; - to notify the network of a Type-1 CG PUSCH to carry UE-initiated/event-driven beam report for mode-B.. …. pusch-ResourceOfModeB Indicates Type-1 CG PUSCH resource for the UE initiated CSI reporting in mode-B..

According to current spec 38.214[2], the type-1 CG PUSCH resource should be configured on the same Cell as the Cell configured with the CSI-ReportConfig:

38.214 ##### 5.2.1.5.4 UE Initiated reporting … After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. […] The UE sends the CSI report - … - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. …

In short, for PUSCH/PUCCH resources of a UEI beam report configuration, we have observation as below:

Observation 1:	For a UEI report configuration (CSI-ReportConfig) configured on a SCell, PUCCH resource of the UEI report configuration can be configured on a PUCCH Cell, which can be the SCell or a different Cell. A Mode-B type-1 CG of the UEI report configuration of a SCell has to be configured on the SCell.

In addition, given that a Cell group can be configured with a maximum of 4 TAGs, it is likely that PUCCH resource of a UEI report configuration is configured on a Cell different from the Cell on which the UEI report configuration is configured (e.g., when a sTAG’s SCell’s CSI-ReportConfig’s pucch-cell is set to spCell).

An example of the resource configuration is illustrated as follows:

Observation 2:	The PUCCH resource of a UEI report configuration of a Cell can be configured on a PUCCH Cell associated with a different TAG from the Cell.

In the previous meetings, it was agreed that

- when the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report, and

- when TAT of sTAG expires, the UE releases PUCCH resource and type-1 CG when the PUCCH/type-1 CG is “pointed to” a SCell.

(RAN2#130) When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG. (RAN2#131) If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact

Based on previous observation, there are multiple combination of PUCCH resource and PUSCH resource being configured on different Cells with different TAGs. PUCCH release of some of the combinations are covered by the previous agreements, while in the case where PUCCH is on a different TAG with the UEI report-concerned SCell, PUCCH resources are not released for a UEI report configuration when the sTAG SCell associated with the UEI report configuration experiences TAT expiry:

Release of PUCCH release for different scenarios, when TAT of the Cell with the UEI report configuration expires:

| When TAT of the Cell configured with the CSI-ReportConfig expires: | The Cell is a pTAG Cell | The Cell is a sTAG SCell |
| --- | --- | --- |
| PUCCH is on the same TAG as the Cell | PUCCH is released | PUCCH is released |
| PUCCH is on different TAG from the Cell | PUCCH is released | PUCCH is not released |

Observation 3:	Currently, when a TAG of a SCell expires, the PUCCH resource of a UEI report configuration of the SCell is not released when the PUCCH resource is configured on a Cell with a different TAG.

2-2.  Clarification on previously raised questions

In our view, once the SCell experiences TAT expiry, it would be unable to provide CSI report to the network, and the transmission for UEIRI is no longer needed and should be released to avoid wasting UE power and corresponding PUCCH resource can be released for other uses. In addition, it’d be reasonable to release the resources along with other cases of TAT expiry of a Cell. Therefore, the PUCCH resource for UEI beam report configuration of a Serving Cell should be released upon TAT expiry of the said Cell, regardless of which Cell/TAG the PUCCH is actually on.

On the other hand, based on online discussion in RAN2#131, some companies commented that the PUCCH resource does not need to be released with various reasons. In this section, we collect comments received during online discussion and provide some input for discussion.

- PUCCH resource can be shared between different UEI report configuration

Some companies worry that since PUCCH resource can be shared different multiple UEI report configurations, if PUCCH resource is released for TAT expiry of a SCell, the other UEI report configurations will also be affected and cannot perform UEIRI transmission. While it is true that PUCCH resource for UEIRI can be shared between multiple UEI report configurations, it is restricted in RAN1 that only UEI report configurations on the same CC (i.e., same Cell) can share the same PUCCH resource:

5.2.1.5.4.1d UE Initiated CSI reporting for multiple CSI configurations For a UE configured with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, the UE expects - the multiple CSI-ReportConfig to be configured in the same CC, - the multiple CSI-ReportConfig to be associated with a same CSI trigger state if reportTransmissionMode is configured as ‘ModeA’, else - the same configuredPUSCHResourceOfModeB if reportTransmissionMode is configured as ‘ModeB’.

Therefore, there won’t be a case where the PUCCH resource for UEIRI reporting is shared by two Cells associated with different TAGs.

Observation 4: Only UEI report configurations on the same CC (i.e., same Cell) can share the same PUCCH resource.

- If PUCCH is released, the NW has to reconfigure PUCCH resource for UEI report of the SCell.

When a TAT expires for a SCell, it could be either an error case where TA command is not received by the UE, or an intended behaviour by the network to release SCells that are no longer needed for UL transmission. The former should be a corner case as HARQ operation should guarantee a confirmation of reception of TA command MAC CE, and for the latter case, the NW may not desire to reconfigure the SCell any time soon, so releasing the resource for other uses and to avoiding unnecessary UL transmission would be beneficial to both the NW and UE

- PUCCH resource does not need to be released if the corresponding TAT is not expired:

When the PUCCH resource is configured on a Cell with a different TAG, it is true that the PUCCH resource is still valid for the UE when the corresponding TAG’s TAT is still running, and transmitting UEIRI via PUCCH alone when UEI report is triggered may provide some information for the NW (e.g., hinting the NW to provide reconfiguration for the UEI report or to re-obtain TA for the concerned Cell). One issue may occur according to the current physical layer spec as the PHY layer will transmit PUSCH after transmission of the PUCCH:

After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission.

In the scenario where PUSCH is no longer available, the UE behaviour is unclear and should be clarified. For example, the UE could, due to no Type-1 CG PUSCH available for CSI report transmission caused by TAT expiry, not transmit the PUCCH for a triggered UEI report on the SCell, or the UE could perform the PUCCH transmission and does not proceed to transmit the CSI report.

Observation 5:	If the PUCCH resource for a UEI report configuration is not released but PUSCH for CSI report is unavailable due to TAT expiry, PHY behaviour on whether to transmit the PUCCH for the UEI report needs to be clarified

In short, the pros and cons to release/not release PUCCH can be summarized as below:

| Upon TAT expiry on a SCell | Pros | Cons |
| --- | --- | --- |
| Releasing PUCCH on a different TAG for UEI report of the SCell | - Avoid unnecessary UEIRI transmission - PUCCH resource can be released for other uses. | - NW has to reconfigure PUCCH for UEI report of the SCell once the NW determines to recover the SCell |
| Not releasing PUCCH on a different TAG for UEI report of the SCell | - UEIRI can still be transmitted for the SCell to provide partial information to the NW. | - PHY behaviour after UEIRI transmission while no PUSCH resource is unclear |

Based on the above analysis, we propose RAN2 to select from one of the options for handling PUCCH resource for a mode-B UEI report configuration of a SCell upon SCell TAT expiry, when the PUCCH resource is associated with a different TAG than the SCell (and the TAG associated with the PUCCH is running):

- Option 1: the UE releases PUCCH resources configured in a UEI report configuration of the SCell.

- Option 2a: the UE does not release PUCCH resources in a UEI report configuration of the SCell. If a UEI beam report is triggered for the SCell, the UE transmits the PUCCH and does not transmit the PUSCH.

- Option 2b: the UE does not release PUCCH resources in a UEI report configuration of the SCell, If a UEI beam report is triggered for the SCell, the UE does not transmit the PUCCH and the PUSCH.

Proposal 1:	(MAC-Issue-1) For mode-B UEI reporting, RAN2 to select from one of the options for UEI beam reporting in the following scenario: Upon STAG TAT expiry associated with a SCell configured with a mode-B UEI report configuration, if the PUCCH resource of the UEI report configuration is configured on a PCell or a PUCCH-SCell of a different TAG with a running TAT, the UE:

      - Option 1: releases the PUCCH resource.

      - Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the UEIRI on PUCCH and does not transmit the Type-1 CG PUSCH for the UEI beam report.

      - Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the UEIRI on PUCCH and the Type-1 CG PUSCH for the UEI beam report.

2-3.  Spec Impact analysis for each solution

In this section, we discuss whether there will be any spec impact for each option.

Option 1: releases the PUCCH resource

In the current running RRC CR [1], the UE releases PUCCH resource of CSI-ReportConfig only upon indication of PUCCH release of pucch-cells:

### 5.3.12 UE actions upon PUCCH/SRS release request Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall: 1> release PUCCH-CSI-Resources configured in CSI-ReportConfig; 1> release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig; 1> release SchedulingRequestResourceConfig instances configured in PUCCH-Config. 1> if the indicated serving cell is referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig; 2> release pucch-Resource indicated in the associated CSI-ReportUE-IBR;

To release PUCCH resource on pucch-cell for UEI beam report configuration of a SCell upon SCell TAT expiry, some changes may be needed, for example:

(Text proposal for option 1)

### 5.3.12 UE actions upon PUCCH/SRS release request Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall: - release PUCCH-CSI-Resources configured in CSI-ReportConfig; 1> release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig; 1> release SchedulingRequestResourceConfig instances configured in PUCCH-Config; - release pucch-Resource configured in the CSI-ReportUE-IBR, if reportTransmissionMode is configured as ‘modeB’; 1> if the indicated serving cell is referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig; 2> release pucch-Resource indicated in the associated CSI-ReportUE-IBR;

In addition, based on discussion on MAC open issues in email discussion [217], another TP for option 1 suggested by other companies with MAC changes could also be considered:

(Text proposal 2 for option 1)

## 5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 2> else: 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; 5> if there is PUCCH resource configured on a serving cell referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig that provides these configured uplink grants: 6> notify RRC to release the PUCCH resource for all such serving cells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG. …

Option 2: not release the PUCCH resource

Option 2 seems to be supported by current RAN2 spec, but one clarification may be needed in MAC spec to confirm that TAT expiry only release PUCCH resource configured “on” the TAT expired Cell:

(Text proposal for option 2)

## 5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 2> else: 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured on all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG. …

In addition, option2a and option2b require additional clarification in the spec (either in PHY or in MAC):

(Text proposal for option 2a)

(38.214) ##### 5.2.1.5.4 UE Initiated reporting … If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. If reportTransmissionMode is configured as ‘ModeB’, and timeAlignmentTimer associated with the Cell configured with the CSI-ReportConfig expires, the UE does not send a CSI report via Type-1 CG PUSCH resource after transmission of UEIRI.

(Text proposal for option 2b)

(38.214) ##### 5.2.1.5.4 UE Initiated reporting … If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount and if reportTransmissionMode is configured as ‘ModeB’, timeAlignmentTimer associated with the Cell configured with the CSI-ReportConfig is running, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig.

In addition, another TP for option 2b with MAC change suggested by rapporteur in email discussion [217] is as follows:

(Text proposal 2 for option 2b)

## 5.2 Maintenance of Uplink Time Alignment … For UE-initiated CSI reporting, if either the PUCCH or the associated PUSCH is associated with a TAG for which is not running, the PUCCH and the associated PUSCH are not transmitted.

(Text proposal 3 for option 2b)

## 5.2 Maintenance of Uplink Time Alignment … For mode-B UE-initiated CSI reporting, if the configured grant Type 1 for mode-B UE-initiated CSI reporting is associated with a TAG of an expired timeAlignmentTimer, the UE Initiated Report Indication on the PUCCH is not transmitted.

Proposal 2:	(MAC-Issue-1) Based on the selected option in P1, RAN2 to discuss spec impact for handling mode-B UEI report upon SCell TAT expiry.

- Issue-10

3-1.  Discussion on DRX active time for Mode-A UEI reporting

[Issue-10] DRX active time stop condition for mode-A UEI report if no corresponding PDCCH is received

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time between transmission of a UEIRI and receiving a PDCCH scheduling PUSCH for mode-A UEI CSI report (as shown in Figure 1):

- a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]).

Figure 1

However, if PDCCH for CSI report is never received (either the PDCCH is not scheduled by NW after the NW receives the UEIRI or the NW fails to receive the UEIRI on PUCCH), the UE will have to keep staying in DRX active time due to the condition being always met, causing unnecessary power consumption (as shown in Fig.2).

Figure 2

Observation 6:	If PDCCH for CSI report is never received after transmission of a UEIRI on PUCCH for mode-A UEI reporting, the UE will keep staying in DRX active time, causing unnecessary power consumption.

In contrast to DRX active time due to SR transmission:

Another DRX active time period the UE considers is when an SR is pending:

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT;

In the SR pending case, the UE will eventually cancel SR after SR reaches maximum number of transmission. Since currently there is no pending or cancellation mechanism in UEI reporting, additional handling for PDCCH monitoring is needed for UEI reporting.

In contrast to DRX active time due to RAR reception:

Still another active time is after the UE receives an RAR for a CFRA and not yet receives a PDCCH:

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

Since RAR is controlled by NW, after RAR transmission, the NW should be able to provide PDCCH properly without having the UE to monitor PDCCH for a long time.

To solve this issue, a maximum time period for the UE to stay in DRX active for monitoring PDCCH for mode-A CSI report should be discussed. In this document, we propose two options:

Option 1: If PDCCH scheduling a mode-A UEI CSI report is not received, the UE stays in DRX active time until the next PUCCH resource for the UEIRI transmission.

In this option, after transmitting UEIRI, the UE stays in DRX active time before either 1) receiving the corresponding PDCCH or 2) until the next PUCCH resource of the corresponding UEI report configuration. The advantage of this option is no additional time period/window needs to be configured, and if the UE transmits another UEIRI at the next PUCCH resource (if event instance is determined again in the PHY layer), the UE can stay in active time for PDCCH monitoring.

Figure for Option1

Option 2: If PDCCH scheduling a mode-A UEI CSI report is not received, the UE stays in DRX active time for a period of maximum PDCCH monitoring time.

Another option is to configure or define a maximum time period for monitoring PDCCH for mode-A CSI report (e.g., similar to latency requirement of SL CSI reporting). The time period starts from transmission of UEIRI for a UEI reporting. The advantage of this option is a more configurable time period providing flexibility for NW scheduling. However, this may require additional parameter for CSI report configuration.

Figure for Option2

Proposal 3:	(MAC-Issue-10) RAN2 to discuss if a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI (e.g., the NW does not receive UEIRI from the UE or the NW decides not to schedule the PDCCH), when the UE stops staying in DRX active time.

- Option 1: until the next PUCCH resource for the UEIRI.

- Option 2: Introduce a maximum PDCCH monitoring time period.

3-2.  Text proposal

Text proposal for Option 1: until the next PUCCH resource for the UEIRI.

(38.321) When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]) and until reception of a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH; or - after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]) and until the next PUCCH resource for transmitting UE Initiated Report Indication in case a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH is not received.

Text proposal for Option 2: Introduce a maximum PDCCH monitoring time period (e.g., modeAPDCCHschedulingwindow).

(38.321) RRC controls DRX operation by configuring the following parameters: - modeAPDCCHschedulingwindow: the maximum duration until a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH is received; When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]) and until reception of a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH; or - during the period of modeAPDCCHschedulingwindow in case a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH is not received.

- Conclusion

We have the following observations and proposals for UEI report:

Observation 1:	For a UEI report configuration (CSI-ReportConfig) configured on a SCell, PUCCH resource of the UEI report configuration can be configured on a PUCCH Cell, which can be the SCell or a different Cell. A Mode-B type-1 CG of the UEI report configuration of a SCell has to be configured on the SCell.

Observation 2:	The PUCCH resource of a UEI report configuration of a Cell can be configured on a PUCCH Cell associated with a different TAG from the Cell.

Observation 3:	Currently, when a TAG of a SCell expires, the PUCCH resource of a UEI report configuration of the SCell is not released when the PUCCH resource is configured on a Cell with a different TAG.

Observation 4: Only UEI report configurations on the same CC (i.e., same Cell) can share the same PUCCH resource.

Observation 5:	If the PUCCH resource for a UEI report configuration is not released but PUSCH for CSI report is unavailable due to TAT expiry, PHY behaviour on whether to transmit the PUCCH for the UEI report needs to be clarified

Proposal 1:	(MAC-Issue-1) For mode-B UEI reporting, RAN2 to select from one of the options for UEI beam reporting in the following scenario: Upon STAG TAT expiry associated with a SCell configured with a mode-B UEI report configuration, if the PUCCH resource of the UEI report configuration is configured on a PCell or PUCCH-SCell of a different TAG with a running TAT, the UE:

      - Option 1: releases the PUCCH resource.

      - Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the UEIRI on PUCCH and does not transmit the Type-1 CG PUSCH for the UEI beam report.

      - Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the UEIRI on PUCCH and the Type-1 CG PUSCH for the UEI beam report.

Proposal 2:	(MAC-Issue-1) Based on the selected option in P1, RAN2 to discuss spec impact for handling mode-B UEI report upon SCell TAT expiry.

Observation 6:	If PDCCH for CSI report is never received after transmission of a UEIRI on PUCCH for mode-A UEI reporting, the UE will keep staying in DRX active time, causing unnecessary power consumption.

Proposal 3:	(MAC-Issue-10) RAN2 to discuss if a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI (e.g., the NW does not receive UEIRI from the UE or the NW decides not to schedule the PDCCH), when the UE stops staying in DRX active time.

- Option 1: until the next PUCCH resource for the UEIRI.

- Option 2: Introduce a maximum PDCCH monitoring time period.

- Reference

[1] R2-2506587, Introduction of MIMO Phase5, Ericsson

[2] 38.214 v19.0.0

[3] Report of Rel-19 MIMO MAC open issues for maintenance, Samsung