3GPP TSG-RAN WG2 Meeting #131bis		       R2-2507543

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	10.3.3

Source:	ASUSTeK

Title:	Discussion on 6G AIML framework

Document for:	Discussion and Decision

- Introduction

In the 6G SID [1], it has been agreed to study AI/ML for 6GR, leveraging 5G AI/ML framework:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we would like to provide our views on the objective and discuss about the way forward.

- Discussion

In TR 38.843, several aspects are identified for the LCM of AI/ML functionalities:

The following aspects, including the definition of components (if needed) and necessity, are studied in LCM: - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - Functionality/model identification - Model delivery/transfer - Model inference operation - Functionality/model selection, activation, deactivation, switching, and fallback operation. - Including: Decision by the network (either network initiated or UE-initiated and requested to the network), decision by the UE (event-triggered as configured by the network, UE’s decision reported to the network, or UE-autonomous either with UE’s decision reported to the network or without it) - Functionality/model monitoring - Model update - UE capability

The current 5G AI/ML framework supports functionality-based LCM based on individual configurations. We believe that this per-configuration approach for applicability reporting, inference configuration, and monitoring is a suitable starting point for 6G AI/ML.

Proposal 1: The current 5G AI/ML framework supporting functionality-based LCM (e.g., applicability reporting, monitoring) can be used as starting point for 6G.

During the discussion of 5G AI/ML framework, model-based LCM are not supported. In 6G, to fully utilize the potential of AI/ML and enable efficient NW management, we believe it is crucial to have some management of AI/ML models on top of the functionality management.

Proposal 2: RAN2 discuss whether to support model-based LCM in 6G.

In the SID [1], it is specified that 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML. During the discussion of 5G AI/ML framework, the functionality with and without AI/ML lacks coordination. For example, switching between legacy and AI/ML models are handled by the NW (e.g., via applicability report). While this approach can minimize spec impact, the latency of exchanging multiple RRC messages is notable and impacts the overall NW performance. We believe it could be enhanced in 6G.

Additionally, to fully utilize the potential of AI/ML, the NW may use different settings for non-AI/ML UEs and AI/ML UEs. For example, the NW may transmit reduced beams for AI/ML UEs, but may not reduce beams for non-AI/ML UEs. In the current framework, the NW may only enable AI/ML on a cell, and dedicate the cell to serve AI/ML UEs and not non-AI/ML UEs, which affects the flexibility of fully utilizing NW capacity.

Proposal 3: Coordination between functionality or UEs with AI/ML and without AI/ML to minimize extra handling should be considered in 6G.

To support the various potential AI/ML use cases, the AI/ML framework aims to be extensible/unified. However, AI/ML use cases are unique in their own ways such that the configuration and reporting may be different, or managed by different entities (e.g., CU, DU). Therefore, it may be hard to have a unified framework for every use case.

However, for UE side data collection, NW side data collection, and model/dataset transfer, these features share some similar characteristics. For example, the collected data are non-latency sensitive, low priority, and occupies large resource, and sometimes collecting the exact same data (e.g., measurements). We think it may be efficient to have a framework to unify them. For example, the UE may not be configured UE side data collection and NW side data collection simultaneously in separate configurations when the collected data (e.g., measured beams) are the same. For another example, the UE may not transmit a message for UE side data collection and another message for NW side data collection where the two messages may contain the same data.

Proposal 4: 6G should consider unifying UE side data collection, NW side data collection, and model/dataset transfer.

- Conclusion

Proposal 1: The current 5G AI/ML framework supporting functionality-based LCM (e.g., applicability reporting, monitoring) can be used as starting point for 6G.

Proposal 2: RAN2 discuss whether to support model-based LCM in 6G.

Proposal 3: Coordination between functionality or UEs with AI/ML and without AI/ML to minimize extra handling should be considered in 6G.

Proposal 4: 6G should consider unifying UE side data collection, NW side data collection, and model/dataset transfer.

References

[1] RP-251881

[2] TR 38.843

[3] Introduction of AI/ML for NR Air interface feature in TS38.300

Annex