3GPP TSG-RAN WG2 Meeting #131bis		R2-2507556

Prague, Czech Republic, 13th – 17th Oct. 2025

Source:	TCL

Title:	Discussion on basic procedures of control plane for 6GR

Agenda Item:	10.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN#108 meeting, the revised SID [1] for 6G radio was agreed. Besides, SA1 study the 6G use cases and service requirements in TR22.870 [2], and RAN study 6G scenarios and requirements in TR38.914 [3].  Based on the SID and specifications, some requirements are summarized as follows.

- Energy efficiency: 6G systems shall achieve substantial improvements in energy efficiency for both networks and devices. On the device side, modem power consumption (including baseband and RF) has increased due to AI, computing, and high-volume data transmission. Energy efficiency design at the UE is therefore essential to extend battery life while maintaining service quality. For multiple access networks (TN and NTN), as energy consumption models differ across access types, the 6G system should select the most energy-efficient network for a given service area and user group. At the slice level, 6G is expected to support energy control across network slices, ensuring resource efficiency while maintaining service requirements.

- System simplification: The 5G system supports various features to accommodate a wide range of use cases. It may lead to operational challenges, further introducing complexity in system management. 6G should minimize optionality and configuration complexity. Simplified cell/UE management and streamlined UE capability reporting are key to lower cost and avoid redundant configurations. System simplification improves ease of operation, reduces implementation burden on UEs, and enhances maintainability, which is essential for large-scale 6G deployment.

- Diverse device types: 6G shall support a broader range of device types, from ultra-high performance devices to low-cost, low-power IoT nodes. The system design should provide scalability and forward compatibility to accommodate heterogeneous devices under a common framework. Furthermore, 6G must address the specific requirements of vertical industries such as industrial automation, automotive, and non-terrestrial communications. Flexibility in protocol, capability configuration, and resource allocation is therefore required.

- Latency: 6G is expected to achieve further latency reduction in both user plane and control plane for immersive communication and HRLLC. Specifically, 4 ms for immersive communication and 1 ms for HRLLC in the user plane. For the control plane, the latency requirement is 20 ms, with 10 ms encouraged, to enable rapid transition from a battery-efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE). These stringent latency objectives are fundamental to supporting interactive services and mission-critical applications.

In the subsequent sections, we will discuss the basic procedures of control plane for the requirements above, including system information, paging, access control, and UE capability.

- Discussion

  - System information

In NR, some system information blocks are broadcast in a unified manner for all UEs with different device types or capabilities. For example, SIB1 is delivered regardless of whether the UE is a high-performance smartphone or an IoT device, even though many fields are irrelevant for IoT UEs. Similarly, SIB2 and SIB3 carry frequency reselection information for bands that certain UEs do not support but must nonetheless decode. Besides, the system information in the NTN cell and TN cell is different. When NTN and TN are supported over a unified air interface in 6G, the NR system information mechanism will also lead to irrelevant information for NTN and TN devices.

Observation 1: In NR, the same system information is broadcast for different device types, which leads to increased UE power consumption.

NR inherits the LTE architecture of defining multiple SIBs according to content type, with different categories of parameters carried by different SIBs (e.g., cell access parameters and RACH configuration in SIB1, neighbor information in SIB4/5). However, this content-based organization exposes limitations in  6G with diverse service requirements. While modularization facilitates logical classification, the amount of system information is excessive, and the content of system information becomes fragmented, requiring UEs to collect multiple SIBs for obtaining a complete configuration. This results in increased latency for system information acquisition, as well as higher network complexity (e.g., mapping multiple SIBs into one SI message, defining area applicability).

Observation 2: The amount of system information is excessive, resulting in increased latency for SI acquisition and network complexity.

Based on the analysis above, the NR system mechanism results in increased UE power consumption, increased latency for SI acquisition, and network complexity. To support the diverse device types, system simplification, and latency targets in 6G, the current system information should be remodeled.

Proposal 1: Remodeling of system information should be supported for diverse device types and a smaller number of SI blocks.

  - Paging

In LTE and NR, several power-saving mechanisms have been defined—such as DRX, eDRX, GWUS, PEI, and LP-WUS—that operate on paging monitoring for NON-CONNECTED UEs. When DRX is enabled, the UE does not continuously monitor the paging. Instead, the UE monitors the paging only during a PO within each DRX cycle and remains in low-power sleep mode at all other times, thereby reducing the overall active time of the radio receiver. Extended Discontinuous Reception (eDRX) is an enhancement to the DRX mechanism that allows the NON-CONNECTED UE to adopt significantly longer sleep cycles for paging monitoring. Thus, it achieves even lower power consumption as the following Figure in TR38.875 [4], showing the relationship between eDRX and the UE battery lifetime.

Figure 1: RedCap UE battery lifetime in RRC_INACTIVE and RRC_IDLE state

Regarding the PEI/LP-WUS/GWUS mechanisms, before each PO, the network transmits a short indication signal to inform the UE whether it needs to monitor the PO. If the indication shows that no paging for that UE is expected, the UE may skip monitoring of the PO. This reduces the probability of unnecessary wake-up for the UE, thereby avoiding superfluous energy consumption. In practice, the network groups UEs into different groups, with each group being associated with a specific indication or wake-up signal. Based on the text below illustrated from TR38.869 [5], LP-WUS achieves paging latency reduction and UE power saving gain compared with eDRX.

- Compared with existing eDRX operation, significant paging latency reduction and moderate UE power saving gain is observed, if LP-WUS monitoring and the corresponding paging monitoring after MR wake-up is performed not restricted within existing PTW of eDRX. Significant UE power gain and moderate paging latency increase is observed if LP-WUS monitoring is restricted within existing PTW of eDRX and existing paging occasion determination is reused.

PEI and LP-WUS each present distinct advantages and limitations. LP-WUS, which relies on a low-power receiver to monitor a low-complexity physical wake-up signal, achieves greater power saving but introduces additional paging latency. Conversely, PEI uses the primary receiver to monitor DCI, resulting in lower paging latency but reduced power-saving efficiency, since the primary receiver is active and the wake-up signal itself is a DCI. LTE GWUS provides a compromise between the energy efficiency of LP-WUS and the latency of PEI. Specifically, LTE GWUS still employs the primary receiver to monitor the wake-up signal, but the signal is realized as a low-complexity physical layer signal rather than a DCI. This yields better power saving compared to PEI, while retaining the low-latency advantage characteristic of PEI.

Observation 3: The DRX/eDRX and GWUS/PEI/LP-WUS mechanisms in LTE and NR are beneficial for UE power saving.

In NR, DRX and eDRX mechanisms were introduced in different releases to reduce UE power consumption. At present, the two mechanisms are largely overlapping: both reduce UE paging monitoring time within a cycle to achieve power saving, with the only distinction being that eDRX supports longer cycles with an additional PTW. Similarly, PEI and LP-WUS in different releases are also overlapping. The difference lies only in the type of indication signal and the receiver used by the UE to detect it. Such duplication results in management complexity and signalling overhead for both UE and network. Concretely, separate procedures and parameters are required in the NAS and RRC layers.

Observation 4: The DRX and eDRX provide similar functionalities, which results in system complexity. The same problem exists between the PEI and LP-WUS mechanisms.

In 6G, mechanisms similar to DRX/eDRX and PEI/LP-WUS should continue to be supported to ensure efficient power control for paging monitoring in NON-CONNECTED states. These mechanisms have demonstrated clear benefits in LTE and NR for extending UE battery life. However, the system complexity proposed above should be avoided to achieve both simplified protocol implementation and power saving.

Proposal 2: The DRX/eDRX and PEI/LP-WUS mechanisms should be supported in a simplified manner to achieve both power saving and simplified protocol implementation.

  - Access control

In LTE and NR, in order to ensure network stability and optimize access efficiency, a set of overload and access control mechanisms were introduced, such as RACH back off, RRC Connection Reject, RRC Connection Release, and UE-based access barring mechanisms.

- RACH back off: When the network detects congestion on the random access channel, the RAR message may include a backoff indication. Upon receiving the backoff indication, the UE selects a random backoff time between 0 and the backoff indication, and shall not initiate a new random access attempt until the expiry of the random backoff time.

- RRC Connection Reject: When the network load is high, the gNB may reject an RRC connection establishment/resume attempt by sending an RRC Connection Reject message. The reject message may include a wait time, during which the UE shall not initiate a new RRC connection establishment/resume request.

- RRC Connection Release: When the network load is elevated, or when no valid user data transfer has been detected for a prolonged period, the network may actively release the UE connection by sending an RRC Connection Release message, thereby conserving radio resources.

- UE-based access barring: Each UE access attempt is associated with the specific Access Category and Access Identities. The network broadcasts the barring information for all Access Categories and Access Identities. Based on the received barring information, the UE autonomously determines whether its access attempt is permitted.

Observation 5: In LTE and NR, access control mechanisms are beneficial to avoid network overload.

In 6G, a unified air interface is envisioned to simultaneously support diverse device types, including eMBB and IoT devices, TN and NTN devices, etc. However, the NR access control parameters do not vary across different device types. For example, the UE-based access barring defines the Access Categories and Access Identities only per service (e.g., MO data, MO signalling, MT access, emergency, MO MMTel voice) in TS24.501 [6] as the Table below.

Table 1: Mapping table for access identities/access categories and RRC establishment cause when establishing N1 NAS signalling connection via E-UTRA connected to 5GCN

| Rule # | Access identities | Access categories | RRC establishment cause is set to |
| --- | --- | --- | --- |
| 1 | 1 | Any category | highPriorityAccess |
| 2 | 2 | Any category | highPriorityAccess |
| 3 | 11, 15 | Any category | highPriorityAccess |
| 4 | 12,13,14, | Any category | highPriorityAccess |
| 5 | 0, 3 | 0 (= MT_acc) | mt-Access |
|  |  | 1 (= delay tolerant) | Not applicable (NOTE 1) |
|  |  | 2 (= emergency) | emergency |
|  |  | 3 (= MO_sig) | mo-Signalling |
|  |  | 4 (= MO MMTel voice) | mo-VoiceCall |
|  |  | 5 (= MO MMTel video) | mo-VoiceCall |
|  |  | 6 (= MO SMS and SMSoIP) | mo-Data |
|  |  | 7 (= MO_data) | mo-Data |
|  |  | 9 (= MO IMS registration related signalling) | mo-Data |
|  |  | 10 (= MO exception data) | mo-ExceptionData (NOTE 3) |
| NOTE 1: A UE using access category 1 for the access barring check will determine a second access category in the range 3 to 7 that is to be used for determination of the RRC establishment cause. See subclause 4.5.2, table 4.5.2.2, NOTE 6. NOTE 2: See subclause 4.5.2, table 4.5.2.1 for use of the access identities of 0, 1, 2, and 11-15. NOTE 3: This applies to the UE in NB-N1 mode. |  |  |  |

Observation 6: NR access control mechanisms can not differentiate between diverse device types.

While existing overload and access control mechanisms provide certain protection against network overload, they were not designed to address the diverse device types in 6G. For example, the number of IoT devices is excessive, and a large number of  IoT devices may request RRC connection establishment simultaneously in order to report water meter readings. To better support and manage different devices in 6G, the access control mechanism should be studied for a unified design considering diverse device types.

Proposal 3: Access control mechanisms should be studied for a unified design considering diverse device types.

  - UE capability

In LTE and NR, different devices demonstrate distinct operating characteristics. eMBB devices are primarily optimized for high throughput and multimedia services, requiring sustained bandwidth utilization and frequent radio activity. As a result, eMBB devices generally exhibit higher power consumption, and their coverage performance is limited by link budget constraints and antenna configurations. By contrast, IoT devices are specifically designed with an emphasis on low complexity, low power consumption, and extended coverage to support massive, low-rate, and intermittent connectivity.

From a power efficiency perspective, IoT devices typically employ simplified RF architectures, optimized modulation and coding schemes, and long-cycle discontinuous reception (DRX/eDRX) or similar power-saving mechanisms. These allow IoT devices to significantly reduce energy consumption in both NON-CONNECTED and CONNECTED modes. From a coverage perspective, IoT devices are often configured with more relaxed throughput requirements but enhanced link budget targets. Mechanism such as transmission repetition allows for much deeper coverage.

Observation 7: In LTE and NR, IoT devices have lower power consumption and wider coverage compared to eMBB devices.

In 6G, diverse device types will be supported over the same air interface. For achieving the 6G energy efficiency requirement, etc, considering IoT devices have lower power consumption and wider coverage compared to eMBB devices, a device with multiple capabilities (e.g., eMBB and IoT) can be supported. The device can switch between different capabilities to on demand achieve high throughput and power saving.

Proposal 4: A device with multiple capabilities (e.g., eMBB and IoT) can be supported to on demand achieve high throughput and power saving.

- Conclusion

In this contribution, we discuss the basic procedures of control plane for some 6G requirements. Based on the discussion, we have the following observations and proposals.

System information

Observation 1: In NR, the same system information is broadcast for different device types, which leads to increased UE power consumption.

Observation 2: The amount of system information is excessive, resulting in increased latency for SI acquisition and network complexity.

Proposal 1: Remodeling of system information should be supported for diverse device types and a smaller number of SI blocks.

Paging

Observation 3: The DRX/eDRX and GWUS/PEI/LP-WUS mechanisms in LTE and NR are beneficial for UE power saving.

Observation 4: The DRX and eDRX provide similar functionalities, which results in system complexity. The same problem exists between the PEI and LP-WUS mechanisms.

Proposal 2: The DRX/eDRX and PEI/LP-WUS mechanisms should be supported in a simplified manner to achieve both power saving and simplified protocol implementation.

Access control

Observation 5: In LTE and NR, access control mechanisms are beneficial to avoid network overload.

Observation 6: NR access control mechanisms can not differentiate between diverse device types.

Proposal 3: Access control mechanisms should be studied for a unified design considering diverse device types.

UE capability

Observation 7: In LTE and NR, IoT devices have lower power consumption and wider coverage compared to eMBB devices.

Proposal 4: A device with multiple capabilities (e.g., eMBB and IoT) can be supported to on demand achieve high throughput and power saving.

- References

- RP-251881, New SID: Study on 6G Radio

- TR22.870 v0.3.1

- TR38.914 v0.1.0

- TR38.875 v17.0.0

- TR38.869 v18.0.0

- TS24.501 v19.1.1