3GPP TSG-RAN WG2 Meeting #131bis                                                     	R2-2507575

Prague, Czech Republic 13th–17th OCT, 2025

Source:	Google Inc.

Title:	Discussion on 6G AS User plane design

Agenda Item:	10.3.1

Document for:	Discussion and Decision

# 1. Introduction

The study item on 6G Scenarios and Requirements (TR 38.914) [1] defines a new set of demanding Key Performance Indicators (KPIs) that fundamentally expand the role of the mobile network. Beyond enhancing mobile broadband, 6G introduces transformative use cases such as Integrated Sensing and Communication (ISAC) and Hyper-Reliable and Low-Latency Communication (HRLLC), which place unprecedented demands on the Access Stratum (AS) user plane.

According to these new KPIs, this contribution discusses the essential design requirements for the 6G AS user plane.

# 2. Discussion

The 5G NR user plane protocol stack, with its four distinct sublayers (SDAP, PDCP, RLC, MAC), provides a logical and well-understood foundation. This architecture has proven to be robust and flexible for 5G services. Given the need for a clear evolutionary path, it is logical to consider this structure as the starting point for the 6G design.

Proposal 1: The 6G UP protocol stack design should take the 5G NR 4-layer user plane architecture (SDAP, PDCP, RLC, MAC) as the baseline.

While the 4-layer structure is a suitable baseline, the functions within each layer are significantly challenged by the new 6G KPIs and use cases. In the following, some specifics of these KPIs are listed and discussed with respect to the NR UP radio protocols and how they need to be reflected in the study.

# 2.1 Enhancements for HRLLC

The 6G study [1] specifies stringent requirements for HRLLC, with a user plane latency of 1 ms and reliability of 1-10⁻⁵. The RLC protocol's reliability mechanism in 5G is based on reactive retransmissions (ARQ), which is fundamentally too slow to consistently meet this latency target, as a single retransmission cycle can consume several milliseconds.

To meet the joint requirements of high reliability and low latency for HRLLC, the user plane cannot afford to wait for a failure to occur. It must be designed to be proactively reliable. This necessitates native support for proactive reliability schemes, such as packet duplication, where data is transmitted over multiple paths simultaneously to ensure at least one copy arrives successfully within the latency budget.

Proposal 2: The 6G user plane shall support proactive reliability mechanisms, such as packet duplication, to meet HRLLC requirements.

# 2.2 Enhancements for New Services and Service Diversity

The 6G architecture must support new services like ISAC and a diverse range of concurrent services. This requires a more flexible and intelligent user plane than what is available in 5G.

In 5G, sensing is being introduced as an add-on "5G wireless sensing service." It is treated as an application that runs on top of the existing 5G architecture. The network's role is to provide a service for sensing by collecting and delivering sensing data, much like it handles communication data. As such, the impact on the core user plane protocols is minimal.

In 6G, this evolves to Integrated Sensing and Communication (ISAC), which is a native, integrated core capability of the network itself. The network does not just provide a sensing service; the network is the sensor, using its own radio signals for both communication and sensing the physical environment. In essence, 5G is learning to use sensing as a new application, while 6G is being built with sensing as a native sense. This fundamental shift has profound implications for the user plane, as the protocols must now natively handle a new type of traffic and a new set of functions.

Therefore, the existing QoS-based framework is insufficient to manage this. The user plane must be inherently service-aware, with functions to identify and differentiate traffic based on its fundamental service type, not just its QoS profile. This will likely impact the SDAP layer, which will need to handle new service types beyond traditional QoS flows.

Proposal 3: The 6G user plane shall be service-aware, with functions to differentiate and manage new service types such as sensing traffic.

Furthermore, the introduction of ISAC means the network must manage radio resources for two distinct purposes: communication data and sensing. The 5G MAC scheduler is designed only to arbitrate resources for communication data. In 6G, the MAC layer's design must evolve to support the joint scheduling of communication and sensing resources.

Proposal 4: The 6G MAC layer shall support the joint scheduling of communication data and sensing resources.

Finally, a single device in the 6G era may run multiple, diverse services concurrently (e.g., immersive video, background IoT updates, and an HRLLC session). A single, rigid protocol stack configuration is inefficient for this. To optimize performance and energy consumption, the user plane can be designed as a set of functional blocks that can be dynamically configured on a per-bearer basis. For example:

- Bearer 1 (eMBB): Uses the full, standard 4-layer protocol stack, similar to today.

- Bearer 2 (HRLLC): the RLC layer's retransmission function is completely bypassed and replaced by a proactive reliability function (like packet duplication) to meet the 0.1 ms latency target.

- Bearer 3 (ISAC): Uses a specialized data path where the SDAP layer has unique logic to handle "sensing data," and the MAC layer uses a different scheduling algorithm to manage sensing signals.

Proposal 5: The 6G user plane can be designed as a set of functional blocks that can be dynamically configured on a per-bearer basis.

# 3. Conclusion

In conclusion, we propose the following design requirements for the 6G user plane:

Proposal 1: The 6G UP protocol stack design should take the 5G NR 4-layer user plane architecture (SDAP, PDCP, RLC, MAC) as the baseline.

Proposal 2: The 6G user plane shall support proactive reliability mechanisms, such as packet duplication, to meet HRLLC requirements.

Proposal 3: The 6G user plane shall be service-aware, with functions to differentiate and manage new service types such as sensing traffic.

Proposal 4: The 6G MAC layer shall support the joint scheduling of communication data and sensing resources.

Proposal 5: The 6G user plane can be designed as a set of functional blocks that can be dynamically configured on a per-bearer basis.

# 4. References

[1] 3GPP TR 38.914 V0.2.0, "Study on 6G Scenarios and Requirements"